| Repairing Alexandru Boico Street, Ungheni mun. | WinСмета2000 Form No. 1 |
| --- | --- |
| (name of the site) |  |

List with quantities of works No. 2-1-1

Bid value: USD

Repairing works

(name of works)

| No. | Symbol of the norm and resource code | Type of works | U.M. | Quantity |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Preparatory works |  |  |
|  |  | Chapter 1.1. Demolishing the road system (asphalt) |  |  |
| 1 | DI155Ek=0.8 | Cutting with the miller of the used asphalt concrete layer, having a drum width of 2000 mm, the depth of the layer: 10 cm (Hmed=8cm) | m2 | 6 037.000 |
| 2 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 1 062.600 |
| 3 | TsC51C | Works for unloading the soil in the storage, field category III | 100 m3 | 4.830 |
| 4 | TsC50C | Repairing and maintaining the natural roads when transporting the soil, for every 0.5 *2=1.0 km, land field category III | 100 m3 | 4.830 |
|  |  | Chapter 1.2. Demolishing the road system (crushed stone) |  |  |
| 5 | TsC03G1 | Mechanic digging with excavator of 0.40 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in trucks, ground cat. III (demolishing the foundation of crushed stone Hmed=15cm) | 100 m3 | 9.060 |
| 6 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 1 177.800 |
| 7 | TsC51C | Works for unloading the soil in the storage, field category III | 100 m3 | 9.060 |
| 8 | TsC50C | Repairing and maintaining the natural roads when transporting the soil, for every 0.5 *2=1.0 km, land field category III | 100 m3 | 9.060 |
|  |  | Chapter 1.3. Demolishing the curbs |  |  |
| 9 | TsC02D1 | Mechanic digging with pneumatic excavator of 0.21-0.39 m3, with hydraulic command, in grounds with natural humidity, and auto unloading of field of cat. II. (dismantling the big curbs БР100.30.15=1380pieces) | 100 m3 | 0.620 |
| 10 | TsC02D1 | Mechanic digging with pneumatic excavator of 0.21-0.39 m3, with hydraulic command, in grounds with natural humidity, and auto unloading of field of cat. II. (dismantling the concrete foundations under the big curbs БР100.30.15=1380pieces) | 100 m3 | 0.760 |
| 11 | TsC02D1 | Mechanic digging with pneumatic excavator of 0.21-0.39 m3, with hydraulic command, in grounds with natural humidity, and auto unloading of field of cat. II. (dismantling the foundation of crushed stone under big curbs БР100.30.15=1380pieces) | 100 m3 | 0.390 |
| 12 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 389.000 |
| 13 | TsC51C | Works for unloading the soil in the storage, field category III | 100 m3 | 1.770 |
| 14 | TsC50C | Repairing and maintaining the natural roads when transporting the soil, for every 0.5 *2=1.0 km, land field category III | 100 m3 | 1.770 |
|  |  | Chapter 1.4. Dismantling road signs |  |  |
| 15 | DF21A | Dismantling road traffic signs | pcs | 24.000 |
| 16 | DF22A | Removing the pillars for road traffic signs | pcs | 17.000 |
| 17 | TRI1AA04C1 | Loading materials from group A – light and small ones, by throwing from the ramp or field, in auto of category 1 | t | 0.803 |
| 18 | TsI51A2 | Transportation of soil with the dumper of 10 t at a distance of 2 km | t | 0.803 |
|  |  | Chapter 1.5. Cutting trees |  |  |
| 19 | TsG06B | Removing manually the stubs of coniferous (excluding fir trees) or softwood, and transportation of timber to warehouses, outside or within the site area, the transportation being carried out manually, for the stubs or roots of a diameter of 31 ... 50 cm | pcs | 5.000 |
| 20 | TRI1AA04C1 | Loading materials from group A – light and small ones, by throwing from the ramp or field, in auto of category 1 | t | 2.200 |
| 21 | TsI51A3 | Transportation of soil with the dumper of 10 t at a distance of 3 km | t | 2.200 |
| 22 | TsC51C | Works for unloading the soil in the storage, field category III | 100 m3 | 0.025 |
| 23 | TsC50C | Repairing and maintaining the natural roads when transporting the soil, for every 0.5 *2=1.0 km, land field category III | 100 m3 | 0.025 |
|  |  | Chapter 1.6. Rearranging wells (aqueduct) |  |  |
| 24 | PJ05B | Demolishing concrete from elevations to abutments, piles, support walls with no explosive, with compressed air hammer (Partial dismantling (concrete) of the existing well) | m3 | 2.900 |
| 25 | TsA02B | Manual excavation of land in confined spaces , having 1.00m or more in width, made without support, with sloping embankment foundations, channels, basements, drainers, stairs in non-cohesive or poorly cohesive land, depth up to 0.75 m middle ground | m3 | 3.800 |
| 26 | DE16B | Assembling in the ditches and culverts, etc, the elements pre-manufactured from reinforced concrete between 0.02 m3/piece and 0.100 m3/piece inclusively (Raising the wells at the pavement level with КЦ 0.1) | pcs | 19.000 |
| 27 | Dl119 | Monolithic foundations of concrete C16/20(XO) at artificial buildings (monolithic rings) | m3 | 1.900 |
| 28 | AcE07C | Mounting iron or iron-concrete covers without the support element, at the manholes of the water and sewerage supply installations, carriageway of type III A and B (th.95kg) | pcs | 19.000 |
|  |  | Chapter 2. Earthworks |  |  |
| 29 | TsC03B1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in the storage of ground cat. II (backfill from the box; Y=1.9t/m3 ) | 100 m3 | 3.680 |
| 30 | TsC26B1k=6 | Mechanical dislocation of the soil from the new storage, non-compacted and pushing it up to 5 m with the bulldozer on tractor of 81-180 HP on land of cat. I or II (=30m) | 100 m3 | 3.680 |
| 31 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. II (excavation; box; Y=1.9t/m3 ) | 100 m3 | 25.310 |
| 32 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 4 808.900 |
| 33 | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 25.310 |
| 34 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 25.310 |
| 35 | DI96 | Compacting the earthwork in the ground of cat. II with pneumatic compactor of 25 t, 8 movements on one pathway (ONLY Compactor 25t) | 100 m3 | 3.680 |
| 36 | TsD14A01 | Mechanical watering of soil layers with a 5-8 t tanker truck, provided with a spraying device, to complete the humidity necessary for mechanical compaction, as well as for watering surfaces for other purposes | m3 | 36.800 |
|  |  | Chapter 3. Finishing and planning works |  |  |
| 37 | TsE05B | Levelling with motor grader up to 175 HP of the natural land field and the groundwork platforms, by cutting the bumps and pushing the dug soil in the holes, ground cat. II (earthworks) | 100 m2 | 60.210 |
|  |  | Chapter 4. Reinforcement works |  |  |
|  |  | Chapter 4.1. Reinforcing the green area with a vegetal layer H=15cm |  |  |
| 38 | TsC59A | Mechanic excavation of soil with “single bin” excavator with the volume of the bin of 0.15 m3, unloading in the dumper: ground category 1 | 100 m3 | 0.731 |
| 39 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 87.660 |
| 40 | TsC51A | Works for unloading the soil in the storage, ground category I | 100 m3 | 0.731 |
| 41 | TsC50Ak=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, ground category I | 100 m3 | 0.731 |
| 42 | TsH09C | Planting the lawns on the bank areas with 1 kg of seeds per 100 m2 | 100 m2 | 4.870 |
| 43 | TsH12A | Watering the areas with the hose from the hydrants | 100 m2 | 4.870 |
|  |  | Chapter 5. Setting the road traffic system |  |  |
| 44 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. II (loading crushed stone from demolishing from the storage) | 100 m3 | 7.700 |
| 45 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 1 001.000 |
| 46 | DA12C | Foundation or re-profiling layer from crushed stone, for roads with mechanical covering, executed without wedging or renewal (ONLY WORK - crushed stone from demolishing) | m3 | 611.111 |
| 47 | Dl110 | Arranging the leveling layer from the ballast (drained from the mixture of optimal aggregates 0-32 Ga75 conf.SM-EN 13242+A1 Н=10сm Kcomp-1.22) | m3 | 783.400 |
| 48 | DA12B | Foundation or re-profiling layer from crushed stone LA30, fr.32-63mm;fr.8-16mm conf.SM-EN 13242+A1;2008, for roads with mechanical covering, executed with wedging without renewal (H=20cm) | m3 | 1 446.200 |
| 49 | DZ01A | Preparation in fixed stations of the mixture for stabilized road stations, made of ballast, with 4% cement (Preparation - ONLY WORK - Recycling type "M" h=10cm - enriched with 4% cement (Y=1.2t/m3); 40%-crushed stone LA30(Y=1.3t/m3) ) | m3 | 662.900 |
| 50 | price | cement (CEMII/B-LL32.5R) | t | 61.200 |
| 51 | price | Crushed stone LA30 fr.16-32mm | m3 | 470.800 |
| 52 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 1 591.340 |
| 53 | DA10B | Road layer of granular material, stabilized with cement or lime and granulated slag using the method of mixing in fixed stations, with mechanical laying (Recycling type "M" h=10cm - enriched with 4% cement (Y=1.2t/m3);40% - crushed stone LA30(Y=1.3t/m3)) (ONLY WORK) | m3 | 662.900 |
| 54 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete ( 0.6l/m2 ) | t | 4.000 |
| 55 | DB19G | Asphalt concrete covering with big aggregates, executed in hot conditions, in thickness of 6.0 cm with mechanic laying (BAD 22.4,conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 6 026.000 |
| 56 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete (0.3l/m2 ) | t | 2.000 |
| 57 | DB16H | Asphalt concrete covering with small aggregates, executed in hot conditions, in thickness of 4.0 cm with mechanical laying (MAS16 ,conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 6 026.000 |
|  |  | Chapter 6. Construction for water removal |  |  |
|  |  | Chapter 6.1. Arranging the rectangular gutter on Hajdeu street, on the right |  |  |
| 58 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. II ( Y=1.9t/m3) | 100 m3 | 0.060 |
| 59 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 11.940 |
| 60 | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 0.060 |
| 61 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 0.060 |
| 62 | TsC03B1 | Mechanic digging with excavator of 0,40-0,70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading on the field storage of cat. II. | 100 m3 | 0.020 |
| 63 | TsC54B | Foundation layer from crushed stone Sparta (LA30 fr.16-32mm;Kcomp=1.26;h=0.1m ) | m3 | 0.700 |
| 64 | Dl119 | Monolithic foundations of concrete C30/37 XC4 XD3 XF4 at artificial elements (h=0.15m) | m3 | 2.700 |
| 65 | Dl126 | Installing the monolith concrete casing for artificial elements on the roads (A240 d6mm) | t | 0.023 |
| 66 | Dl126 | Installing the monolith concrete casing for artificial elements on the roads (A500C) | t | 0.064 |
| 67 | AcE06A | Assembling the grills with cast iron frame at the water drainage points | pcs | 12.000 |
| 68 | TsD01B | Spreading with the shovel of light earth in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from the middle ground | m3 | 2.000 |
| 69 | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of land separately, with the thickness of 10 cm of cohesive ground | m3 | 2.000 |
|  |  | Chapter 6.2. Arranging the rectangular gutter PC4+60-PC5+00, on the left |  |  |
| 70 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. II ( Y=1.9t/m3) | 100 m3 | 0.240 |
| 71 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 45.600 |
| 72 | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 0.240 |
| 73 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 0.240 |
| 74 | TsC03B1 | Mechanic digging with excavator of 0,40-0,70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading on the field storage of cat. II. | 100 m3 | 0.080 |
| 75 | TsC54B | Foundation layer from crushed stone Sparta (LA30 fr.16-32mm;Kcomp=1.26;h=0.1m ) | m3 | 3.100 |
| 76 | Dl119 | Monolithic foundations of concrete C30/37 XC4 XD3 XF4 at artificial elements (h=0.15m) | m3 | 10.600 |
| 77 | Dl126 | Installing the monolith concrete casing for artificial elements on the roads (A240 d6mm) | t | 0.089 |
| 78 | Dl126 | Installing the monolith concrete casing for artificial elements on the roads (A500C) | t | 0.252 |
| 79 | Dl119 | Monolithic foundations of concrete C30/37 XC4 XD3 XF4 at artificial elements (portal h=0.15m) | m3 | 0.100 |
| 80 | AcE06A | Assembling the grills with cast iron frame at the water drainage points | pcs | 50.000 |
| 81 | TsD01B | Spreading with the shovel of light earth in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from the middle ground | m3 | 8.000 |
| 82. | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of land separately, with the thickness of 10 cm of cohesive ground | m3 | 8.000 |
|  |  | Chapter 6.3. Repairing the existing gutter |  |  |
| 83 | TsC03G1 | (cleaning the existing capturing wells) Mechanic digging with excavator of 0.40 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. III | 100 m3 | 0.012 |
| 84 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km | t | 2.400 |
| 85 | RpAcH18B | Mechanical cleaning of the public channels with self-cleaner with under-pressure water jet, the channels with a diameter of 450-600 mm | m | 78.000 |
| 86 | PF01A | Cement mortar plaster of 100 brand, 2 cm thick, smooth (Mp200 = 1.3m3) | m2 | 65.000 |
| 87 | AcE06Ak=0.6 | Dismantling the grills with cast iron frame at the water drainage points | pcs | 13.000 |
| 88 | AcE06A | Assembling the grills with cast iron frame at the water drainage points | pcs | 13.000 |
|  |  | Chapter 7. Side roads, installing road signs |  |  |
|  |  | Chapter 7.1. Arranging the side roads |  |  |
| 89 | TsC21B1 | Mechanic digging with moto grader up to 175 HP, including spreading of the ground up to 10m, in ground of category II. | 100 m3 | 3.585 |
| 90 | TsC22K1k=2 | Increase in use of hours-equipment art. TsC21В1 for transporting the ground for each additional 10 m, exceeding the distance envisaged for the respective item, for grounds of category II (+20m) | 100 m3 | 3.585 |
| 91 | DI96 | Compacting the earthwork in the ground of cat. II with pneumatic compactor of 25 t, 8 movements on one pathway (ONLY Compactor 25t) | 100 m3 | 3.585 |
| 92 | TsD14A01 | Mechanical watering of soil layers with a 5-8 t tanker truck, provided with a spraying device, to complete the humidity necessary for mechanical compaction, as well as for watering surfaces for other purposes | m3 | 35.850 |
| 93 | Dl110 | Arranging the leveling layer from the ballast (drained from the mixture of optimal aggregates 0-32 Ga75 conf.SM-EN 13242+A1 Н=10сm Kcomp-1.22) | m3 | 119.500 |
| 94 | DA12B | Foundation or re-profiling layer from crushed stone LA30, fr.32-63mm;fr.8-16mm conf.SM-EN 13242+A1;2008, for roads with mechanical covering, executed with wedging without renewal (H=24cm) | m3 | 286.800 |
| 95 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete ( 0.6l/m2 ) | t | 0.717 |
| 96 | DB19G | Asphalt concrete covering with big aggregates, executed in hot conditions, in thickness of 6.0 cm with mechanic laying (BAD 22.4,conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 1195.000 |
| 97 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete (0.3l/m2 ) | t | 0.359 |
| 98 | DB16H | Asphalt concrete covering with small aggregates, executed in hot conditions, in thickness of 4.0 cm with mechanical laying (BA16, conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 1195.000 |
|  |  | Chapter7.2. Arranging the entries into the courtyards |  |  |
| 99 | TsC03F1 | Mechanic digging with excavator of 0.40 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in trucks, ground cat. II. (box; Y=1.9t/m3) | 100 m3 | 0.520 |
| 100 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 98.800 |
| 101 | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 0.520 |
| 102 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 0.520 |
| 103 | DI96 | Compacting the earthwork in the ground of cat. II with pneumatic compactor of 25 t, 8 movements on one pathway (ONLY Compactor 25t) | 100 m3 | 0.520 |
| 104 | TsD14A01 | Mechanical watering of soil layers with a 5-8 t tanker truck, provided with a spraying device, to complete the humidity necessary for mechanical compaction, as well as for watering surfaces for other purposes | m3 | 5.200 |
| 105 | TsE05B | Levelling with motor grader up to 175 HP of the natural land field and of the groundwork platforms, by cutting the bumps and pushing the dug soil in the holes, land cat. II (earthworks) | 100 m2 | 2.250 |
| 106 | Dl110 | Arranging the leveling layer from the ballast (drained from the mixture of optimal aggregates 0-32 Ga75 conf.SM-EN 13242+A1 Н=10сm Kcomp-1.22) | m3 | 22.500 |
| 107 | DA12B | Foundation or re-profiling layer from crushed stone LA30, fr.32-63mm;fr.8-16mm conf.SM-EN 13242+A1;2008, for roads with mechanical covering, executed with wedging without renewal (H=20cm) | m3 | 33.750 |
| 108 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete ( 0.6l/m2 ) | t | 0.135 |
| 109 | DB16H | Asphalt concrete covering with small aggregates, executed in hot conditions, in thickness of 4.0 cm with mechanical laying (BA16, conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 225.000 |
| 110 | DE11A | Small edging, precast from concrete with section of 10x15 cm, for framing green spaces, sidewalks, alleys, etc., placed on a concrete foundation C12/15 XO, of 10x20 cm (БР100.20.8) | m | 100.000 |
|  |  | Chapter 7.3. Arranging the platforms for auto stations |  |  |
| 111 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor-cars, ground cat. II (box; Y=1.9t/m3) | 100 m3 | 2.470 |
| 112 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 469.300 |
| 113 | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 2.470 |
| 114 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 2.470 |
| 115 | DI96 | Compacting the earthwork in the ground of cat. II with pneumatic compactor of 25 t, 8 movements on one pathway (ONLY Compactor 25t) | 100 m3 | 1.610 |
| 116 | TsD14A01 | Mechanical watering of soil layers with a 5-8 t tanker truck, provided with a spraying device, to complete the humidity necessary for mechanical compaction, as well as for watering surfaces for other purposes | m3 | 16.100 |
| 117 | TsE05B | Levelling with motor grader up to 175 HP of the natural land field and the groundwork platforms, by cutting the bumps and pushing the dug soil in the holes, land cat. II (earthworks) | 100 m2 | 5.380 |
| 118 | Dl110 | Arranging the leveling layer from the ballast (drained from the mixture of optimal aggregates 0-32 Ga75 conf.SM-EN 13242+A1 Н=10сm Kcomp-1.22) | m3 | 53.800 |
| 119 | DA12B | Foundation or re-profiling layer from crushed stone LA30, fr.32-63mm;fr.8-16mm conf.SM-EN 13242+A1;2008, for roads with mechanical covering, executed with wedging without renewal (H=24cm) | m3 | 129.120 |
| 120 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete (0.6l/m2) | t | 0.323 |
| 121 | DB19G | Asphalt concrete covering with big aggregates, executed in hot conditions, in thickness of 6.0 cm with mechanic laying (BAD 22.4,conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 538.000 |
| 122 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete (0.3l/m2) | t | 0.161 |
| 123 | DB16H | Asphalt concrete covering with small aggregates, executed in hot conditions, in thickness of 4.0 cm with mechanical laying (BA16, conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 538.000 |
|  |  | Chapter 7.4. Arranging the sidewalk |  |  |
| 124 | TsA01B1 | Manual digging of soil in wide spaces, excavating, open ditches, borrowing holes, for removing the vegetative layer of 10-30 cm, in soil with natural humidity, throwing in the storage or in the vehicle at a height of H< 0.60 m, in medium ground (Y=1.9t/m3) | m3 | 33.000 |
| 125 | TsC02C1 | Mechanic digging with pneumatic excavator of 0,21-0,39 m3, with hydraulic command, in grounds with natural humidity, and auto unloading of field of cat. I (loading) | 100 m3 | 0.330 |
| 126 | TsI51A5 | Transportation of soil with the dumper of 10 t at a distance of 5 km (to the storage) | t | 62.700 |
| 127. | TsC51B | Works for unloading the soil in the storage, ground category II | 100 m3 | 0.330 |
| 128 | TsC50Bk=2 | Repairing and maintaining the natural roads when transporting the soil, for every 0.5*2=1.0 km, field category II | 100 m3 | 0.330 |
| 129 | Dl111 | Mechanized execution of foundation layers with h = 12 cm from crushed stone in sidewalks (LA30 fr.16-32mm; fr.8-16mm ) | m2 | 256.000 |
| 130 | Dl107 | Priming the surface of the main layers in order to apply a layer of asphaltic concrete ( 0.6l/m2 ) | t | 0.200 |
| 131 | DB16B | Asphalt concrete covering with small aggregates, executed in hot conditions, in thickness of 3.0 cm with mechanical laying (BA16, conf.CP D.02.25:2021 with road bitumen 50/70) | m2 | 256.000 |
| 132. | DE11A | Small edging, precast from concrete with section of 10x15 cm, for framing green spaces, sidewalks, alleys, etc., placed on a concrete foundation C12/15 XO, of 10x20 cm (БР100.20.8) | m | 79.000 |
|  |  | Chapter 7.5. Arranging big curbs |  |  |
| 133. | DE10A | Pre-manufactured concrete borders, for pavements 20x25 cm, on concrete foundation (C16/20 XF1) 30x15 cm (БР100.30.18 according to SM EN 1340:2010+AC) | m | 1 679.000 |
|  |  | Chapter 7.6. Arranging the road signs |  |  |
| 134. | DF18A | Planting the pillars for industrially-manufactured metallic road traffic signs (on concrete foundation (C 16/20 XO)) (CKM2.35=15piece;CKM2.40=7piece) | pcs | 22.000 |
| 135 | DF20A | Installing in localities the road traffic signs made of steel or aluminum, placing them on existing special pillars for existing indicators | pcs | 50.000 |
|  |  | Chapter 7.7. Setting the road marking |  |  |
| 136. | DF17A | Longitudinal, transversal and various markings, mechanically executed, on road surfaces, coldplast type | m2 | 122.300 |
| 137 | DF17A | Longitudinal, transversal and various markings, mechanically executed, on road surfaces (transversal), coldplast type - white lines on red carpet | m2 | 213.200 |

|  |  | Total Incl. salary | USD USD |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CAS, 24% | USD |  |  |
|  |  | Transportation of materials, % | USD |  |  |
|  |  | Total | USD |  |  |
|  |  | Overhead costs, % | USD |  |  |
|  |  | Total | USD |  |  |
|  |  | Benefit, % | USD |  |  |
|  |  | Total cost of the offer Incl. salary | USD USD |  |  |
|  | Bidder |  |  |  |  |
|  | (position, signature, name, surname) |  |  |  |  |

STAMP PLACE