List with quantities of works

for the price offer call

1. Name of the beneficiary: Crihana Veche Mayoralty.

2. Organizer of the procurement procedure: UNDP Moldova/EU4 Moldova: Focal Regions Programme.

3. Procurement object Reconstruction and partial change of destination of Buildings A, A' in the cultural and recreational construction in the business development center in Crihana Veche village, Cahul district

ROOF, FASADE.

Quotation – offer 1-AR/R

Bid currency -  USD

| № | Symbol of the norm and resource code | Name of works | Unit of measure | Volume |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Roof |  |  |
| 1 | RpCB18G | Demolishing the old concrete by mechanic means, reinforced concrete (battlement plate) | m3 | 3.10 |
| 2 | RpCI42F | Dismantling roof elements – bituminous membranes in one or two layers | m2 | 64.00 |
| 3 | RpCK42A | Dismantling the cold flooring from concrete or cement mortar | m2 | 64.00 |
| 4 | RpIzC45B | Dismantling the insulating or protective materials: granular (slag, granular or similar) | m3 | 12.80 |
| 5 | RpCG29C | Demolishing full brick walls, autoclaved aerated concrete, clay or light concrete blocks, perforated bricks, except for scaffold and cleaning the bricks (Note: demolishing two brick layers vents) | m3 | 0.01 |
| 6 | TrI1AA02F1 | Loading group A materials (heavy masonry debris), by transportation means up to 10 m distance – using ramp or from the ground, by car, category 1 | t | 13.20 |
| 7 | TsI50F | Transportation of loads with the trucks at a distance of 10 km | t | 13.20 |
| 8 | CB02C | Formwork from reusable panels using short and very short softwood planks to pour the concrete into slabs, beams and columns at heights of up to 20m inclusively | m2 | 110.00 |
| 9 | RCsB21A | Mechanical drilling of holes with diameter of 5 cm, in concrete elements, with up to 20 cm thickness Subsidiary materials=1.05 | pcs | 148.00 |
| 10 | CC04A | On-site shaped concrete steel reinforcement OB 37, with bars up to 8 mm diameter inclusively (Note: Anchorage) | kg | 53.70 |
| 11 | CC04A | On-site shaped concrete steel reinforcement OB 37, with bars up to 8 mm diameter inclusively | kg | 128.24 |
| 12 | CC04D3 | On-site shaped concrete steel reinforcement PC 52, with bars up to 8 mm in diameter and mounted into wall formworks with fixed cross-sectional area, using sliding formworks at heights up to 35 m | kg | 569.56 |
| 13 | CL17C | Diverse metallic confections, mounted apparently (external stairs in case of fire, etc.) exclusively parapets, railings, manhole covers, etc. (Note: Strip of 6x80x340) | kg | 23.10 |
| 14 | CA04F1 | Concrete poured into slab, beam, column formworks prepared at the concrete plant or bought ready-made according to article CA01 and poured by classical means (Note: concrete B15 - monolithic belt, columns, foundation, pillars) Subsidiary materials (softwood planks, nails, staples) =1.03 | m3 | 7.15 |
| 15 | CD70A | Ordinary limestone masonry blocks for the walls with the manual preparation of the mortar M-50, with the height of the floor up to 4 m | m3 | 16.40 |
| 16 | CE41A | Mounting rafters treated with antiseptic | m3 | 17.37 |
| 17 | CE30B | Sheathing or valley flashing made of roofing tiles, Eternit type plates etc., rough softwood planks (24 mm thick), planed on one side, for ordinary construction. The standards of the resources with 0 (zero) value are taken according to the design. (Note: Resinous wood boards 11.64 m3) | m2 | 880.00 |
| 18 | CE17A | Additional polymer layer, Ondutiss type, placed under the top layer of tiles, made of corrugated or ribbed sheets Subsidiary materials=1.03 | m2 | 880.00 |
| 19 | CE42A | Execution of dormers | pcs | 6.00 |
| 20 | CN50A | Fireproof treatment of the carpentry: trusses, arches, beams, rafters, plates. | m3 | 29.01 |
| 21 | CE07A | Corrugated metal sheets (tile effect) for roof covering (Lindab type) Subsidiary materials=1.05 | m2 | 880.00 |
| 22 | CE20A | Brass-type gutter assembly made of sheet metal with anti-corrosive coating (Note: gutters d=125 mm, steel brackets – 150 pcs, end caps – 6 pcs, joiners – 58 pcs, drop outlets – 18 pcs, gutter corner – 4 pcs) Subsidiary materials=1.03 | m | 156.00 |
| 23 | CE22A | Brass-type downspout assembly made of sheet metal with anti-corrosive coating (Note: downspouts d=90 mm, connectors – 132 pcs, drain drip caps – 36 pcs, end caps – 18 pcs, strainers, clips, joiners – zero price) Subsidiary materials=1.02 | m | 174.00 |
| 24 | CE07A | Ribbed sheet covering (tile effect) for drip edge (Lindab type) (Note: folded sheet – eave) Subsidiary materials=1.05 | m2 | 128.00 |
| 25 | CE40A | Building the frame using rectangular hollow bars treated with antiseptic (Note: ventilation ducts) | m3 | 0.15 |
| 26 | IzF14B | Sound insulation layer with mineral wool boards for general insulation, inserted between double walls, supported with galvanized mild steel wire (ventilation ducts, 50mm thick mineral wool boards) | m2 | 6.20 |
| 27 | CD07B1 | Walls made of galvanized corrugated sheets, fastened by self-tapping screws, installed at a height of 6 m minimum (ventilation ducts) | m2 | 8.50 |
| 28 | CD07B1 | Walls made of galvanized corrugated sheets, fastened by self-tapping screws, installed at a height of 6 m minimum (Note: Lindab sheets, ventilation ducts) | m2 | 4.00 |
| 29 | CE24I | Cap made of 0.5 mm galvanized sheet, for smoke, chimneys and vents covering Subsidiary materials (coal, hard water) = 1.03 | pcs | 1.00 |
| 30 | CL17B | Diverse metalwork, surface-mounted: rail, grids, manhole covers, snow guards, grills (snow guards – galvanized steel sheet, polyester protective coating 0.5 mm thick 1ml/0.81kg, 142m.l.x0.81=115.02kg) | kg | 115.02 |
| 31 | IzF10A | Terrace roofs, roofs and ceilings insulation, with thermal-insulating boards from autoclaved aerated concrete, perlite, fibro-perlite, etc. (semi-rigid mineral wool boards Y=125kg/m3, 150mm thick) | m2 | 590.00 |
| 32 | CG01A | Supporting layer for flooring executed from cement mortar M 100-T of 3 cm thickness with delicately smoothed surface | m2 | 590.00 |
| 33 | CL17B | Various metal garments, mounted visibly: rail, grids, manhole covers, snow stops, grills (protection barrier) | kg | 344.00 |
| 34 | CN20B | Internal or external painting applied for the metal carpentry with alkyd enamel in 2 layers , including the plaster | m2 | 25.44 |
| 35 | CK23B | Plastic windows with one or more leafs with heights up to 35 m inclusively, having an area of the casement between 1.00 and 2.5 m2 inclusively | m2 | 8.10 |
|  |  | Total Roof Incl. salary | $ $ |  |
|  |  | 2. Carpentry |  |  |
| 36 | CK23B | Plastic windows with one or more leafs with heights up to 35 m inclusively, having an area of the casement between 1.00 and 2.5 m2 inclusively | m2 | 414.70 |
| 37 | CK25A | Doors made of plastic profiles, including the casement and the necessary accessories for assembling doors in any type of masonry in constructions with the height up to 35 m inclusively, in one leaf, with the surface of the case up to 7 m2 inclusively | m2 | 57.67 |
| 38 | CK26B | Sills assembled at the windows from aluminum | m | 208.00 |
| 39 | CK26C | Sills assembled at the windows or doors from plastic | m | 208.00 |
|  |  | Total Carpentry. Incl. salary | $ $ |  |
|  |  | 3. Facade |  |  |
| 40 | CB14A | Tubular metallic scaffold for works on vertical areas for heights up to 30 m inclusively, with immobilization of the scaffold for 25 days (200 hours) | m2 | 1 300.00 |
| 41 | CN53A | Coating the external surfaces of the walls and ceilings (coating with Betonocontact) | m2 | 1 058.00 |
| 42 | IzF55C | External thermal insulation of the walls of the buildings with fine plaster based on thermal insulators (rigid fixation systems of the thermal insulation), smooth wall surface: with plates of mineral wool (note: mineral wool of 10 cm thick, density of 145 kg/m3, aluminum cornice 448 m.l) Subsidiary materials (cloth, foam) = 1.01 | m2 | 820.00 |
| 43 | IzF55B | External thermal insulation of the walls of the buildings with fine plaster based on thermal insulators (rigid fixation systems of the thermal insulation), smooth wall surface: with polystyrene plates thickness 10 cm Subsidiary materials (cloth, foam) = 1.01 | m2 | 120.00 |
| 44 | IzF55E | External thermal insulation of the sills of the buildings with fine plaster based on thermal insulators (rigid fixation systems of the thermal insulation), smooth wall surface: with fireproof plates of mineral wool th. 40 mm Subsidiary materials (foam, cloth, angles) = 1.01 | m2 | 118.00 |
| 45 | CN54B | Manual application of the quartz ground “Gleta" in one layer, for the exterior walls of the facade (walls + sills) | m2 | 1 062.00 |
| 46 | CF30A | Exterior coating of 2-3 mm thickness, executed manually, with “TINC” mixture on the walls. | m2 | 938.00 |
| 47 | CI21A | Plating the walls with ceramic-granite plates: size up to 400 x 400 mm. Subsidiary materials (disk, cloth) = 1.01 | m2 | 124.00 |
| 48 | RpCL14A | Repairing the stairs from concrete Bc 7.5 (B 100), poured on the site, executed roughly so as to be plated | m | 15.00 |
| 49 | CI24A | Plating the steps with ceramic-granite tiles, bonded with glue, with thickness under 15 mm | m2 | 11.00 |
|  |  | Total Facade Incl. salary | $ $ |  |
|  |  | 4. Pitching, rampant |  |  |
| 50 | CB02A | Reusable formwork panels with short and very short wood boarding planks to pour the concrete in bearings, foundations and foundations glass and foundation equipment including support | m2 | 23.00 |
| 51 | CC03C | Assembling sealed meshes at heights lower or equal to 35 m, for plates (Vr I 4/150x150 thickness 1.32 kg/m2) | kg | 306.26 |
| 52 | CG22A | Simple concrete flooring class C 10/8 (Bc 10 / B 150) in thickness of 10 cm, continuous field, leveled, poured on the site, in rooms with areas over 16 m2 (concrete B15) | m2 | 234.00 |
| 53 | CA04F | Concrete poured in slabs, beams, columns, prepared with the concrete plant or concrete CA01 and pouring with classical means Subsidiary materials (softwood planks, nails, staples) = 1.03 | m3 | 2.40 |
| 54 | CL19D | Ready-made metallic grating: from profiled steel or forged steel at stairs and balconies, exclusively the current mine | kg | 99.60 |
|  |  | Total, Pitching, rampant Incl. salary | $ $ |  |
|  |  | 5. Interior finishing works |  |  |
| 55 | RpCG29A | Demolishing the walls from brick masonry or autoclaved aerated concrete 6-8 cm thick | m2 | 125.00 |
| 56 | RpCG29C | Demolishing the walls from full brick masonry, autoclaved aerated concrete, ceramic or light concrete blocks, GVP bricks, exclusively the scaffold and cleaning the bricks | m3 | 42.00 |
| 57 | RpCJ35A | Removing external or internal coating from walls or ceilings | m2 | 660.00 |
| 58 | RpCK42A | Dismantling the cold flooring from concrete or cement mortar | m2 | 1 306.80 |
| 59 | TrI1AA02F1 | Loading group A materials (heavy masonry debris), by transportation means up to 10 m distance – using ramp or from the ground, by car, category 1 | t | 141.00 |
| 60 | TsI50F | Transportation of loads with the trucks at a distance of 10 km | t | 141.00 |
| 61 | CD66С | Brick masonry, format 250 x 120 x 65 mm at the separating reinforced walls, with thickness 1/2 brick, with manual preparation of the mortar M-50, the height of the floor up to 4m | 100 m2 | 1.25 |
| 62 | CF02A | Exterior coating of 2 cm thickness, levelled, executed manually, on the walls and pylons, on plain surfaces, with cement-lime mortar of brand M 25-T for sprit and lime-cement mortar M 10-T for primer and visible layer, on brick masonry or small concrete blocks | m2 | 700.00 |
| 63 | CF50B | Interior coating of 5 mm thickness, executed manually, with gypsum-based dry mixture, for walls and dividing walls, manual preparation of the mortar. | m2 | 380.00 |
| 64 | СF59B | Coating the surfaces with a layer of plasterboards executing the simple plain metallic carcass, with the height up to 4 m: inclined ceilings without insulation Subsidiary materials (water, sanding paper, etc.) = 1.00 | m2 | 870.00 |
| 65 | CF57A | Manual application of the gypsum-based putty "Eurofin" thickness 1,0 mm on the ceilings’, walls’ and columns’ surfaces. | m2 | 870.00 |
| 66 | CN06A | Interior painting with paints based on vinyl copolymers in water emulsion, applied in 2 layers on the existing fillings, executed manually. | m2 | 1 250.00 |
| 67 | CI07A | Plywood of glazed, unglazed, matte or glossy pottery executed on flat surfaces on walls and pillars, including sills fixed with adhesive paste with alternating grouts and grouting with putty, in premises with an area over 10 m2, in constructions where via the architectural design specific requirements are imposed for the architectural quality and value Subsidiary materials (diluent, rags, water, etc.) = 1.02 | m2 | 309.00 |
| 68 | CG01A | Supporting layer for flooring executed from cement mortar M 100-T of 3 cm thickness with delicately smoothed surface | m2 | 1 306.80 |
| 69 | IzF31A | Waterproofing the concrete surfaces (vertical ones, horizontal ones, including the ceilings) with “Penetron” mixture - 2 layers: smooth surface | m2 | 76.80 |
| 70 | CG47D | Ceramic tile floors, including the adhesives (dry mix), tile size: 300 x 300 mm Subsidiary materials (cloth) = 1.01 | m2 | 1 306.80 |
| 71 | CK28B | Suspended ceilings made on site from PFL or melamine chipboard (PAL) with joints covered with plastic profiles Subsidiary material (self-tapping screws, with notched head, glue, nails) = 1.03 | m2 | 76.80 |
| 72 | CK29F | Suspended ceiling from pre-manufactured panels "Armstrong", including the grid-system | m2 | 1 230.00 |
|  |  | Total Interior finishing works Incl. salary | $ $ |  |
|  |  | Total Direct estimate costs Incl. salary | $ $ |  |
|  |  | Social Insurance, 24% | $ |  |
|  |  | Transportation of materials, % | $ |  |
|  |  | Total | $ |  |
|  |  | Overhead costs, % | $ |  |
|  |  | Total | $ |  |
|  |  | Estimate benefit, % | $ |  |
|  |  | Total estimates: Incl. salary | $ $ |  |

Bidder	_____________________________________________________________

(certified estimates’ developer, signature, name, surname)

Verified	_______________________________________________________________

(position, signature, name, surname)