| Construction of a hall made of metal elements in Zârnești village for organising the activity of the public services operator ServComSud SRL from Cahul district (Car hall) | Form No 1 WinSmeta |
| --- | --- |
| (name of the project) |  |

1-1. Bill of quantities

of works, Architectural Solutions (AS), Reinforced Concrete Structures (RCS), Structural Steel Structures (SS), General Plan (GP)

Currency of offer: US Dollars

| Seq No | Norm symbol and Resource code | Works and costs | M.U. | Quantity as per project data | Estimated cost, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— including wages | Total ————— including wages |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Earthworks |  |  |  |  |  |  |  |  |  |  |
| 1 |  | TsH61A |  | Random rubbish removal |  |  | 100 m2 |  | 2.80 |  |  |  |  |  |
| 2 |  | TsC21B1 |  | Mechanical digging with a motor grader up to 175 hp, including earth spreading at 10 m, in soil of cat. II |  |  | 100 m3 |  | 0.84 |  |  |  |  |  |
| 3 |  | TsC03F1 |  | Mechanical digging with excavator of 0.40-0.70 cubic meters, with internal combustion engine and hydraulic control, in soil with natural humidity, unloading in vehicles, cat. II soil |  |  | 100 m3 |  | 1.02 |  |  |  |  |  |
| 4 |  | TsA03E |  | Manual digging of soil in confined spaces, less than 1.00 m wide, performed without shoring, with sloping embankment at foundations, canals, etc., medium or very cohesive soil, up to 1.5 m deep, medium soil |  |  | m3 |  | 11.50 |  |  |  |  |  |
| 5 |  | TsI51A5 |  | Transporting soil by 10 t dump truck over a distance of 5 km |  |  | t |  | 93.00 |  |  |  |  |  |
| 6 |  | TsD01B |  | Spreading of loosened soil in uniform layers, 10-30 cm thick, by throwing up to 3 m from the heaps, including the breaking up of the clods, in medium soil |  |  | m3 |  | 42.00 |  |  |  |  |  |
| 7 |  | TsD05A |  | Compacting with a 150-200 kg plate compactor of the fills in successive layers of 20-30 cm thickness, excluding the watering of each layer, the fills consisting of cohesionless soil. |  |  | 100 m3 |  | 0.42 |  |  |  |  |  |
|  |  |  |  | Total Earthworks Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Foundations |  |  |  |  |  |  |  |  |  |  |
| 8 |  | CB02A |  | Formwork from reusable panels made of short and shortened softwood planks for pouring concrete into stem walls, pocket footings and foundation for equipment, including supports |  |  | m2 |  | 46.00 |  |  |  |  |  |
| 9 |  | CA03F01 |  | Plain concrete poured by conventional means, in foundations, plinths, retaining walls, walls below ground level, prepared by concrete mixer on site, poured by conventional means, plain concrete grade... (concrete grade C2.8/3.5) |  |  | m3 |  | 7.20 |  |  |  |  |  |
| 10 |  | CA03F |  | Plain concrete poured by conventional means, in foundations, plinths, retaining walls, walls below ground level, prepared with concrete mixing plant or ready-mixed concrete according to art. CA01, poured by conventional means, plain concrete grade... (concrete grade C12/15) |  |  | m3 |  | 32.71 |  |  |  |  |  |
| 11 |  | CA03F |  | Plain concrete poured by conventional means, in foundations, plinths, retaining walls, walls below ground level, prepared with concrete mixing plant or ready-mixed concrete according to art. CA01, poured by conventional means, plain concrete grade... (concrete grade C16/20) |  |  | m3 |  | 20.30 |  |  |  |  |  |
| 12 |  | CC01E |  | Concrete steel reinforcement OB 37, fabricated in site workshops and installed in existing building elements, with bar diameters up to 8 mm, including in continuous and raft foundations |  |  | kg |  | 29.43 |  |  |  |  |  |
| 13 |  | CC01F1 |  | Concrete steel reinforcement PC 52, fabricated in site workshops and installed in existing building elements, with bar diameters over 8 mm, including in continuous and raft foundations |  |  | kg |  | 639.51 |  |  |  |  |  |
| 14 |  | CL18A |  | Various metal structures made of laminated profiles, sheet metal, corrugated sheet metal, concrete steel, pipes for supports or coverings, fully or partially embedded in concrete (anchor bolts, Pi1 part) |  |  | kg |  | 400.50 |  |  |  |  |  |
|  |  |  |  | Total Foundations Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Floors |  |  |  |  |  |  |  |  |  |  |
| 15 |  | TsC54B |  | Crushed stone foundation layer (fraction 20-40, h – 150mm) |  |  | m3 |  | 30.00 |  |  |  |  |  |
| 16 |  | CC03C |  | Mounting of welded meshes at heights less than or equal to 35 m, in slabs (VrI 150x150x4mm) |  |  | kg |  | 250.00 |  |  |  |  |  |
| 17 |  | CG22A1 |  | Concrete floors of plain concrete grade C 10/8 (Bc 10/B 150), 10 cm thick, in continuous pour, leveled, site-poured, in rooms with a surface less than or equal to 16 sq.m. |  |  | m2 |  | 51.10 |  |  |  |  |  |
| 18 |  | CA02C |  | Plain concrete poured in slopes, excavations, for levelling, at heights up to 35 m inclusively, prepared with a concrete mixing plant according to art. CA01 or ready-mixed concrete, poured by conventional means (concrete layer – 150 mm). |  |  | m3 |  | 17.85 |  |  |  |  |  |
| 19 |  | IzF31A |  | Waterproofing of concrete surfaces (vertical, horizontal, including ceilings) with Penetron mixture, in 2 layers, smooth surface |  |  | m2 |  | 4.95 |  |  |  |  |  |
| 20 |  | CG01A |  | Leveling layer for flooring, prepared from M 300-T cement mortar, 10 cm thick, sanded surface |  |  | m2 |  | 199.88 |  |  |  |  |  |
| 21 |  | CG47D |  | Ceramic tile flooring, including adhesives (dry mix), tile size: over 300 x 300 mm |  |  | m2 |  | 51.10 |  |  |  |  |  |
|  |  |  |  | Total Floors Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Metal Structures |  |  |  |  |  |  |  |  |  |  |
| 22 |  | CL18A |  | Various metal structures made of laminated profiles, sheet metal, corrugated sheet metal, concrete steel, pipes for supports or coverings, fully or partially embedded in concrete (metal frames) |  |  | kg |  | 9 580,67 |  |  |  |  |  |
| 23 |  | CL18A |  | Various metal structures made of laminated profiles, sheet metal, corrugated sheet metal, concrete steel, pipes for supports or coverings, fully or partially embedded in concrete (linear roof elements) |  |  | kg |  | 2 431,04 |  |  |  |  |  |
| 24 |  | CL18A |  | Various metal structures made of laminated profiles, sheet metal, corrugated sheet metal, concrete steel, pipes for supports or coverings, fully or partially embedded in concrete (bracing) |  |  | kg |  | 1 008,84 |  |  |  |  |  |
|  |  |  |  | Total Metal Structures Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Walls, roofs |  |  |  |  |  |  |  |  |  |  |
| 25 |  | CD73A |  | Walls made of lightweight, profiled, sheet metal skinned, heat-insulating sandwich panels mounted on metal or reinforced concrete rails at heights below 12 m: positioned in front of the rails. Note: * the brand and type of sealing of the panels is determined according to the project. |  |  | m2 |  | 330.00 |  |  |  |  |  |
| 26 |  | CE44A |  | Coverings made of lightweight, profiled, sheet metal skinned, heat-insulating sandwich panels mounted on metal rails. Note: * the brand and type of sealing of the panels is determined according to the project. |  |  | m2 |  | 235.40 |  |  |  |  |  |
| 27 |  | CE20A |  | Brass type gutter systems made of corrosion-protected sheet metal |  |  | m |  | 42.80 |  |  |  |  |  |
| 28 |  | CE22A |  | Brass type downpipe systems made of corrosion-protected sheet metal |  |  | m |  | 32.40 |  |  |  |  |  |
|  |  |  |  | Total walls, roofs Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Carpentry |  |  |  |  |  |  |  |  |  |  |
| 29 |  | CK25D |  | Doors made of plastic profiles, including the fittings and accessories necessary for the doors, mounted in masonry of any kind, in buildings up to 35 m high inclusively, with double leaves, with a frame surface up to 7 sq.m. inclusively. |  |  | m2 |  | 13.08 |  |  |  |  |  |
| 30 |  | CK23B |  | Plastic windows, with one or more leaves, in buildings up to and including 35 m in height, having a frame area of between 1.00 and 2.5 sq.m. inclusively |  |  | m2 |  | 25.11 |  |  |  |  |  |
| 31 |  | CK26C |  | Sills mounted on plastic windows or doors |  |  | m |  | 28.40 |  |  |  |  |  |
| 32 |  | CK13B |  | Single-leaf sliding or folding metal doors and gates, mounted in masonry of any kind, including the necessary accessories, having a surface area exceeding 7 sq.m. |  |  | m2 |  | 56.70 |  |  |  |  |  |
|  |  |  |  | Total Carpentry Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. Mesh fencing (Eurogard) |  |  |  |  |  |  |  |  |  |  |
| 33 |  | RpCA01A |  | Manual digging of soil in confined spaces, less than 1.00 m wide and not more than 1.5 m deep, with vertical slope, for polygonal foundation pits, ditches, canals, etc., executed in quantities of up to 20 cubic meters with unsupported banks |  |  | m3 |  | 21.60 |  |  |  |  |  |
| 34 |  | CB02A |  | Formwork from reusable panels made of short and shortened softwood planks for pouring concrete into stem walls, pocket footings and foundation for equipment, including supports |  |  | m2 |  | 136.00 |  |  |  |  |  |
| 35 |  | CA03B2 |  | Concrete poured in foundations, plinths, retaining walls, walls below ground level, prepared with a concrete mixer on site and poured by conventional means, plain concrete grade C 10/8 (Bc 10/B 150) Subsidiary materials (softwood lumber, nails, staples)=1,01 |  |  | m3 |  | 40.30 |  |  |  |  |  |
| 36 |  | CO06Bf |  | Wire mesh fencing with fence panels (Eurogard 2.5x1.5 m – 95 sections, 50x50x3 posts, L=2.1 m – 96 pcs., 100x100x4 posts, L=2.5 m, 3 pcs., 7.5 sq.m gate, 1.5 sq.m small gate) Subsidiary materials (nails, steel wire)=1,03 |  |  | m |  | 234.00 |  |  |  |  |  |
|  |  |  |  | Total Mesh fencing (Eurogard) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Pavement |  |  |  |  |  |  |  |  |  |  |
| 37 |  | TsC21A1 |  | Mechanical digging with a motor grader up to 175 hp, including earth spreading at 10 m, in soil of cat. I |  |  | 100 m3 |  | 4.94 |  |  |  |  |  |
| 38 |  | TsA02A |  | Manual digging of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, performed without shoring, with vertical slope, of foundations, canals, basements, drains, embankment steps, in cohesionless or weakly cohesive soil, depth < 0.75 m, light soil |  |  | m3 |  | 9.30 |  |  |  |  |  |
| 39 |  | TsE06A |  | Preparation of a soil platform for the asphalting of an insulating or repair layer of sand or ballast, by manual levelling and compaction with a 10-12 t self-propelled static roller compactor, in cohesionless soil. |  |  | 100 m2 |  | 16.46 |  |  |  |  |  |
| 40 |  | TsC54B |  | Granite crushed stone foundation layer (fraction 20-70 mm) |  |  | m3 |  | 399.50 |  |  |  |  |  |
| 41 |  | DE10E |  | Precast concrete curbs, for sidewalks 24x25 cm, on concrete foundation 35x15 cm (100x30x15 cm curbs) |  |  | m |  | 96.00 |  |  |  |  |  |
| 42 |  | DE11A |  | Small, prefabricated concrete kerbs with a section of 10x15 cm, for framing green spaces, sidewalks, alleys, etc., laid on a 10x20 cm concrete foundation. |  |  | m |  | 59.00 |  |  |  |  |  |
| 43 |  | DE18B |  | Pavements made of precast concrete paving slabs, laid on a layer of dry cement and sand mixture, 1:6 ratio, grouted with dry cement and sand mixture, 10 cm thick layer (8 cm thick). |  |  | m2 |  | 1 566,00 |  |  |  |  |  |
| 44 |  | DE18B |  | Pavements made of precast concrete paving slabs, laid on a layer of dry cement and sand mixture, 1:6 ratio, grouted with dry cement and sand mixture, 10 cm thick layer (4 cm thick). |  |  | m2 |  | 80.00 |  |  |  |  |  |
|  |  |  |  | Total Pavement Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | USD |  |  |  |  |  |  |  |
|  |  |  |  | CASM (ПЗ) * 24 % |  |  | 24.00% |  |  |  |  |  |  |  |
|  |  |  |  | Transportation of materials |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Estimate profit |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total estimates: Including wages |  |  |  |  |  |  |  |  |  |  |

| Bidder |  |
| --- | --- |
| (position)(signature)(name, surname) |  |
| Certified Estimating Professional |  |