| Interior repair and modernization works of the science classrooms at the LT ''M. Sadoveanu’’ Ocnița |  |
| --- | --- |
| (name of the objective) |  |

Bill of quantities of works

Architectural and construction solutions (ACS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 |  | 2 |  | 3 |  |  |  |  | 4 |  | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Demolition works |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Elevation 3.300 |  |  |  |  |  |  |  |  |
| 1 |  | RpCO56A |  | Dismantling of wooden joinery (doors). |  |  |  |  | m2 |  | 7,98 |  |
| 2 |  | RpCG29C1 |  | Mechanical demolition of masonry walls made of solid bricks, BCA (aerated concrete), ceramic blocks, or lightweight concrete, GVP bricks, excluding scaffolding and cleaning of bricks, for mechanical demolition. |  |  |  |  | m3 |  | 1,98 |  |
| 3 |  | RpCJ35A |  | Removal of interior or exterior plaster, troweled on walls or ceilings. |  |  |  |  | m2 |  | 149,00 |  |
| 4 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, including the underlay or just the carpet. |  |  |  |  | m2 |  | 106,04 |  |
| 5 |  | RpCK42A |  | Removal of cold flooring made of concrete or cement mortar. |  |  |  |  | m2 |  | 106,04 |  |
| 6 |  | RpCB09A |  | Surface roughening of the top concrete layer of floors (scoring) |  |  |  |  | m2 |  | 9,30 |  |
| 7 |  | TsH92B |  | Loading soil (earth) with boulders and stones into a dump truck. |  |  |  |  | t |  | 14,10 |  |
| 8 |  | TsI51A8 |  | Transportarea pamintului cu autobasculanta de 10 t la distanta de: 8 km |  |  |  |  | t |  | 14,10 |  |
| 9 |  | TsC51C |  | Works for unloading soil into the deposit, soil category III |  |  |  |  | 100 m3 |  | 0,07 |  |
|  |  |  |  | Total 1.1 Elevation 0.000 |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Elevation 6.600 |  |  |  |  |  |  |  |  |
| 10 |  | RpCO56A |  | Dismantling of wooden joinery (doors). |  |  |  |  | m2 |  | 4,20 |  |
| 11 |  | RpCG29C1 |  | Mechanical demolition of masonry walls made of solid bricks, BCA (aerated concrete), ceramic blocks, or lightweight concrete, GVP bricks, excluding scaffolding and cleaning of bricks, for mechanical demolition. |  |  |  |  | m3 |  | 2,18 |  |
| 12 |  | RpCJ35A |  | Removal of interior or exterior plaster, troweled on walls or ceilings. |  |  |  |  | m2 |  | 154,50 |  |
| 13 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, including the underlay or just the carpet. |  |  |  |  | m2 |  | 106,04 |  |
| 14 |  | RpCK42A |  | Removal of cold flooring made of concrete or cement mortar. |  |  |  |  | m2 |  | 106,04 |  |
| 15 |  | TsH92B |  | Loading soil (earth) with boulders and stones into a dump truck. |  |  |  |  | t |  | 14,80 |  |
| 16 |  | TsI51A8 |  | Transportarea pamintului cu autobasculanta de 10 t la distanta de: 8 km |  |  |  |  | t |  | 14,80 |  |
| 17 |  | TsC51C |  | Works for unloading soil into the deposit, soil category III |  |  |  |  | 100 m3 |  | 0,08 |  |
|  |  |  |  | Total 1.2 Elevation 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Elevation 9.900 |  |  |  |  |  |  |  |  |
| 18 |  | RpCO56A |  | Dismantling of wooden joinery (doors). |  |  |  |  | m2 |  | 13,86 |  |
| 19 |  | RpCG29C1 |  | Mechanical demolition of masonry walls made of solid bricks, BCA (aerated concrete), ceramic blocks, or lightweight concrete, GVP bricks, excluding scaffolding and cleaning of bricks, for mechanical demolition. |  |  |  |  | m3 |  | 7,24 |  |
| 20 |  | RpCJ35A |  | Removal of interior or exterior plaster, troweled on walls or ceilings. |  |  |  |  | m2 |  | 183,50 |  |
| 21 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, including the underlay or just the carpet. |  |  |  |  | m2 |  | 161,41 |  |
| 22 |  | RpCK42A |  | Removal of cold flooring made of concrete or cement mortar. |  |  |  |  | m2 |  | 161,41 |  |
| 23 |  | RpCB09A |  | Surface roughening of the top concrete layer of floors (scoring) |  |  |  |  | m2 |  | 9,30 |  |
| 24 |  | TsH92B |  | Loading soil (earth) with boulders and stones into a dump truck. |  |  |  |  | t |  | 46,00 |  |
| 25 |  | TsI51A8 |  | Transport of soil with a 10-ton dump truck over a distance of 8 km. |  |  |  |  | t |  | 46,00 |  |
| 26 |  | TsC51C |  | Earthworks for unloading soil in a deposit, category III land. |  |  |  |  | 100 m3 |  | 0,23 |  |
|  |  |  |  | Total 1.3 Elevation 9.900 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 1 Demolition works |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Elevation Finish 3.300 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Ceiling |  |  |  |  |  |  |  |  |
| 27 |  | CK29F |  | Suspended ceilings made of prefabricated "Armstrong" panels, including the grid system. Panel dimensions: 600x600x13 mm. RAL 9003. |  |  |  |  | m2 |  | 105,77 |  |
|  |  |  |  | Total 2.1 Ceiling |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.1. Par1 |  |  |  |  |  |  |  |  |
| 28 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 116,00 |  |
| 29 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -116,00 |  |
| 30 |  | CG49A |  | Execution of antibacterial covering with 2 mm thick homogeneous vinyl carpet, class 34-43, for floors with baseboard installation. Small materials (rags) = 1.0050 |  |  |  |  | m2 |  | 116,00 |  |
|  |  |  |  | Total 2.2.1 Par1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.2. Par2 |  |  |  |  |  |  |  |  |
| 31 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 1,50 |  |
| 32 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -1,50 |  |
| 33 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 1,50 |  |
| 34 |  | CG17D |  | Porcelain stoneware flooring, RAL 7001, wear resistance class 4, anti-slip, 600x600 mm, thickness 9 mm, with joint filling using a grout resistant to disinfectants, including the adhesive mortar support layer, applied on surfaces larger than 16 m². |  |  |  |  | m2 |  | 1,50 |  |
| 35 |  | CI14B |  | Linear elements made of porcelain stoneware tiles fixed with adhesive, height 100 mm Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 1,50 |  |
|  |  |  |  | Total 2.2.2 Par2 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 2.2 Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.1. Finish type 1 |  |  |  |  |  |  |  |  |
| 36 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 153,30 |  |
| 37 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 153,30 |  |
| 38 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 222,29 |  |
| 39 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 153,30 |  |
| 40 |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 153,30 |  |
| 41 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 153,30 |  |
| 42 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 153,30 |  |
| 43 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 153,30 |  |
| 44 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 153,30 |  |
| 45 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 153,30 |  |
|  |  |  |  | Total 2.3.1 Finish type 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.2. Finish type 2 |  |  |  |  |  |  |  |  |
| 46 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 6,23 |  |
| 47 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 6,23 |  |
| 48 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 9,04 |  |
| 49 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 6,23 |  |
| 50 |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 6,23 |  |
| 51 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 6,23 |  |
| 52 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 6,23 |  |
| 53 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 6,23 |  |
| 54 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 6,23 |  |
| 55 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 6,23 |  |
|  |  |  |  | Total 2.3.2 Finish type 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.3. Finish type 3 |  |  |  |  |  |  |  |  |
| 56 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 3,33 |  |
| 57 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 3,33 |  |
| 58 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 4,83 |  |
| 59 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 3,33 |  |
| 60 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 3,33 |  |
| 61 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 3,33 |  |
|  |  |  |  | Total 2.3.3 Finish type 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.4. Finish type 7 |  |  |  |  |  |  |  |  |
| 62 |  | CD72D |  | Partition walls with a thickness of 100 mm, made of two layers of drywall (PGC) on both sides, with 50 mm mineral wool insulation, and a metal frame made of identical CW50 and UW50 profiles, up to a height of 4 m. Small materials (water, rags, etc.) = 1.0050 |  |  |  |  | m2 |  | 8,18 |  |
| 63 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 16,63 |  |
| 64 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 16,63 |  |
|  |  |  |  | Total 2.3.4 Finish type 7 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.5. Other works |  |  |  |  |  |  |  |  |
| 65 |  | CD74A |  | P2. Masonry of exterior and interior walls made of "Porotherm" ceramic blocks, using M100 cement and sand mortar, up to a floor height of 4 m. |  |  |  |  | m3 |  | 1,80 |  |
|  |  |  |  | Total 2.3.5 Other works |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.6. Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
| 66 |  | RCsB29A |  | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 20 mm, L=400 mm (K=3.6) Labor coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 67 |  | CP21A |  | Execution of M 100-T mortar for bonding, monolithic bonding, or sealing of joints up to a height of 35 m, filling technological and installation gaps in prefabricated elements, including finishing of surfaces. |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 68 |  | CP21C |  | Execution of M 100-T mortar for joint bonding, monolithic bonding, or sealing of joints up to a height of 35 m, sealing of horizontal joints between wall panels and ceiling panels, or vertical joints between wall panels. |  |  |  |  | m |  | 12,00 |  |
| 69 |  | RpCU02A |  | Steel beam lintels, installed in masonry, including cutting the beams to size. |  |  |  |  | kg |  | 188,00 |  |
| 70 |  | IzD10C |  | Anti-corrosion painting by hand brushing of metal structures and constructions with a coat of primer GF-021 and two layers of enamel PF-115, applied to metal structures and constructions made of profiles with thicknesses up to 7 mm inclusive. |  |  |  |  | t |  | 0,19 |  |
| 71 |  | CK35B |  | Metal expansion anchors M12x120 mm fixed in reinforced concrete walls |  |  |  |  | pcs (pieces) |  | 4,00 |  |
|  |  |  |  | Total 2.3.6 Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 2.3. Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Carpentry |  |  |  |  |  |  |  |  |
| 72 |  | CK03A |  | Interior door with a single leaf, 40 mm thick, featuring honeycomb stabilization filling, HPL laminate finish in natural ash wood (RAL 1014), matte lacquer coating, smooth surface, vertical window (240x1500 mm) with 4+4 mm transparent triplex glass, 1 mm stainless steel protective plate at the bottom, stainless steel door handle set, anodized aluminum frame, laminated glass 3.1.3, steel corner profile door frame (100 mm), powder-coated in RAL 7044, minimum 3 hinges per leaf, lock, strike plate, maximum 20 mm threshold height, and door frame dimensions (1000x2200(h) mm). |  |  |  |  | m2 |  | 2,42 |  |
| 73 |  | CK22C |  | Interior glazing made of aluminum profiles, white color, with a maximum threshold of 20 mm, filled with semi-transparent tempered glass, consisting of fixed panels and a door leaf (750x2200(h) mm) (V2 - 2200x2200(h) mm). |  |  |  |  | m2 |  | 4,84 |  |
|  |  |  |  | Total 2.4 Carpentry |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 2. Elevation Finish 3.300 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Elevation Finish 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1 Ceiling |  |  |  |  |  |  |  |  |
| 74 |  | CK29F |  | Suspended ceilings made of prefabricated "Armstrong" panels, including the grid system. Panel dimensions: 600x600x13 mm. RAL 9003. |  |  |  |  | m2 |  | 105,60 |  |
|  |  |  |  | Total 3.1 Ceiling |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2.1. Par 1 |  |  |  |  |  |  |  |  |
| 75 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 97,70 |  |
| 76 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -97,70 |  |
| 77 |  | CG49A |  | Execution of antibacterial covering with 2 mm thick homogeneous vinyl carpet, class 34-43, for floors with baseboard installation. Small materials (rags) = 1.0050 |  |  |  |  | m2 |  | 97,70 |  |
|  |  |  |  | Total 3.2.1 Par 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2.2. Par 2 |  |  |  |  |  |  |  |  |
| 78 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 8,69 |  |
| 79 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -8,69 |  |
| 80 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 8,69 |  |
| 81 |  | CG17D |  | Porcelain stoneware flooring, RAL 7001, wear resistance class 4, anti-slip, 600x600 mm, thickness 9 mm, with joint filling using a grout resistant to disinfectants, including the adhesive mortar support layer, applied on surfaces larger than 16 m². |  |  |  |  | m2 |  | 8,69 |  |
| 82 |  | CI14B |  | Linear elements made of porcelain stoneware tiles fixed with adhesive, height 100 mm Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 9,33 |  |
|  |  |  |  | Total 3.2.2 Par 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 3.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3 Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
| 83 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 128,83 |  |
| 84 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 128,83 |  |
| 85 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 186,81 |  |
| 86 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 128,83 |  |
| 87 |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 128,83 |  |
| 88 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 128,83 |  |
| 89 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 128,83 |  |
| 90 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 128,83 |  |
| 91 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 128,83 |  |
| 92 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 128,83 |  |
|  |  |  |  | Total 3.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
| 93 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 13,79 |  |
| 94 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 13,79 |  |
| 95 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 9,04 |  |
| 96 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 13,79 |  |
| 97 |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 13,79 |  |
| 98 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 13,79 |  |
| 99 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 13,79 |  |
| 100 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 13,79 |  |
| 101 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 13,79 |  |
| 102 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 13,79 |  |
|  |  |  |  | Total 3.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
| 103 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 8,90 |  |
| 104 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 8,90 |  |
| 105 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 12,91 |  |
| 106 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 8,90 |  |
| 107 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 8,90 |  |
| 108 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 8,90 |  |
|  |  |  |  | Total 3.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.4 Fin. 9. |  |  |  |  |  |  |  |  |
| 109 |  | CD72D |  | Partition walls with a thickness of 100 mm, made of two layers of drywall (PGC) on both sides, with 50 mm mineral wool insulation, and a metal frame made of identical CW50 and UW50 profiles, up to a height of 4 m. Small materials (water, rags, etc.) = 1.0050 |  |  |  |  | m2 |  | 8,18 |  |
| 110 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 16,63 |  |
| 111 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 16,63 |  |
|  |  |  |  | Total 3.3.4 Fin. 9 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.5 Other works |  |  |  |  |  |  |  |  |
| 112 |  | CD74A |  | P2. Masonry of exterior and interior walls made of "Porotherm" ceramic blocks, using M100 cement and sand mortar, up to a floor height of 4 m. |  |  |  |  | m3 |  | 1,80 |  |
|  |  |  |  | Total 3.3.5 Other works |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.6. Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
| 113 |  | RCsB29A |  | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 20 mm, L=400 mm (K=3.6) Labor coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 114 |  | CP21A |  | Execution of M 100-T mortar for bonding, monolithic bonding, or sealing of joints up to a height of 35 m, filling technological and installation gaps in prefabricated elements, including finishing of surfaces. |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 115 |  | CP21C |  | Execution of M 100-T mortar for joint bonding, monolithic bonding, or sealing of joints up to a height of 35 m, sealing of horizontal joints between wall panels and ceiling panels, or vertical joints between wall panels. |  |  |  |  | m |  | 12,00 |  |
| 116 |  | RpCU02A |  | Steel beam lintels, installed in masonry, including cutting the beams to size. |  |  |  |  | kg |  | 188,00 |  |
| 117 |  | IzD10C |  | Anti-corrosion painting by hand brushing of metal structures and constructions with a coat of primer GF-021 and two layers of enamel PF-115, applied to metal structures and constructions made of profiles with thicknesses up to 7 mm inclusive. |  |  |  |  | t |  | 0,19 |  |
| 118 |  | CK35B |  | Metal expansion anchors M12x120 mm fixed in reinforced concrete walls |  |  |  |  | pcs (pieces) |  | 4,00 |  |
|  |  |  |  | Total 3.3.6 Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4 Carpentry |  |  |  |  |  |  |  |  |
| 119 |  | CK03A |  | Interior door with a single leaf, 40 mm thick, featuring honeycomb stabilization filling, HPL laminate finish in natural ash wood (RAL 1014), matte lacquer coating, smooth surface, vertical window (240x1500 mm) with 4+4 mm transparent triplex glass, 1 mm stainless steel protective plate at the bottom, stainless steel door handle set, anodized aluminum frame, laminated glass 3.1.3, steel corner profile door frame (100 mm), powder-coated in RAL 7044, minimum 3 hinges per leaf, lock, strike plate, maximum 20 mm threshold height, and door frame dimensions (1000x2200(h) mm). |  |  |  |  | m2 |  | 2,64 |  |
| 120 |  | CK03A |  | Interior door with a single leaf, 40 mm thick, featuring honeycomb stabilization filling, HPL laminate finish in natural ash wood (RAL 1014), matte lacquer coating, smooth surface, vertical window (240x1500 mm) with 4+4 mm transparent triplex glass, 1 mm stainless steel protective plate at the bottom, stainless steel door handle set, anodized aluminum frame, laminated glass 3.1.3, steel corner profile door frame (100 mm), powder-coated in RAL 7044, minimum 3 hinges per leaf, lock, strike plate, maximum 20 mm threshold height, and door frame dimensions (1000x2200(h) mm). |  |  |  |  | m2 |  | 1,65 |  |
| 121 |  | CK22C |  | Interior glazing made of aluminum profiles, white color, with a maximum threshold of 20 mm, filled with semi-transparent tempered glass, consisting of fixed panels and a door leaf (850x2200(h) mm) (V2 - 2200x2200(h) mm). |  |  |  |  | m2 |  | 4,84 |  |
|  |  |  |  | Total 3.4 Carpentry |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 3 Elevation Finish 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Elevation Finish 9.900 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.1 Ceiling |  |  |  |  |  |  |  |  |
| 122 |  | CK29F |  | Suspended ceilings made of prefabricated "Armstrong" panels, including the grid system. Panel dimensions: 600x600x13 mm. RAL 9003. |  |  |  |  | m2 |  | 161,77 |  |
|  |  |  |  | Total 4.1 Ceiling |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.2.1. Par 2 |  |  |  |  |  |  |  |  |
| 123 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 154,36 |  |
| 124 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -154,36 |  |
| 125 |  | CG49A |  | Execution of antibacterial covering with 2 mm thick homogeneous vinyl carpet, class 34-43, for floors with baseboard installation. Small materials (rags) = 1.0050 |  |  |  |  | m2 |  | 154,36 |  |
|  |  |  |  | Total 4.2.1 Par 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.2.2. Par 3 |  |  |  |  |  |  |  |  |
| 126 |  | CG56A |  | Self-leveling compound "Nivelir" screed, thickness 10mm Small materials (expanded polystyrene for deformation joints, rags, markers) = 1.0050 |  |  |  |  | m2 |  | 10,64 |  |
| 127 |  | CG56A1 |  | Correction to standard CG56A: subtract 1mm thickness: k=5 Labor coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -10,64 |  |
| 128 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 10,64 |  |
| 129 |  | CG17D |  | Porcelain stoneware flooring, RAL 7001, wear resistance class 4, anti-slip, 600x600 mm, thickness 9 mm, with joint filling using a grout resistant to disinfectants, including the adhesive mortar support layer, applied on surfaces larger than 16 m². |  |  |  |  | m2 |  | 10,64 |  |
| 130 |  | CI14B |  | Linear elements made of porcelain stoneware tiles fixed with adhesive, height 100 mm Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 12,70 |  |
|  |  |  |  | Total 4.2.2 Par 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 4.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3 Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
| 131 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 209,25 |  |
| 132 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 209,25 |  |
| 133 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 324,82 |  |
| 134 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 209,25 |  |
| 135 |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 209,25 |  |
| 136 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 209,25 |  |
| 137 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 209,25 |  |
| 138 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 209,25 |  |
| 139 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 209,25 |  |
| 140 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 209,25 |  |
|  |  |  |  | Total 4.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
| 141 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 34,89 |  |
| 142 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 34,89 |  |
| 143 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 50,59 |  |
| 144 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 34,89 |  |
|  |  | CF17C |  | Glass fiber plaster layer with a weight of 80 kg/m², applied as part of the plastering system with a dry mix. |  |  |  |  | m2 |  | 34,89 |  |
| 145 |  | CF50B |  | Interior plastering with a thickness of 5 mm, applied manually, using a dry mix based on gypsum, for walls and partition walls, with manual preparation of the mortar. |  |  |  |  | m2 |  | 34,89 |  |
| 146 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 34,89 |  |
| 147 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 34,89 |  |
| 148 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 34,89 |  |
| 149 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 34,89 |  |
|  |  |  |  | Total 4.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
| 150 |  | CN53A |  | Priming of interior surfaces with a quartz sand-based primer. |  |  |  |  | m2 |  | 12,60 |  |
| 151 |  | CF02B |  | Interior plastering with a thickness of 2 cm, troweled, applied manually on walls or columns, on flat surfaces with cement-lime mortar, M 100-T brand for spattering, priming, and visible layers, on brick or small concrete block masonry. |  |  |  |  | m2 |  | 12,60 |  |
| 152 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm, on walls (Weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 18,27 |  |
| 153 |  | CF17B k=2 |  | Various works - thickness difference, +5 mm to the primer layer applied to the walls, executed with M100 cement mortar Labor coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 12,60 |  |
| 154 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 12,60 |  |
| 155 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 12,60 |  |
|  |  |  |  | Total 3.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.4 Fin. 7. |  |  |  |  |  |  |  |  |
| 156 |  | CD72D |  | Partition walls with a thickness of 100 mm, made of two layers of drywall (PGC) on both sides, with 50 mm mineral wool insulation, and a metal frame made of identical CW50 and UW50 profiles, up to a height of 4 m. Small materials (water, rags, etc.) = 1.0050 |  |  |  |  | m2 |  | 8,18 |  |
| 157 |  | CF57A |  | Manual application of gypsum-based putty with a thickness of 1.0 mm on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 8,18 |  |
| 158 |  | CF56A |  | Manual application of plaster for interior works, with a thickness of 0.5 mm, on the surfaces of walls, columns, and ceilings. |  |  |  |  | m2 |  | 8,18 |  |
| 159 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 8,18 |  |
| 160 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over the existing plaster, executed manually, RAL according to the "Interior Design" project, with the following paint characteristics: - Coverage power: 2 - high coverage power; - Wet abrasion resistance according to DIN EN class 13300: 2 - abrasion-resistant. |  |  |  |  | m2 |  | 8,18 |  |
| 161 |  | CN53A |  | Priming of interior wall and ceiling surfaces |  |  |  |  | m2 |  | 8,48 |  |
| 162 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pillars, and sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 8,48 |  |
|  |  |  |  | Total 4.3.4 Fin. 7 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.5 Other works |  |  |  |  |  |  |  |  |
| 163 |  | CD74A |  | P2. Masonry of exterior and interior walls made of "Porotherm" ceramic blocks, using M100 cement and sand mortar, up to a floor height of 4 m. |  |  |  |  | m3 |  | 2,82 |  |
| 164 |  | СF60A |  | Cladding of surfaces in two layers (1 layer of 18 mm OSB III + 1 layer of 12.5 mm gypsum board), with the installation of an identical metal frame, up to 4 m in height: walls without insulation Small materials (water, sandpaper, etc.) = 1.0050 |  |  |  |  | m2 |  | 3,73 |  |
| 165 |  | CK33B |  | 300x300 mm steel inspection hatch, factory-finished with anti-corrosion coating |  |  |  |  | pcs (pieces) |  | 3,00 |  |
|  |  |  |  | Total 4.3.5 Other works |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3.6. Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
| 166 |  | RCsB29A |  | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 20 mm, L=400 mm (K=3.6) Labor coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 167 |  | CP21A |  | Execution of M 100-T mortar for bonding, monolithic bonding, or sealing of joints up to a height of 35 m, filling technological and installation gaps in prefabricated elements, including finishing of surfaces. |  |  |  |  | pcs (pieces) |  | 4,00 |  |
| 168 |  | CP21C |  | Execution of M 100-T mortar for joint bonding, monolithic bonding, or sealing of joints up to a height of 35 m, sealing of horizontal joints between wall panels and ceiling panels, or vertical joints between wall panels. |  |  |  |  | m |  | 12,00 |  |
| 169 |  | RpCU02A |  | Steel beam lintels, installed in masonry, including cutting the beams to size. |  |  |  |  | kg |  | 188,00 |  |
| 170 |  | IzD10C |  | Anti-corrosion painting by hand brushing of metal structures and constructions with a coat of primer GF-021 and two layers of enamel PF-115, applied to metal structures and constructions made of profiles with thicknesses up to 7 mm inclusive. |  |  |  |  | t |  | 0,19 |  |
| 171 |  | CK35B |  | Metal expansion anchors M12x120 mm fixed in reinforced concrete walls |  |  |  |  | pcs (pieces) |  | 4,00 |  |
|  |  |  |  | Total 4.3.6 Reinforcement frame RF-1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4 Carpentry |  |  |  |  |  |  |  |  |
| 172 |  | CK03A |  | Interior door with a single leaf, 40 mm thick, featuring honeycomb stabilization filling, HPL laminate finish in natural ash wood (RAL 1014), matte lacquer coating, smooth surface, vertical window (240x1500 mm) with 4+4 mm transparent triplex glass, 1 mm stainless steel protective plate at the bottom, stainless steel door handle set, anodized aluminum frame, laminated glass 3.1.3, steel corner profile door frame (100 mm), powder-coated in RAL 7044, minimum 3 hinges per leaf, lock, strike plate, maximum 20 mm threshold height, and door frame dimensions (1000x2200(h) mm). |  |  |  |  | m2 |  | 1,58 |  |
| 173 |  | CK03A |  | Interior door with a single leaf, 40 mm thick, featuring honeycomb stabilization filling, HPL laminate finish in natural ash wood (RAL 1014), matte lacquer coating, smooth surface, vertical window (240x1500 mm) with 4+4 mm transparent triplex glass, 1 mm stainless steel protective plate at the bottom, stainless steel door handle set, anodized aluminum frame, laminated glass 3.1.3, steel corner profile door frame (100 mm), powder-coated in RAL 7044, minimum 3 hinges per leaf, lock, strike plate, maximum 20 mm threshold height, and door frame dimensions (1000x2200(h) mm). |  |  |  |  | m2 |  | 7,68 |  |
| 174 |  | CK22C |  | Interior glazing made of aluminum profiles, white color, with a maximum threshold of 20 mm, filled with semi-transparent tempered glass, consisting of fixed panels and a door leaf (850x2200(h) mm) (V2 - 2200x2200(h) mm). |  |  |  |  | m2 |  | 4,84 |  |
|  |  |  |  | Total 4.4 Carpentry |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.5 Blinds |  |  |  |  |  |  |  |  |
| 175 |  | CK32B |  | Blinds (jalousies) for windows, Venetian type, made of anodized aluminum, including accessories / Fabric roller blinds, fixed on the window profile, color RAL 9016, 50% transparency. |  |  |  |  | m2 |  | 66,00 |  |
|  |  |  |  | Total 4.5 Blinds |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 4 Elevation Finish 9.900 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  | lei |  |  |  |  |  |  |
|  |  |  |  | Social insurance |  | 24 % |  |  |  |  |  |  |
|  |  |  |  | Transport expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Supply and storage expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Estimated profit |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Total Including salary |  |  |  |  |  |  |  |  |

Bill of quantities of works

Interior water supply and sewerage networks (WSSN)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 16–25 cm thick, using mechanized methods | pcs (pieces) | 5,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 26–50 cm thick, using mechanized methods | pcs (pieces) | 3,00 |
| 3 | RpEJ09A | Sealing of openings in walls with gypsum mortar | pcs (pieces) | 3,00 |
| 4 | RpEJ09C | Sealing of openings in slabs with cement mortar | pcs (pieces) | 5,00 |
| 5 | SD07A | Ball-butterfly valve with socket and Dutch connection FI-FI, 1/2", PN16 Small materials (hemp fiber, lead primer, etc.) = 1.0200 | pcs (pieces) | 6,00 |
| 6 | SA15B | Polypropylene pipe PN16, joined by butt fusion welding, for connecting pipes to sanitary fixtures, residential and social-cultural buildings, with a diameter of 20 mm. | m | 60,00 |
| 7 | SA37B | Brace for securing water and gas supply pipes, made of steel or PVC, mounted by embedding, with pipes having a diameter of 3/4". | pcs (pieces) | 20,00 |
| 8 | RpIF09A | Insulation of pipes with special insulation sleeves made of elastomeric foam based on synthetic rubber, inserted onto the pipes, type ARMAFLEX AF, with a diameter and thickness of D=22x7.5 mm Small materials = 1.0500 | m | 60,00 |
| 9 | SF01C | Performing the pressure sealing test of the hot or cold water installation made of heavy-duty polyvinyl chloride (PVC) pipes or plastic material, with a diameter of 16-110 mm. | m | 60,00 |
| 10 | SF02C | Performing the operation test of the cold water installation made with heavy-duty polyvinyl chloride (PVC) pipes or plastic material, with a diameter of 16 - 110 mm. | 10 m | 6,00 |
| 11 | SF05C | Flushing of the cold or hot water installation made of plastic pipes, with a diameter of 20-75 mm. | m | 60,00 |
| 12 | SB08E | Polypropylene (PP-R) pipe for drainage, joined with a rubber gasket, installed surface-mounted or embedded under the floor, with a diameter of 110 mm. | m | 20,00 |
| 13 | SB08C | Polypropylene (PP-R) pipe for drainage, joined with a rubber gasket, installed surface-mounted or embedded under the floor, with a diameter of 50 mm. | m | 13,00 |
| 14 | SA37F | Brace for securing water and gas supply pipes, made of steel or PVC, mounted by embedding, with pipes having a diameter of 2". | pcs (pieces) | 13,00 |
| 15 | SB09E | 45° elbow, joined with a rubber gasket, with a diameter of 110 mm. | pcs (pieces) | 2,00 |
| 16 | SB09E | Excentric reducer joined with a rubber gasket, with a diameter of 110x50 mm. | pcs (pieces) | 1,00 |
| 17 | SB09C | 45° elbow joint with rubber gasket, diameter 50 mm | pcs (pieces) | 6,00 |
| 18 | SB10C | Connection piece (simple branch) made of plastic material for drainage, joined with a rubber gasket, with a diameter of 50 mm, 45° angle. | pcs (pieces) | 3,00 |
| 19 | SB09C | Cleaning piece made of PP-R with a threaded plug, for drainage, joined with a rubber gasket, with a diameter of 50 mm | pcs (pieces) | 2,00 |
| 20 | SB09E | Cleaning piece made of PP-R with a threaded plug, for drainage, joined with a rubber gasket, with a diameter of 110 mm | pcs (pieces) | 1,00 |
| 21 | SA40A | Installation of fire-resistant fittings, D=50 mm, fixed to the ceiling slab with dowels | pcs (pieces) | 5,00 |
| 22 | SB09C | Cleaning piece made of PP-R with a threaded plug, for drainage, joined with a rubber gasket, with a diameter of 50 mm | pcs (pieces) | 1,00 |
| 23 | SF04A | Performing the leak and operation test of the sewage system made of cast iron pipes, for drainage, unplasticized polyvinyl chloride (PVC) pipes, light type or plastic material, ductile cast iron pipes, with a diameter of up to 100 mm inclusive. | 10 m | 3,30 |
| 24 | SC04C | Semi-porcelain sink, sanitary porcelain, etc., including for people with disabilities, with a plastic drain pipe, mounted on a pedestal Small materials (plaster, wooden dowels, etc.) = 1.0200 | pcs (pieces) | 3,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 1 Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 25 | SD04A | Mixer faucet with swinging arm for sink, regardless of the closing mechanism, including for people with disabilities, with a diameter of 1/2" | pcs (pieces) | 3,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 26 | Supplier price | Instant water heater, electric, 230V, 13A, 3.0 kW, hot water performance 1.2-1.7 l/min | set | 3,00 |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Heating, ventilation, and air conditioning (HVAC)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works 1.1 Heating |  |  |
| 1 | RpIB12A | Dismantling of convectors with casing | pcs (pieces) | 24,00 |
| 2 | RpIC19A | Dismantling of black steel pipes used in central heating installations, welded, in residential and socio-cultural buildings, for connecting appliances, distribution risers, etc., with a diameter of 3/8" – 3/4" | m | 130,00 |
| 3 | IC35B | Cross-linked polyethylene pipe for high pressure, type RAUTITAN stabil, d=20x2.9 mm, installed for connecting heating units or appliances in central heating systems (K=1.4 for fittings) Labor coefficient = 1.4000 Material coefficient = 1.4000 Equipment coefficient = 1.4000 | m | 95,00 |
| 4 | IC36C | Cross-linked polyethylene pipe for high pressure, type RAUTITAN stabil, d=32x4.7 mm, installed in risers for central heating systems (K=1.4 for fittings) Labor coefficient = 1.4000 Material coefficient = 1.4000 Equipment coefficient = 1.4000 | m | 35,00 |
| 5 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length up to 1000 mm, type 22K, 600x700 mm | pcs (pieces) | 2,00 |
| 6 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length up to 1000 mm, type 22K, 600x700 mm | pcs (pieces) | 1,00 |
| 7 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length 1001–1500 mm, type 22K, 600x1400 mm | pcs (pieces) | 1,00 |
| 8 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length up to 1000 mm, type 33K, 600x400 mm | pcs (pieces) | 1,00 |
| 9 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length 1001–1500 mm, type 33K, 600x1100 mm | pcs (pieces) | 4,00 |
| 10 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length 1001–1500 mm, type 33K, 600x1200 mm | pcs (pieces) | 1,00 |
| 11 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with mounting elements, length 1001–1500 mm, type 33K, 600x1400 mm | pcs (pieces) | 9,00 |
| 12 | ID13A | RTR-G type valve, straight, DN 15 | pcs (pieces) | 19,00 |
| 13 | ID01A | Angle valve with return and drain function for radiators, nominal diameter 1/2", type RLV | pcs (pieces) | 19,00 |
| 14 | IE01A | Pressure tightness test of the central heating installation, consisting of steel, copper, polyethylene or polypropylene pipes, with heating units, heating appliances, fittings, distributors, collectors, and air vents or valves, with a total heating surface area of up to 100 m² | m2 | 94,20 |
| 15 | IE02A | Expansion-contraction and operational testing of the central heating installation, consisting of steel, copper, polyethylene or polypropylene pipes, with heating units, heating appliances, fittings, distributors, collectors, and air vents or valves, with a total heating surface area of up to 100 m² | m2 | 94,20 |
| 16 | IE08A | Flushing of the interior central heating installation with potable water, with a total heating surface area of up to 100 m² | m2 | 94,20 |
| 17 | IE03A | Pressure tightness test of the supply pipes for heating appliances (air heaters, fan convectors, skirting convectors, etc.) with a diameter of 3/8" to 1" | m | 95,00 |
| 18 | IE03B | Pressure tightness test of the supply pipes for heating appliances (air heaters, fan convectors, skirting convectors, etc.) with a diameter of 1 1/4" to 2" | m | 35,00 |
| 19 | IE04A | Expansion-contraction and operational testing of the supply pipes for heating appliances (air heaters, fan convectors, skirting convectors, etc.) with a diameter of 3/8" to 1" | m | 95,00 |
| 20 | IE04B | Expansion-contraction and operational testing of the supply pipes for heating appliances (air heaters, fan convectors, skirting convectors, etc.) with a diameter of 1 1/4" to 2" | m | 35,00 |
|  |  | Total 1.1 Heating |  |  |
|  |  | 1.2 Ventilation |  |  |
| 21 | VA19A | Installation of ventilation ducts, class H, at a height from the floor up to 3 m, made of galvanized sheet metal with a thickness of 0.5 mm, with a circular section diameter of 100 - 180 mm Small materials (rags, silicone) = 1.0100 | m2 | 4,21 |
| 22 | Supplier price | Check valve for ducts, D=100 mm, type KOM100 | pcs (pieces) | 2,00 |
| 23 | IzH07B | Thermal insulation mineral wool with an aluminum foil layer, thickness 50 mm (fire-resistant EI30) | m2 | 7,67 |
| 24 | VB27A | Protective cap according to series 3K 00.000, for circular ducts with a perimeter of 230 - 700 mm Small materials = 1.0500 | pcs (pieces) | 4,00 |
| 25 | CL20A | Pre-assembled ventilation grilles with double regulator, mounted in the door, type MV450R/2 | pcs (pieces) | 6,00 |
| 26 | CL20A | Air supply and exhaust ventilation grilles, dimensions 200x200 mm | pcs (pieces) | 1,00 |
| 27 | CL20A | Air supply and exhaust ventilation grilles, dimensions 100x100 mm | pcs (pieces) | 1,00 |
| 28 | RCsB29P | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 170 mm, L=400 mm (K=3.6) Labor coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 | pcs (pieces) | 9,00 |
| 29 | RCsB29S | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 220 mm, L=400 mm (K=4.84) Labor coefficient = 4.8400 Material coefficient = 4.8400 Equipment coefficient = 4.8400 | pcs (pieces) | 2,00 |
| 30 | RCsB29K | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 120 mm, L=400 mm (K=3.6) Labor coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 | pcs (pieces) | 2,00 |
|  |  | Total 1.2 Ventilation |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 1 Construction works |  |  |
|  |  | 2. Installation works |  |  |
|  |  | 2.1 Ventilation |  |  |
| 31 | VB17A | Air intake valve with thermostat | pcs (pieces) | 7,00 |
| 32 | CL20A | Air intake valve with hygroregulation system and noise absorption, type EHA2 22-55 | pcs (pieces) | 1,00 |
| 33 | VC07A | Installation of the double-flow air intake-exhaust system Small materials and installation (screws, oxygen, carbide, etc.) = 1.0100 | pcs (pieces) | 10,00 |
| 34 | 11-05-001-01 | Execution mechanism, weight, kg, up to: 20 Fans | pcs (pieces) | 2,00 |
|  |  | Total 2.1 Ventilation |  |  |
|  |  | 2.2 Conditioning |  |  |
| 35 | 11-04-002.01 | Tabletop device, t, up to: 0.015 - Air cooler | pcs (pieces) | 4,00 |
|  |  | Total 2.2 Conditioning |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
|  |  | 3.1 Ventilation |  |  |
| 36 | Supplier price | Air intake valve with thermostat, type VTK-160 or similar. | pcs (pieces) | 6,00 |
| 37 | Supplier price | Air intake valve with hygroregulation system and noise absorption, type EHA2 22-55 or similar. | pcs (pieces) | 1,00 |
| 38 | Supplier price | Double-flow air intake-exhaust system with a copper heat exchanger, air preheating function, features: inlet air flow 110 m³/h, exhaust air flow 105 m³/h, power 4-12 W/h, 230V, installation diameter 150 mm, thermally insulated housing, remote control system via PRANA 150C remote control or similar. | pcs (pieces) | 8,00 |
| 39 | Supplier price | Double-flow air intake-exhaust system with a copper heat exchanger, air preheating function, features: inlet air flow 220 m³/h, exhaust air flow 200 m³/h, power 4 - 35 W/h, 230V, installation diameter 200 mm, thermally insulated housing, remote control system via PRANA 200C remote control or similar. | pcs (pieces) | 2,00 |
| 40 | Supplier price | Ventilator with a productivity of 85 m³/h, P=49Pa, N=0.016kW, 2300 rpm, 230V, type VENTS 100M or similar | pcs (pieces) | 2,00 |
|  |  | Total 3.1 Ventilation |  |  |
|  |  | 3.2. Conditioning |  |  |
| 41 | Supplier price | Air cooler type Trotec PAE 61 or similar | pcs (pieces) | 4,00 |
|  |  | Total 3.2 Conditioning |  |  |
|  |  | Total | lei |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3 Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Electrical power equipment. Interior electric lighting (EPE/IEL)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 16–25 cm thick, using mechanized methods | pcs (pieces) | 6,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 26–50 cm thick, using mechanized methods | pcs (pieces) | 11,00 |
| 3 | RpEJ09A | Sealing of openings in walls with gypsum mortar | pcs (pieces) | 17,00 |
| 4 | RpEJ08C | Execution of grooves with a depth of up to 5 cm in stone or reinforced concrete walls of 5x50 cm² | m | 50,00 |
| 5 | RpEJ09D | Cutting of grooves in walls up to 50 cm² | m | 50,00 |
| 6 | RCsB29G | Drilling of perforated holes in brick and stone masonry using a diamond core drill machine with a diameter of: 80 mm (K=0.5) Labor coefficient = 0.5000 Material coefficient = 0.5000 Equipment coefficient = 0.5000 | pcs (pieces) | 150,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 7 | RpEG17D | Disassembly of electrical panels: metal from panel, cabinet, cell, console | pcs (pieces) | 1,00 |
| 8 | RpEF23A | Disassembly of lighting fixtures of any type, including rods and globes | pcs (pieces) | 109,00 |
| 9 | RpEE24A | Disassembly of electrical devices for high voltage: single-pole (switches, breakers, sockets for removal or replacement, remote control buttons) | pcs (pieces) | 51,00 |
| 10 | RpED21A | Disassembly of electrical cables made of copper or aluminum, with PVC insulation or sheath, flame-retardant, with cross-section ranging from 1-2-3x1.5 to 1-2-3x10 mm² | m | 365,00 |
| 11 | 08-03-572-3 | Execution control block type cabinet or distribution point (cabinet), wall-mounted, height and width up to 600x600 mm | pcs (pieces) | 3,00 |
| 12 | 08-03-575-1 | Device - automatic circuit breakers, load break switches | pcs (pieces) | 48,00 |
| 13 | 08-03-530-4 | General-purpose magnetic starter, separate unit, mounted on structure on wall or column, current up to 40 A | pcs (pieces) | 2,00 |
| 14 | 08-02-407-1 | Steel conduit on installed structures, mounted on walls, fixed with clamps, diameter up to 25 mm | 100 m | 14,00 |
| 15 | Supplier price | Rigid PVC conduit for electrical installation, d=25 mm | m.l. | 1 400,00 |
| 16 | 08-02-397-1 | Mounting perforated profile, length 2 m | 100 pcs (pieces) | 0,15 |
| 17 | Supplier price | Perforated profile type K108, L=2.0 m | pcs (pieces) | 15,00 |
| 18 | 08-02-396-6 | Metal duct on walls and ceilings, length 3 m | 100 m | 1,20 |
| 19 | Supplier price | Perforated galvanized metal cable tray, cross-section 50x50 mm | m.l. | 120,00 |
| 20 | Supplier price | 90° elbow for galvanized metal cable tray 50x50 mm | pcs (pieces) | 20,00 |
| 21 | Supplier price | Wall bracket for cable tray mounting, B=60 mm | pcs (pieces) | 40,00 |
| 22 | 10-08-01901 | Wall-mounted junction box | pcs (pieces) | 155,00 |
| 23 | Supplier price | Corrosion-protected metal distribution box, IP54, 171x171x101 mm, type У995 | pcs (pieces) | 25,00 |
| 24 | Supplier price | Carbolite distribution box, IP44, D=102 mm, H=44 mm, type KOP-74 | pcs (pieces) | 85,00 |
| 25 | Supplier price | Distribution box У-192 | pcs (pieces) | 40,00 |
| 26 | 08-02-412-2 | Insertion of conductors into laid metal pipes and hoses: first single-wire or multi-wire conductor in common braid, total cross-section up to 6 mm² | 100 m | 8,00 |
| 27 | 08-02-412-3 | Insertion of conductors into laid metal pipes and hoses: first single-wire or multi-wire conductor in common braid, total cross-section up to 16 mm² | 100 m | 5,50 |
| 28 | 08-02-398-1 | Conductor in cable trays, cross-section up to 6 mm² | 100 m | 1,10 |
| 29 | 08-02-398-2 | Conductor in cable trays, cross-section up to 35 mm² | 100 m | 0,55 |
| 30 | 08-02-398-3 | Conductor in cable trays, cross-section up to 120 mm² | 100 m | 0,05 |
| 31 | Supplier price | Cable ВВГнг-LS-0.66 3x1.5 mm² | m.l. | 700,00 |
| 32 | Supplier price | Cable ВВГнг-LS-0.66 3x2.5 mm² | m.l. | 550,00 |
| 33 | Supplier price | Cable ВВГнг-LS-0.66 4x1.5 mm² | m.l. | 110,00 |
| 34 | Supplier price | Cable ВВГнг-LS-0.66 4x2.5 mm² | m.l. | 45,00 |
| 35 | Supplier price | Cable ВВГнг-LS-0.66 5x6 mm² | m.l. | 10,00 |
| 36 | Supplier price | Cable ВВГнг-LS-0.66 5x16 mm² | m.l. | 5,00 |
| 37 | Supplier price | Cable ВВГнг(A)-FRLS 3x1.5 mm² | m.l. | 150,00 |
| 38 | 08-03-59310 | Wall-mounted lighting indicators | 100 pcs (pieces) | 0,05 |
| 39 | 08-03-594-1 | Lighting fixture with LED modules | 100 pcs (pieces) | 0,92 |
| 40 | 08-03-591-2 | Single-rocker switch, flush-mounted, for closed installation, including mounting box | 100 pcs (pieces) | 0,18 |
| 41 | Supplier price | Single-rocker switch for closed installation with grounding, IP20, 10A, 230V | pcs (pieces) | 18,00 |
| 42 | 08-03-591-5 | Double-rocker switch, flush-mounted, for closed installation, including mounting box | 100 pcs (pieces) | 0,18 |
| 43 | Supplier price | Double-rocker switch for closed installation with grounding, IP20, 10A, 230V | pcs (pieces) | 18,00 |
| 44 | 08-03-591-7 | Changeover switch, flush-mounted, for closed installation, including mounting box | 100 pcs (pieces) | 0,02 |
| 45 | Supplier price | Changeover switch for closed installation with grounding, single rocker, IP20, 10A, 230V | pcs (pieces) | 2,00 |
| 46 | 10-08-01901 | Floor-mounted junction box | pcs (pieces) | 3,00 |
| 47 | 08-03-591-9 | Socket outlet, flush-mounted, for closed installation, including mounting box | 100 pcs (pieces) | 1,40 |
| 48 | Supplier price | Socket outlet for closed installation with grounding, IP20, 16A, 230V | pcs (pieces) | 125,00 |
| 49 | Supplier price | 2-module horizontal frame | set | 70,00 |
| 50 | Supplier price | 3-module horizontal frame | set | 11,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 51 | Supplier price | Surface-mounted metal distribution board with a capacity of 72 modules, IP41, equipped with a lock and DIN, N, and PE bars | pcs (pieces) | 1,00 |
| 52 | Supplier price | Surface-mounted metal distribution board with a capacity of 24 modules, IP41, equipped with a lock and DIN, N, and PE bars | pcs (pieces) | 2,00 |
| 53 | Supplier price | Modular load break switch, Un=400V, 3P, In=50A | pcs (pieces) | 1,00 |
| 54 | Supplier price | Modular load break switch, Un=400V, 3P, In=25A | pcs (pieces) | 2,00 |
| 55 | Supplier price | Single-pole circuit breaker, characteristic B, In=10A, U=230V | pcs (pieces) | 17,00 |
| 56 | Supplier price | Residual current circuit breaker (RCCB), double-pole, characteristic "AC", In=20A, 30mA, Un=230V | pcs (pieces) | 25,00 |
| 57 | Supplier price | Three-pole circuit breaker, characteristic C, In=63A, Un=400V | pcs (pieces) | 1,00 |
| 58 | Supplier price | Three-pole circuit breaker, characteristic C, In=32A, Un=400V | pcs (pieces) | 2,00 |
| 59 | Supplier price | Electromagnetic starter 380V, 39A, in IP65 enclosure, supplied with STOP button | pcs (pieces) | 2,00 |
| 60 | Supplier price | Indoor general lighting fixture, 595 x 595 mm, IP41, for installation in suspended ceiling, LED module 32W, opal reflector, luminous flux 3600 lm, 4000K, supply voltage 230V | pcs (pieces) | 87,00 |
| 61 | Supplier price | Emergency lighting fixture with “Exit” orientation sign, permanent type, IP20, surface-mounted on ceiling or wall, LED 3W, luminous flux 40 lm, supply voltage 230V, 3-hour operating autonomy | pcs (pieces) | 5,00 |
| 62 | Supplier price | Lighting fixture for school board illumination with wall spacer bracket, dimensions 1445x630x250 mm, with 48 LED modules, 20W, 2150 lm, IP41, type LE-SBO-14-020-7005-20D or equivalent | pcs (pieces) | 4,00 |
| 63 | Supplier price | Floor box with frame for mounting 2 socket outlets | set | 3,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Fire signaling and security (FS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 16–25 cm thick, using mechanized methods | pcs (pieces) | 6,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 26–50 cm thick, using mechanized methods | pcs (pieces) | 3,00 |
| 3 | RpEJ09A | Sealing of openings in walls with gypsum mortar | pcs (pieces) | 9,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 4 | 08-03-572-3 | Execution control block type cabinet or distribution point (cabinet), wall-mounted, height and width up to 600x600 mm | pcs (pieces) | 1,00 |
| 5 | 10-08-001-01 | Receptor devices: "SI" type reception and control units, for start-up. Concentrator: base block for 10 zones | pcs (pieces) | 1,00 |
| 6 | 10-08-001-07 | Installation of event recording module | 10 pcs (pieces) | 0,10 |
| 7 | 08-03-575-1 | Device - Radio transmitter | pcs (pieces) | 1,00 |
| 8 | 08-03-575-1 | Device – Battery | pcs (pieces) | 2,00 |
| 9 | 08-03-593-10 | Optical and acoustic signal device | 100 pcs (pieces) | 0,05 |
| 10 | 10-08-002-02 | Automatic "SI" detectors: smoke type | pcs (pieces) | 33,00 |
| 11 | 08-03-591-3 | Manual fire alarm call point | 100 pcs (pieces) | 0,04 |
| 12 | 08-03-593-10 | Illuminated exit sign panel "EXIT" | 100 pcs (pieces) | 0,04 |
| 13 | 10-02-050-04 | Control panel equipment: Installation of resistors | pcs (pieces) | 6,00 |
| 14 | Supplier price | Electrical resistance | pcs (pieces) | 6,00 |
| 15 | 08-02-409-2 | Vinyl plastic conduit on installed structures, on walls and columns, fixed with clamps, diameter up to 50 mm | 100 m | 0,20 |
| 16 | Supplier price | Rigid PVC conduit for electrical installation, d=50 mm | m.l. | 20,00 |
| 17 | 08-02-409-1 | Vinylplast pipe on installed constructions, on walls and risers, fixed with clamps, diameter up to 25 mm | 100 m | 4,00 |
| 18 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm | m.l. | 400,00 |
| 19 | 08-02-390-1 | Plastic cable ducts with cover, cross-section 22x10 mm, including all necessary accessories (elbows, end caps, tees, crosses, etc.) | 100 m | 1,50 |
| 20 | 08-02-411-1 | Metal hose, outer diameter up to 48 mm | 100 m | 0,20 |
| 21 | Supplier price | Flexible metal conduit with PVC insulation, D=21 mm | m.l. | 20,00 |
| 22 | 08-02-412-2 | Insertion of conductors into laid metal pipes and hoses: first single-wire or multi-wire conductor in common braid, total cross-section up to 6 mm² | 100 m | 5,00 |
| 23 | Supplier price | Fire alarm signal cable with 2 conductors, cross-section S=1.5 mm², with a fire resistance rating of at least 30 minutes, FE180 E30 – JE-H(St)H-1x2x1.5 mm² | m.l. | 450,00 |
| 24 | Supplier price | Cable ВВГнг(A)-FRLS 3x1.5 mm² | m.l. | 180,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 25 | Supplier price | Metal enclosure type ЩМП-3-0 УХЛ3, IP31, dimensions 650 x 500 x 220 mm | pcs (pieces) | 1,00 |
| 26 | Supplier price | Conventional fire alarm control panel, type Teletek MAG 8Plus, 8–16 zones, LED, EN54 certified or equivalent | pcs (pieces) | 1,00 |
| 27 | Supplier price | Event recording module compatible with the fire alarm control panel | pcs (pieces) | 1,00 |
| 28 | Supplier price | Radio transmitter type ATS-100 or equivalent | pcs (pieces) | 1,00 |
| 29 | Supplier price | 12V, 17Ah battery | pcs (pieces) | 2,00 |
| 30 | Supplier price | External fire siren with strobe, type SF 200, LED, 96dB, IP54, EN54 certified or equivalent | pcs (pieces) | 1,00 |
| 31 | Supplier price | Internal fire siren with strobe, type SF 105, LED, 105dB, EN54 certified or equivalent | pcs (pieces) | 3,00 |
| 32 | Supplier price | Smoke detector type SensoMAG S30, LED, 360°, EN54 certified or equivalent, with detector base and additional box for detector installation | pcs (pieces) | 23,00 |
| 33 | Supplier price | Fire alarm call point type SensoMAG MCP50, LED, IP40, EN54 compliant or equivalent | pcs (pieces) | 3,00 |
| 34 | Supplier price | Non-addressable audio-visual indicator “EXIT”, sound signal 92dB, 18–30V, current consumption 50mA | set | 3,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Automatic security signaling (ASS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 16–25 cm thick, using mechanized methods | pcs (pieces) | 3,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 26–50 cm thick, using mechanized methods | pcs (pieces) | 8,00 |
| 3 | RpEJ09A | Sealing of openings in walls with gypsum mortar | pcs (pieces) | 1,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 4 | 10-04-067-23 | Video surveillance device – video camera | pcs (pieces) | 8,00 |
| 5 | 10-08-002-06 | Structure for video camera installation | pcs (pieces) | 8,00 |
| 6 | 10-08-019-01 | Video camera junction box | pcs (pieces) | 8,00 |
| 7 | Supplier price | 100x100x70 mm junction box, IK08, for video camera connection | pcs (pieces) | 8,00 |
| 8 | 10-03-01305 aplicat | Access switch | pcs (pieces) | 1,00 |
| 9 | 10-04-030-01 | Power strip | pcs (pieces) | 1,00 |
| 10 | 10-04-030-01 | UPS | pcs (pieces) | 1,00 |
| 11 | 10-04-077-01 | Control panel equipment: Video recorder | pcs (pieces) | 1,00 |
| 12 | 10-04-101-10 | Subscriber equipment and miscellaneous equipment: 3.5" HDD | pcs (pieces) | 1,00 |
| 13 | 10-04-100-07 | Desktop equipment: System unit + Monitor | pcs (pieces) | 1,00 |
| 14 | 08-02-409-1 | Vinylplast pipe on installed constructions, on walls and risers, fixed with clamps, diameter up to 25 mm | 100 m | 4,00 |
| 15 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm | m.l. | 400,00 |
| 16 | 08-02-390-1 | Plastic cable ducts with cover, cross-section 15x10 mm, including all necessary accessories (elbows, end caps, tees, crosses, etc.) | 100 m | 1,20 |
| 17 | 08-02-411-1 | Metal hose, outer diameter up to 48 mm | 100 m | 0,50 |
| 18 | Supplier price | Flexible metal conduit with PVC insulation, D=25 mm | m.l. | 50,00 |
| 19 | 08-02-412-1 | Insertion of conductors into laid metal pipes and hoses: first single-wire or multi-wire conductor in common braid, total cross-section up to 2.5 mm² | 100 m | 4,80 |
| 20 | Supplier price | UTP Cat.5E 4P CU Solid cable | m.l. | 480,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 21 | Supplier price | IP Video Camera 4Mp, model Uniview IPC354SR3-ADNPF28-F with the following features: IR 30m; fixed 2.8mm lens; 1/3" CMOS; resolution: 2688 x 1520, 30fps; viewing angle: 95.8°; WDR 120dB; Ultra 265, H.265, H.264, MJPEG; intelligent functions - motion detection, human detection, intrusion detection, audio detection; audio: G.711, bidirectional audio support, built-in microphone; power supply: 12V DC, PoE; with Micro SD - 256 GB; PoE, IP67; or equivalent with camera mounting bracket. | pcs (pieces) | 7,00 |
| 22 | Supplier price | POE Switch ZKTeco PE082-120-C, 8xPOE, 2xLAN Uplink, max. 120W or equivalent | pcs (pieces) | 1,00 |
| 23 | Supplier price | 4-socket power strip 2P+E, 16A, 200-240V, 50/60Hz with safety switch | pcs (pieces) | 1,00 |
| 24 | Supplier price | UPS 1500VA (3-step AVR, CPU-controlled, USB), metal case, LCD display | pcs (pieces) | 1,00 |
| 25 | Supplier price | 16-channel IP video recorder, minimum 1xSATA interfaces with video and audio input/output signals | pcs (pieces) | 1,00 |
| 26 | Supplier price | 3.5" HDD SATA3, 10TB, 7200rpm, 256MB cache | pcs (pieces) | 1,00 |
| 27 | Supplier price | System unit - Computer with Core i7-13700 processor / GeForce RTX 3060 graphics card / 16GB RAM / Internal storage - 2TB SSD / Windows 11 operating system / Mouse / Keyboard | pcs (pieces) | 1,00 |
| 28 | Supplier price | 28" VA Monitor, Screen Resolution 2560x1440, 4 ms, 75 Hz, CR 4000:1, HDMI+DP, Pivot | pcs (pieces) | 1,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | 4. Adjustment and commissioning |  |  |
| 29 | 10-06-068-15 | Adjustment of synchronous digital transmission systems. Adjustment of the control and steering system: Configuration and adjustment of network components | pcs (pieces) | 1,00 |
|  |  | Total 4. Adjustment and commissioning |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Telephone and signaling communications (TSC)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 16–25 cm thick, using mechanized methods | pcs (pieces) | 3,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls, 26–50 cm thick, using mechanized methods | pcs (pieces) | 5,00 |
| 3 | RpEJ09A | Sealing of openings in walls with gypsum mortar | pcs (pieces) | 8,00 |
| 4 | RpEJ08C | Execution of grooves with a depth of up to 5 cm in stone or reinforced concrete walls of 5x50 cm² | m | 15,00 |
| 5 | RpEJ09D | Cutting of grooves in walls up to 50 cm² | m | 15,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 6 | 10-06-037-05 | Wall-mounted cabinet, 19" size, 12U | pcs (pieces) | 1,00 |
| 7 | 10-04-030-01 | 19" ventilation module with thermostat, 1U | pcs (pieces) | 1,00 |
| 8 | 10-03-01305 aplicat | Access / Distribution switch | pcs (pieces) | 2,00 |
| 9 | 10-04-030-01 | ODF Panel 19", 1U | pcs (pieces) | 1,00 |
| 10 | 10-04-030-01 | Modular patch panel Cat. 6e fully equipped with 24xRJ45 modules, 19", 1U | pcs (pieces) | 1,00 |
| 11 | 10-04-030-01 | 1U Cable management | pcs (pieces) | 1,00 |
| 12 | 10-04-030-01 | Power strip | pcs (pieces) | 1,00 |
| 13 | 10-04-030-01 | UPS | pcs (pieces) | 1,00 |
| 14 | 10-04-066-07 | Wall-mounted device: RJ-45 socket | pcs (pieces) | 5,00 |
| 15 | Supplier price | Socket with 2 RJ45 Cat.6e modules - frame, mounting box, and support, wall installation | pcs (pieces) | 5,00 |
| 16 | 10-02-051-03 | Cable connection bridges between sections of a rack: with installation in cable tray - UTP Patch Cord | 100 pcs (pieces) | 0,15 |
| 17 |  | Patch Cord cable 0.5 m, copper, UTP Cat.6e | pcs (pieces) | 10,00 |
| 18 |  | Patch Cord cable 1.5 m, copper, UTP Cat.6e | pcs (pieces) | 5,00 |
| 19 | 08-02-409-1 | Vinylplast pipe on installed constructions, on walls and risers, fixed with clamps, diameter up to 25 mm | 100 m | 6,00 |
| 20 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm | m.l. | 600,00 |
| 21 | 08-02-409-2 | Vinyl plastic conduit on installed structures, on walls and columns, fixed with clamps, diameter up to 50 mm | 100 m | 0,15 |
| 22 | Supplier price | Rigid PVC pipe for electrical installation, diameter 50 mm | m.l. | 15,00 |
| 23 | 08-02-390-1 | Plastic channel with cover, section 25x16 mm, including all necessary accessories (elbows, plugs, tees, crosses, etc.) | 100 m | 0,80 |
| 24 | 08-02-396-6 | Metal cable channel for walls and ceilings, 3 meters in length | 100 m | 0,60 |
| 25 | Supplier price | Metal cable channel with dimensions 50(h)x80 mm with cover | m.l. | 60,00 |
| 26 | 08-02-412-1 | Introduction of conductors into metal pipes and hoses: the first conductor, single-core or multi-core in common braid, cross-section up to 2.5 mm² | 100 m | 7,00 |
| 27 | Supplier price | UTP Cable Cat.6E 4P CU Solid | m.l. | 700,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 28 | Supplier price | 9U wall-mounted cabinet, 600x600 mm, 19" black, unassembled (set includes feet, wheels), with fans for hot air extraction produced by equipment and thermostat, with perforated front/back doors and tool-less mounting | pcs (pieces) | 1,00 |
| 29 | Supplier price | Access switch with 24 RJ-45 POE+ auto-negotiating 1 Gbps + at least 2 SFP+ 10 Gbps | pcs (pieces) | 1,00 |
| 30 | Supplier price | Distribution switch type CRS310-1G-5S-4S+IN or similar | pcs (pieces) | 1,00 |
| 31 | Supplier price | ODF Patch-Panel 19", 1U | pcs (pieces) | 1,00 |
| 32 | Supplier price | Modular patch panel Cat. 6e fully equipped with 24xRJ45 modules, 19", 1U | pcs (pieces) | 1,00 |
| 33 | Supplier price | Cable management 1U (Cable organizer), horizontal | pcs (pieces) | 1,00 |
| 34 | Supplier price | Power strip with at least 8 sockets 2P+E, 16A, 200-240V, 50/60Hz with safety button, 1U | pcs (pieces) | 1,00 |
| 35 | Supplier price | UPS Rack Mounting 2U SRV2000VA (SRV2KRI+SRVRK1), 2000VA/1600W, Double-conversion On-line UPS, 4 x IEC Sockets ( 4 IEC C13, all 4 Battery Backup + Surge Protected), Multifunction LCD Display, DB- 9 RS-232, USB, with Rail Kit | pcs (pieces) | 1,00 |
| 36 | Supplier price | Wall clock with independent power source | set | 3,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |