| Interior repair and modernization works of the science classrooms at the LT ''Alexei Mateevici’’ Dondușeni |  |
| --- | --- |
| (name of the objective) |  |

Bill of quantities of works

Architectural and construction solutions (ACS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 |  | 2 |  | 3 |  |  |  |  | 4 |  | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Demolition works |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Elevation 0.000 |  |  |  |  |  |  |  |  |
| 1 |  | RpCO56A |  | Dismantling of wooden joinery (doors). |  |  |  |  | m2 |  | 12,33 |  |
| 2 |  | RpCO56C |  | Dismantling of window sills. |  |  |  |  | m2 |  | 4,62 |  |
| 3 |  | RpCQ11C |  | Dismantling of glass brick walls. |  |  |  |  | m2 |  | 7,00 |  |
| 4 |  | RpCM33A |  | Dismantling of ceramic wall and floor tiles (faience, gresie, ceramic). |  |  |  |  | m2 |  | 17,80 |  |
| 5 |  | RpCG29C1 |  | Demolition using mechanical means of masonry walls made of solid bricks, AAC blocks, ceramic blocks or lightweight concrete, GVP bricks; excluding scaffolding and cleaning of bricks, for demolition using mechanical means. |  |  |  |  | m3 |  | 2,02 |  |
| 6 |  | RpCJ35A |  | Removal of interior or exterior plaster (float finish) on walls or ceilings. |  |  |  |  | m2 |  | 334,80 |  |
| 7 |  | RpCK41C |  | Removal of PVC flooring (with or without textile backing), carpet. |  |  |  |  | m2 |  | 132,10 |  |
| 8 |  | RpCK42A |  | Removal of cold floors made of concrete or cement mortar. |  |  |  |  | m2 |  | 132,10 |  |
| 9 |  | RpCB09A |  | Scoring of the superficial concrete layer on floors (grooving). |  |  |  |  | m2 |  | 37,82 |  |
| 10 |  | TsH92B |  | Loading into truck of soil (earth) with boulders and stones. |  |  |  |  | t |  | 22,00 |  |
| 11 |  | TsI51A8 |  | Transportation of soil with 10 t dump truck over a distance of: 8 km. |  |  |  |  | t |  | 22,00 |  |
| 12 |  | TsC51C |  | Unloading of soil at the deposit site, land category III. |  |  |  |  | 100 m3 |  | 0,12 |  |
|  |  |  |  | Total 1.1 Elevation 0.000 |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Elevation 6.600 |  |  |  |  |  |  |  |  |
| 13 |  | RpCO56A |  | Dismantling of wooden joinery (doors). |  |  |  |  | m2 |  | 18,51 |  |
| 14 |  | RpCO56C |  | Dismantling of window sills. |  |  |  |  | m2 |  | 5,32 |  |
| 15 |  | RpCQ11C |  | Dismantling of glass brick walls. |  |  |  |  | m2 |  | 7,00 |  |
| 16 |  | RpCG29C1 |  | Demolition using mechanical means of masonry walls made of solid bricks, AAC blocks, ceramic blocks or lightweight concrete, GVP bricks; excluding scaffolding and cleaning of bricks, for demolition using mechanical means. |  |  |  |  | m3 |  | 13,75 |  |
| 17 |  | RpCJ35A |  | Removal of interior or exterior plaster (float finish) on walls or ceilings. |  |  |  |  | m2 |  | 297,20 |  |
| 18 |  | RpCK41C |  | Removal of PVC flooring (with or without textile backing), carpet. |  |  |  |  | m2 |  | 161,53 |  |
| 19 |  | RpCK42A |  | Removal of cold floors made of concrete or cement mortar. |  |  |  |  | m2 |  | 161,53 |  |
| 20 |  | RpCB09A |  | Scoring of the superficial concrete layer on floors (grooving). |  |  |  |  | m2 |  | 65,65 |  |
| 21 |  | TsH92B |  | Loading into truck of soil (earth) with boulders and stones. |  |  |  |  | t |  | 42,36 |  |
| 22 |  | TsI51A8 |  | Transportation of soil with 10 t dump truck over a distance of: 8 km. |  |  |  |  | t |  | 42,36 |  |
| 23 |  | TsC51C |  | Unloading of soil at the deposit site, land category III. |  |  |  |  | 100 m3 |  | 0,22 |  |
|  |  |  |  | Total 1.2 Elevation 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 1 Demolition works |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Finish Elevation 0.000 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Ceiling |  |  |  |  |  |  |  |  |
| 24 |  | CK29F |  | Suspended ceilings made of prefabricated "Armstrong" panels, including the grid system. Panel dimensions: 600x600x13 mm. RAL 9003. |  |  |  |  | m2 |  | 151,11 |  |
|  |  |  |  | Total 2.1 Ceiling |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.1. Par1 |  |  |  |  |  |  |  |  |
| 25 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 37,82 |  |
| 26 |  | CG17D |  | Flooring made of porcelain gres tiles RAL 7001, wear resistance class 4, non-slip, 600x600 mm, thickness 9 mm, with joint filling using disinfectant-resistant grout, including adhesive mortar base layer, executed on surfaces larger than 16 m². |  |  |  |  | m2 |  | 37,82 |  |
| 27 |  | CI14B |  | Linear elements made of porcelain gres tiles fixed with adhesive, h = 100 mm. Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 34,95 |  |
|  |  |  |  | Total 2.2.1 Par1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.2. Par2 |  |  |  |  |  |  |  |  |
| 28 |  | CG56A |  | Self-leveling screed made of "Nivelir" mixture: thickness 10 mm. Small materials (expanded polystyrene for expansion joints, rags, guides) = 1.0050 |  |  |  |  | m2 |  | 130,72 |  |
| 29 |  | CG56A1 |  | Correction to norm CG56A: subtract for 1 mm thickness: k=5 Labour coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -130,72 |  |
| 30 |  | CG49A |  | Execution of antibacterial covering made of 2 mm thick homogeneous vinyl sheet, class 34-43, on floors including skirting installation. Small materials (rags) = 1.0050 |  |  |  |  | m2 |  | 130,72 |  |
|  |  |  |  | Total 2.2.2 Par2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.3. Par3 |  |  |  |  |  |  |  |  |
| 31 |  | CG56A |  | Self-leveling screed made of "Nivelir" mixture: thickness 10 mm. Small materials (expanded polystyrene for expansion joints, rags, guides) = 1.0050 |  |  |  |  | m2 |  | 2,00 |  |
| 32 |  | CG56A1 |  | Correction to norm CG56A: subtract for 1 mm thickness: k=5 Labour coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -2,00 |  |
| 33 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 2,00 |  |
| 34 |  | CG17D |  | Flooring made of porcelain gres tiles RAL 7001, wear resistance class 4, non-slip, 600x600 mm, thickness 9 mm, with joint filling using disinfectant-resistant grout, including adhesive mortar base layer, executed on surfaces larger than 16 m². |  |  |  |  | m2 |  | 2,00 |  |
| 35 |  | CI14B |  | Linear elements made of porcelain gres tiles fixed with adhesive, h = 100 mm. Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 1,00 |  |
|  |  |  |  | Total 2.2.3 Par3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.1. Finish type 1 |  |  |  |  |  |  |  |  |
| 36 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 272,79 |  |
| 37 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 272,79 |  |
| 38 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 395,55 |  |
| 39 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 272,79 |  |
| 40 |  | CF17C |  | Layer of fiberglass mat, weight 80 kg/m², applied as part of plaster with dry mix. |  |  |  |  | m2 |  | 272,79 |  |
| 41 |  | CF50B |  | Interior plaster, 5 mm thick, applied manually with gypsum-based dry mix, on walls and partition walls, with manual mortar preparation. |  |  |  |  | m2 |  | 272,79 |  |
| 42 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 272,79 |  |
| 43 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 272,79 |  |
| 44 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 272,79 |  |
| 45 |  | CN06A |  | Fin 1. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 264,50 |  |
| 46 |  | CN06A |  | Fin 1.2. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 6,97 |  |
| 47 |  | CN06A |  | Fin 1.3. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 1,32 |  |
|  |  |  |  | Total 2.3.1 Finish type 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.2. Finish type 2 |  |  |  |  |  |  |  |  |
| 48 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 2,20 |  |
| 49 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 2,20 |  |
| 50 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 3,19 |  |
| 51 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 2,20 |  |
| 52 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 2,20 |  |
| 53 |  | CI22B |  | Cladding with ceramic tiles (on walls, columns, pilasters and window jambs) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm. Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 2,20 |  |
|  |  |  |  | Total 2.3.2 Finish type 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.3. Finish type 3 |  |  |  |  |  |  |  |  |
| 54 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 18,07 |  |
| 55 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 18,07 |  |
| 56 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 26,21 |  |
| 57 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 18,07 |  |
| 58 |  | CF17C |  | Layer of fiberglass mat, weight 80 kg/m², applied as part of plaster with dry mix. |  |  |  |  | m2 |  | 18,07 |  |
| 59 |  | CF50B |  | Interior plaster, 5 mm thick, applied manually with gypsum-based dry mix, on walls and partition walls, with manual mortar preparation. |  |  |  |  | m2 |  | 18,07 |  |
| 60 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 18,07 |  |
| 61 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 18,07 |  |
| 62 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 18,07 |  |
| 63 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 18,07 |  |
|  |  |  |  | Total 2.3.3 Finish type 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.4. Finish type 4 |  |  |  |  |  |  |  |  |
| 64 |  | CD72D |  | Partition walls with 100 mm thickness, made of gypsum plasterboard in two layers on both sides, with 50 mm mineral wool insulation, including identical metal frame made of CW50 and UW50 profiles, with height up to 4 m. Small materials (water, rags, etc.) = 1.0050 |  |  |  |  | m2 |  | 8,32 |  |
| 65 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 16,63 |  |
| 66 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns and ceilings. |  |  |  |  | m2 |  | 16,63 |  |
| 67 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 16,63 |  |
| 68 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 16,63 |  |
|  |  |  |  | Total 2.3.4 Finish type 4 |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3.5. Other works |  |  |  |  |  |  |  |  |
| 69 |  | CD74A |  | P7. Masonry of exterior and interior walls made of "Porotherm" ceramic blocks with cement-sand mortar M100; floor height up to 4 m. |  |  |  |  | m3 |  | 1,81 |  |
| 70 |  | СF60A |  | Cladding of surfaces in two layers (1 layer OSB III 18 mm + 1 layer gypsum plasterboard 12.5 mm) with identical metal framing, height up to 4 m: walls without insulation. Small materials (water, sandpaper, etc.) = 1.0050 |  |  |  |  | m2 |  | 3,20 |  |
| 71 |  | CK33B |  | Inspection hatch made of steel, 300x300 mm, with anti-corrosion finish under factory conditions. |  |  |  |  | pcs (pieces) |  | 1,00 |  |
| 72 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 6,93 |  |
| 73 |  | RpCJ06C |  | Repair of interior plaster around frames and sills, at doors and windows, 2 cm thick, float finish, executed with cement-lime mortar grade 25 T, with straight jambs, width between 25–35 cm. |  |  |  |  | m |  | 23,10 |  |
| 74 |  | IzF55C |  | External thermal insulation of building walls with fine plaster on insulation (systems with rigid fixation of the insulation), smooth wall surface: with mineral wool boards, thickness 40 mm, density 150/80 kg/m³, thermal conductivity coefficient – 0.0371. Small materials (rags, foam) = 1.0100 |  |  |  |  | m2 |  | 6,93 |  |
| 75 |  | CK07B |  | MDF window sills, width 300 mm, L = 1600 mm, made of wood, installed at windows. |  |  |  |  | m |  | 19,20 |  |
| 76 |  | CK07B |  | MDF window sills, width 300 mm, L = 1950 mm, made of wood, installed at windows. |  |  |  |  | m |  | 3,90 |  |
|  |  |  |  | Total 2.3.5 Other works |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Carpentry |  |  |  |  |  |  |  |  |
| 77 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (700x2200(h) mm). |  |  |  |  | m2 |  | 1,54 |  |
| 78 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (800x2200(h) mm). |  |  |  |  | m2 |  | 3,52 |  |
| 79 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (900x2200(h) mm). |  |  |  |  | m2 |  | 1,98 |  |
| 80 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (1100x2200(h) mm). |  |  |  |  | m2 |  | 7,26 |  |
| 81 |  | CK22B |  | Interior glass partitions made of aluminum profiles, white color, with semi-transparent tempered glass filling, fixed panels mounted at floor level (V1 - 2800x2500(h) mm). |  |  |  |  | m2 |  | 7,00 |  |
| 82 |  | CH06A |  | Protective railing made of stainless steel (including current handrail), mounted on supports with a height of 96 cm, placed at 0.50 m distances and provided with covers for fixing points, fixed to the concrete floor, made of stainless steel pipe D=1 1/2" / D=1 1/4", straight, including fixing elements. |  |  |  |  | m |  | 3,00 |  |
| 83 |  | CK22C |  | Interior glass partitions made of aluminum profiles, white color, with a maximum 20 mm threshold, with semi-transparent tempered glass filling, including fixed panels and door leaf (850x2200(h) mm) (V2 - 2200x2200(h) mm). |  |  |  |  | m2 |  | 4,84 |  |
|  |  |  |  | Total 2.4 Carpentry |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 2. Elevation Finish 0.000 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Elevation Finish 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1 Ceiling |  |  |  |  |  |  |  |  |
| 84 |  | CK29F |  | Suspended ceilings made of prefabricated "Armstrong" panels, including the grid system. Panel dimensions: 600x600x13 mm. RAL 9003. |  |  |  |  | m2 |  | 233,76 |  |
|  |  |  |  | Total 3.1 Ceiling |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Floors |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2.1. Par 1 |  |  |  |  |  |  |  |  |
| 85 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 65,65 |  |
| 86 |  | CG17D |  | Flooring made of porcelain gres tiles RAL 7001, wear resistance class 4, non-slip, 600x600 mm, thickness 9 mm, with joint filling using disinfectant-resistant grout, including adhesive mortar base layer, executed on surfaces larger than 16 m². |  |  |  |  | m2 |  | 65,65 |  |
| 87 |  | CI14B |  | Linear elements made of porcelain gres tiles fixed with adhesive, h = 100 mm. Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 38,34 |  |
|  |  |  |  | Total 3.2.1 Par 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2.2. Par 2 |  |  |  |  |  |  |  |  |
| 88 |  | CG56A |  | Self-leveling screed made of "Nivelir" mixture: thickness 10 mm. Small materials (expanded polystyrene for expansion joints, rags, guides) = 1.0050 |  |  |  |  | m2 |  | 84,95 |  |
| 89 |  | CG56A1 |  | Correction to norm CG56A: subtract for 1 mm thickness: k=5 Labour coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -84,95 |  |
| 90 |  | CG49A |  | Execution of antibacterial covering made of 2 mm thick homogeneous vinyl carpet, class 34-43, on floors including skirting installation. Small materials (rags) = 1.0050 |  |  |  |  | m2 |  | 84,95 |  |
|  |  |  |  | Total 3.2.2 Par 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2.3. Par 3 |  |  |  |  |  |  |  |  |
| 91 |  | CG56A |  | Self-leveling screed made of "Nivelir" mixture: thickness 10 mm. Small materials (expanded polystyrene for expansion joints, rags, guides) = 1.0050 |  |  |  |  | m2 |  | 83,16 |  |
| 92 |  | CG56A1 |  | Correction to norm CG56A: subtract for 1 mm thickness: k=5 Labour coefficient = 5.0000 Material coefficient = 5.0000 Equipment coefficient = 5.0000 |  |  |  |  | m2 |  | -83,16 |  |
| 93 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 83,16 |  |
| 94 |  | CG17D |  | Flooring made of porcelain gres tiles RAL 7001, wear resistance class 4, non-slip, 600x600 mm, thickness 9 mm, with joint filling using disinfectant-resistant grout, including adhesive mortar base layer, executed on surfaces larger than 16 m². |  |  |  |  | m2 |  | 83,16 |  |
| 95 |  | CI14B |  | Linear elements made of porcelain gres tiles fixed with adhesive, h = 100 mm. Small materials (rags, water, etc.) = 1.0500 |  |  |  |  | m |  | 52,84 |  |
|  |  |  |  | Total 3.2.3 Par 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3 Walls |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
| 96 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 316,30 |  |
| 97 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat, and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 316,30 |  |
| 98 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 458,64 |  |
| 99 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 316,30 |  |
| 100 |  | CF17C |  | Layer of fiberglass mat, weight 80 kg/m², applied as part of plaster with dry mix. |  |  |  |  | m2 |  | 316,30 |  |
| 101 |  | CF50B |  | Interior plaster, 5 mm thick, applied manually with gypsum-based dry mix, on walls and partition walls, with manual mortar preparation. |  |  |  |  | m2 |  | 316,30 |  |
| 102 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 316,30 |  |
| 103 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 316,30 |  |
| 104 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 316,30 |  |
| 105 |  | CN06A |  | Fin 1. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 294,50 |  |
| 106 |  | CN06A |  | Fin 1.2. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 13,60 |  |
| 107 |  | CN06A |  | Fin 1.3. Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 8,20 |  |
|  |  |  |  | Total 3.3.1 Fin. 1 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
| 108 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 17,08 |  |
| 109 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat, and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 17,08 |  |
| 110 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 24,77 |  |
| 111 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 17,08 |  |
| 112 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 17,08 |  |
| 113 |  | CI22B |  | Cladding with ceramic tiles (on walls, columns, pilasters, and window sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm. Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 17,08 |  |
|  |  |  |  | Total 3.3.2 Fin. 2 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
| 114 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 20,70 |  |
| 115 |  | CF02B |  | Interior plaster, 2 cm thick, float finish, applied manually on walls or columns, on flat surfaces with cement-lime mortar M 100-T for spritz, base coat, and visible layer, on brick masonry or small concrete blocks. |  |  |  |  | m2 |  | 20,70 |  |
| 116 |  | CC03A |  | Installation of galvanized welded mesh 20x20 mm, wire diameter 1.5 mm on walls (weight 1.45 kg/m²), with mesh weight up to 3 kg/m². |  |  |  |  | kg |  | 20,70 |  |
| 117 |  | CF17B k=2 |  | Various works – thickness difference, +5 mm to the base coat applied on walls, executed with cement mortar M100. Labour coefficient = 2.0000 Material coefficient = 2.0000 Equipment coefficient = 2.0000 |  |  |  |  | m2 |  | 20,70 |  |
| 118 |  | CF17C |  | Layer of fiberglass mat, weight 80 kg/m², applied as part of plaster with dry mix. |  |  |  |  | m2 |  | 20,70 |  |
| 119 |  | CF50B |  | Interior plaster, 5 mm thick, applied manually with gypsum-based dry mix, on walls and partition walls, with manual mortar preparation. |  |  |  |  | m2 |  | 20,70 |  |
| 120 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 20,70 |  |
| 121 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 20,70 |  |
| 122 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 20,70 |  |
| 123 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 20,70 |  |
|  |  |  |  | Total 3.3.3 Fin. 3 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.4 Fin. 4. Fin. 5. |  |  |  |  |  |  |  |  |
| 124 |  | CD72D |  | Partition walls with 100 mm thickness, made of gypsum plasterboard in two layers on both sides, with 50 mm mineral wool insulation, including identical metal frame made of CW50 and UW50 profiles, with height up to 4 m. Small materials (water, rags, etc.) = 1.0050 |  |  |  |  | m2 |  | 10,77 |  |
| 125 |  | CF57A |  | Manual application of gypsum-based filler, 1.0 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 8,40 |  |
| 126 |  | CF56A |  | Manual application of interior putty, 0.5 mm thick, on wall surfaces, columns, and ceilings. |  |  |  |  | m2 |  | 8,40 |  |
| 127 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 8,40 |  |
| 128 |  | CN06A |  | Interior painting with washable paint, applied in 2 layers over existing putty, executed manually, RAL as per "Interior Design" project, with the following paint characteristics: - Coverage power: 2 – high coverage power; - Wet abrasion resistance according to DIN EN 13300: class 2 – abrasion resistant. |  |  |  |  | m2 |  | 8,40 |  |
| 129 |  | CN53A |  | Priming of interior surfaces of walls and ceilings. |  |  |  |  | m2 |  | 8,21 |  |
| 130 |  | CI22B |  | Cladding with ceramic tiles (on walls, columns, pilasters, and window sills) fixed with adhesive (dry mix), tile dimensions: 300x300x9 mm. Small materials (rags, disc) = 1.0100 |  |  |  |  | m2 |  | 8,21 |  |
|  |  |  |  | Total 3.3.4 Fin. 4. Fin. 5 |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3.5 Other works |  |  |  |  |  |  |  |  |
| 131 |  | CD66С |  | P1. Brick masonry, size 250 x 120 x 65 mm, on reinforced partition walls, with a thickness of 1/2 brick, with manual preparation of mortar M-50, floor height up to 4 m. |  |  |  |  | 100m2 |  | 0,173 |  |
| 132 |  | CD74A |  | P7. Exterior and interior masonry walls made of "Porotherm" ceramic blocks with cement and sand mortar M100; floor height up to 4 m. |  |  |  |  | m3 |  | 1,89 |  |
| 133 |  | СF60A |  | Cladding of surfaces in two layers (1 layer OSB III 18 mm + 1 layer gypsum plasterboard 12.5 mm) with identical metal frame, height up to 4 m: walls without insulation. Small materials (water, sandpaper, etc.) = 1.0050 |  |  |  |  | m2 |  | 6,40 |  |
| 134 |  | CK33B |  | Inspection hatch made of steel, 300x300 mm, with anti-corrosion finish under factory conditions. |  |  |  |  | pcs (pieces) |  | 2,00 |  |
| 135 |  | CN53A |  | Priming of interior surfaces with quartz sand-based primer. |  |  |  |  | m2 |  | 5,01 |  |
| 136 |  | RpCJ06C |  | Repair of interior plaster around frames and sills, at doors and windows, 2 cm thick, float finish, executed with cement-lime mortar grade 25 T, with straight jambs, width between 25–35 cm. |  |  |  |  | m |  | 16,70 |  |
| 137 |  | IzF55C |  | External thermal insulation of building walls with fine plaster on insulation (systems with rigid fixation of the insulation), smooth wall surface: with mineral wool boards, thickness 40 mm, density 150/80 kg/m³, thermal conductivity coefficient – 0.0371. Small materials (rags, foam) = 1.0100 |  |  |  |  | m2 |  | 5,01 |  |
| 138 |  | CK07B |  | MDF window sills, width 300 mm, L = 1600 mm, made of wood, installed at windows. |  |  |  |  | m |  | 12,80 |  |
| 139 |  | CK07B |  | MDF window sills, width 300 mm, L = 1950 mm, made of wood, installed at windows. |  |  |  |  | m |  | 3,90 |  |
|  |  |  |  | Total 3.3.5 Other works |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4 Carpentry |  |  |  |  |  |  |  |  |
| 140 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (700x2200(h) mm). |  |  |  |  | m2 |  | 1,54 |  |
| 141 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (800x2200(h) mm). |  |  |  |  | m2 |  | 1,76 |  |
| 142 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (1000x2100(h) mm). |  |  |  |  | m2 |  | 2,10 |  |
| 143 |  | CK03A |  | Interior door with one leaf. The leaf thickness is 40 mm with the following characteristics: core – stabilizing honeycomb; leaf finish – HPL laminate, model – natural ash wood color RAL 1014, coated with matte lacquer; smooth surface. Window leaf – vertical window with dimensions 240x1500 mm and 4+4 mm transparent triplex glass, protected with a 1 mm stainless steel plate at the lower part and a set of stainless steel handle with anodized aluminum frame, laminated glass 3.1.3. Door frame material – Corner steel, 100 mm profile, electrostatically painted RAL 7044; Minimum 3 hinges per leaf, Lock, Threshold with maximum height of 20 mm, Frame (1100x2200(h) mm). |  |  |  |  | m2 |  | 9,68 |  |
| 144 |  | CK22B |  | Interior glass partitions made of aluminum profiles, white color, with semi-transparent tempered glass filling, fixed panels mounted at floor level (V1 - 2800x2500(h) mm). |  |  |  |  | m2 |  | 7,00 |  |
| 145 |  | CH06A |  | Protective railing, made of stainless steel (including the handrail), mounted on supports with a height of 96 cm, placed at 0.50 m distances and provided with covers for fixing points, fixed to the concrete floor, made of stainless steel pipe D=1 1/2" / D=1 1/4", straight, including fixing elements. |  |  |  |  | m |  | 3,00 |  |
| 146 |  | CK22C |  | Interior glass partitions made of aluminum profiles, white color, with a maximum 20 mm threshold, with semi-transparent tempered glass filling, including fixed panels and door leaf (850x2200(h) mm) (V2 - 2200x2200(h) mm). |  |  |  |  | m2 |  | 4,84 |  |
|  |  |  |  | Total 3.4 Carpentry |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.5 Blinds. |  |  |  |  |  |  |  |  |
| 147 |  | CK32B |  | Blinds (jalousies) for windows, venetian blinds made of anodized aluminum, including accessories / Roller blinds made of textile material, fixed on the window profile, color RAL 9016, 50% transparency. |  |  |  |  | m2 |  | 66,00 |  |
|  |  |  |  | Total 3.5 Blinds. |  |  |  |  |  |  |  |  |
|  |  |  |  | Total 3 Elevation Finish 6.600 |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  | lei |  |  |  |  |  |  |
|  |  |  |  | Social insurance |  | 24 % |  |  |  |  |  |  |
|  |  |  |  | Transport expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Supply and storage expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead expenses |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Estimated profit |  | % |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |
|  |  |  |  | Total Including salary |  |  |  |  |  |  |  |  |

Bill of quantities of works

Interior water supply and sewerage networks (WSSN)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | TsA02B | Manual excavation of soil in confined spaces, with a width of less than 1.00 m, executed without support, with vertical slope, for trenches, in non-cohesive or weakly cohesive soil, depth < 0.75 m, medium soil (Trenches 6 m). | m3 | 4,00 |
| 2 | AcF03A | Backfilling in trenches for water supply or sewage pipes, as substrate, protective layer, insulation layer, or filter layer for drainage pipes, executed with sand. | m3 | 0,36 |
| 3 | TsD18B | Compacted backfilling in trenches for buried cables of high-voltage power lines, executed with soil from medium soil. | m3 | 2,00 |
| 4 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 16-25 cm thickness, for mechanical penetration execution. | pcs (pieces) | 8,00 |
| 5 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 26-50 cm thickness, for mechanical penetration execution. | pcs (pieces) | 6,00 |
| 6 | RpEJ09A | Closing of openings in walls with gypsum mortar. | pcs (pieces) | 6,00 |
| 7 | RpEJ09C | Closing of openings in slabs with cement mortar. | pcs (pieces) | 8,00 |
| 8 | SD07A | Ball butterfly valve with socket and Dutch connection FI-FI, 1/2", PN16. Small materials (hemp fiber, lead primer, etc.) = 1.0200 | pcs (pieces) | 5,00 |
| 9 | SA15B | PN16 polypropylene pipe, joined by fusion welding, in connecting pipes, for sanitary fixtures, residential and social-cultural buildings, with a diameter of 20 mm. | m | 45,00 |
| 10 | SA37B | Brace for fixing water and gas supply pipes, made of steel or PVC, installed by embedding, with pipes having a diameter of 3/4". | pcs (pieces) | 13,00 |
| 11 | RpIF09A | Insulation of pipes with special insulation sleeves, made of elastomeric foam based on synthetic rubber, applied to pipes, of the ARMAFLEX AF type, with a diameter and thickness of D=22x7.5 mm. Small materials = 1.0500 | m | 45,00 |
| 12 | SF01C | Performing a pressure leak test on the hot or cold water installation made of heavy-duty polyvinyl chloride (PVC) pipe or plastic material, with a diameter of 16-110 mm. | m | 45,00 |
| 13 | SF02C | Performing the operational test of the cold water installation made with heavy-duty polyvinyl chloride (PVC) pipe or plastic material, with a diameter of 16-110 mm. | 10 m | 4,50 |
| 14 | SF05C | Washing of the cold or hot water installation, made of plastic pipes, with a diameter of 20-75 mm. | m | 45,00 |
| 15 | SB08E | Polypropylene (PP-R) pipe for drainage, joined with a rubber gasket, installed either exposed or buried under the floor, with a diameter of 110 mm. | m | 20,00 |
| 16 | SB08C | Polypropylene (PP-R) pipe for drainage, joined with a rubber gasket, installed either exposed or buried under the floor, with a diameter of 50 mm. | m | 20,00 |
| 17 | SA37F | Brace for fixing water and gas supply pipes, made of steel or PVC, installed by embedding, with pipes having a diameter of 2". | pcs (pieces) | 20,00 |
| 18 | SB09E | 45° elbow, joined with a rubber gasket, with a diameter of 110 mm. | pcs (pieces) | 4,00 |
| 19 | SB09E | Excentric reducer, joined with a rubber gasket, with a diameter of 110x50 mm. | pcs (pieces) | 2,00 |
| 20 | SB09C | 45° elbow, joined with a rubber gasket, with a diameter of 50 mm. | pcs (pieces) | 6,00 |
| 21 | SB10C | Connecting piece (simple branching) made of plastic material for drainage, joined with a rubber gasket, with a diameter of 50 mm, 45°. | pcs (pieces) | 3,00 |
| 22 | SB09C | Cleaning piece made of PP-R with a threaded plug, for drainage, joined with a rubber gasket, with a diameter of 50 mm. | pcs (pieces) | 4,00 |
| 23 | SB09E | Cleaning piece made of PP-R with a threaded plug, for drainage, joined with a rubber gasket, with a diameter of 110 mm. | pcs (pieces) | 2,00 |
| 24 | SA40A | Installation of fireproof couplings D=50 mm, fixed to the ceiling slab with dowels. | pcs (pieces) | 6,00 |
| 25 | SB09C | Ventilation cap made of PP-R, for drainage, joined with a rubber gasket, with a diameter of 50 mm. | pcs (pieces) | 2,00 |
| 26 | SF04A | Performing the tightness and operation test of the drainage system made of cast iron pipes, for drainage, unplasticized polyvinyl chloride (PVC) pipe, lightweight type, or plastic material, ductile cast iron pipe, with a diameter of up to 100 mm, inclusive. | 10 m | 4,00 |
| 27 | SC04C | Washbasin made of semi-porcelain, sanitary porcelain, etc., including for persons with disabilities, with a drainage pipe made of plastic material, mounted on a pedestal. Small materials (sealant, wood dowel, plaster, etc.) = 1.0200 | pcs (pieces) | 3,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 1 Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 28 | SD04A | Mixer faucet with swiveling arm for washbasin, regardless of the closing mechanism, including for persons with disabilities, with a diameter of 1/2". | pcs (pieces) | 3,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 29 | Supplier price | Instant water heater, electric 230V, 13A, 3.0 kW, hot water performance 1.2-1.7 l/min. | set | 3,00 |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Heating, ventilation, and air conditioning (HVAC)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works 1.1 Heating |  |  |
| 1 | RpIB11A | Removal of position radiators for dismantling. | m2 | 76,80 |
| 2 | RpIC19A | Dismantling of black pipes, for installations, welded to the central heating system, in residential and social-cultural buildings, for connecting appliances, distribution columns, etc., with a diameter of 3/8" - 3/4". | m | 32,00 |
| 3 | RpIC19B | Dismantling of black pipes, for installations, welded to the central heating system, in residential and social-cultural buildings, for connecting appliances, distribution columns, etc., with a diameter of 1" - 1 1/4". | m | 90,00 |
| 4 | IC35B | Cross-linked polyethylene pipe for high-pressure applications, type RAUTITAN stabil d=20x2.9 mm, installed for connecting components or appliances in central heating systems (K=1.4 for fittings). Labour coefficient = 1.4000 Material coefficient = 1.4000 Equipment coefficient = 1.4000 | m | 35,00 |
| 5 | IC36D | Cross-linked polyethylene pipe for high-pressure applications, type RAUTITAN stabil d=40x5.5 mm, installed in columns for central heating systems (K=1.4 for fittings). Labour coefficient = 1.4000 Material coefficient = 1.4000 Equipment coefficient = 1.4000 | m | 100,00 |
| 6 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 500x800 mm. | pcs (pieces) | 3,00 |
| 7 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 500x1000 mm. | pcs (pieces) | 1,00 |
| 8 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 500x600 mm. | pcs (pieces) | 3,00 |
| 9 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 500x800 mm. | pcs (pieces) | 1,00 |
| 10 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 500x1000 mm. | pcs (pieces) | 4,00 |
| 11 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of 1001 - 1500 mm, type 33K, 500x1100 mm. | pcs (pieces) | 1,00 |
| 12 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of 1001 - 1500 mm, type 33K, 500x1200 mm. | pcs (pieces) | 6,00 |
| 13 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 600x600 mm. | pcs (pieces) | 1,00 |
| 14 | IB06A | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of up to 1000 mm, type 22K, 600x900 mm. | pcs (pieces) | 3,00 |
| 15 | IB06B | Steel radiators, monoblock with lateral connection, built-in air vent, with fixing elements, with a length of 1001 - 1500 mm, type 33K, 600x1200 mm. | pcs (pieces) | 1,00 |
| 16 | IE01A | Performing the pressure leak test of the central heating system, with steel, copper, polyethylene, or polypropylene pipes, heating bodies, heating appliances, fittings, distributors, collectors, and air vents, with a total heating body area of up to 100 m². | m2 | 67,70 |
| 17 | IE02A | Performing the expansion-contraction and operation test of the central heating system, with steel, copper, polyethylene, or polypropylene pipes, heating bodies, heating appliances, fittings, distributors, collectors, and air vents, with a total heating body area of up to 100 m². | m2 | 67,70 |
| 18 | IE08A | Washing with potable water of the interior central heating system, with a total heating body area of up to 100 m². | m2 | 67,70 |
| 19 | IE03A | Performing the pressure leak test of the supply pipes for heating appliances (fan heaters, convectors, skirting heaters, etc.), with a diameter of 3/8" ... 1". | m | 35,00 |
| 20 | IE03B | Performing the pressure leak test of the supply pipes for heating appliances (fan heaters, convectors, skirting heaters, etc.), with a diameter of 1 1/4" ... 2". | m | 100,00 |
| 21 | IE04A | Performing the expansion-contraction and operation test of the supply pipes for heating appliances (fan heaters, convectors, skirting heaters, etc.), with a diameter of 3/8" ... 1". | m | 35,00 |
| 22 | IE04B | Performing the expansion-contraction and operation test of the supply pipes for heating appliances (fan heaters, convectors, skirting heaters, etc.), with a diameter of 1 1/4" ... 2". | m | 100,00 |
| 23 | ID13A | RTR-G type valve, straight, DN 15. | pcs (pieces) | 24,00 |
| 24 | ID01A | Angle valve with vent, return, with the possibility of system draining, for radiators, with a nominal diameter of 1/2", type RLV. | pcs (pieces) | 24,00 |
| 25 | ID04B | Ball valve, with sockets for central heating installations, with a nominal diameter of 1 1/4". | pcs (pieces) | 8,00 |
|  |  | Total 1.1 Heating |  |  |
|  |  | 1.2 Ventilation |  |  |
| 26 | VA19A | Installation of ventilation ducts class H, at a height from the floor up to 3 m, made of galvanized sheet metal 0.5 mm thick, with a circular section diameter of 100 - 180 mm. Small materials (wiping rags, silicone) = 1.0100 | m2 | 4,21 |
| 27 | Supplier price | Check valve for ducts D=100 mm, type KOM100. | pcs (pieces) | 2,00 |
| 28 | IzH07B | Thermal insulation made of mineral wool with an aluminum foil covering, d=50 mm (fire-resistant EI30). | m2 | 7,67 |
| 29 | VB27A | Protective cap according to series 3K 00.000, for circular ducts with a perimeter of 230 - 700 mm. Small materials = 1.0500 | pcs (pieces) | 4,00 |
| 30 | CL20A | Pre-made ventilation grilles with double regulator, mounted in a door, type MV450R/2. | pcs (pieces) | 6,00 |
| 31 | CL20A | Ventilation grilles for air supply and exhaust, dimensions 200x200 mm. | pcs (pieces) | 1,00 |
| 32 | CL20A | Ventilation grilles for air supply and exhaust, dimensions 100x100 mm. | pcs (pieces) | 1,00 |
| 33 | RCsB29P | Drilling penetration holes in brick and stone masonry using a diamond core drill machine with a diameter of 170 mm, L=400 mm (K=3.6). Labour coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 | pcs (pieces) | 8,00 |
| 34 | RCsB29S | Drilling penetration holes in brick and stone masonry using a diamond core drill machine with a diameter of 220 mm, L=400 mm (K=4.84). Labour coefficient = 4.8400 Material coefficient = 4.8400 Equipment coefficient = 4.8400 | pcs (pieces) | 2,00 |
| 35 | RCsB29K | Drilling penetration holes in brick and stone masonry using a diamond core drill machine with a diameter of 120 mm, L=400 mm (K=3.6). Labour coefficient = 3.6000 Material coefficient = 3.6000 Equipment coefficient = 3.6000 | pcs (pieces) | 2,00 |
|  |  | Total 1.2 Ventilation |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 1 Construction works |  |  |
|  |  | 2. Installation works |  |  |
|  |  | 2.1 Ventilation |  |  |
| 36 | VB17A | Admission valve with thermostat. | pcs (pieces) | 7,00 |
| 37 | CL20A | Admission valve with hygroscopic regulation system and noise absorption, type EHA2 22-55. | pcs (pieces) | 1,00 |
| 38 | VC07A | Installation of the air supply and exhaust system with double flow. Small materials and installation materials (screws, oxygen, carbide, etc.) = 1.0100 | pcs (pieces) | 10,00 |
| 39 | 11-05-001-01 | Execution mechanism, table, kg, up to: 20 Ventilators. | pcs (pieces) | 2,00 |
|  |  | Total 2.1 Ventilation |  |  |
|  |  | 2.2 Conditioning |  |  |
| 40 | 11-04-002.01 | Apparatus weight, t, up to: 0.015 - Air cooler. | pcs (pieces) | 5,00 |
|  |  | Total 2.2 Conditioning |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
|  |  | 3.1 Ventilation |  |  |
| 41 | Supplier price | Admission valve with thermostat, type VTK-160 or similar. | pcs (pieces) | 7,00 |
| 42 | Supplier price | Admission valve with hygroscopic regulation system and noise absorption, type EHA2 22-55 or similar. | pcs (pieces) | 1,00 |
| 43 | Supplier price | Air supply and exhaust system with double flow, with a copper heat exchanger, air preheating function, with the following characteristics: admitted air flow rate 110 m³/h, exhausted air flow rate 105 m³/h, N=4-12 W/h, 230V, pipe diameter - 150 mm, in thermal-insulated casing, with remote control system via PRANA 150C type remote or similar. | pcs (pieces) | 8,00 |
| 44 | Supplier price | Air supply and exhaust system with double flow, with a copper heat exchanger, air preheating function, with the following characteristics: admitted air flow rate 220 m³/h, exhausted air flow rate 200 m³/h, N=4 - 35 W/h, 230V, pipe diameter - 200 mm, in thermal-insulated casing, with remote control system via PRANA 200C type remote or similar. | pcs (pieces) | 2,00 |
| 45 | Supplier price | Ventilator with a productivity of 85 m³/h, P=49Pa, N=0.016 kW, 2300 rpm, 230V, type VENTS 100M or similar. | pcs (pieces) | 2,00 |
|  |  | Total 3.1 Ventilation |  |  |
|  |  | 3.2. Conditioning |  |  |
| 21 | Supplier price | Air cooler, type Trotec PAE 61 or similar. | pcs (pieces) | 5,00 |
|  |  | Total 3.2 Conditioning |  |  |
|  |  | Total | lei |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3 Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Electrical power equipment. Interior electric lighting (EPE/IEL)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 16-25 cm thickness, for mechanical penetration execution. | pcs (pieces) | 6,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 26-50 cm thickness, for mechanical penetration execution. | pcs (pieces) | 11,00 |
| 3 | RpEJ09A | Closing of openings in walls, with gypsum mortar. | pcs (pieces) | 17,00 |
| 4 | RpEJ08C | Execution of trenches with a depth of up to 5 cm in stone or reinforced concrete walls, with dimensions of 5x50 cm. | m | 50,00 |
| 5 | RpEJ09D | Closing of trenches in walls up to 50 cm². | m | 50,00 |
| 6 | RCsB29G | Drilling penetration holes in brick and stone masonry using a diamond core drill machine with a diameter of 80 mm (K=0.5). Labour coefficient = 0.5000 Material coefficient = 0.5000 Equipment coefficient = 0.5000 | pcs (pieces) | 121,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 7 | RpEG17D | Dismantling of electrical panels: metal from panel, cabinet, cell, control desk. | pcs (pieces) | 1,00 |
| 8 | RpEF23A | Dismantling of light fixtures of any type, including rods and globes. | pcs (pieces) | 38,00 |
| 9 | RpEE24A | Dismantling of electrical devices for high current: single-pole (switches, circuit breakers, outlets for removal or replacement, remote control button). | pcs (pieces) | 35,00 |
| 10 | RpED21A | Dismantling of electrical cables made of copper or aluminum, with PVC insulation or sheath, with increased flame propagation resistance, having a cross-section of 1-2-3x1.5 to 1-2-3x10 mm². | m | 190,00 |
| 11 | 08-03-572-3 | Execution control block, type cabinet or distribution point (cabinet), mounted on the wall, height and width, mm, up to 600x600. | pcs (pieces) | 2,00 |
| 12 | 08-03-575-1 | Device - automatic circuit breakers, load separators. | pcs (pieces) | 41,00 |
| 13 | 08-03-530-4 | Magnetic starter for general purpose, separate, mounted on the construction on the wall or column, with current up to 40 A. | pcs (pieces) | 2,00 |
| 14 | 08-02-407-1 | Steel pipe on structures installed on walls, fixed with nails, with a diameter of up to 25 mm. | 100 m | 16,25 |
| 15 | Supplier price | Rigid PVC pipe for electrical installation, d=25 mm. | m.l. | 1 625,00 |
| 16 | 08-02-397-1 | Perforated mounting profile, length 2 m. | 100 pcs (pieces) | 0,05 |
| 17 | Supplier price | K108 type perforated profile, L=2.0 m. | pcs (pieces) | 5,00 |
| 18 | 08-02-396-6 | Metal channel on walls and ceilings, length 3 m. | 100 m | 0,90 |
| 19 | Supplier price | Perforated galvanized metal conduit, section 50x50 mm. | m.l. | 90,00 |
| 20 | Supplier price | 90° elbow for galvanized metal conduit 50x50 mm. | pcs (pieces) | 10,00 |
| 21 | Supplier price | Wall bracket for fixing the conduit, B=60 mm. | pcs (pieces) | 30,00 |
| 22 | 10-08-01901 | Junction box on the wall. | pcs (pieces) | 90,00 |
| 23 | Supplier price | Distribution box made of corrosion-protected metal, IP54, 171x171x101 mm, type У995. | pcs (pieces) | 20,00 |
| 24 | Supplier price | Distribution box made of carbolite, IP44, D=102 mm, H=44 mm, type KOP-74. | pcs (pieces) | 50,00 |
| 25 | Supplier price | Distribution box У-192. | pcs (pieces) | 20,00 |
| 26 | 08-02-412-2 | Introducing conductors into metal pipes and hoses laid: the first conductor, single-core or multi-core, in common braid, total cross-section up to 6 mm². | 100 m | 3,70 |
| 27 | 08-02-412-3 | Introducing conductors into metal pipes and hoses laid: the first conductor, single-core or multi-core, in common braid, total cross-section up to 16 mm². | 100 m | 12,55 |
| 28 | 08-02-398-1 | Conductor in conduits, cross-section up to 6 mm². | 100 m | 0,05 |
| 29 | 08-02-398-2 | Conductor in conduits, cross-section up to 35 mm². | 100 m | 0,45 |
| 30 | 08-02-398-3 | Conductor in conduits, cross-section up to 70 mm². | 100 m | 0,50 |
| 31 | 08-02-398-4 | Conductor in conduits, cross-section up to 120 mm². | 100 m | 0,05 |
| 32 | Supplier price | Cable ВВГнг-LS-0.66 3x1.5 mm². | m.l. | 255,00 |
| 33 | Supplier price | Cable ВВГнг-LS-0.66 3x2.5 mm². | m.l. | 1 220,00 |
| 34 | Supplier price | Cable ВВГнг-LS-0.66 x2.5 mm². | m.l. | 80,00 |
| 35 | Supplier price | Cable ВВГнг-LS-0.66 5x10 mm². | m.l. | 50,00 |
| 36 | Supplier price | Cable ВВГнг-LS-0.66 5x16 mm². | m.l. | 5,00 |
| 37 | Supplier price | Cable ВВГнг(A)-FRLS 3x1.5 mm². | m.l. | 120,00 |
| 38 | 08-03-59310 | Wall-mounted lighting indicators. | 100 pcs (pieces) | 0,04 |
| 39 | 08-03-594-1 | Lighting fixture with LED modules. | 100 pcs (pieces) | 0,87 |
| 40 | 08-03-591-2 | Single-pole switch, recessed type, for closed installation, including mounting box for the device. | 100 pcs (pieces) | 0,15 |
| 41 | Supplier price | Switch for closed installation with grounding connection, 1 rocker, IP20, 10A, 230V. | pcs (pieces) | 15,00 |
| 42 | 08-03-591-5 | Double-pole switch, recessed type, for closed installation, including mounting box for the device. | 100 pcs (pieces) | 0,14 |
| 43 | Supplier price | Switch for closed installation with grounding connection, 2 rockers, IP20, 10A, 230V. | pcs (pieces) | 14,00 |
| 44 | 08-03-591-7 | Recessed type commutator, for closed installation, including mounting box for the device. | 100 pcs (pieces) | 0,02 |
| 45 | Supplier price | Commutator for closed installation with grounding connection, 1 rocker, IP20, 10A, 230V. | pcs (pieces) | 2,00 |
| 46 | 10-08-01901 | Junction box on the floor. | pcs (pieces) | 22,00 |
| 47 | 08-03-591-9 | Recessed type socket, for closed installation, including mounting box for the device. | 100 pcs (pieces) | 1,50 |
| 48 | Supplier price | Socket for closed installation with grounding connection, IP20, 16A, 230V. | pcs (pieces) | 150,00 |
| 49 | Supplier price | Horizontal frame 2M. | set | 53,00 |
| 50 | Supplier price | Horizontal frame 3M. | set | 10,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 51 | Supplier price | Metal distribution panel, mounted exposed on the wall, with a capacity of 72 modules, IP41, equipped with a lock and DIN, N, and PE rails. | pcs (pieces) | 2,00 |
| 52 | Supplier price | Modular load separator, Un=400V, 3P, In=40A. | pcs (pieces) | 2,00 |
| 53 | Supplier price | Single-pole automatic circuit breaker, characteristic B, In=10A, U=230V. | pcs (pieces) | 12,00 |
| 54 | Supplier price | Differential circuit breaker, bipolar, with "AC" characteristic, In=20A, 30mA, Un=230V. | pcs (pieces) | 25,00 |
| 55 | Supplier price | Three-pole automatic circuit breaker, characteristic C, In=63A, Un=400V. | pcs (pieces) | 2,00 |
| 56 | Supplier price | Electromagnetic starter 230V, 9A, in IP65 box, set with thermal relay and START/STOP buttons. | pcs (pieces) | 2,00 |
| 57 | Supplier price | Lighting fixture for general interior lighting 595 x 595 mm, IP41, for installation in suspended ceiling, LED module 32W, opal reflector, luminous flux 3600 lm, 4000K, supply voltage 230V. | pcs (pieces) | 84,00 |
| 58 | Supplier price | Emergency lighting fixture with orientation inscription "Exit", permanent type, IP20, for surface mounting on ceiling or wall, LED 3W, luminous flux 40Lm, supply voltage 230V, operating autonomy of 3 hours. | pcs (pieces) | 4,00 |
| 59 | Supplier price | Lighting fixture for illuminating school boards with wall distance support, dimensions 1445x630x250 mm, with 48 LED modules 20 W, 2150 lm, IP41, type LE-SBO-14-020-7005-20D or similar. | pcs (pieces) | 4,00 |
| 60 | Supplier price | Floor box with frame for mounting a box to install 2 sockets. | set | 22,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Fire signaling and security (FS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 16-25 cm thickness, for mechanical penetration execution. | pcs (pieces) | 2,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 26-50 cm thickness, for mechanical penetration execution. | pcs (pieces) | 7,00 |
| 3 | RpEJ09A | Closing of openings in walls, with gypsum mortar. | pcs (pieces) | 9,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 4 | 10-08-001-04 | Receiver devices: "SI" device for: 4 beams. | pcs (pieces) | 1,00 |
| 5 | 10-08-001-07 | Installation of the communicator module. | 10 pcs (pieces) | 0,10 |
| 6 | 08-03-575-1 | Device - Radio transmitter. | pcs (pieces) | 1,00 |
| 7 | 10-08-003-03 | Power and control block. | pcs (pieces) | 1,00 |
| 8 | 08-03-575-1 | Device - Accumulator. | pcs (pieces) | 2,00 |
| 9 | 08-03-593-10 | Optical and acoustic signalizer. | 100 pcs (pieces) | 0,03 |
| 10 | 10-08-002-02 | Automatic "SI" alarm: smoke. | pcs (pieces) | 20,00 |
| 11 | 08-03-591-3 | Manual fire alarm trigger. | 100 pcs (pieces) | 0,03 |
| 12 | 08-03-593-10 | Illuminated indication panel "EXIT". | 100 pcs (pieces) | 0,04 |
| 13 | 10-08-019-01 | Miscellaneous equipment - Optical signaling indicator. | pcs (pieces) | 4,00 |
| 14 | 08-02-409-2 | Vinyl plastic pipe on structures installed on walls and columns, fixed with nails, diameter up to 50 mm. | 100 m | 0,15 |
| 15 | Supplier price | Rigid PVC pipe for electrical installation, d=50 mm. | m.l. | 15,00 |
| 16 | 08-02-409-1 | Vinyl plastic pipe on structures installed on walls and columns, fixed with nails, diameter up to 25 mm. | 100 m | 2,30 |
| 17 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm. | m.l. | 230,00 |
| 18 | 08-02-390-1 | Plastic mass ducts with cover, section 22x10 mm, including all necessary accessories (elbows, plugs, tees, crosses, etc). | 100 m | 0,50 |
| 19 | 08-02-412-2 | Introducing conductors into metal pipes and hoses laid: the first conductor, single-core or multi-core in common braid, total cross-section up to 6 mm². | 100 m | 3,15 |
| 20 | Supplier price | Fire alarm cable, with 2 conductors, cross-section S=0.8 mm², with a fire resistance limit of at least 30 minutes, FE180 E30 - JE-H(St)H-1x2x0.8 mm². | m.l. | 250,00 |
| 21 | Supplier price | Cable ВВГнг(A)-FRLS 3x1.5 mm². | m.l. | 65,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 22 | Supplier price | Fire panel - fire detection control panel with 4 security zones, with output for siren and radio transmitter, type PREVIDIA-MSG or similar, in a red metal cabinet with a glass door and key. | pcs (pieces) | 1,00 |
| 23 | Supplier price | Communicator module, type PREVIDIA-C-COM LAN or similar. | pcs (pieces) | 1,00 |
| 24 | Supplier price | Radio transmitter, type ATS-100 or similar. | pcs (pieces) | 1,00 |
| 25 | Supplier price | Backup power supply with switching, 12 V, 10 A. | pcs (pieces) | 1,00 |
| 26 | Supplier price | Accumulator 12V, 17A/h. | pcs (pieces) | 2,00 |
| 27 | Supplier price | Non-addressable light and sound siren, 18-30 V, 86-101 dB, 10-40mA, type IS2010RE or similar. | pcs (pieces) | 2,00 |
| 28 | Supplier price | Outdoor light and sound siren, uncovered, 12V, 86-103 dB, type Ivy-R or similar. | pcs (pieces) | 1,00 |
| 29 | Supplier price | Smoke detector, 0.09/40 mA, type ID100 or similar, with socket for detectors and additional box for detector installation. | pcs (pieces) | 20,00 |
| 30 | Supplier price | Manual fire alarm trigger, non-addressable, with a resettable element secured with a key, type IC0020 or similar. | pcs (pieces) | 3,00 |
| 31 | Supplier price | Light and sound indicator, non-addressable, "EXIT" with sound signal 92dB, 18-30 V, current consumption 50mA. | set | 4,00 |
| 32 | Supplier price | Optical signaling indicator, type RI31. | pcs (pieces) | 4,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Automatic security signaling (ASS)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 16-25 cm thickness, for mechanical penetration execution. | pcs (pieces) | 2,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 26-50 cm thickness, for mechanical penetration execution. | pcs (pieces) | 4,00 |
| 3 | RpEJ09A | Closing of openings in walls, with gypsum mortar. | pcs (pieces) | 6,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 4 | 10-04-067-23 | Video control device - video camera. | pcs (pieces) | 6,00 |
| 5 | 10-08-002-06 | Structure for video camera installation. | pcs (pieces) | 6,00 |
| 6 | 10-08-019-01 | Video camera connection box. | pcs (pieces) | 6,00 |
| 7 | Supplier price | Box 100x100x70 mm IK08 for video camera connection. | pcs (pieces) | 6,00 |
| 8 | 10-03-013-05 aplicat | Access switch. | pcs (pieces) | 1,00 |
| 9 | 10-04-030-01 | Power socket block. | pcs (pieces) | 1,00 |
| 10 | 10-04-030-01 | UPS | pcs (pieces) | 1,00 |
| 11 | 10-04-077-01 | Central equipment: Video recorder. | pcs (pieces) | 1,00 |
| 12 | 10-04-101-10 | Subscriber equipment and miscellaneous equipment: 3.5" HDD. | pcs (pieces) | 1,00 |
| 13 | 10-04-100-07 | Table apparatus: System block + Monitor. | pcs (pieces) | 1,00 |
| 14 | 08-02-409-1 | Vinyl plastic pipe on installed structures, on walls and columns, fixed with nails, diameter up to 25 mm. | 100 m | 3,00 |
| 15 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm. | m.l. | 300,00 |
| 16 | 08-02-390-1 | Plastic mass ducts with cover, section 15x10 mm, including all necessary accessories (elbows, plugs, tees, crosses, etc). | 100 m | 1,10 |
| 17 | 08-02-411-1 | Metallic hose, outer diameter up to 48 mm. | 100 m | 0,20 |
| 18 | Supplier price | Flexible metal tube with PVC insulation, D=25 mm. | m.l. | 20,00 |
| 19 | 08-02-412-1 | Introducing conductors into laid metal pipes and hoses: the first conductor, single-core or multi-core in common braid, total cross-section up to 2.5 mm². | 100 m | 4,10 |
| 20 | Supplier price | UTP Cable Cat.5E 4P CU Solid. | m.l. | 410,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 21 | Supplier price | IP Video Camera 4Mp, type Uniview IPC354SR3-ADNPF28-F, with the following features: IR30m; fixed 2.8 mm lens; 1/3" CMOS; resolution: 2688 x 1520, 30fps; viewing angle: 95.8°; WDR 120dB; Ultra 265, H.265, H.264, MJPEG; smart functions - motion detection, people detection, intrusion detection, audio detection; audio: G.711, supports two-way audio, built-in microphone; power: 12V DC, PoE; with Micro SD - 256 GB; POE, IP67 or similar, with camera mount support. | pcs (pieces) | 6,00 |
| 22 | Supplier price | POE Switch ZKTeco PE082-120-C, 8xPOE, 2xLAN Uplink, max. 120W or similar. | pcs (pieces) | 1,00 |
| 23 | Supplier price | 4-socket block 2P+E, 16A, 200-240V, 50/60Hz with safety button. | pcs (pieces) | 1,00 |
| 24 | Supplier price | UPS 1500VA (3 steps of AVR, CPU controlled, USB), metal housing, LCD display. | pcs (pieces) | 1,00 |
| 25 | Supplier price | Video recorder, 16-ch IP, minimum 1xSATA interfaces with video and audio signal input/output. | pcs (pieces) | 1,00 |
| 26 | Supplier price | 3.5" HDD SATA3, 10TB, 7200rpm, 256MB cache. | pcs (pieces) | 1,00 |
| 27 | Supplier price | System block - Computer with Core i7-13700 processor / GeForce RTX 3060 video card / 16GB RAM / Internal storage - 2TB SSD / Windows 11 operating system / Mouse / Keyboard. | pcs (pieces) | 1,00 |
| 28 | Supplier price | 28" VA Monitor, Screen resolution 2560x1440, 4 ms, 75 Hz, CR4000:1, HDMI+DP, Pivot. | pcs (pieces) | 1,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | 4. Adjustment and commissioning |  |  |
| 29 | 10-06-068-15 | Adjustment of synchronous digital transmission systems. Adjustment of the control and management system: Configuration and adjustment of network components. | pcs (pieces) | 1,00 |
|  |  | Total 4. Adjustment and commissioning |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total Including salary |  |  |

Bill of quantities of works

Telephone and signaling communications (TSC)

| № crt. | Standard symbol and Resource code. | Works and expenses. | U.M. | Quantity according to project data. |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | 1. Construction works |  |  |
| 1 | RpCU05F1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 16-25 cm thickness, for mechanical penetration execution. | pcs (pieces) | 2,00 |
| 2 | RpCU05I1 | Execution of penetrations for pipes or tie rods in stone or reinforced concrete walls of 26-50 cm thickness, for mechanical penetration execution. | pcs (pieces) | 5,00 |
| 3 | RpEJ09A | Closing of openings in walls, with gypsum mortar. | pcs (pieces) | 7,00 |
| 4 | RpEJ08C | Execution of trenches with a depth of up to 5 cm in stone or reinforced concrete walls, 5x50 cm. | m | 12,00 |
| 5 | RpEJ09D | Closing of trenches in walls up to 50 cm². | m | 12,00 |
|  |  | Total 1. Construction works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total 1. Construction works |  |  |
|  |  | 2. Installation works |  |  |
| 6 | 10-06-037-05 | Wall-mounted cabinet, size 19" 12U. | pcs (pieces) | 1,00 |
| 7 | 10-04-030-01 | Ventilation module with thermostat 19", 1U. | pcs (pieces) | 1,00 |
| 8 | 10-03-01305 aplicat | Access / distribution switch. | pcs (pieces) | 2,00 |
| 9 | 10-04-030-01 | ODF Panel 19", 1U. | pcs (pieces) | 1,00 |
| 10 | 10-04-030-01 | Modular patch panel Cat. 6e fully equipped with 24xRJ45 modules, 19", 1U. | pcs (pieces) | 1,00 |
| 11 | 10-04-030-01 | Cable management 1U. | pcs (pieces) | 1,00 |
| 12 | 10-04-030-01 | Power socket block. | pcs (pieces) | 1,00 |
| 13 | 10-04-030-01 | UPS | pcs (pieces) | 1,00 |
| 14 | 10-04-066-07 | Wall-mounted device: RJ-45 socket. | pcs (pieces) | 4,00 |
| 15 | Supplier price | Socket with 2 RJ45 Cat. 6e modules - frame, mounting box, and support, wall installation. | pcs (pieces) | 4,00 |
| 16 | 10-02-051-03 | Cable connection bridges between sections of a rack: with placement in cable duct - Patch Cord UTP. | 100 pcs (pieces) | 0,08 |
| 17 | Supplier price | Patch Cord cable 0.5 m, copper, UTP Cat. 6e. | pcs (pieces) | 4,00 |
| 18 | Supplier price | Patch Cord cable 1.5 m, copper, UTP Cat. 6e. | pcs (pieces) | 4,00 |
| 19 | 08-02-409-1 | Vinyl plastic pipe on installed structures, on walls and columns, fixed with nails, diameter up to 25 mm. | 100 m | 4,00 |
| 20 | Supplier price | Corrugated U-PVC pipe, 750N, d=20 mm. | m.l. | 400,00 |
| 21 | 08-02-409-2 | Vinyl plastic pipe on installed structures, on walls and columns, fixed with nails, diameter up to 50 mm. | 100 m | 0,10 |
| 22 | Supplier price | Rigid PVC pipe for electrical installation, d=50 mm. | m.l. | 10,00 |
| 23 | 08-02-390-1 | Plastic mass ducts with cover, section 25x16 mm, including all necessary accessories (elbows, plugs, tees, crosses, etc). | 100 m | 0,80 |
| 24 | 08-02-396-6 | Metal duct on walls and ceilings, length 3 m. | 100 m | 0,60 |
| 25 | Supplier price | Metal cable duct, section 50(h)x80 mm with cover. | m.l. | 60,00 |
| 26 | 08-02-412-1 | Introducing conductors into laid metal pipes and hoses: the first conductor, single-core or multi-core in common braid, total cross-section up to 2.5 mm². | 100 m | 4,50 |
| 27 | Supplier price | UTP Cable Cat.6E 4P CU Solid. | m.l. | 450,00 |
|  |  | Total 2 Installation works |  |  |
|  |  | Total | lei |  |
|  |  | Social insurance | 24 % |  |
|  |  | Transport expenses | % |  |
|  |  | Supply and storage expenses | % |  |
|  |  | Total |  |  |
|  |  | Overhead expenses | % |  |
|  |  | Total |  |  |
|  |  | Estimated profit | % |  |
|  |  | Total |  |  |
|  |  | Total 2 Installation works |  |  |
|  |  | 3. Equipment |  |  |
| 28 | Supplier price | Wall-mounted rack 9U, 600x600 mm, 19" black, unassembled (includes feet, wheels), with fans for hot air extraction produced by equipment and thermostat, with perforated front/back doors, tool-less mounting. | pcs (pieces) | 1,00 |
| 29 | Supplier price | Access switch, with 24 RJ-45 POE+ auto-negotiating 1 Gbps + minimum 2 SFP+ 10 Gbps. | pcs (pieces) | 1,00 |
| 30 | Supplier price | Distribution switch, model CRS310-1G-5S-4S+IN or similar. | pcs (pieces) | 1,00 |
| 31 | Supplier price | ODF Patch Panel 19", 1U. | pcs (pieces) | 1,00 |
| 32 | Supplier price | Modular patch panel Cat. 6e fully equipped with 24xRJ45 modules, 19", 1U. | pcs (pieces) | 1,00 |
| 33 | Supplier price | Cable management 1U (Cable organizer), horizontal. | pcs (pieces) | 1,00 |
| 34 | Supplier price | Power strip with minimum 8 2P+E sockets, 16A, 200-240V, 50/60Hz with safety button, 1U. | pcs (pieces) | 1,00 |
| 35 | Supplier price | UPS Rack Mounting 2U SRV2000VA (SRV2KRI+SRVRK1), 2000VA/1600W, Double-conversion On-line UPS, 4 x IEC Sockets (4 IEC C13, all 4 Battery Backup + Surge Protected), Multifunction LCD Display, DB-9 RS-232, USB, with Rail Kit. | pcs (pieces) | 1,00 |
| 36 | Supplier price | Wall clock with independent power source | set | 2,00 |
|  |  | Total 3. Equipment |  |  |
|  |  | Total |  |  |
|  |  | Supply and storage expenses | 1,2 % |  |
|  |  | Total 3. Equipment |  |  |
|  |  | Total Including salary |  |  |