## ANNEX 1: SCHEDULE OF REQUIREMENTS

To comply with national e-Government standards and ensure high performance, security, and maintainability, here are the additional functionalities for the Servers:

| Server Manufacturer's warranty: 5 years. Quantity: 1 PCS |  |
| --- | --- |
| CPUs | 2 processors, each with 8 cores, each with 22.5MB cache, Base frequency 2.6GHz; each with 16 threads |
| RAM | 4x32GB DDR5 (5600 MT/s), Minimum supported slots 1.0 TB speeds up to 5600 MT/s. |
| Boot Device | 2 x 480GB self-encrypting (SED) NVMe drives in a hardware RAID1 configuration, supporting hot-swap installation and removal without opening the chassis or shutting down the server. The controller must not use PCIe slots reserved for expansion. |
| Storage Controller | RAID controller, RAID levels 0,1,5,6,10,50,60 Non-volatile cache memory installed at the controller level with a minimum of 8 GB. |
| Storage | 4x 1.92TB SSD SAS, Read Intensive, up to 24Gbps 512e |
| Drive bays | 10 bays 2.5” for data, Support Hot-Swap drives. 2 bays for OS boot, RAID 1, Support Hot-Swap drives. |
| Expansions Slots | For the installation of IO modules, the equipment will have PCI Express slots: minim 3 Gen4 slots extension, 1x16 LP Slots, 2x8 LP Slot (for additional NICs, HBAs, GPUs etc.) |
| Network | 4 ports X 1GbE BASE-T |
| Ports | 2 x USB ports. 1 x VGA port. |
| Power Supply | Redundant PSU support. |
| Supported operating system | Microsoft Windows Server. Microsoft Hyper-V. Red Hat Enterprise Linux. VMware (ESXi, vSphere, vSAN 7.x-8.x). |
| Management | Dedicated 1 GbE management port A web-based solution for KVM must be included with full functionality for manage and monitoring, including at least: - View information about the state of the managed server; - Inventory and monitoring of network adapters and data storage subsystems without software agents in the OS; - View inventory information; - View information from sensors; - Monitoring and control of electricity consumption; - Turn on/off the server; - Remote update of BIOS, firmware of network and RAID controllers; - Virtual console, virtual media devices; - OS installation from virtual media devices and through network shared folders. Centralized resource management solution for hardware management must be included, with full functionality that provides agent-free management, automates discovery, inventory, tracking, updates, monitoring, and provisioning". |
| Form Factor | Compatible with standard 19” racks. Maximum server height 1U. Kit with rack mounting/installation elements. Locator LED for identifying its position in the rack. Front panel with a locking mechanism to secure access to drives, and LCD or LEDs to indicate the system’s operating status. |
| Warranty | Global Warranty: 5 Years, Official Manufacturer Service (On-site, Next Business Day, Keep your Drive). Hardware replacement included min. 5 years. Hardware and software support from vendor, with available customer portal. |
| Note | All requirements are minimum and mandatory. One requirement must not limit another requirement. All components must be current and not advertised as EOS / EOL. The expansion of memory (RAM) and storage capacity must not include any hardware or software limitations. Faulty storage drives are not the subject of return in order to be compliant with the security requirements. |
| Power conditions | Frequency AC. 50Hz, Nominal voltage 220V, type of electric plug: type F “Schuko” plug and socket with side grounding contacts. |

Note: The technical requirements specified above must be strictly adhered to submissions must fully comply with the requested parameters. Any offer that does not meet these criteria will not be considered.

Delivery Requirements

| Delivery Requirements |  |
| --- | --- |
| Delivery date and time | ☒ Bidder shall deliver the goods to the addresses indicated below not later than 60 calendar days after the contract signature. |
| Delivery Terms (INCOTERMS 2020) | ☒ DDP – Chisinau, Republic of Moldova |
| Customs clearance (must be linked to INCOTERM | ☒ Supplier/Bidder UNDP shall provide a Tax Exemption letter for Customs clearance if necessary. |
| Exact Address(es) of Delivery Location(s) | Ministry of Labor and Social Protection, 1 Vasile Alecsandri street, Chișinău, Republic of Moldova, MD-2009 |
| Warranty Period | ☒ Global Warranty: 5 Years, Official Manufacturer Service (On-site, Next Business Day, Keep your Drive). Hardware replacement included min. 5 years. Hardware and software support from vendor, with available customer portal. |
| After-sales service and local service support requirements | ☒ Brand new replacement (in the shortest period possible) if Purchased Unit is beyond repair (within warranty period) |