| Major renovation and modernisation of the "Mihai Eminescu" Public Institution, Telenesti District, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 2-1-1

Study block. SAC

Prepared at current prices

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Demolition |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Level 0.000 |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCJ35A |  | Removal of rough-cast exterior plaster from walls (sheet 5) |  |  | m² |  | 624.0500 |  |  |  |  |  |
| 2 |  | RpCJ35A |  | Removal of roughcast exterior plaster from ceilings (sheet 5) |  |  | m2 |  | 421.4000 |  |  |  |  |  |
| 3 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (sheet 5 ) |  |  | m2 |  | 35.2300 |  |  |  |  |  |
| 4 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (sheet 5 ) |  |  | m2 |  | 40.5000 |  |  |  |  |  |
| 5 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks (sheet 5 ) |  |  | m³ |  | 7.1400 |  |  |  |  |  |
| 6 |  | RpCK42C |  | Removal of cold floors made of concrete slabs, marble, stone, stoneware, ceramic tiles, etc. (sheet 5) |  |  | m2 |  | 385.2400 |  |  |  |  |  |
| 7 |  | RpCP44B |  | Dismantling of metal structures without material recovery (demolition of the balustrade – 11.08 m, sheet 31) |  |  | kg |  | 403.7000 |  |  |  |  |  |
| 8 |  | RpCB18F |  | Demolition of old concrete using mechanical means, plain concrete (dismantling of the canopy) (sheet 31) |  |  | m³ |  | 11.0800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Share 0.000 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Share + 2,800 |  |  |  |  |  |  |  |  |  |  |
| 9 |  | RpCJ35A |  | Removal of rough-cast exterior plaster from walls (sheet 9 ) |  |  | m² |  | 699.0600 |  |  |  |  |  |
| 10 |  | RpCJ35A |  | Removal of roughcast exterior plaster from ceilings (sheet 9 ) |  |  | m2 |  | 394.8000 |  |  |  |  |  |
| 11 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (sheet 9) |  |  | m² |  | 28.8600 |  |  |  |  |  |
| 12 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (sheet 9 ) |  |  | m2 |  | 45.4500 |  |  |  |  |  |
| 13 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks (sheet 9 ) |  |  | m3 |  | 5.3900 |  |  |  |  |  |
| 14 |  | RpCJ35A |  | Removal of rough-cast external plaster from walls or ceilings (sheet 31) |  |  | m2 |  | 414.6300 |  |  |  |  |  |
| 15 |  | RpCM33B |  | Removal of marble, stone, granite or similar cladding (plinth) (sheet 31 ) |  |  | m2 |  | 42.9300 |  |  |  |  |  |
| 16 |  | RpCI42A |  | Removal of roof elements – gutters ( ( sheet 31 ) |  |  | m |  | 64.6000 |  |  |  |  |  |
| 17 |  | RpCI42A |  | Removal of roof elements - downpipes ( sheet 31 ) |  |  | m |  | 54.1400 |  |  |  |  |  |
| 18 |  | RpCI42B |  | Removal of roof elements – sheet metal, asbestos cement, PVC, cardboard, felt, reed, etc., including cutting of recoverable sheet metal (sheet 31) |  |  | m² |  | 600.0000 |  |  |  |  |  |
| 19 |  | RpCH32B |  | Dismantling of wooden decking and roof elements – roof truss with dismantling of all component parts, including sorting of resulting materials (sheet 31) |  |  | m2 |  | 493.5700 |  |  |  |  |  |
| 20 |  | RpCP45D |  | Dismantling of railings, grilles, parapets and metal fencing, etc. (sheet 31) |  |  | kg |  | 110.8000 |  |  |  |  |  |
| 21 |  | RpCB18G |  | Demolition of old concrete using mechanical means, reinforced concrete (dismantling of stairs and ramp) (sheet 31) |  |  | m³ |  | 6.0400 |  |  |  |  |  |
| 22 |  | RpCB18G |  | Demolition of old concrete using mechanical means, reinforced concrete (dismantling of the floor slab) (sheet 31) |  |  | m3 |  | 2.6100 |  |  |  |  |  |
| 23 |  | RpCK41A |  | Removal of flooring from wooden floorboards, cupboards, etc. (sheet 9) |  |  | m2 |  | 387.5700 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Share + 2,800 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Waste disposal |  |  |  |  |  |  |  |  |  |  |
| 24 |  | RpCU09C |  | Manual transport by carrying directly over a distance of 60 m with a load of up to 50 kg (for every 1 m carried vertically, 10 m horizontally is considered) |  |  | t |  | 313.2700 |  |  |  |  |  |
| 25 |  | TrI1AA01C2 |  | Loading of Group A materials – heavy and small items by throwing – from a ramp or ground into a Category 2 vehicle (construction waste) |  |  | t |  | 313.2700 |  |  |  |  |  |
| 26 |  | TsI51B5 |  | Transport of construction waste by 10-tonne tipper lorry over a distance of: 15 km |  |  | t |  | 313.2700 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total waste disposal Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Demolition Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Reinforcement of the gable (sheet 77) |  |  |  |  |  |  |  |  |  |  |
| 27 |  | RpCB04D |  | Reinforced concrete cast in columns, beams, slabs, landings, stairs, retaining walls, reinforced concrete bearings, floor slabs, diaphragms, stair ramps, etc., for existing buildings, prepared on-site using a concrete mixer and cast using conventional methods with ready-mixed concrete supplied from batching plants (reinforced concrete class C-25/20) |  |  | m³ |  | 10.2000 |  |  |  |  |  |
| 28 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, with 15 mm plywood |  |  | m2 |  | 51.0000 |  |  |  |  |  |
| 29 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with a diameter of up to 8 mm inclusive, cut to size in site workshops and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diam. 6 A240) |  |  | kg |  | 148.0000 |  |  |  |  |  |
| 30 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 12 B500B) |  |  | kg |  | 266.0000 |  |  |  |  |  |
| 31 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 16 B500B) |  |  | kg |  | 372.0000 |  |  |  |  |  |
| 32 |  | CD71C |  | Construction of external and internal walls from lightweight concrete blocks using adhesive without cladding, including filling of voids and joints: for floor heights up to 4 m (installation of the gable) |  |  | m³ |  | 9.4600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Reinforcement of the gable (sheet 77) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Walls |  |  |  |  |  |  |  |  |  |  |
| 33 |  | CD60С |  | Brickwork, 250 x 120 x 65 mm bricks with M - 50 under site conditions for reinforced partition walls with a thickness of 1/2 brick, height up to 4 m (level 0.000 - 15.06 m³/0.12 m = 125.5 m²) (sheet 6) |  |  | 100 m² |  | 1.2500 |  |  |  |  |  |
| 34 |  | CD60С |  | Brickwork, 250 x 120 x 65 mm bricks with M mortar - 50 on site for reinforced partition walls with a thickness of 1/2 brick, height up to 4 m (elevation +2.800 – 9.81 m³/0.12 m = 81.75 m²) (sheet 10) |  |  | 100 m² |  | 0.8200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Walls Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Thermal insulation of wall P-2 (walls below floor S=119.04 m², panel 25, panel 32) |  |  |  |  |  |  |  |  |  |  |
| 35 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with a vertical slope, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 99.2000 |  |  |  |  |  |
| 36 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil originating from medium soil |  |  | m3 |  | 84.9000 |  |  |  |  |  |
| 37 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer; the fill is carried out using non-cohesive soil |  |  | 100 m³ |  | 0.8500 |  |  |  |  |  |
| 38 |  | IzF04G |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including the flashings and valleys in the standard waterproofing on slopes exceeding 40% or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or bitumen with rubber additive (base, waterproofing layer in 2 layers, K=2) |  |  | m² |  | 119.0400 |  |  |  |  |  |
| 39 |  | IzF10C |  | Thermal insulation layer for terraces, roofs and floors, made of cellular polystyrene boards, hemp fibre, PEL, porous, stabilised, etc., bonded with bituminous mastic to surfaces with a slope of over 40% or vertical surfaces (extruded polystyrene, 100 mm thick, on Technonicol N27 adhesive) |  |  | m² |  | 119.0400 |  |  |  |  |  |
| 40 |  | IzF04G |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including the flashings and valleys in the standard waterproofing on surfaces with a slope of over 40% or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or bitumen with rubber additive (Izoflex membrane) |  |  | m² |  | 119.0400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Thermal insulation of wall P-2 (walls below floor S=119.04 m², panel 25, panel 32) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Roof (sheet 32, sheet 78) |  |  |  |  |  |  |  |  |  |  |
| 41 |  | RpCB04C |  | Reinforced concrete cast in columns, beams, slabs, landings, stairs, retaining walls, reinforced concrete bearings, floor slabs, diaphragms, stair ramps, etc., for existing buildings, prepared with a concrete mixer on site and cast using conventional methods with Class C- 25/20 Bc25 (B330) reinforced concrete ( access hatch to the roof ) |  |  | m3 |  | 0.1100 |  |  |  |  |  |
| 42 |  | CE40A |  | Installation of the framework elements made of beams (bars) with antiseptic treatment |  |  | m3 |  | 4.4300 |  |  |  |  |  |
| 43 |  | CE41A |  | Installation of rafters treated with antiseptic |  |  | m3 |  | 3.8100 |  |  |  |  |  |
| 44 |  | CN50A |  | Fireproofing of timberwork; trusses, arches, beams, rafters, purlins. |  |  | m3 |  | 8.2400 |  |  |  |  |  |
| 45 |  | CE30A |  | Battens for roofing or battens for roofing made of tiles, asbestos cement sheets, etc., from rough softwood planks (24 mm thick), for standard constructions (plank consumption – 0.0065 m³/m²) |  |  | m2 |  | 572.2400 |  |  |  |  |  |
| 46 |  | CN51A |  | Antiseptic treatment of timberwork on concealed surfaces using antiseptic pastes: trusses, arches, rafters. |  |  | m³ |  | 3.7500 |  |  |  |  |  |
| 47 |  | CN50A |  | Fireproofing of timberwork; trusses, arches, beams, rafters, purlins. |  |  | m3 |  | 3.7500 |  |  |  |  |  |
| 48 |  | CE17A |  | Additional Ondutiss-type polymeric layer installed beneath the roof covering of tiles, corrugated sheets or embossed sheets |  |  | m² |  | 572.2400 |  |  |  |  |  |
| 49 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or ribbed, installed on metal panels, carried out on areas larger than 40 m² using profiled sheet metal sheets fastened with special clips and mechanical screws from the upper surface, including the installation of valleys, flashings, connections to chimneys, etc. |  |  | m² |  | 572.2400 |  |  |  |  |  |
| 50 |  | CE22A |  | Brass-type downpipe systems made of corrosion-resistant sheet metal |  |  | m |  | 54.1400 |  |  |  |  |  |
| 51 |  | CE20A |  | Brass-type gutter systems made of corrosion-resistant sheet metal |  |  | m |  | 64.6000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Roof (sheet 32, sheet 78) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Ventilation duct (sheet 44) |  |  |  |  |  |  |  |  |  |  |
| 52 |  | CD50I |  | Brickwork, 250 x 120 x 65 mm, on the walls of the discharge pits and channels (1.3 m²) |  |  | m3 |  | 0.1600 |  |  |  |  |  |
| 53 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement panels with visible fixing, with architectural detailing covering up to 30% of the total wall area (insulation panels – mineral wool, 50 mm thick, RAL-1013) |  |  | m2 |  | 1.3000 |  |  |  |  |  |
| 54 |  | CN54B |  | Manual application of a single coat of ‘Gleta’ quartz-based primer to external walls on the façade |  |  | m2 |  | 1.3000 |  |  |  |  |  |
| 55 |  | CN11A1 |  | Exterior painting with water-based acrylic polymer paint, applied in 3 coats to facades on existing plaster, applied mechanically (RAL-1013) |  |  | m² |  | 1.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation duct (sheet 44) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. Doors |  |  |  |  |  |  |  |  |  |  |
| 56 |  | CK12D |  | Metal doors made from rolled steel sections, cold-formed steel strip sections, including the necessary fittings and accessories for doors installed in masonry of any kind in buildings up to and including 35 m in height, double-leaf, with a door frame area up to and including 7 m² (Door without threshold, fire-resistant and fitted with a self-closing system, door Ui-1, Ui-2) |  |  | m² |  | 8.1900 |  |  |  |  |  |
| 57 |  | CK12D |  | Metal doors made from rolled steel profiles, cold-formed steel strip profiles, including the necessary fittings and accessories for doors installed in masonry of any kind in buildings with a height of up to 35 m inclusive, double-leaf, with a door frame area of up to 7 m² inclusive (threshold-free door, fire-rated door EI-30, Ui-13 - Ui-15) |  |  | m² |  | 10.0800 |  |  |  |  |  |
| 58 |  | CK33A |  | Automatic door closing device |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 59 |  | CK03A |  | Single-leaf interior wooden doors, on hinges and balcony doors, including water-repellent and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-3, Ui-4, door without threshold) |  |  | m² |  | 23.1000 |  |  |  |  |  |
| 60 |  | CK03A |  | Single-leaf interior wooden doors, on lining and balcony doors, including waterproof and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-5, Ui-6, door without threshold) |  |  | m² |  | 15.1200 |  |  |  |  |  |
| 61 |  | CK03A |  | Single-leaf interior wooden doors, on lining and balcony doors, including waterproof and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-7, door without threshold) |  |  | m2 |  | 4.3200 |  |  |  |  |  |
| 62 |  | CK03A |  | Single-leaf interior wooden doors, on linings and balcony doors, including waterproof and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-8, door without threshold) |  |  | m2 |  | 3.4200 |  |  |  |  |  |
| 63 |  | CK03A |  | Single-leaf interior wooden doors, on linings and balcony doors, including waterproof and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-9, Ui-10, door without threshold) |  |  | m² |  | 3.3600 |  |  |  |  |  |
| 64 |  | CK03A |  | Single-leaf interior wooden doors, on lining and balcony doors, including waterproof and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-11, door without threshold) |  |  | m2 |  | 1.8900 |  |  |  |  |  |
| 65 |  | CK03C |  | Double-leaf interior wooden doors, on hinges, and balcony doors, including water-repellent and thermal insulation of the door frame, fitted to existing door frames in buildings up to 35 m high (wooden/MDF door Ui-12, door without threshold) |  |  | m² |  | 2.3100 |  |  |  |  |  |
| 66 |  | CK21D |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings up to and including 35 m in height, double-leaf, with a frame area up to and including 7 m² (Ue-1 Thermal insulation doors comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium, door with threshold) |  |  | m² |  | 2.8200 |  |  |  |  |  |
| 67 |  | CK21D |  | Doors made from aluminium profiles, including the fittings and accessories required for doors installed in masonry of any kind, in buildings up to and including 35 m in height, double-leaf, with a frame area up to and including 7 m² (Ue-2 Thermal insulation doors comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium, door with threshold) |  |  | m² |  | 7.5600 |  |  |  |  |  |
| 68 |  | CK21A |  | Doors made from aluminium profiles, including the fittings and accessories required for doors installed in masonry of any kind, in buildings up to and including 35 m in height, single-leaf, with a frame area up to and including 7 m² (Ue-3, Ue-4 Thermal insulation doors comprising 3 layers of glass separated by a space filled with inert gas ( ). Frames made of 5-chamber aluminium, door with threshold) |  |  | m² |  | 6.3000 |  |  |  |  |  |
| 69 |  | RpCJ06B |  | Repairs to interior plasterwork, around door and window frames and sills, 2 cm thick, rough-cast, executed with 25 T grade cement-lime mortar, with straight edges, between 15–25 cm wide |  |  | m |  | 229.5000 |  |  |  |  |  |
| 70 |  | Supplier price |  | Aluminium angle profile |  |  | m |  | 229.5000 |  |  |  |  |  |
| 71 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 72 |  | CF61A |  | Continuous skimming of the surface (single-layer plaster) with dry plaster mix: window sills and flat doors |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 73 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast reinforced concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 74 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 75 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 45.9000 |  |  |  |  |  |
| 76 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 77 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m² |  | 45.9000 |  |  |  |  |  |
| 78 |  | CK07B |  | Wooden mouldings fitted to doors or windows |  |  | m |  | 156.6000 |  |  |  |  |  |
| 79 |  | CK26B |  | Window sills fitted to aluminium doors |  |  | m |  | 37.7000 |  |  |  |  |  |
| 80 |  | CK33C |  | Yalle system surface-mounted lock |  |  | pc |  | 47.0000 |  |  |  |  |  |
| 81 |  | CK33B |  | Chrome-plated sheet metal handle for metal entrance doors in buildings |  |  | pc |  | 47.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Doors Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Windows |  |  |  |  |  |  |  |  |  |  |
| 82 |  | CK57F apl |  | Installation of aluminium windows: tilt-and-turn (folding, swing-out) with an opening area of over 2 m² in two sashes (F-1 – Thermal insulation glazing comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium) |  |  | m² |  | 56.7000 |  |  |  |  |  |
| 83 |  | CK57F apl |  | Installation of aluminium windows: tilt-and-turn (folding, swing-out) with an opening area exceeding 2 m² in two sashes (F-2 – Thermal insulation glazing comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium) |  |  | m2 |  | 22.5000 |  |  |  |  |  |
| 84 |  | CK57C |  | Installation of PVC windows: tilt-and-turn (folding, swing-out) with an opening area of less than 2 m² in a single sash (Thermal insulation glazing comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium) |  |  | m² |  | 1.8000 |  |  |  |  |  |
| 85 |  | RpCJ06B |  | Repairs to interior plasterwork, around door and window frames and sills, 2 cm thick, rough-cast, executed with 25 T grade cement-lime mortar, with straight edges, between 15–25 cm wide |  |  | m |  | 109.5000 |  |  |  |  |  |
| 86 |  | Supplier price |  | Aluminium angle profile |  |  | m |  | 109.5000 |  |  |  |  |  |
| 87 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 21.9000 |  |  |  |  |  |
| 88 |  | CF61A |  | Continuous skimming of the surface (single-layer plaster) using a dry e plaster mix: window sills and flat doors |  |  | m2 |  | 21.9000 |  |  |  |  |  |
| 89 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast reinforced concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 21.9000 |  |  |  |  |  |
| 90 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 21.9000 |  |  |  |  |  |
| 91 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 21.9000 |  |  |  |  |  |
| 92 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 21.9000 |  |  |  |  |  |
| 93 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m2 |  | 21.9000 |  |  |  |  |  |
| 94 |  | CK26D |  | Window or door sills for aluminium windows or doors (drip mouldings) (width 200 mm) |  |  | m |  | 57.2000 |  |  |  |  |  |
| 95 |  | CK26C |  | Sills fitted to plastic windows or doors (width 350 mm) |  |  | m |  | 57.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Windows Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9. Facade (wall type 1, sheet 25, sheet 33) |  |  |  |  |  |  |  |  |  |  |
| 96 |  | CF02A |  | 2 cm thick interior plaster, rough-textured, applied by hand to walls or pillars, on flat surfaces using M 25-T lime-cement mortar for the base coat and M 10-T mortar for the primer and finish coat, on brickwork or small concrete blocks |  |  | m² |  | 544.8000 |  |  |  |  |  |
| 97 |  | CF17B |  | Miscellaneous works – thickness variation, ±5 mm for the primer coat applied to walls, executed with mortar (add 10 mm, K=2) |  |  | m² |  | 544.8000 |  |  |  |  |  |
| 98 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement panels with visible fixing, with architectural detailing covering up to 30% of the total wall area (insulation boards – mineral wool, 150 mm thick, RAL-1013) |  |  | m² |  | 501.8600 |  |  |  |  |  |
| 99 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement cladding and visible fixings, with architectural detailing covering up to 30% of the total wall area (insulation boards – mineral wool, 150 mm thick, RAL-9016) |  |  | m² |  | 42.9300 |  |  |  |  |  |
| 100 |  | CN54B |  | Manual application of a single coat of ‘Gleta’ quartz-based primer to external walls on the façade |  |  | m2 |  | 544.8000 |  |  |  |  |  |
| 101 |  | CN11A1 |  | Exterior painting with water-based acrylic polymer paint, applied in 3 coats to façades on existing plaster, applied mechanically (RAL-1013) |  |  | m² |  | 501.8600 |  |  |  |  |  |
| 102 |  | CN11A1 |  | Exterior painting with water-based acrylic polymer paint, applied in 3 coats to facades on existing plaster, applied mechanically (RAL-9016) |  |  | m² |  | 42.9300 |  |  |  |  |  |
| 103 |  | RpCH31A |  | Erection and dismantling of scaffolding for construction works – tubular metal scaffolding for interior and exterior works up to 7 m in height, including materials required for the construction of working platforms, PFL guardrails or safety netting |  |  | m2 |  | 638.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Facade (wall type 1, panel 25, panel 33) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10. Floors |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10.1. Porch |  |  |  |  |  |  |  |  |  |  |
| 104 |  | IzF04J1 |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including the flashings and valleys of the existing waterproofing: separation layer associated with waterproofing works, executed with single-ply bituminous membrane, laid with unsealed overlaps (2-layer roll-type waterproofing, K=2) |  |  | m² |  | 434.0100 |  |  |  |  |  |
| 105 |  | IzF11B |  | Thermal insulation layer for terraces, roofs and floors, laid with granulate, on horizontal or sloping surfaces up to 7% (thickness 230 mm) |  |  | m³ |  | 99.8200 |  |  |  |  |  |
| 106 |  | IzF18C K=1.67 |  | Levelling or protective base layer for insulation, including associated battens, made with ready-mixed M100-T cement mortar without lime additive, rough-textured, on horizontal or sloping surfaces up to and including 40%, applied to an average thickness of 3 cm (50 mm, K=1.67, layer 2 for equipment installation) |  |  | m² |  | 434.0100 |  |  |  |  |  |
| 107 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on panels (welded mesh 100x100x4 – weight 1.75 kg/m²) |  |  | kg |  | 759.5200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Porch Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Flooring Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11. Emergency staircase (sheets 61, 62) |  |  |  |  |  |  |  |  |  |  |
| 108 |  | RpCB04D |  | Reinforced concrete cast in columns, beams, slabs, landings, stairs, retaining walls, reinforced concrete bearings, floor slabs, diaphragms, stair ramps, etc., for existing buildings, prepared on-site using a concrete mixer and cast using conventional methods with ready-mixed concrete procured from batching plants (reinforced concrete class C30/37) |  |  | m³ |  | 1.2000 |  |  |  |  |  |
| 109 |  | RpCC01A |  | Mixed formwork made from reusable panels, constructed from 15 mm thick P-type beech plywood, including supports, for concrete in elevation, for retaining walls, plinths, etc., for repairs to existing structures, for straight walls up to 3.00 m in height |  |  | m2 |  | 5.9200 |  |  |  |  |  |
| 110 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut to size in site workshops and installed in existing structural elements; cutting and installation of reinforcement for straight slabs, columns, beams, etc. (diam. 14 B500B) |  |  | kg |  | 8.0000 |  |  |  |  |  |
| 111 |  | CL18A |  | Various metal fabrications from rolled sections, sheet metal, corrugated sheet metal, reinforced concrete, pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 15.0000 |  |  |  |  |  |
| 112 |  | CL01A |  | Ready-made steel columns, delivered fully assembled, installed at heights up to 35 m, weighing up to 1 t inclusive (columns made from 160×160×4 profile pipes) |  |  | t |  | 0.1600 |  |  |  |  |  |
| 113 |  | CL03A |  | Ready-made solid-core steel beams, delivered fully assembled, installed at heights up to 35 m, weighing up to 1 t inclusive (beams made from U-section No. 16) |  |  | t |  | 0.1900 |  |  |  |  |  |
| 114 |  | RpCB04C |  | Reinforced concrete cast in columns, beams, slabs, landings, stairs, retaining walls, reinforced concrete bearings, floor slabs, diaphragms, stair ramps, etc., for existing buildings, mixed on-site using a concrete mixer and cast using conventional methods with reinforced concrete of class C30/37 |  |  | m3 |  | 0.3000 |  |  |  |  |  |
| 115 |  | RpCC02A |  | Reusable composite formwork panels, made of plywood (excluding supports), for reinforced concrete, for extensions or repairs to existing structures, in walls and slabs, with 15 mm plywood |  |  | m2 |  | 1.9100 |  |  |  |  |  |
| 116 |  | CB11A |  | Supports with extendable props from stock, used for the installation of precast slabs and precast floor panels, for the casting of partially or fully monolithic floors with beams or for monolithic beams with precast floor panels of type PE 3100 R |  |  | pcs |  | 6.0000 |  |  |  |  |  |
| 117 |  | RpCD08C |  | Installation of STM or STNB welded mesh on slabs (welded mesh 100x100x6, weight 4.4 kg/m²) |  |  | kg |  | 17.4000 |  |  |  |  |  |
| 118 |  | CH10C |  | Manufacture of straight metal staircases in quantities exceeding 50 kg and steps |  |  | kg |  | 403.6600 |  |  |  |  |  |
| 119 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.7540 |  |  |  |  |  |
| 120 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.7540 |  |  |  |  |  |
| 121 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.7540 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Evacuation staircase (sheets 61, 62) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12. Conversion of window opening into door opening (sheet 56) |  |  |  |  |  |  |  |  |  |  |
| 122 |  | RpCB03B |  | Reinforced concrete cast in formwork, ring beams, insulated cornices, partition walls, stair and balcony parapets, decorative columns, reinforced concrete bases for the installation of metal columns on existing buildings, prepared using a concrete mixer ( ) on site and cast using conventional methods with C-20/16 Bc20 class reinforced concrete (B250) |  |  | m3 |  | 0.1600 |  |  |  |  |  |
| 123 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, with 15 mm plywood |  |  | m2 |  | 1.3500 |  |  |  |  |  |
| 124 |  | CB11A |  | Supports with extendable props from stock, used for installing precast slabs and precast floor panels, for casting partially or fully monolithic floors with beams or for monolithic beams with precast floor panels of type PE 3100 R |  |  | pcs |  | 3.0000 |  |  |  |  |  |
| 125 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with bar diameters up to and including 8 mm, cut to size on site and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diam. 6 A240) |  |  | kg |  | 1.0000 |  |  |  |  |  |
| 126 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 12 B500B) |  |  | kg |  | 4.0000 |  |  |  |  |  |
| 127 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks |  |  | m³ |  | 0.3800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Conversion of window opening into door opening (sheet 56) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13. Lintel above the designed door opening (sheet 57) |  |  |  |  |  |  |  |  |  |  |
| 128 |  | RpCU06D |  | Cutting grooves up to 5 cm deep in 5 x 100 cm² brick masonry walls |  |  | m |  | 3.2000 |  |  |  |  |  |
| 129 |  | RpCB03B |  | Reinforced concrete cast in formwork, ring beams, insulated cornices, partition walls, stair and balcony parapets, decorative columns, reinforced concrete bases for the installation of metal columns on existing buildings, prepared with a concrete mixer on site and cast using conventional methods with reinforced concrete class C-20/16 Bc20 (B250) |  |  | m3 |  | 0.1800 |  |  |  |  |  |
| 130 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, with 15 mm plywood |  |  | m2 |  | 1.1200 |  |  |  |  |  |
| 131 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with a diameter of up to 8 mm inclusive, cut to size in site workshops and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diam. 6 A240) |  |  | kg |  | 2.0000 |  |  |  |  |  |
| 132 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 14 B500B) |  |  | kg |  | 60.0000 |  |  |  |  |  |
| 133 |  | CD71C |  | Erection of external and internal walls from lightweight concrete blocks using adhesive without cladding, including filling of voids and joints: for floor heights up to 4 m |  |  | m³ |  | 0.6700 |  |  |  |  |  |
| 134 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks |  |  | m3 |  | 0.7600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total lintel above the designed door opening (sheet 57) Including labour |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14. Widening of the opening in non-load-bearing walls (mod. 3 – 3 units) (sheet 58) |  |  |  |  |  |  |  |  |  |  |
| 135 |  | RpCU06D1 |  | Excavation of trenches up to 5 cm deep in 5 x 100 cm² brick masonry walls, for mechanical execution |  |  | m |  | 8.4000 |  |  |  |  |  |
| 136 |  | RpCB03B |  | Reinforced concrete cast in formwork, ring beams, insulated cornices, partition walls, stair and balcony parapets, decorative columns, reinforced concrete bases for the installation of metal columns on existing buildings, prepared with a concrete mixer on site and cast using conventional methods with reinforced concrete class C-20/16 Bc20 (B250) |  |  | m3 |  | 0.0900 |  |  |  |  |  |
| 137 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, with 15 mm plywood |  |  | m2 |  | 1.8900 |  |  |  |  |  |
| 138 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with a diameter of up to 8 mm inclusive, cut to size in site workshops and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diam. 6 A240) |  |  | kg |  | 0.6000 |  |  |  |  |  |
| 139 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 10 B500B) |  |  | kg |  | 6.0000 |  |  |  |  |  |
| 140 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks |  |  | m³ |  | 0.1500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Widening of openings in non-load-bearing walls (mod. 3 – 3 units) (sheet 58) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 15. Creation of openings in non-load-bearing walls (mod. 4, mod. 5 – 6 units and mod. 6 – 1 unit) (sheet 59) |  |  |  |  |  |  |  |  |  |  |
| 141 |  | RpCU06D1 |  | Excavation of trenches up to 5 cm deep in 5 x 100 cm² brick masonry walls, for mechanical excavation |  |  | m |  | 21.4000 |  |  |  |  |  |
| 142 |  | RpCB03B |  | Reinforced concrete cast in formwork, ring beams, insulated cornices, partition walls, stair and balcony parapets, decorative columns, reinforced concrete bases for the installation of metal columns on existing buildings, prepared with a concrete mixer on site and cast using conventional methods with C-20/16 Bc20 class reinforced concrete (B250) |  |  | m3 |  | 0.2100 |  |  |  |  |  |
| 143 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, with 15 mm plywood |  |  | m2 |  | 3.0200 |  |  |  |  |  |
| 144 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with a diameter of up to 8 mm inclusive, cut to size on site and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diam. 6 A240) |  |  | kg |  | 1.4000 |  |  |  |  |  |
| 145 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters over 8 mm, cut on-site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 10 B500B) |  |  | kg |  | 15.0000 |  |  |  |  |  |
| 146 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, BCA, ceramic or lightweight concrete blocks, GVP bricks, excluding scaffolding and clearing of bricks |  |  | m³ |  | 1.8100 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Creation of openings in non-load-bearing walls (mod. 4, mod. 5 – 6 units and mod. 6 – 1 unit) (sheet 59) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 16. Concreting of side walls, staircase 3 |  |  |  |  |  |  |  |  |  |  |
| 147 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m3 |  | 0.0900 |  |  |  |  |  |
| 148 |  | CA03E2 |  | Cast concrete in foundations, plinths, retaining walls, walls below ground level, mixed on-site using a concrete mixer and poured using conventional methods, reinforced concrete class C 16/20 |  |  | m3 |  | 2.0000 |  |  |  |  |  |
| 149 |  | CL18A |  | Various metalwork made from rolled sections, sheet metal, corrugated sheet metal, structural steel, and pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total concreting of side walls, staircase 3 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17. Steps and ramp No. 1 (sheet 65) |  |  |  |  |  |  |  |  |  |  |
| 150 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 3.8500 |  |  |  |  |  |
| 151 |  | CA04F |  | Concrete cast in slabs, beams, columns, prepared at a concrete plant or using ready-mixed concrete in accordance with item CA01 and cast using conventional methods ( concrete C 30/37 ) |  |  | m3 |  | 9.5000 |  |  |  |  |  |
| 152 |  | CB02C |  | Formwork made of reusable panels, with formwork struts of short and extra-short softwood planks for the casting of concrete into slabs and beams, excluding supports at heights up to and including 20 m |  |  | m2 |  | 13.6000 |  |  |  |  |  |
| 153 |  | CC01C |  | OB 37 reinforced concrete steel reinforcement, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 27.0000 |  |  |  |  |  |
| 154 |  | CC01F1 |  | PC 52 reinforced concrete steel reinforcement cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diam. 8 B500B) |  |  | kg |  | 292.0000 |  |  |  |  |  |
| 155 |  | CC01F1 |  | PC 52 concrete steel reinforcement cut on-site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 12 B500B) |  |  | kg |  | 82.0000 |  |  |  |  |  |
| 156 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (weight 1 kg/m²) |  |  | kg |  | 4.0000 |  |  |  |  |  |
| 157 |  | TsD01B |  | Spreading of fine-grained soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil being sourced from medium-grade land |  |  | m³ |  | 7.6500 |  |  |  |  |  |
| 158 |  | TsD04A |  | Compaction with a hand tamper of backfill in horizontal or 1/4-inclined trenches, including wetting each individual layer of soil, 10 cm thick, non-cohesive soil |  |  | m3 |  | 7.6500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Steps and Ramp No. 1 (sheet 65) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18. Steps and ramp No. 2 (sheet 67) |  |  |  |  |  |  |  |  |  |  |
| 159 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with a vertical face, at foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m3 |  | 4.3500 |  |  |  |  |  |
| 160 |  | CA04F |  | Concrete cast in slabs, beams, columns, prepared at a concrete plant or using ready-mixed concrete in accordance with item CA01 and cast using conventional methods ( concrete C 30/37 ) |  |  | m3 |  | 13.5000 |  |  |  |  |  |
| 161 |  | CB02C |  | Formwork made of reusable panels, with formwork struts of short and extra-short softwood planks for the casting of concrete into slabs and beams, excluding supports at heights up to and including 20 m |  |  | m2 |  | 17.2000 |  |  |  |  |  |
| 162 |  | CC01C |  | OB 37 concrete steel reinforcement, cut to size in site workshops and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 28.0000 |  |  |  |  |  |
| 163 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 B500B) |  |  | kg |  | 326.0000 |  |  |  |  |  |
| 164 |  | CC01F1 |  | PC 52 concrete steel reinforcement cut on-site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 12 B500B) |  |  | kg |  | 94.0000 |  |  |  |  |  |
| 165 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (weight 1 kg/m²) |  |  | kg |  | 8.0000 |  |  |  |  |  |
| 166 |  | TsD01B |  | Spreading of fine-grained soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil being sourced from medium-grade land |  |  | m³ |  | 7.5600 |  |  |  |  |  |
| 167 |  | TsD04A |  | Compaction using a hand tamper of backfill in horizontal or 1/4-inclined trenches, including wetting each individual layer of soil, 10 cm thick, non-cohesive soil |  |  | m3 |  | 7.5600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total steps and ramp No. 2 (sheet 67) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19. Cover No. 1 |  |  |  |  |  |  |  |  |  |  |
| 168 |  | CL18A |  | Various metal fabrications from rolled sections, sheet metal, corrugated sheet metal, structural steel, pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 20.0000 |  |  |  |  |  |
| 169 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in lightweight construction |  |  | t |  | 0.6350 |  |  |  |  |  |
| 170 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.6350 |  |  |  |  |  |
| 171 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.6350 |  |  |  |  |  |
| 172 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.6350 |  |  |  |  |  |
| 173 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or ribbed, mounted on metal panels, installed on areas larger than 40 m² using profiled sheet metal sheets fastened with special clips and mechanical screws on the upper surface, including the installation of valleys, flashings, connections to chimneys, etc. |  |  | m² |  | 14.7500 |  |  |  |  |  |
| 174 |  | CL54B |  | Ventilated façade system with metal cladding (external panelling) without thermal insulation, with architectural detailing covering up to 30% of the total wall area |  |  | m2 |  | 6.5400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Canopy No. 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 20. Cover No. 2 |  |  |  |  |  |  |  |  |  |  |
| 175 |  | CL18A |  | Various metal structures made from rolled sections, sheet metal, corrugated sheet metal, reinforced concrete, pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 20.0000 |  |  |  |  |  |
| 176 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in lightweight construction |  |  | t |  | 0.5400 |  |  |  |  |  |
| 177 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.5400 |  |  |  |  |  |
| 178 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.5400 |  |  |  |  |  |
| 179 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.5400 |  |  |  |  |  |
| 180 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or ribbed, mounted on metal panels, installed on areas larger than 40 m² using profiled sheet metal sheets fastened with special clips and mechanical screws on the upper surface, including the installation of valleys, flashings, connections to chimneys, etc. |  |  | m² |  | 9.2000 |  |  |  |  |  |
| 181 |  | CL54B |  | Ventilated façade system with metal cladding (external panelling) without thermal insulation, with architectural detailing covering up to 30% of the total wall area |  |  | m² |  | 5.1600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Canopy No. 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 21. Cover No. 3 |  |  |  |  |  |  |  |  |  |  |
| 182 |  | CL18A |  | Various metal structures made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 20.0000 |  |  |  |  |  |
| 183 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in lightweight construction |  |  | t |  | 0.4300 |  |  |  |  |  |
| 184 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.4300 |  |  |  |  |  |
| 185 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.4300 |  |  |  |  |  |
| 186 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.4300 |  |  |  |  |  |
| 187 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or ribbed, mounted on metal panels, installed on areas larger than 40 m² using profiled sheet metal sheets secured with special clips and mechanical screws, top-mounted, including the installation of flashings, trims, connections to chimneys, etc. |  |  | m² |  | 7.8800 |  |  |  |  |  |
| 188 |  | CL54B |  | Ventilated façade system with metal cladding (external panelling) without thermal insulation, with architectural detailing covering up to 30% of the total wall area |  |  | m2 |  | 4.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Canopy No. 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 22. Balustrades |  |  |  |  |  |  |  |  |  |  |
| 189 |  | CH06A |  | Metal handrail mounted on 15 cm high supports, spaced at intervals of 1....1.2 m, fitted with welded washers, fixed to a brick wall or concrete parapet, made of D=1 1/4" steel pipe and rolled steel, straight |  |  | m |  | 31.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Balustrades Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead expenses |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Estimated profit |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including salary |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |