| Major repair and modernisation of the Public Institution "Mihai Eminescu", Telenesti district, 8 Codrilor Street, cadastral number 8901224. 151. 02 No. 8/13a-25 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-2

Study module: Internal water supply and drainage networks     (RAC brand)

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. System A1 |  |  |  |  |  |  |  |  |  |  |
| 1 |  | IC38B apl |  | Connection fitting (fitting), with 2 joints, made of polypropylene fused to the reinforced polypropylene pipe, with an external diameter of 25.0 mm PPR valve PN 25 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 2 |  | IC38A apl |  | Connection fitting (fitting), with 2 joints, made of polypropylene joined by polyfusion to the reinforced polypropylene pipe, with an external diameter of up to 20.0 mm, inclusive PPR valve PN 20 |  |  | pcs |  | 6.000 |  |  |  |  |  |
| 3 |  | IC38A apl |  | Connection fitting (fitting), with 2 joints, made of polypropylene joined by polyfusion to the reinforced polypropylene pipe, with an external diameter of up to 20.0 mm, inclusive PPR valve PN 15 |  |  | pc |  | 21.000 |  |  |  |  |  |
| 4 |  | SD01A |  | Service valve, single or double with connection, regardless of the closing mechanism, with a diameter of 3/8" - 1/2" - FE-type angled water valve with hose connection DN15 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 5 |  | SD01B |  | Service tap, single or double with connection, regardless of shut-off type, with a diameter of 3/4" - 1" - FE-type angled water tap with hose connection DN20 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 6 |  | SA16C |  | Plastic pipe joined by fusion welding, in columns, for residential and social/cultural buildings, with a diameter of 32 mm (PP-R PN10 D32 pipe) |  |  | m |  | 5.000 |  |  |  |  |  |
| 7 |  | SA15B |  | Polyfusion-welded plastic pipe, used in connecting pipes, for sanitary fittings, and in residential and socio-cultural buildings, with a diameter of 20–25 mm (PP-R PN10 D25 pipe) |  |  | m |  | 45.000 |  |  |  |  |  |
| 8 |  | SA15B |  | Plastic pipe joined by fusion welding, for connecting pipes, sanitary fittings, residential and socio-cultural facilities, with a diameter of 20–25 mm (PP-R PN10 D20 pipe) |  |  | m |  | 40.000 |  |  |  |  |  |
| 9 |  | SF01C |  | Carrying out pressure leak testing of hot or cold water installations made from heavy-duty polyvinyl chloride (PVC) or plastic pipes, with a diameter of 16–110 mm |  |  | m |  | 90.000 |  |  |  |  |  |
| 10 |  | SF02C |  | Carrying out the functional test of the cold water system made of heavy-duty polyvinyl chloride (PVC) or plastic pipes, with a diameter of 16–110 mm |  |  | 10 m |  | 9.000 |  |  |  |  |  |
| 11 |  | SF05C |  | Flushing of cold or hot water systems made of plastic pipes with a diameter of 20–75 mm. |  |  | m |  | 90.000 |  |  |  |  |  |
| 12 |  | RpIF09B |  | Insulation of pipes using special insulation sleeves, fitted over the pipe , with diameters and thicknesses ranging from D=12x9 to D=54x9 mm (PEP thermal insulation tubes, d-32mm) |  |  | m |  | 5.000 |  |  |  |  |  |
| 13 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes, with diameters and thicknesses ranging from D=12x9 to D=54x9 mm (PEP thermal insulation tubes d-25mm) |  |  | m |  | 45.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total System A1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. System T3.T4 |  |  |  |  |  |  |  |  |  |  |
| 14 |  | IC38A apl |  | Connection fitting (fitting), with 2 joints, made of polypropylene fused to the reinforced polypropylene pipe, with an external diameter of up to 20.0 mm, including PPR valve PN 20 |  |  | pc |  | 6.000 |  |  |  |  |  |
| 15 |  | IC38A apl |  | Connection fitting (fitting), with 2 joints, made of polypropylene joined by polyfusion to the reinforced polypropylene pipe, with an external diameter of up to 20.0 mm, inclusive PPR valve PN 15 |  |  | pc |  | 12.000 |  |  |  |  |  |
| 16 |  | SA15B |  | Plastic pipe joined by polyfusion welding, for connecting pipes, in sanitary installations, in residential and socio-cultural buildings, with a diameter of 20–25 mm (PP-R PN10 D25 pipe) |  |  | m |  | 45.000 |  |  |  |  |  |
| 17 |  | SA15B |  | Polyfusion-welded plastic pipe, used in connecting pipes, for sanitary fittings, and in residential and socio-cultural buildings, with a diameter of 20–25 mm (PP-R PN10 D20 pipe) |  |  | m |  | 70.000 |  |  |  |  |  |
| 18 |  | SF01C |  | Carrying out pressure leak testing of hot or cold water installations made from heavy-duty polyvinyl chloride (PVC) or plastic- e pipes, with a diameter of 16–110 mm |  |  | m |  | 115.000 |  |  |  |  |  |
| 19 |  | SF02C |  | Carrying out a functional test on a cold water system made of heavy-duty polyvinyl chloride (PVC) or plastic pipes, with a diameter of 16–110 mm |  |  | 10 m |  | 11.500 |  |  |  |  |  |
| 20 |  | SF05C |  | Flushing of cold or hot water systems made of plastic pipes with a diameter of 20–75 mm. |  |  | m |  | 115.000 |  |  |  |  |  |
| 21 |  | RpIF09B |  | Insulation of pipes using special insulation sleeves fitted onto the pipes, with diameters and thicknesses ranging from D=12x9 to D=54x9 mm (PEP thermal insulation tubes, d-25mm) |  |  | m |  | 45.000 |  |  |  |  |  |
| 22 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes, with diameters and thicknesses ranging from D=12x9 to D=54x9 mm (PEP thermal insulation tubes, d-20mm) |  |  | m |  | 70.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total System T3.T4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. System C1 |  |  |  |  |  |  |  |  |  |  |
| 23 |  | SC07B1 |  | Toilet bowl, fully equipped, made of semi-porcelain, sanitary porcelain, etc., including for disabled persons, floor-mounted, with the cistern mounted on the bowl, at full or half height, with an internal P-type siphon. ( WC bowl with side outlet and cistern ) |  |  | set |  | 14.000 |  |  |  |  |  |
| 24 |  | SC04A |  | Washbasin made of semi-porcelain, sanitary porcelain, etc., including models for disabled users, fitted with a plastic drainpipe, mounted on brackets fixed to walls constructed of brick or reinforced concrete. (Ceramic washbasin with a ‘bottle’ trap and tap) |  |  | each |  | 16.000 |  |  |  |  |  |
| 25 |  | SC05A |  | Enamelled cast iron or acid-resistant ceramic sink, with plastic drainpipe, mounted on brickwork walls (Enamelled steel sink with "bottle" type trap and tap) |  |  | each |  | 1.000 |  |  |  |  |  |
| 26 |  | SB02A1 |  | Cast iron sewer fittings, sealed with white rope and cement, with a nominal diameter of 50 mm siphon for shower tray |  |  | pc |  | 2.000 |  |  |  |  |  |
| 27 |  | SB26A |  | Enamelled cast iron drain (receiver) for draining water from terraces and roofs, with a diameter of 100–150 mm (DN 110 drain) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 28 |  | SB09E |  | Plastic connecting piece for drainage, fitted with a rubber gasket, with a diameter of 110 mm (Cleaning piece with threaded plug PP DN 110) |  |  | pc |  | 8.000 |  |  |  |  |  |
| 29 |  | SB09C |  | Plastic sewer connection fitting, fitted with a rubber gasket, with a diameter of 50 mm. (Cleaning fitting) |  |  | pc |  | 18.000 |  |  |  |  |  |
| 30 |  | SB09E |  | Plastic sewer connection fitting, fitted with a rubber gasket, with a diameter of 110 mm (D110 fire-resistant socket) |  |  | pc |  | 8.000 |  |  |  |  |  |
| 31 |  | SB08C |  | Plastic sewer pipe, fitted with a rubber gasket, installed above ground or buried under the floor, with a diameter of 50 mm (PP sewer pipe DN50) |  |  | m |  | 20.000 |  |  |  |  |  |
| 32 |  | SB11A |  | Plastic sewer pipe fitting (double branch), fitted with a rubber gasket, with a diameter of 50 mm (PP pipe fittings, 50 mm (30%) |  |  | pc |  | 6.000 |  |  |  |  |  |
| 33 |  | SB08E |  | Plastic sewer pipe, fitted with a rubber gasket, installed exposed or buried under the floor, with a diameter of 110 mm (PP sewer pipe DN100) |  |  | m |  | 50.000 |  |  |  |  |  |
| 34 |  | SB11C |  | Plastic sewer connection piece (double branch), fitted with a rubber gasket, with a diameter of 110 mm. (Fittings PP pipes DN100 (30%) |  |  | pcs |  | 15.000 |  |  |  |  |  |
| 35 |  | SB05E |  | Lightweight (U) unplasticised PVC pipe for drainage, joined by welding, installed exposed or buried under the floor, with a diameter of 110 mm (Polyvinyl chloride (PVC) pipes ф110 |  |  | m |  | 35.000 |  |  |  |  |  |
| 36 |  | SB06D |  | Fittings (elbows, clean-out fittings, double sockets, expansion joints and reducers) made from lightweight (U-type) PVC pipe, installed by welding, with a diameter of 110 mm (fittings 30% of the length of the pipe ф110) |  |  | pcs |  | 11.000 |  |  |  |  |  |
| 37 |  | SF04A |  | Carrying out leak and functional tests on the drainage system made of cast iron pipes, for drainage, unplasticised PVC pipes, light-type or plastic, ductile iron pipes with a diameter of up to 100 mm inclusive |  |  | 10 m |  | 1.050 |  |  |  |  |  |
| 38 |  | SF02C |  | Testing the operation of the cold water system made of heavy-duty polyvinyl chloride (PVC) pipes or plastic pipes, with a diameter of 16–110 mm |  |  | 10 m |  | 1.050 |  |  |  |  |  |
| 39 |  | SF05C |  | Flushing of cold or hot water systems made of plastic pipes with a diameter of 20–75 mm. |  |  | m |  | 105.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total System C1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social and health insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Semi-finished product and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
| 40 |  | SC02A |  | Enamelled cast iron, enamelled sheet metal, polymethyl methacrylate, etc. shower tray (Shower tray with drain and mixer tap) |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Storage and costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Machinery |  |  |  |  |  |  |  |  |  |  |
| 41 |  | Supplier price |  | Shower tray |  |  | each |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |