| Major repairs and modernisation of the "Mihai Eminescu" Public Institution, Telenesti District, 8 Codrilor Street, cadastral number 8901224. 151. 02 No. 106-2025-3-IVC | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 2-1-3

-study block. IVC

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Heating – Kermi PROFIL - K steel monobloc radiators |  |  |  |  |  |  |  |  |  |  |
| 1 |  | IB06A |  | Steel radiators, monobloc, up to 1000 mm in length, including installation. Side-connected steel panel radiators, TYPE 11K 400X500, Kermi PROFIL - K |  |  | pcs |  | 1.0000 |  |  |  |  |  |
| 2 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TYPE 22K 600X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 3 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TYPE 22K 1000X500 Kermi PROFIL - K |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 4 |  | IB06C |  | Steel radiators, monobloc with a length of 1501–2000 mm - Steel panel radiators with side connection TYPE 22K 1200X500 Kermi PROFIL - K |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 5 |  | IB06C |  | Steel radiators, monobloc, length 1501–2000 mm - Steel panel radiators with side connection TYPE 22K 1400X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 6 |  | IB06C |  | Steel radiators, monobloc with a length of 1501–2000 mm - Steel panel radiators with side connection TYPE 22K 1600X500 Kermi PROFIL - K |  |  | pc |  | 5.0000 |  |  |  |  |  |
| 7 |  | IB06A |  | Steel radiators, monobloc, up to 1000 mm in length, including TIP 33K 600X900 Kermi PROFIL side-connected steel panel radiators - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 8 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TYPE 33K 800X900 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 9 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TYPE 33K 800X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 10 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TYPE 33K 900X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 11 |  | IB06A |  | Steel radiators, monobloc, up to 1000 mm in length, including Heating steel panel radiators with side connection TYPE 33K 1000X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 12 |  | IB06C |  | Steel radiators, monobloc, with a length of 1501–2000 mm - Heating steel panel radiators with side connection TYPE 33K 1200X500 Kermi PROFIL - K |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 13 |  | IB06C |  | Steel radiators, monobloc, length 1501–2000 mm - Steel panel radiators with side connection TYPE 33K 1400X500 Kermi PROFIL - K |  |  | pc |  | 13.0000 |  |  |  |  |  |
| 14 |  | IB06C |  | Steel radiators, monobloc, length 1501–2000 mm - Steel panel radiators with side connection TYPE 33K 1600X500 Kermi PROFIL - K |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 15 |  | IB06C |  | Steel radiators, monobloc, 1501–2000 mm in length - TIP 33K 1800×500 Kermi PROFIL side-connected steel panel radiators - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 16 |  | IB06C |  | Steel radiators, monobloc with a length of 1501–2000 mm - Steel panel radiators with side connection TYPE 33K 2300X500 Kermi PROFIL - K |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 17 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" - RLV angle shut-off valve Db=15 . DANFOSS |  |  | pc |  | 41.00 |  |  |  |  |  |
| 18 |  | ID04A |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1/2" - 1" - Direct thermostatic valve Dn15 Type RA-N ("Danfoss" Austria) |  |  | pc |  | 41.0000 |  |  |  |  |  |
| 19 |  | ID04A |  | Gate or check valve with sockets for central heating systems, Thermostatic head Dn15 P=10 bar. Type RA2991 ("Danfoss" Austria) |  |  | pc |  | 31.0000 |  |  |  |  |  |
| 20 |  | ID04A |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1/2"–1" Ball valve φ20 |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 21 |  | ID04B |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1 1/4" - 1 1/2" -Ball valve ф=32 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 22 |  | Supplier price |  | DN50 PN16 Genebre welded ball valve |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 23 |  | RpIC31C |  | Oxy-acetylene flame welding of insulated installation pipes, diameter 2" DN50 PN16 under-weld ball valve |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 24 |  | ID04B |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1 1/4"–1 1/2" - Manual balancing valve db=32 USV-1 Danfoss |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 25 |  | ID04A |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1/2" - 1" (Shut-off and measuring valve dв=20. ASV-M 20 Danfoss) |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 26 |  | IC46A |  | Manifold for heating points and substations, mounted on a ready-made support 65–100 mm ( , φ40, L=500 mm) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 27 |  | ID06A.F |  | Air vent valve with ball valve for central heating systems, (Automatic air vent valve with ball valve, 15 mm) |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 28 |  | ID06A.F |  | Air vent valve with ball valve for central heating systems, ( -type drain valve, , 15 mm) |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 29 |  | IC11F |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings, pipe diameter 2" - Steel pipe, seamless, 60.3 x 4.0 mm, SM EN 10217 P265GH |  |  | m |  | 32.0000 |  |  |  |  |  |
| 30 |  | IC11D |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by column welding, in central heating systems for residential and social-cultural buildings, pipe diameter 1 1/4" - Steel pipe , 42.4 x 4.0 mm, SM EN 10217 P265GH |  |  | m |  | 10.0000 |  |  |  |  |  |
| 31 |  | IzA06D |  | Anti-corrosive paintwork on metal joinery, technological equipment and metal structures with alkyd enamel (one coat of GF-21 primer and two coats of BT-177l enamel) |  |  | m² |  | 9.1800 |  |  |  |  |  |
| 32 |  | IE03B |  | Carrying out pressure leak tests on the supply pipes of heating appliances (fan heaters, convector heaters, baseboard heaters, etc.) with a diameter of 1 1/4" ... 2" |  |  | m |  | 42.0000 |  |  |  |  |  |
| 33 |  | IC35B |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed at the connection points of heating units or appliances in central heating systems, Pipe made of polypropylene . ф20х3.4 PP-R 20 |  |  | m |  | 167.0000 |  |  |  |  |  |
| 34 |  | IC37B |  | Polyethylene fittings with two joints, press-fitted with a threaded connection to the reinforced polyethylene pipe, with an external diameter of 20.0 mm - Fittings for polypropylene pipe account for 60% of the pipe length |  |  | pcs |  | 100.0000 |  |  |  |  |  |
| 35 |  | IC35C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed at the connection points of heating units or appliances in central heating systems, Polypropylene pipe . ф25х4.2 PP-R 25 |  |  | m |  | 500,000 |  |  |  |  |  |
| 36 |  | IC37C |  | Polyethylene fittings with two joints, press-fitted with a threaded connection to reinforced polyethylene pipe, with an external diameter of 25.0 mm - Fittings for polypropylene pipe account for 60% of the pipe length |  |  | pcs |  | 300.0000 |  |  |  |  |  |
| 37 |  | SF01C |  | Carrying out a pressure leak test on hot or cold water installations made of heavy-duty polyvinyl chloride (PVC) pipe or plastic, with a diameter of 16–110 mm |  |  | m |  | 667.0000 |  |  |  |  |  |
| 38 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes Basalt wool cylinders with aluminium foil coating FLEXOROCK b=40 mm DN 50 ROCKWOOL |  |  | m |  | 10,000 |  |  |  |  |  |
| 39 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes Basalt wool cylinders with aluminium foil coating FLEXOROCK b=40 mm DN 32 ROCKWOOL |  |  | m |  | 32.0000 |  |  |  |  |  |
| 40 |  | RpIF09B |  | Insulation of pipes using special insulation sleeves, fitted onto the pipes - 9 mm porous polyethylene hose, 20 mm diameter PEP |  |  | m |  | 167.0000 |  |  |  |  |  |
| 41 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm, 25 mm diameter PEP |  |  | m |  | 480.0000 |  |  |  |  |  |
| 42 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines St 3) |  |  | kg |  | 270.0000 |  |  |  |  |  |
| 43 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 14.0400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating - Kermi PROFIL steel monobloc radiators - K Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Ventilation PV 1 |  |  |  |  |  |  |  |  |  |  |
| 44 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet air ducts 100x100 mm , thickness 0.5 mm, class "H" |  |  | m2 |  | 6.2000 |  |  |  |  |  |
| 45 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a pitch of 600 mm. - Galvanised sheet metal air ducts 200x100 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 6.6000 |  |  |  |  |  |
| 46 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 600 mm. - air ducts made of galvanised sheet metal 200x150 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 6.3000 |  |  |  |  |  |
| 47 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of galvanised sheet metal 200x200 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 1.2000 |  |  |  |  |  |
| 48 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 250x100 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 1.4000 |  |  |  |  |  |
| 49 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 250x200 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 4.5000 |  |  |  |  |  |
| 50 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x150 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.4500 |  |  |  |  |  |
| 51 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x200 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 1.5000 |  |  |  |  |  |
| 52 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x250 mm , б=0.7 mm class "Н" |  |  | m² |  | 2.2000 |  |  |  |  |  |
| 53 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of galvanised sheet 300x300 mm , б=0.7 mm class "Н" |  |  | m² |  | 24.6000 |  |  |  |  |  |
| 54 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 350x100 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.4500 |  |  |  |  |  |
| 55 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of up to 1200 mm. - Air ducts made of 400x150 mm galvanised sheet metal, thickness 0.7 mm |  |  | m² |  | 2.2000 |  |  |  |  |  |
| 56 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x200 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.6000 |  |  |  |  |  |
| 57 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x300 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 1.4000 |  |  |  |  |  |
| 58 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: 1800 mm. - Galvanised sheet metal air ducts 500x150 mm , б=0.7 mm class "Н" |  |  | m² |  | 0.6500 |  |  |  |  |  |
| 59 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Air ducts made of galvanised sheet 500x200 mm , б=0.7 mm class "Н" |  |  | m² |  | 0.7000 |  |  |  |  |  |
| 60 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet air ducts 500x500 mm , thickness 0.7 mm, class "H" |  |  | m2 |  | 2.0000 |  |  |  |  |  |
| 61 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium 0.7 mm thick, with a circular cross-section diameter of: 500–560 mm. (circular cross-section) Air ducts made of galvanised sheet metal 315 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 1.4800 |  |  |  |  |  |
| 62 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 100x100 mm , б=0.5 mm class "П" |  |  | m² |  | 3.2000 |  |  |  |  |  |
| 63 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet metal air ducts 200x100 mm , thickness 0.5 mm, class "П" |  |  | m2 |  | 2.4000 |  |  |  |  |  |
| 64 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet metal air ducts 200x150 mm , thickness 0.5 mm, class "П" |  |  | m² |  | 5.6000 |  |  |  |  |  |
| 65 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 250x200 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 7.2000 |  |  |  |  |  |
| 66 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pcs |  | 23.0000 |  |  |  |  |  |
| 67 |  | Company price |  | P350x100 supply air control grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 68 |  | Company price |  | P400x200 supply air control grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 69 |  | Company price |  | P400x150 supply air control grille |  |  | pc |  | 5.0000 |  |  |  |  |  |
| 70 |  | Company price |  | P500x150 supply air control grille |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 71 |  | Company price |  | P200x150 exhaust air control grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 72 |  | Company price |  | P250x100 adjustable exhaust grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 73 |  | Company price |  | P300x150 adjustable exhaust grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 74 |  | Company price |  | P400x150 adjustable exhaust grille |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 75 |  | Company price |  | air intake grille P500x500 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 76 |  | Company price |  | exhaust grille P400x300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 77 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry (Throttle valve) |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 78 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 79 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 80 |  | Company price |  | Throttle valve 100x250 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 81 |  | Company price |  | Throttle valve 150x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 82 |  | Company price |  | Throttle valve 250x200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 83 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 84 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 85 |  | VB19A |  | Air outlet, for type I air shower - -type hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 86 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 87 |  | IzH22A |  | Pipe insulation with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 78.0000 |  |  |  |  |  |
| 88 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | each |  | 4.0000 |  |  |  |  |  |
| 89 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 60.0000 |  |  |  |  |  |
| 90 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 3.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Ventilation PV 2 |  |  |  |  |  |  |  |  |  |  |
| 91 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet air ducts 100x100 mm , thickness 0.5 mm, class "H" |  |  | m2 |  | 4.2000 |  |  |  |  |  |
| 92 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a pitch of 600 mm. - Galvanised sheet metal air ducts 150x100 mm , thickness 0.5 mm, class "H" |  |  | m2 |  | 0.2500 |  |  |  |  |  |
| 93 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet air ducts 200x100 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 2.4000 |  |  |  |  |  |
| 94 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a pitch of 600 mm. - Galvanised sheet metal air ducts 200x150 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 6.3000 |  |  |  |  |  |
| 95 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 200x200 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 2.0000 |  |  |  |  |  |
| 96 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 250x100 mm , thickness 0.5 mm, class "H" |  |  | m2 |  | 1.0500 |  |  |  |  |  |
| 97 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 250x200 mm , thickness 0.5 mm, class "H" |  |  | m² |  | 18.9000 |  |  |  |  |  |
| 98 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x100 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.4000 |  |  |  |  |  |
| 99 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x300 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 22.8000 |  |  |  |  |  |
| 100 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x150 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 2.7500 |  |  |  |  |  |
| 101 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x200 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.6000 |  |  |  |  |  |
| 102 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x300 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 1.4000 |  |  |  |  |  |
| 103 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal with a perimeter of 1800 mm. - Galvanised sheet metal air ducts 500x150 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 3.9000 |  |  |  |  |  |
| 104 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Galvanised sheet metal air ducts 500x200 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 0.7000 |  |  |  |  |  |
| 105 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet metal air ducts 500x500 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 2.0000 |  |  |  |  |  |
| 106 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium 0.7 mm thick, with a circular cross-section diameter of: 500–560 mm. (circular cross-section) Air ducts made of galvanised sheet metal 315 mm , thickness 0.7 mm, class "H" |  |  | m² |  | 2.9670 |  |  |  |  |  |
| 107 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 100x100 mm , б=0.5 mm class "П" |  |  | m² |  | 1.6000 |  |  |  |  |  |
| 108 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet metal air ducts 200x150 mm , thickness 0.5 mm, class "П" |  |  | m² |  | 2.8000 |  |  |  |  |  |
| 109 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Galvanised sheet air ducts 250x200 mm, thickness 0.5 mm, class "H" |  |  | m² |  | 14.4000 |  |  |  |  |  |
| 110 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 25.0000 |  |  |  |  |  |
| 111 |  | Company price |  | R400x150 adjustable supply air grille |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 112 |  | Company price |  | P400x200 supply air control grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 113 |  | Company price |  | R500x150 supply air control grille |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 114 |  | Company price |  | P150x100 exhaust air control grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 115 |  | Company price |  | P200x150 adjustable exhaust grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 116 |  | Company price |  | P300x100 adjustable exhaust grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 117 |  | Company price |  | P400x150 adjustable exhaust grille |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 118 |  | Company price |  | air intake grille P500x500 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 119 |  | Company price |  | exhaust grille P400x300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 120 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( -type damper) |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 121 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 122 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 123 |  | Company price |  | Throttle valve 100x250 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 124 |  | Company price |  | Throttle valve 150×200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 125 |  | Company price |  | Throttle valve 250x200 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 126 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 127 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 128 |  | VB19A |  | Air outlet, for type I air shower - -type hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 129 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 130 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 85.0000 |  |  |  |  |  |
| 131 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 132 |  | IC42A |  | Supports and fasteners for supporting pipes, boilers, appliances and containers, weighing up to 2 kg per piece (Steel for pipe fastening St 3) |  |  | kg |  | 60.0000 |  |  |  |  |  |
| 133 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 3.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Ventilation PV 3 |  |  |  |  |  |  |  |  |  |  |
| 134 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet air ducts 100x100 mm, thickness 0.5 mm, class "H" |  |  | m2 |  | 14.4000 |  |  |  |  |  |
| 135 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 150x100 mm galvanised sheet, thickness 0.5 mm, class "H" |  |  | m2 |  | 1.0000 |  |  |  |  |  |
| 136 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 600 mm. - Air ducts made of 200x100 mm galvanised sheet metal, thickness 0.5 mm, class "H" |  |  | m² |  | 11.7000 |  |  |  |  |  |
| 137 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 200x150 mm galvanised sheet, thickness 0.5 mm, class "H" |  |  | m2 |  | 1.4000 |  |  |  |  |  |
| 138 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 200x200 mm galvanised sheet, thickness 0.5 mm, class "H" |  |  | m² |  | 2.0000 |  |  |  |  |  |
| 139 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 600 mm. - Air ducts made of 250x100 mm galvanised sheet metal, thickness 0.5 mm, class "H" |  |  | m² |  | 0.3500 |  |  |  |  |  |
| 140 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 250x200 mm galvanised sheet, thickness 0.5 mm, class "H" |  |  | m² |  | 21.6000 |  |  |  |  |  |
| 141 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 250x250 mm galvanised sheet, thickness 0.5 mm, class "H" |  |  | m2 |  | 3.1500 |  |  |  |  |  |
| 142 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal with a perimeter of up to 1200 mm. - Air ducts made of 300x150 mm galvanised sheet metal, thickness 0.7 mm, class "H" |  |  | m² |  | 0.9000 |  |  |  |  |  |
| 143 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 300x200 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 0.5000 |  |  |  |  |  |
| 144 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 300x250 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 2.2000 |  |  |  |  |  |
| 145 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal with a perimeter of up to 1200 mm. - Air ducts made of 300x300 mm galvanised sheet metal, thickness 0.7 mm, class "H" |  |  | m² |  | 18.6000 |  |  |  |  |  |
| 146 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 350x100 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 0.4500 |  |  |  |  |  |
| 147 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of galvanised sheet metal 350x300 mm, thickness 0.7 mm, class "H" |  |  | m² |  | 9.1000 |  |  |  |  |  |
| 148 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal with a perimeter of up to 1200 mm. - Air ducts made of 400x150 mm galvanised sheet metal, thickness 0.7 mm, class "H" |  |  | m² |  | 2.2000 |  |  |  |  |  |
| 149 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 400x200 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 0.6000 |  |  |  |  |  |
| 150 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 400x300 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 1.4000 |  |  |  |  |  |
| 151 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of 1800 mm. - Air ducts made of 500x150 mm galvanised sheet metal, thickness 0.7 mm, class "H" |  |  | m² |  | 3.5300 |  |  |  |  |  |
| 152 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Galvanised sheet air ducts 500x200 mm, thickness 0.7 mm, class "H" |  |  | m2 |  | 0.7000 |  |  |  |  |  |
| 153 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet air ducts 500x500 mm, thickness 0.7 mm, class "H" |  |  | m² |  | 2.0000 |  |  |  |  |  |
| 154 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet, with a circular cross-sectional diameter of 500–560 mm. (circular cross-section) Air ducts made of 315 mm galvanised sheet, thickness 0.7 mm, class "H" |  |  | m² |  | 1.4800 |  |  |  |  |  |
| 155 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | each |  | 27.0000 |  |  |  |  |  |
| 156 |  | Company price |  | P300x150 supply air control grille |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 157 |  | Company price |  | P350x100 supply air control grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 158 |  | Company price |  | P500x150 supply air control grille |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 159 |  | Company price |  | P250x100 exhaust air control grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 160 |  | Company price |  | P300x150 adjustable exhaust grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 161 |  | Company price |  | P400x150 supply air control grille |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 162 |  | Company price |  | air intake grille P500x500 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 163 |  | Company price |  | exhaust grille P400x300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 164 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( -type damper) |  |  | pc |  | 11.0000 |  |  |  |  |  |
| 165 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 166 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 167 |  | Company price |  | Throttle valve 250x200 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 168 |  | VB02A |  | (Installation only) Control damper, butterfly type, mounted on rectangular ducts of types CFR-I, CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 100x100 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 169 |  | Market price |  | -Fire damper 100x100 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 170 |  | VB02A |  | (Installation only) Adjustable damper, butterfly type, for installation on rectangular ducts of types CFR-I and CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 100×150 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 171 |  | Market price |  | -Fire damper 100x150 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 172 |  | VB02A |  | (Installation only) Control damper, butterfly type, mounted on rectangular ducts of types CFR-I, CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 100x120 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 173 |  | Market price |  | -Fire damper 100x200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 174 |  | VB02A |  | (Installation only) Control damper, butterfly type, mounted on rectangular ducts of types CFR-I, CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 200×250 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 175 |  | Market price |  | -Fire damper 200x250 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 176 |  | VB19A |  | Discharge outlet, for Type I air shower – PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 177 |  | Company price |  | PT 614 L35 Pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 178 |  | VB19A |  | Air outlet, for type I air shower - PT 614 L36 L37 air duct cleaning hatch ( ) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 179 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 180 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 95.0000 |  |  |  |  |  |
| 181 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 5.0000 |  |  |  |  |  |
| 182 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg each (Steel for securing pipelines St 3) |  |  | kg |  | 75.0000 |  |  |  |  |  |
| 183 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 3.9000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.5. Ventilation PV 4 |  |  |  |  |  |  |  |  |  |  |
| 184 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 200x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 6.9000 |  |  |  |  |  |
| 185 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 600 mm. - Air ducts made of 200x150 mm galvanised sheet metal, thickness 0.5 mm |  |  | m² |  | 2.4500 |  |  |  |  |  |
| 186 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet ( ) with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 200x200 mm, thickness 0.5 mm |  |  | m² |  | 2.8000 |  |  |  |  |  |
| 187 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised sheet metal or aluminium, with a perimeter of 800–1000 mm. - Air ducts made of 250x200 mm galvanised sheet metal, thickness 0.5 mm |  |  | m2 |  | 38.2500 |  |  |  |  |  |
| 188 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal, with a cross-sectional area of 800–1000 mm. - Galvanised sheet metal air ducts 250x250 mm, thickness 0.5 mm |  |  | m2 |  | 3.5000 |  |  |  |  |  |
| 189 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 300x150 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 0.4500 |  |  |  |  |  |
| 190 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 300x200 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 0.5000 |  |  |  |  |  |
| 191 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x300 mm, thickness 0.7 mm |  |  | m² |  | 5.4000 |  |  |  |  |  |
| 192 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 0.7 mm thick galvanised sheet, 400x150 mm |  |  | m2 |  | 3.3000 |  |  |  |  |  |
| 193 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 400x200 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 0.6000 |  |  |  |  |  |
| 194 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x300 mm, thickness 0.7 mm |  |  | m² |  | 28.7000 |  |  |  |  |  |
| 195 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 400x400 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 1.6000 |  |  |  |  |  |
| 196 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: from 1800 mm. - Air ducts made of 500x150 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 5.2000 |  |  |  |  |  |
| 197 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of 1800 mm. - Galvanised sheet metal air ducts 500x200 mm, thickness 0.7 mm |  |  | m2 |  | 0.7000 |  |  |  |  |  |
| 198 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. Air ducts made of 600x500 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 2.2000 |  |  |  |  |  |
| 199 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | each |  | 28.0000 |  |  |  |  |  |
| 200 |  | Company price |  | P300x150 adjustable supply air grille |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 201 |  | Company price |  | P500x150 supply air control grille |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 202 |  | Company price |  | P400x150 exhaust air control grille |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 203 |  | Company price |  | air intake grille R600x500 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 204 |  | Company price |  | exhaust grille P400x400 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 205 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( -type damper) |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 206 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 207 |  | Company price |  | Throttle valve 250x200 |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 208 |  | VB02A |  | (Installation only) Control damper, butterfly type, mounted on rectangular ducts of types CFR-I, CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 100x120 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 209 |  | Market price |  | -Fire damper 100x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 210 |  | VB02A |  | (Installation only) Adjustable damper, butterfly type, for installation on rectangular ducts of types CFR-I and CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 200×250 |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 211 |  | Market price |  | -Fire damper 200x250 |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 212 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 213 |  | Company price |  | PT 614 L35 Pitot tube |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 214 |  | VB19A |  | Air outlet, for type I air shower - PT 614 L36 L37 air duct cleaning hatch ( ) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 215 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 216 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 100.0000 |  |  |  |  |  |
| 217 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 218 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 60.0000 |  |  |  |  |  |
| 219 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 3.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV 4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.6. In 5 |  |  |  |  |  |  |  |  |  |  |
| 220 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised sheet metal or aluminium, with a perimeter of 600 mm. - Air ducts made of 100x100 mm galvanised sheet metal, thickness 0.5 mm |  |  | m2 |  | 1.0000 |  |  |  |  |  |
| 221 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 150x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.5000 |  |  |  |  |  |
| 222 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 150x150 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 3.0000 |  |  |  |  |  |
| 223 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal, with a perimeter of 600 mm. - Air ducts made of 200x100 mm galvanised sheet metal, thickness 0.5 mm |  |  | m² |  | 0.3000 |  |  |  |  |  |
| 224 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 200x150 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 4.9000 |  |  |  |  |  |
| 225 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 300x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.4000 |  |  |  |  |  |
| 226 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 227 |  | Company price |  | R200x100 supply air control grille |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 228 |  | Company price |  | P300x100 supply air control grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 229 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( valve) |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 230 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 231 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 232 |  | VB30A |  | General-purpose hood, types A, B, C, made from 0.5 mm thick black sheet metal (300 x 250 mm hood, 1 pc) |  |  | m |  | 0.2600 |  |  |  |  |  |
| 233 |  | VB19A |  | Air outlet, for type I air duct – PT 614 L35 pitot tube |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 234 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 235 |  | VB19A |  | Air outlet, for Type I air shower - Access hatch for cleaning PT 614 L36 L37 air ducts |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 236 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 237 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 7.5000 |  |  |  |  |  |
| 238 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 239 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 12.0000 |  |  |  |  |  |
| 240 |  | CN20B |  | Interior or exterior paint applied to metal joinery with two coats of alkyd enamel, including primer. Painting of fixings |  |  | m² |  | 0.6200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total В 5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.7. В6 |  |  |  |  |  |  |  |  |  |  |
| 241 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 100x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.8000 |  |  |  |  |  |
| 242 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 150x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.5000 |  |  |  |  |  |
| 243 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal, with a perimeter of 600 mm. - Air ducts made of 150x150 mm galvanised sheet metal, thickness 0.5 mm |  |  | m² |  | 6.3000 |  |  |  |  |  |
| 244 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 200x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.3000 |  |  |  |  |  |
| 245 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 300x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.4000 |  |  |  |  |  |
| 246 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 247 |  | Company price |  | R200x100 adjustable exhaust grille |  |  | pc |  | 5.0000 |  |  |  |  |  |
| 248 |  | Company price |  | P300x100 adjustable exhaust grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 249 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( valve) |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 250 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 251 |  | Company price |  | Throttle valve 100x200 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 252 |  | VB02A |  | (Installation only) Control damper, butterfly type, mounted on rectangular ducts of types CFR-I, CFR-II, with a perimeter of 1000–1600 mm - -Fire damper 100x150 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 253 |  | Market price |  | -Fire damper 100x150 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 254 |  | VB30A |  | General-purpose hood, types A, B, C, made from 0.5 mm thick black sheet metal (250x250 cover, 1 pc) |  |  | m² |  | 0.2100 |  |  |  |  |  |
| 255 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 256 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 257 |  | VB19A |  | Air outlet, for type I air shower - PT 614 L36 L37 air duct cleaning hatch ( ) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 258 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 259 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 3.5000 |  |  |  |  |  |
| 260 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | each |  | 1.0000 |  |  |  |  |  |
| 261 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg each (Steel for securing pipelines St 3) |  |  | kg |  | 9.0000 |  |  |  |  |  |
| 262 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 0.4700 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B6 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.8. ВЕ1-ВЕ10 |  |  |  |  |  |  |  |  |  |  |
| 263 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of 150x150 mm galvanised sheet, b=0.5 mm |  |  | m2 |  | 2.4000 |  |  |  |  |  |
| 264 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal, with a cross-sectional area of 800–1000 mm. - Galvanised sheet metal air ducts 200x200 mm, thickness 0.5 mm |  |  | m2 |  | 12.8000 |  |  |  |  |  |
| 265 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pcs |  | 10.0000 |  |  |  |  |  |
| 266 |  | Company price |  | R400x100 adjustable exhaust grille |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 267 |  | Company price |  | P500x150 adjustable exhaust grille |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 268 |  | VB30A |  | General-purpose extractor hood, types A, B, C, made from 0.5 mm thick black sheet metal (250 x 250 mm hood, 1 pc) |  |  | m2 |  | 0.2100 |  |  |  |  |  |
| 269 |  | VB30A |  | General-purpose hood, types A, B, C, made of 0.5 mm thick black sheet metal (300 x 300 mm, 9 pcs) |  |  | m2 |  | 2.7000 |  |  |  |  |  |
| 270 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 15.2000 |  |  |  |  |  |
| 271 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 272 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 47.0000 |  |  |  |  |  |
| 273 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 2.4400 |  |  |  |  |  |
| 274 |  | CL20A |  | Ready-made ventilation grilles made from black sheet metal, with manually adjustable louvres, painted and fitted into the masonry |  |  | pc |  | 22.0000 |  |  |  |  |  |
| 275 |  | Supplier price |  | PR300X150 supply air control grille for doors |  |  | pc |  | 22.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total BE1-BE10 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.9. Cold storage |  |  |  |  |  |  |  |  |  |  |
| 276 |  | IC31A |  | Copper pipe, welded, for connecting heating units and appliances in central heating systems, with an external diameter of up to 15.0 mm inclusive (Freon pipes D 6.35) |  |  | m |  | 31.5000 |  |  |  |  |  |
| 277 |  | IC31A |  | Copper pipe, welded, for connecting radiators and heating appliances in central heating systems, with an external diameter of up to 15.0 mm inclusive (Freon pipes D 12.7) |  |  | m |  | 31.5000 |  |  |  |  |  |
| 278 |  | IC31A |  | Copper pipe, fitted by welding, used to connect radiators and heating appliances in central heating systems, with an external diameter of up to and including 15.0 mm (Freon pipes D 9.52) |  |  | m |  | 10.5000 |  |  |  |  |  |
| 279 |  | IC31A |  | Copper pipe, fitted by welding, for connecting radiators and heating appliances in central heating systems, with an external diameter of up to 15.0 mm inclusive (Freon pipes D 15.88) |  |  | m |  | 10.5000 |  |  |  |  |  |
| 280 |  | IE07A |  | Pressure testing of installed gas pipes, including valves, pipes with a diameter of up to 1", inclusive |  |  | m |  | 84.0000 |  |  |  |  |  |
| 281 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes, b=13mm, for freon pipes ф6.35 |  |  | m |  | 31.5000 |  |  |  |  |  |
| 282 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes, b=13mm, for ф12.7 refrigerant pipes |  |  | m |  | 31.5000 |  |  |  |  |  |
| 283 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes b=13mm for freon pipes ф9.52 |  |  | m |  | 10.5000 |  |  |  |  |  |
| 284 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes b=13mm for freon pipes ф15.88 |  |  | m |  | 10.5000 |  |  |  |  |  |
| 285 |  | IC36C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed in columns in central heating systems, with an external diameter of 32.0 mm. Polypropylene pipe . ф32х3.0 |  |  | m |  | 55.0000 |  |  |  |  |  |
| 286 |  | IC36D |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed in columns in central heating systems, with an external diameter of 40.0 mm. Polypropylene pipe . ф40х3.7 |  |  | m |  | 20.0000 |  |  |  |  |  |
| 287 |  | IC36E |  | High-density reinforced polyethylene pipe or reinforced or unreinforced polypropylene pipe, installed in columns in central heating systems, with an external diameter of 50.0 mm. Polypropylene pipe . ф50х4.6 |  |  | m |  | 10.0000 |  |  |  |  |  |
| 288 |  | SF01C |  | Carrying out a pressure leak test on hot or cold water systems made of heavy-duty polyvinyl chloride (PVC) or plastic pipes with a diameter of 16–110 mm |  |  | m |  | 85.0000 |  |  |  |  |  |
| 289 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto the pipes - 9 mm porous polyethylene hose, 32 mm diameter PEP |  |  | m |  | 55.0000 |  |  |  |  |  |
| 290 |  | RpIF09A |  | Insulation of pipes with special insulation sleeves, fitted onto pipes, Porous polyethylene hose 9 mm, ф40 ПЭП |  |  | m |  | 20.0000 |  |  |  |  |  |
| 291 |  | RpIF09A |  | Insulation of pipes with special insulation sleeves fitted onto the pipes, 9 mm porous polyethylene hose, 50 mm diameter, PEP |  |  | m |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cold Supply Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.10. Dismantling |  |  |  |  |  |  |  |  |  |  |
| 292 |  | RpIB11A |  | Removal of radiators for decommissioning (Removal of radiators for decommissioning (10 sections, 41 installation points) |  |  | m2 |  | 122.5900 |  |  |  |  |  |
| 293 |  | RpIC19C |  | Cutting of black steel pipes for plumbing installations, fitted to central heating systems by welding, in residential and social/cultural buildings, for connecting appliances, distribution risers, etc., with diameters of 1 1/2"–2" (45 x 3.0 mm pipes) |  |  | m |  | 300.0000 |  |  |  |  |  |
| 294 |  | CL20A |  | Ready-made ventilation grilles removal К0,8 |  |  | pc |  | 23.0000 |  |  |  |  |  |
| 295 |  | RpVA35B |  | Dismantling of ventilation ducts made of black, galvanised or aluminium sheet, with a rectangular or circular cross-section perimeter of 700–1600 mm (Dismantling of air ducts made of black, galvanised or aluminium sheets, with a rectangular cross-section perimeter of 700–1600 mm) |  |  | m² |  | 64.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Dismantling Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction- e works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 296 |  | ID04A |  | Shut-off or check valve with connectors for central heating systems, with a nominal diameter of 1/2"-1" ASV-PV - Danfoss automatic balancing valve, 20 mm |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 297 |  | ID04B |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1 1/4"–1 1/2" - Manual balancing valve db=32 USV-1 Danfoss |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 298 |  | SE56A1 |  | Drinking water filter, with flanges for pipe mounting, size 1" - 2" - Screen filter Ф50 Danfoss |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 299 |  | SE56A1 |  | Drinking water filter, with flanges for pipe mounting, size 1" - 2" - 20 mm mesh filter Danfoss |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 300 |  | IA18B |  | Precision fittings for central heating boilers: Pressure gauge |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 301 |  | IA18A |  | Precision fittings for central heating boilers: Thermometer |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 302 |  | IA18A |  | Precision fittings for central heating boilers: thermomanometer |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 303 |  | IA18J |  | Precision fittings for central heating boilers: fitting with control valve for fittings Mounting bracket for thermomanometer |  |  | pc |  | 16.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.1. PV1 |  |  |  |  |  |  |  |  |  |  |
| 304 |  | VC16A |  | Installation of an ALTPA-type partial air handling unit within the premises, comprising air intake, filtration, ventilation, heating, cooling and air discharge, with a treated air flow rate of 3, ,000 – 7,500 m³/h (ПВ1 Supply and exhaust unit with heat recovery included: type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1,0000 |  |  |  |  |  |
| 305 |  | VB18A |  | Rectangular sound attenuator with perimeter 1000–2000 mm - Sound attenuator 300×300 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 306 |  | VB18A |  | Noise attenuator, rectangular with a perimeter of 1000–2000 mm - Noise attenuator 300×200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 307 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 300×300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 308 |  | AcB09A |  | Installation of electric actuators for valves with a diameter of 400–600 mm - Electric actuator for an air valve |  |  | each |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.2. PV2 |  |  |  |  |  |  |  |  |  |  |
| 309 |  | VC16A |  | Installation of a partial air handling unit, type ALTPA, within the premises, comprising suction, filtration, ventilation, heating, cooling and discharge, with a treated air flow rate of 3,000–7,500 m³/h ( ПВ1 Supply and exhaust unit with heat recovery included: type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1,0000 |  |  |  |  |  |
| 310 |  | VB18A |  | Rectangular sound attenuator with perimeter 1000–2000 mm - Sound attenuator 300×300 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 311 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 300×300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 312 |  | AcB09A |  | Installation of electric actuators for valves with diameters of 400– 600 mm - Electric actuator for air valve |  |  | each |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.3. PV3 |  |  |  |  |  |  |  |  |  |  |
| 313 |  | VC16A |  | Installation of a partial air handling unit, type ALTPA, within the premises, comprising suction, filtration, ventilation, heating, cooling and discharge, with a treated air flow rate of 3,000–7,500 m³/h ( ПВ1 Supply and exhaust unit with heat recovery included: type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 314 |  | VB18A |  | Rectangular sound attenuator with perimeter 1000–2000 mm - Sound attenuator 300×300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 315 |  | VB18A |  | Noise attenuator, rectangular with a perimeter of 1000–2000 mm - Noise attenuator 350×300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 316 |  | VB04A |  | Counterweight damper, with a perimeter of 800–1600 mm -Servo-operated air damper 300×300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 317 |  | AcB09A |  | Installation of electric actuators for valves, valves with a diameter of 400–600 mm - Electric actuator for air valve |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 318 |  | VB04A |  | Counterweight damper, with a perimeter of 800–1600 mm - Air damper with servo drive 350×300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 319 |  | AcB09A |  | Installation of electric actuators for dampers, with a diameter of 400–600 mm - Electric actuator for air damper |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.4. ПВ4 |  |  |  |  |  |  |  |  |  |  |
| 320 |  | VC16A |  | Installation of an ALTPA-type partial air handling unit within the premises, comprising air intake, filtration, ventilation, heating, cooling and air discharge, with a treated air flow rate of 3,000–7,500 m³/h (ПВ1 Supply and exhaust unit with heat recovery included: type Duplex 2500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 321 |  | VB18A |  | Rectangular sound attenuator with perimeter 1000–2000 mm - Sound attenuator 300×400 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 322 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 300×400 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 323 |  | AcB09A |  | Installation of electric actuators for dampers, dampers with a diameter of 400–600 mm - Electric actuator for air damper |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.5. В5 |  |  |  |  |  |  |  |  |  |  |
| 324 |  | VC01B |  | Installation of centrifugal fans, with a total weight of 50–400 kg, mounted on a metal frame – ВКМц 200 VENTS |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 325 |  | VC24A |  | Elastic fan mounting, with rubber elastic mounts - Soft insert ф200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 326 |  | VB17A |  | AZC-type circular silencer with a circumference of 800–1600 mm ( ф200) |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total В5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.6. В 6 |  |  |  |  |  |  |  |  |  |  |
| 327 |  | VC01B |  | Installation of centrifugal fans, with a total weight of 50–400 kg, mounted on a metal frame -ВКМц 200 VENTS |  |  | pcs |  | 1.0000 |  |  |  |  |  |
| 328 |  | VC24A |  | Elastic fan support, with rubber elastic mounts -Soft insert ф200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 329 |  | VB17A |  | AZC-type circular silencer with a circumference of 800–1600 mm ( ф200) |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B 6 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2.7. BE1–BE 10 |  |  |  |  |  |  |  |  |  |  |
| 330 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry (Hydraulically controlled air intake unit with sound insulation in the upper frame of the E-EMM2 double-glazed unit) |  |  | pc |  | 56.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total BE1-BE 10 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Refrigeration |  |  |  |  |  |  |  |  |  |  |
| 331 |  | VC37G |  | Installation of domestic air conditioning units (split-system) with a motor power of up to 12.5 kW, on the staircase (Gree SMART GWH18QD-K6DNB61 outdoor compressor-condenser unit) Gree |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 332 |  | VC37G |  | Installation of domestic air conditioning units (split systems) with a motor power of up to 12.5 kW, on the staircase (Gree SMART GWH24QD-K6DNB61 outdoor compressor-condenser unit) Gree |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Refrigeration Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 333 |  | Supplier price |  | Manual balancing valve db=32 USV-1 Danfoss |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 334 |  | Company price |  | Automatic balancing valve DN 20 Type ASV-PV-20 "Danfoss" Austria |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 335 |  | Company price |  | Danfoss F50 strainer |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 336 |  | Company price |  | Danfoss F20 mesh filter |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 337 |  | Company price |  | Pressure gauge |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 338 |  | Company price |  | Thermometer |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 339 |  | Company price |  | thermomanometer |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 340 |  | Company price |  | Mounting bracket for the e thermomanometer |  |  | pc |  | 16.0000 |  |  |  |  |  |
| 341 |  | Company price |  | 2000W electric convector heater with temperature control ATLANTIC |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Ventilation PV1 |  |  |  |  |  |  |  |  |  |  |
| 342 |  | Company price |  | PV1 Supply and exhaust unit with heat recovery included: Supply fan L=1260 m³/h; N=0.34 kW. U=230 V 50 Hz : Exhaust fan L=890 m³/h. N=0.17 kW, U=230 V, 50 Hz, soft inserts, F7 filter. M5 filter: Counter-flow heat recovery unit: 1.8 kW electric heater: Direct evaporation air heater/cooler, condensate collection tray, type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 343 |  | Company price |  | Sound absorber 300x300 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 344 |  | Company price |  | Silencer 300x200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 345 |  | Company price |  | Servo-operated air damper 300x300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Ventilation PV2 |  |  |  |  |  |  |  |  |  |  |
| 346 |  | Company price |  | PV2 Supply and exhaust unit with heat recovery included: Supply fan L=1450 m³/h; N=0.46 kW; U=230 V 50 Hz : Exhaust fan L=1175 m³/h. N=0.27 kW, U=230 V, 50 Hz, soft inserts, F7 filter. M5 filter: Counter-flow heat recovery unit: 1.9 kW electric heater: Direct evaporation air heater/cooler, condensate collection tray, type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 347 |  | Company price |  | Sound absorber 300x300 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 348 |  | Company price |  | Servo-operated air damper 300x300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. Ventilation PV3 |  |  |  |  |  |  |  |  |  |  |
| 349 |  | Company price |  | PV3 Supply and exhaust unit with heat recovery included: Supply fan L=1445 m³/h; N=0.46 kW. U=230 V 50 Hz : Exhaust fan L=1220 m³/h. N=0.29 kW, U=230 V, 50 Hz, soft inserts, F7 filter. M5 filter: Counter-flow heat recovery unit: 1.9 kW electric heater: Direct evaporation air heater/cooler, condensate collection tray, type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 350 |  | Company price |  | 300x300 silencer |  |  | each |  | 2.0000 |  |  |  |  |  |
| 351 |  | Company price |  | Silencer 350x300 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 352 |  | Company price |  | Servo-operated air damper 300x300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 353 |  | Company price |  | Servo-operated air valve 350x300 |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.5. Ventilation PV4 |  |  |  |  |  |  |  |  |  |  |
| 354 |  | Company price |  | PV4 Supply and exhaust unit with heat recovery included: Supply fan L=1670 m³/h; N=0.42 kW. U=400 V 50 Hz : Exhaust fan L=1500 m³/h. N=0.31 kW, U=400 V, 50 Hz, soft inserts, F7 filter. M5 filter: Counter-flow heat recovery unit: 3.5 kW electric heater: Direct evaporation air heater/cooler, condensate collection tray, type Duplex 2500 Multi "ARTREA" |  |  | set |  | 1.0000 |  |  |  |  |  |
| 355 |  | Company price |  | Silencer 300x400 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 356 |  | Company price |  | Servo-operated air damper 300x400 |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.6. In 5 |  |  |  |  |  |  |  |  |  |  |
| 357 |  | Company price |  | Extractor fan L=385 m³/h. N=0.69 kW. U=230 V 50 Hz VKMc 200 VENTS |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 358 |  | Company price |  | Soft inserts ф200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 359 |  | Company price |  | Silencer ф200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total In 5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.7. В 6 |  |  |  |  |  |  |  |  |  |  |
| 360 |  | Company price |  | Extractor fan L=335 m³/h. N=0.69 kW. U=230 V 50 Hz VKMts 200 VENTS |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 361 |  | Company price |  | Soft inserts f200 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 362 |  | Company price |  | Silencer ф200 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 363 |  | Company price |  | Hydraulically adjustable air intake device with noise reduction in the upper frame of the double-glazed unit E-EMM2 BE1-BE0 |  |  | pc |  | 56.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total In 6 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.8. Cold storage |  |  |  |  |  |  |  |  |  |  |
| 364 |  | Company price |  | SMART Gree GWH18QD-K6DNB61 outdoor compressor-condenser unit, Qx=5.2 kW, Qt=5.6 kW, Nу=1.5 kW, U=230 V, GWH18QD-R6DNB61 Gree |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 365 |  | Company price |  | Gree SMART GWH24QD-K6DNB61 outdoor compressor-condenser unit, Qx=7.1 kW, Qt=7.8 kW, Nu=2.0 kW, , U=230V, GWH24QD-R6DNB61 Gree |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cold Supply Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |