| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 2-1-4

e and internal electrical networks    . Block  of study rooms

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
| 1 |  | TsA16A1 |  | Manual excavation of soil, in confined spaces, in trenches up to 4 m deep, for high-voltage electrical cables, in soil with natural moisture without shoring, width < 1 m, depth < 1.5 m, soft ground |  |  | m3 |  | 1.9000 |  |  |  |  |  |
| 2 |  | TsD01B |  | Spreading of sifted soil with a spade, in uniform layers 10–30 cm thick, by throwing from piles up to 3 m away, including breaking up clods, soil originating from medium-grade land |  |  | m3 |  | 1.9000 |  |  |  |  |  |
| 3 |  | TsD04A |  | Compaction with a hand tamper of backfill in horizontal or 1/4-inclined trenches, including wetting each individual layer of soil, 10 cm thick, non-cohesive soil |  |  | m3 |  | 1.9000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. PDG |  |  |  |  |  |  |  |  |  |  |
| 4 |  | 08-03-572-8 |  | Control cabinet or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (Metal distribution board with IP31 protection rating and dimensions 800(h)x650x250mm ЩМП-4-0 36 УХЛ3 IP31) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 5 |  | 08-03-521-15 |  | Lever-operated switch on a panel with central or side lever or bar operation, mounted on a metal bracket, three-pole, current, up to 250 A (Load break switch ВР32И-31/3P/100A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 6 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load break switch QF/3P/C25A) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 7 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load separator QF/3P/C20A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 8 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/C10A circuit breaker) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 9 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker QF/1P/B10A) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 10 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (RBD 160A rail-mounted distribution unit) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 11 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Rail set <N/PE> (copper, size 2)) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 12 |  | Supplier price |  | Rail set <N/PE> (copper, size 2), L=218mm |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 13 |  | Supplier price |  | SM-35 bushing insulator |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 14 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (DIN mounting rail 35x1mm 1 m) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 15 |  | Supplier price |  | DIN mounting rail 35x1mm |  |  | m |  | 1.0000 |  |  |  |  |  |
| 16 |  | Supplier price |  | Y-134 cable label |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 17 |  | Supplier price |  | "Danger of electric shock" sticker A5 148 x 210 mm yellow |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDG Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. PDI1 |  |  |  |  |  |  |  |  |  |  |
| 18 |  | 08-03-572-8 |  | Cabinet-type control and execution unit or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (VoltaV U60AT recessed distribution board, IP30) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 19 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load-break switch QS/3P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 20 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/B10A circuit breaker) |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 21 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker QF/1P/B6A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 22 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/B10A/30mА) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 23 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/C16A/10mА) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 24 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic residual current circuit breaker AVDT/2P/C16A/30mA) |  |  | pcs |  | 11.0000 |  |  |  |  |  |
| 25 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting preparation (KT/1P/16A time relay) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 26 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (modular contactor KM/4P/25A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 27 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM/2P/25A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 28 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (RBD 80A rail-mounted distribution block) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 29 |  | Supplier price |  | Rail <N>, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 30 |  | Supplier price |  | <PE> rail, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 31 |  | Supplier price |  | "Danger of electric shock" sticker A5 148 x 210 mm yellow |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDI1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. PDI2 |  |  |  |  |  |  |  |  |  |  |
| 32 |  | 08-03-572-8 |  | Cabinet-type control panel or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (VoltaV U60AT recessed distribution board, IP30) |  |  | pcs |  | 1.0000 |  |  |  |  |  |
| 33 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load-break switch QS/3P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 34 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/B10A circuit breaker) |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 35 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/C16A/10mА) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 36 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/C16A/30mА) |  |  | pc |  | 15.0000 |  |  |  |  |  |
| 37 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (RBD 80A rail-mounted distribution block) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 38 |  | Supplier price |  | Rail <N>, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 39 |  | Supplier price |  | <PE> rail, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 40 |  | Supplier price |  | "Danger of electric shock" sticker A5 148 x 210 mm yellow |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDI2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. PDV |  |  |  |  |  |  |  |  |  |  |
| 41 |  | 08-03-572-8 |  | Cabinet-type control panel or distribution point (cabinet), recessed, height and width, mm, up to 700×850 ( VoltaU36AT recessed distribution board, IP30) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 42 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load-break switch QS/3P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 43 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/B6A circuit breaker) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 44 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM/4P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 45 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM/2P/25A) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 46 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (circuit breakers) (QF/1P/C32A circuit breaker) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 47 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker QF/1P/C16A) |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 48 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/C4A circuit breaker) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 49 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker QF/1P/C2A) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 50 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (RBD 80A rail-mounted distribution block) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 51 |  | Supplier price |  | Rail <N>, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 52 |  | Supplier price |  | <PE> rail, 12 holes |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 53 |  | Supplier price |  | "Danger of electric shock" sticker A5 148 x 210 mm yellow |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total POS Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Set of cables and pipes |  |  |  |  |  |  |  |  |  |  |
| 54 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 26.0000 |  |  |  |  |  |
| 55 |  | Market price |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 2x1.5 mm² - 0.6 kV |  |  | m |  | 15.0000 |  |  |  |  |  |
| 56 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 3x1.5 mm² - 0.6 kV |  |  | m |  | 1,250.0000 |  |  |  |  |  |
| 57 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 3x2.5 mm² - 0.6 kV |  |  | m |  | 1,200.0000 |  |  |  |  |  |
| 58 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 3x6.0 mm² - 0.6 kV |  |  | m |  | 35.0000 |  |  |  |  |  |
| 59 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 5x6.0 mm² - 0.6 kV |  |  | m |  | 100.0000 |  |  |  |  |  |
| 60 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and pre-installed boxes, weight per 1 m up to: 1 kg |  |  | 100 m |  | 2.45 |  |  |  |  |  |
| 61 |  | Market price |  | Fire-resistant copper-core cable that does not propagate fire, with low smoke and gas emission ( ) and low toxicity of combustion products – TU 16-705.496-2011, GOST 31996-2012: VVGng-FRLSLTx 2x1.5 mm² – 0.6 kV |  |  | m |  | 10.0000 |  |  |  |  |  |
| 62 |  | Market price for |  | Fire-resistant copper-core cable that does not propagate fire, with low smoke and gas emission, and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-FRLSLTx 3x1.5 mm² - 0.6 kV |  |  | m |  | 200.0000 |  |  |  |  |  |
| 63 |  | Price from market |  | Fire-resistant copper-core cable that does not propagate fire, with low smoke and gas emission and low toxicity of combustion products: TU 16-705.496-2011, GOST 31996-2012: VVGng-FRLSLTx 3x2.5 mm² - 0.6 kV |  |  | m |  | 35.0000 |  |  |  |  |  |
| 64 |  | 08-02-405-1 |  | Conductor for steel structures and prefabricated panels, cross-section up to 16 mm² |  |  | 100 m |  | 0.8000 |  |  |  |  |  |
| 65 |  | Market price |  | Copper-wired cable, PVC-insulated, GOST 6323-79, cross-section 1x6mm² ПВ-3 |  |  | m |  | 60.0000 |  |  |  |  |  |
| 66 |  | Price from market |  | Copper-wired cable, PVC-insulated, GOST 6323-79, cross-section 1x16mm² ПВ-3 |  |  | m |  | 20.0000 |  |  |  |  |  |
| 67 |  | 08-02-409-1 |  | Vinyl-plastic pipe for building installations, on walls and columns, clamped in place, diameter up to 25 mm |  |  | 100 m |  | 22.2000 |  |  |  |  |  |
| 68 |  | Market price |  | PVC pipe, TU 2248-002-14369938-2008, diameter D16 mm |  |  | m |  | 2,130.0000 |  |  |  |  |  |
| 69 |  | Price from market |  | PVC pipe, TU 2248-002-14369938-2008 with diameter, D25mm |  |  | m |  | 90,0000 |  |  |  |  |  |
| 70 |  | 08-02-411-1 |  | Metal hose, outer diameter up to 48 mm |  |  | 100 m |  | 1.0000 |  |  |  |  |  |
| 71 |  | Market price for |  | PVC-insulated metal hose, РЗ-ЦП: with a diameter of 18 mm |  |  | m |  | 100.0000 |  |  |  |  |  |
| 72 |  | 08-02-407-1 |  | Steel pipe for wall-mounted installations, clamp-fixed, diameter up to 25 mm |  |  | 100 m |  | 1.6000 |  |  |  |  |  |
| 73 |  | Market price |  | Longitudinally welded steel pipe, GOST 10704-91, diameter D18 mm |  |  | m |  | 140.0000 |  |  |  |  |  |
| 74 |  | Price from market |  | Longitudinally welded steel pipe, GOST 10704-91, diameter D25 mm |  |  | m |  | 20,0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Set of cables and pipes Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. Lighting fixtures and lighting |  |  |  |  |  |  |  |  |  |  |
| 75 |  | 08-03-594-17 |  | Lighting fixture with fluorescent lamps in suspended ceilings, fitted in recessed units, number of lamps in the fixture: 4 |  |  | 100 pcs |  | 1.4100 |  |  |  |  |  |
| 76 |  | Supplier price |  | LED luminaire (recessed) 600x600mm, 4000K, 3400lm, 28W , IP20 RC065B G5 34S/840 PSD W60L60 OC SC |  |  | pcs |  | 130.0000 |  |  |  |  |  |
| 77 |  | Supplier price |  | LED luminaire (recessed) 595x595mm, 4000K, 3800lm, 34W, IP54 OWP OPTIMA LED 595 |  |  | pc |  | 11.0000 |  |  |  |  |  |
| 78 |  | 08-03-593-6 |  | Lighting fixture for recessed ceiling or wall lamps with screw mounting for rooms with normal environmental conditions, single-lamp |  |  | 100 pcs |  | 0.2400 |  |  |  |  |  |
| 79 |  | Supplier price |  | Round LED recessed light fitting, 4000K, 1350 lm, 12 W, IP44, Eglo Fueva 5 |  |  | pc |  | 24.0000 |  |  |  |  |  |
| 80 |  | 08-03-594-1 |  | Lighting fixture with fluorescent lamps mounted separately on pivots, number of lamps in the fixture, 1 |  |  | 100 pcs |  | 0.0700 |  |  |  |  |  |
| 81 |  | Supplier price |  | LED luminaire (wall-mounted) 1200mm, 4000K, 1700lm, 18W, IP20 ASM/S LED 1200 SCHOOL |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 82 |  | 08-03-594-6 |  | Lighting fixture with fluorescent lamps mounted separately on suspensions (bars), number of lamps in the fixture, 1 |  |  | 100 pcs |  | 0.0400 |  |  |  |  |  |
| 83 |  | Supplier price |  | LED luminaire (surface-mounted) 1000mm, 3000K, 5000lm, 40W, IP66 PowMr LBLWLW040 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 84 |  | 08-03-593-6 |  | Lighting fixture for recessed ceiling or wall lamps with screw mounting for rooms with normal environmental conditions, single-lamp |  |  | 100 pcs |  | 0.1200 |  |  |  |  |  |
| 85 |  | Supplier price |  | LED luminaire (surface-mounted) 4000K, 1800lm, 18W, IP65, PL-13272P |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 86 |  | 08-03-593-10 apl |  | Wall-mounted lighting indicators |  |  | 100 pcs |  | 0.0700 |  |  |  |  |  |
| 87 |  | Supplier price |  | 8W exit sign, IP20 |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 88 |  | 08-03-593-10 apl |  | Wall-mounted lighting indicators ( lighting unit, emergency lighting) |  |  | 100 pcs |  | 0.1500 |  |  |  |  |  |
| 89 |  | Supplier price |  | LED emergency lighting fixture, 10W |  |  | pc |  | 15.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Lighting fixtures Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.7. Sockets and light s |  |  |  |  |  |  |  |  |  |  |
| 90 |  | 08-03-591-2 |  | Single-pole flush-mounted switch, for concealed installation |  |  | 100 pcs |  | 0.2600 |  |  |  |  |  |
| 91 |  | Supplier price |  | Single-pole staircase switch 10A, 250V for flush mounting, IP20 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 92 |  | Supplier price |  | Single-pole switch 10A, 250V for flush mounting, IP20 |  |  | pc |  | 21.0000 |  |  |  |  |  |
| 93 |  | Supplier price |  | Single-pole switch 10A, 250V for flush mounting, IP54 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 94 |  | 08-03-591-5 |  | Two-pole switch, surface-mounted type, for enclosed circuits |  |  | 100 pcs |  | 0.0900 |  |  |  |  |  |
| 95 |  | Supplier price |  | -pole switch, 10A, 250V, for flush mounting, IP20 |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 96 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (Motion sensor) |  |  | pc |  | 29.0000 |  |  |  |  |  |
| 97 |  | Supplier price |  | IP20 motion sensor |  |  | pc |  | 29.0000 |  |  |  |  |  |
| 98 |  | 08-03-591-9 |  | Surface-mounted socket, for flush-mounted installation |  |  | 100 pcs |  | 1.8200 |  |  |  |  |  |
| 99 |  | Supplier price |  | socket for flush-mounted, Unom=250V, Inom=16A, protection rating IP20 16A / 230V / 2P+PE IP20 |  |  | pc |  | 182.0000 |  |  |  |  |  |
| 100 |  | 08-03-591-8 |  | Surface-mounted socket, for open installation |  |  | 100 pcs |  | 0.1200 |  |  |  |  |  |
| 101 |  | Supplier price |  | -type socket for , surface-mounted, Unom=250V, Inom=16A, IP54 protection rating 16A / 230V / 2P+PE IP54 |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 102 |  | Supplier price |  | У -197 УХЛ4 junction box for concealed wiring |  |  | pc |  | 220.0000 |  |  |  |  |  |
| 103 |  | Supplier price |  | Box for mounting switches and sockets У -196 УХЛ4 |  |  | pc |  | 220.0000 |  |  |  |  |  |
| 104 |  | Supplier price |  | Junction box for open wiring 80x80x40mm, IP54 |  |  | pc |  | 20.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sockets and Switches Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.8. Earth socket |  |  |  |  |  |  |  |  |  |  |
| 105 |  | 08-02-471-4 |  | Earth terminal, vertical, made of round steel, diameter 20 mm |  |  | 10 pcs |  | 0.3000 |  |  |  |  |  |
| 106 |  | 08-02-472-7 |  | Open earthing conductor, on structural supports, made of split steel, cross-section 160 mm² (40 x 4 mm steel strip) |  |  | 100 m |  | 0.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Earth connection Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. PDG |  |  |  |  |  |  |  |  |  |  |
| 107 |  | Supplier price |  | Metal distribution board with IP31 protection rating and dimensions 800(h) x 650 x 250 mm ЩМП-4-0 36 УХЛ3 IP31 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 108 |  | Supplier price |  | Load break switch 3P, In=100A, Un=400V ВР32И-31/3P/100A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 109 |  | Supplier price |  | Circuit breaker, 3P, characteristic C, In=25A, Icn=6kA QF/3P/С25A |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 110 |  | Supplier price |  | Circuit breaker, 3P, characteristic C, In=20A, Icn=6kA QF/3P/C20A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 111 |  | Supplier price |  | Circuit breaker, 1P, characteristic C, In=10A, Icn=6kA QF/1P/C10A |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 112 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=10A, Icn=6kA QF/1P/B10A |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 113 |  | Supplier price |  | DIN rail distribution block (2х7) РБД-160А |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDG Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. PDI1 |  |  |  |  |  |  |  |  |  |  |
| 114 |  | Supplier price |  | Flush-mounted distribution board for the installation of modular devices up to 60 modules, IP40 VoltaV U60AT, IP30 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 115 |  | Supplier price |  | -load separator 3P, In=40A, Un=400V QS/3P/40A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 116 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=10A, Icn=6kA QF/1P/B10A |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 117 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=6A, Icn=6kA QF/1P/B6A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 118 |  | Supplier price |  | Residual current circuit breaker, type B, 2P, Inom=10A, Idif=30mA. AVDT/2P/B10A/30mA |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 119 |  | Supplier price |  | Residual current circuit breaker, type B, 2-pole, rated current = 16A, residual current = 10mA. RCCB/2P/C16A/10mA |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 120 |  | Supplier price |  | Residual current circuit breaker, type B, 2-pole, rated current 16A, residual current 30mA. RCCB/2P/16A/30mA |  |  | pc |  | 11.0000 |  |  |  |  |  |
| 121 |  | Supplier price |  | Time relay, 230V/16A KT/1P/16A |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 122 |  | Supplier price |  | Modular contactor, 4P, In=25A KM/4P/25A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 123 |  | Supplier price |  | Modular contactor, 2P, In=25A KM/2P/25A |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 124 |  | Supplier price |  | DIN rail distribution block (2×7) RBD 80A |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDI1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. PDI2 |  |  |  |  |  |  |  |  |  |  |
| 125 |  | Supplier price |  | Flush-mounted distribution board for the installation of modular devices up to 60 modules, IP40 VoltaV U60AT, IP30 |  |  | each |  | 1.0000 |  |  |  |  |  |
| 126 |  | Supplier price |  | 3P load break switch, In=40A, Un=400V QS/3P/40A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 127 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=10A, Icn=6kA QF/1P/B10A |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 128 |  | Supplier price |  | Residual current circuit breaker, type B, 2P, Inom=16A, Idif=10mA. AVDT/2P/C16A/10mA |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 129 |  | Supplier price |  | Residual current circuit breaker, type B, 2-pole, rated current 16A, residual current 30mA. RCCB/2P/16A/30mA |  |  | pc |  | 15.0000 |  |  |  |  |  |
| 130 |  | Supplier price |  | DIN rail distribution block (2x7) RBD 80A |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDI2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. PDV |  |  |  |  |  |  |  |  |  |  |
| 131 |  | Supplier price |  | Flush-mounted distribution board for the installation of modular devices up to 36 modules, IP40 VoltaVU36AT, IP30 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 132 |  | Supplier price |  | 3P load break switch, In=40A, Un=400V QS/3P/40A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 133 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=6A, Icn=6kA QF/1P/B6A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 134 |  | Supplier price |  | Modular contactor, 4P, In=40A KM/4P/40A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 135 |  | Supplier price |  | Modular contactor, 2-pole, In=25A KM/2P/25A |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 136 |  | Supplier price |  | Circuit breaker, 1P, C-type, In=32A, Icn=6kA QF/1P/С32A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 137 |  | Supplier price |  | Circuit breaker, 1P, C-type, In=16A, Icn=6kA QF/1P/С16A |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 138 |  | Supplier price |  | Circuit breaker, 1P, C-type, In=4A, Icn=6kA QF/1P/С4A |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 139 |  | Supplier price |  | Circuit breaker, 1P, C-type, In=2A, Icn=6kA QF/1P/С2A |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 140 |  | Supplier price |  | DIN rail distribution block (2x7) RBD 80A |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total VAT Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |