| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 2-1-8

-study block. Design

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Ceilings |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1.1. Ground floor |  |  |  |  |  |  |  |  |  |  |
| 1 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m high: ceilings without insulation (moisture-resistant plasterboard – GTS) |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 2 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 3 |  | CF52B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mix, on the ceiling, with mortar prepared by hand |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 4 |  | CF53B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture, on the ceiling, with mortar prepared by hand. Additional or reduced amount for every 1.0 mm (deduct 3 mm from item CF52, K=3) |  |  | m² |  | -21.5000 |  |  |  |  |  |
| 5 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 6 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 21.5000 |  |  |  |  |  |
| 7 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 8 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m² |  | 21.5000 |  |  |  |  |  |
| 9 |  | CK29F apl |  | Suspended ceilings made of prefabricated panels, including the grid system (Rockfon Artic E25 painted ceiling tile 600x600x15 mm) |  |  | m2 |  | 341.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground Floor Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1.2. First floor |  |  |  |  |  |  |  |  |  |  |
| 10 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal frame, up to 4 m high: ceilings without insulation (moisture-resistant plasterboard – GTS) |  |  | m2 |  | 11.3000 |  |  |  |  |  |
| 11 |  | СF59D |  | Cladding of surfaces with a layer of PGC with the construction of a simple flat metal frame, up to 4 m high: ceilings without insulation |  |  | m² |  | 25.1000 |  |  |  |  |  |
| 12 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 36.4000 |  |  |  |  |  |
| 13 |  | CF52B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture on the ceiling, with mortar prepared by hand |  |  | m2 |  | 36.4000 |  |  |  |  |  |
| 14 |  | CF53B |  | 5 mm thick interior plasterwork, applied by hand using a dry gypsum-based mix on the ceiling; mortar prepared by hand. Allowance for deviation of plus or minus 1.0 mm (3 mm deduction under item CF52, K=3) |  |  | m² |  | -36.4000 |  |  |  |  |  |
| 15 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 36.4000 |  |  |  |  |  |
| 16 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 36.4000 |  |  |  |  |  |
| 17 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 36.4000 |  |  |  |  |  |
| 18 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m2 |  | 36.4000 |  |  |  |  |  |
| 19 |  | CK29F apl |  | Suspended ceilings made of prefabricated panels, including the grid system (Rockfon Artic E25 painted ceiling tile 600x600x15 mm) |  |  | m2 |  | 323.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ceilings Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Floors |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2.1. Ground floor: ceramic tiles ( ) |  |  |  |  |  |  |  |  |  |  |
| 20 |  | IzF53A |  | Manual installation of floor underlay with thermal insulation layer made of extruded polystyrene foam boards, 40 mm thick, in a single layer |  |  | m2 |  | 33.0000 |  |  |  |  |  |
| 21 |  | CG01A |  | Floor screed made of 3 cm thick M 100-T cement mortar with a finely trowelled finish |  |  | m2 |  | 33.0000 |  |  |  |  |  |
| 22 |  | CG01A1 |  | Sub-base layer for flooring made of 3 cm thick M 100-T cement mortar with a finely trowelled finish. For every 0.5 cm deviation (either way) from the 3 cm M 100-T mortar sub-base layer, add 2 cm, K=4 |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 23 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 100x100x4 - 1.75 kg/m²) |  |  | kg |  | 57.7500 |  |  |  |  |  |
| 24 |  | IzF04A |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including flashings and valleys in the existing waterproofing on horizontal or sloping surfaces up to 40% gradient, flat or curved, using bitumen mastic applied with a brush or rubber trowel (trowel) |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 25 |  | CG47D |  | Ceramic tile flooring, including the adhesive backing layer (dry mix), tile dimensions: 900 x 900 mm (porcelain tile backing - 6 mm; tile dimensions 900x900 mm (Indic Marengo 90x90 porcelain tile) |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 26 |  | CI24A |  | Tiling of steps with bonded ceramic granite tiles, less than 15 mm thick (ARK IVORY MATT 20MM 60x60 floor tiles) |  |  | m² |  | 12.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground floor tiles ceramic Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2.2. Ground floor linoleum |  |  |  |  |  |  |  |  |  |  |
| 27 |  | IzF53A |  | Manual installation of the floor substrate with a thermal insulation layer of extruded polystyrene foam boards, 40 mm thick, in a single layer |  |  | m² |  | 313.0000 |  |  |  |  |  |
| 28 |  | CG01A |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish |  |  | m2 |  | 313.0000 |  |  |  |  |  |
| 29 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm of deviation (either way) in the thickness of the M 100-T mortar screed, add 2 cm, K=4 |  |  | m2 |  | 313.0000 |  |  |  |  |  |
| 30 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 100x100x4 - 1.75 kg/m²) |  |  | kg |  | 547.7500 |  |  |  |  |  |
| 31 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical linoleum), on floors (Eminent DUSTY WHITE homogeneous polyvinyl chloride (PVC) flooring) (add the perimeter length of the rooms, with the material raised up the wall to a height: H=100 mm) |  |  | m² |  | 166.0000 |  |  |  |  |  |
| 32 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical linoleum) (Eminent DUSTY YELLOW homogeneous polyvinyl chloride (PVC) flooring) (add the perimeter length of the rooms, with the material raised up the wall to a height of H=100 mm) |  |  | m² |  | 147.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground floor linoleum Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2.3. First floor , tiles , ceramic |  |  |  |  |  |  |  |  |  |  |
| 33 |  | IzF53A |  | Manual installation of floor underlay with thermal insulation layer made of extruded polystyrene foam boards, 40 mm thick, in a single layer |  |  | m2 |  | 40.5000 |  |  |  |  |  |
| 34 |  | CG01A |  | Floor screed made of 3 cm thick M 100-T cement mortar with a finely trowelled finish |  |  | m2 |  | 40.5000 |  |  |  |  |  |
| 35 |  | CG01A1 |  | Sub-base for flooring made of 3 cm thick M 100-T cement mortar with a finely trowelled finish. For every 0.5 cm deviation (either way) from the 3 cm M 100-T mortar sub-base, add 2 cm, K=4 |  |  | m² |  | 40.5000 |  |  |  |  |  |
| 36 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 100x100x4 - 1.75 kg/m²) |  |  | kg |  | 70.8800 |  |  |  |  |  |
| 37 |  | IzF04A |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including flashings and valleys in the existing waterproofing on horizontal or sloping surfaces up to 40% gradient, flat or curved, using bitumen mastic applied with a brush or rubber trowel (trowel) |  |  | m² |  | 40.5000 |  |  |  |  |  |
| 38 |  | CG47D |  | Ceramic tile flooring, including the adhesive backing layer (dry mix), tile dimensions: 900 x 900 mm (porcelain tile backing - 6 mm; tile dimensions 900 x 900 mm (Indic Marengo 90x90 porcelain tile) |  |  | m² |  | 25.0000 |  |  |  |  |  |
| 39 |  | CI24A |  | Tiling of steps with bonded ceramic granite tiles, less than 15 mm thick (ARK IVORY MATT 20MM 60x60 floor tiles) |  |  | m² |  | 15.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor tiles ceramic Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2.4. Floor: linoleum |  |  |  |  |  |  |  |  |  |  |
| 40 |  | IzF53A |  | Manual installation of the floor substrate with a thermal insulation layer of extruded polystyrene foam boards, 40 mm thick, in a single layer |  |  | m² |  | 350.0000 |  |  |  |  |  |
| 41 |  | CG01A |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish |  |  | m2 |  | 350.0000 |  |  |  |  |  |
| 42 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 2 cm, K=4 |  |  | m2 |  | 350.0000 |  |  |  |  |  |
| 43 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 100x100x4 - 1.75 kg/m²) |  |  | kg |  | 612.5000 |  |  |  |  |  |
| 44 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical linoleum), on floors (Eminent DUSTY WHITE homogeneous polyvinyl chloride (PVC) flooring) (add the perimeter length of the rooms, with the material raised up the wall to a height: H=100 mm) |  |  | m² |  | 255.0000 |  |  |  |  |  |
| 45 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical linoleum) (Eminent DUSTY YELLOW homogeneous polyvinyl chloride (PVC) flooring) (add the perimeter length of the rooms, with the material raised up the wall to a height of H=100 mm) |  |  | m² |  | 95.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor linoleum Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2.5. Floor: epoxy- |  |  |  |  |  |  |  |  |  |  |
| 46 |  | CG51A |  | Cast epoxy flooring. "FUXRAD" "FX 20" system, 2-layer industrial coating, layer thickness 0.5–0.7 mm on existing substrate |  |  | m2 |  | 12.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor epoxy Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Flooring Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Walls |  |  |  |  |  |  |  |  |  |  |
| 47 |  | CF05A |  | 3 cm thick interior plaster, applied over wire mesh, trowelled, using M 100-T cement-lime mortar for the skim coat, M 50-T cement-lime mortar for the base coat and M 10-T lime-cement mortar for the finish coat, applied by hand to flat surfaces, including the installation of steel reinforcement and wire mesh, applied to walls |  |  | m² |  | 1,501.0000 |  |  |  |  |  |
| 48 |  | CF17B |  | Miscellaneous works – thickness variation, ±5 mm for the primer coat applied to walls, executed with mortar (add 10 mm, K=2) |  |  | m2 |  | 1,501.0000 |  |  |  |  |  |
| 49 |  | CN53A |  | Priming of interior wall surfaces |  |  | m2 |  | 1,501.0000 |  |  |  |  |  |
| 50 |  | CF50B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture, on walls and partition walls, with mortar prepared by hand. |  |  | m2 |  | 1,501.0000 |  |  |  |  |  |
| 51 |  | CF51B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture, on walls and partition walls, with mortar prepared by hand. Additional or reduced difference for every 1.0 mm (2 mm is deducted from item CF50, K=2) |  |  | m² |  | -1,501.0000 |  |  |  |  |  |
| 52 |  | CN53A |  | Priming of interior wall surfaces |  |  | m2 |  | 1,501.0000 |  |  |  |  |  |
| 53 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 1,501.0000 |  |  |  |  |  |
| 54 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 9001) |  |  | m2 |  | 830.0000 |  |  |  |  |  |
| 55 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 6019) |  |  | m² |  | 2.5000 |  |  |  |  |  |
| 56 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 6021) |  |  | m² |  | 188.0000 |  |  |  |  |  |
| 57 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 6017) |  |  | m² |  | 8.5000 |  |  |  |  |  |
| 58 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 5024) |  |  | m² |  | 50.0000 |  |  |  |  |  |
| 59 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 1034) |  |  | m2 |  | 50.0000 |  |  |  |  |  |
| 60 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to the existing skim coat, carried out by hand (washable paint applied in two coats, RAL 1013) |  |  | m² |  | 277.0000 |  |  |  |  |  |
| 61 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually (washable paint applied in two coats, RAL 2012) |  |  | m2 |  | 95.0000 |  |  |  |  |  |
| 62 |  | IzF04G |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including the eaves and the valleys in the existing waterproofing on surfaces with a slope of over 40% or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or bitumen with rubber additive ( waterproofing membrane liquid) |  |  | m² |  | 53.0000 |  |  |  |  |  |
| 63 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pilasters and window sills) fixed with adhesive (dry mix), tile dimensions: up to 200 x 200 mm (FLAT SNOW 5X20 tiles) |  |  | m2 |  | 140.0000 |  |  |  |  |  |
| 64 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pilasters and window sills) fixed with adhesive (dry mix), tile dimensions: up to 600 x 1200 mm (Technical Jean 20x20 cm porcelain tiles) |  |  | m2 |  | 6.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Walls Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Furniture |  |  |  |  |  |  |  |  |  |  |
| 65 |  | CK32A |  | Window blinds with wooden brackets (horizontal blinds) |  |  | m2 |  | 86.4000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Furniture Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.5. Sanitary engineering ( ) |  |  |  |  |  |  |  |  |  |  |
| 66 |  | SC13A |  | Semi-crystal bathroom mirror with polished edges, (Mirror for people with disabilities, 50 cm, chrome) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 67 |  | SC13A |  | Semi-crystal bathroom mirror with polished edges, (Minimalist framed mirror 400x600) |  |  | pc |  | 9.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sanitary equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | 2 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction- e works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation works |  |  |  |  |  |  |  |  |  |  |
| 68 |  | SC11B apl |  | Alvit support bar, adjustable, foldable, 73 cm, chrome |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 69 |  | SC11B wall-mounted |  | Soap dispenser, Esenia stainless steel, 1000ml |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 70 |  | SC11B wall-mounted |  | Folded Towel Dispenser Dimensions: 260 x 280 x 110 mm Material: satin-finish stainless steel |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 71 |  | SC11B wall-mounted |  | Jumbo Mini Toilet Paper Dispenser |  |  | pc |  | 13.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Sanitary equipment |  |  |  |  |  |  |  |  |  |  |
| 72 |  | Supplier price |  | Alvit Care+ support bar, for people with disabilities, foldable, 73 cm, chrome |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 73 |  | Supplier price |  | Esenia stainless steel soap dispenser, 1000ml |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 74 |  | Supplier price |  | Folded Towel Dispenser Dimensions: 260 x 280 x 110 mm Material: satin-finish stainless steel |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 75 |  | Supplier price |  | Jumbo Mini Toilet Paper Dispenser |  |  | pc |  | 13.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sanitary equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |