| Major repair and modernisation of the Public Institution "Mihai Eminescu", Telenesti District, 8 Codrilor Street, cadastral number 8901224. 151. 02 No. 106-2025-1-IVC | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-2-3

Technological Engineering Workshop        Heating,   ventilation     and   air conditioning    (brand   IVC)

Prepared using current prices

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Heating |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including wages |  |  |  |  |  |  |  |  |  | 0.000 |
|  |  |  |  | 1.2. Steel radiators, monobloc, up to 1000 mm in length, including steel panel radiators with side connections, complete with wall mounting brackets and a built-in air vent. 15 units, Kermi PROFIL - K, including: |  |  |  |  |  |  |  |  |  |  |
| 1 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TIP 33K 1000X500 Kermi PROFIL - K |  |  | pc |  | 1.000 |  |  |  |  |  |
| 2 |  | IB06C |  | Steel radiators, monobloc with a length of 1501–2000 mm - Steel panel radiators with side connection TYPE 33K 1200X500 Kermi PROFIL - K |  |  | pc |  | 11.000 |  |  |  |  |  |
| 3 |  | IB06C |  | Steel radiators, monobloc, length 1501–2000 mm - Side-connected steel panel radiators TYPE 33K 1600X500 Kermi PROFIL - K |  |  | pc |  | 2.000 |  |  |  |  |  |
| 4 |  | IB06C |  | Steel radiators, monobloc, 1501–2000 mm in length - TIP 33K 1800×500 Kermi PROFIL side-connected steel panel radiators - K |  |  | pc |  | 1.000 |  |  |  |  |  |
| 5 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" - RLV angle shut-off valve Db=15 . DANFOSS |  |  | pc |  | 15.000 |  |  |  |  |  |
| 6 |  | ID04A |  | Through or check valve with connectors for central heating systems, with a nominal diameter of 1/2" - 1" - Direct thermostatic valve Dn15 Type RA-N ("Danfoss" Austria) |  |  | pc |  | 15.000 |  |  |  |  |  |
| 7 |  | ID04A |  | Gate or check valve with sockets for central heating systems, Thermostatic head Dn15 P=10 bar. Type RA2991 ("Danfoss" Austria) |  |  | pc |  | 14.000 |  |  |  |  |  |
| 8 |  | ID04A |  | Ball valve with sockets for central heating systems, nominal diameter 1/2"–1" Ball valve φ20 |  |  | pc |  | 4.000 |  |  |  |  |  |
| 9 |  | ID04B |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1 1/4" - 1 1/2" -Ball valve ф=32 |  |  | pc |  | 3.000 |  |  |  |  |  |
| 10 |  | ID04A |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1/2" - 1" (Shut-off and measuring valve dв=20. ASV-M 20 Danfoss) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 11 |  | IC46A |  | Manifold for heating points and substations, mounted on a ready-made support 65–100 mm ( , φ40, L=500 mm) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 12 |  | ID06A.F |  | Air vent valve with ball valve for central heating systems (Automatic air vent valve with ball valve, 15 mm) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 13 |  | ID06A.F |  | Air vent valve with ball valve for central heating systems, ( -type drain valve, , 15 mm) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 14 |  | IC11D |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings, pipe diameter 1 1/4" - Steel pipe, seamless, 42.4 x 4.0 mm, SM EN 10217 P265GH |  |  | m |  | 10.000 |  |  |  |  |  |
| 15 |  | IE03B |  | Pressure leak testing of supply pipes for heating appliances (air heaters, convector heaters, baseboard convectors, etc.) with a diameter of 1 1/4" ... 2" |  |  | m |  | 10.000 |  |  |  |  |  |
| 16 |  | IzA06D |  | Anti-corrosive painting of metal joinery, technological equipment and metal structures with alkyd enamel (one coat of GF-21 primer and two coats of BT-177l enamel) |  |  | m² |  | 1.800 |  |  |  |  |  |
| 17 |  | IC35B |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed at the connection points of heating units or appliances in central heating systems, Polypropylene pipe . ф20х3.4 PP-R 20 |  |  | m |  | 46.000 |  |  |  |  |  |
| 18 |  | IC37B |  | Polyethylene fittings with two joints, press-fitted with threaded connections to reinforced polyethylene pipe, with an external diameter of 20.0 mm - Polypropylene pipe fittings account for 60% of the pipe length |  |  | pc |  | 28.000 |  |  |  |  |  |
| 19 |  | IC35C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed at the connection points of heating units or appliances in central heating systems, Polypropylene pipe . ф25х4.2 PP-R 25 |  |  | m |  | 171.000 |  |  |  |  |  |
| 20 |  | IC37C |  | Two-joint polyethylene fittings, press-fitted with threaded connections to reinforced polyethylene pipes, with an external diameter of 25.0 mm - Fittings for polypropylene pipes account for 60% of the pipe length |  |  | pc |  | 103.000 |  |  |  |  |  |
| 21 |  | IC36C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed in columns in central heating systems, with an external diameter of 40.0 mm. Polypropylene pipe . ф40х6.7 |  |  | m |  | 16.000 |  |  |  |  |  |
| 22 |  | IC37D |  | Polyethylene fittings with two joints, press-fitted with a threaded connection to the reinforced polyethylene pipe, with an external diameter of 40.0 mm Fittings for polypropylene pipe account for 60% of the pipe length |  |  | pc |  | 10.000 |  |  |  |  |  |
| 23 |  | SF01C |  | Carrying out a pressure leak test on hot or cold water installations made of heavy-duty polyvinyl chloride (PVC) pipe or plastic, with a diameter of 16–110 mm |  |  | m |  | 233.000 |  |  |  |  |  |
| 24 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes Basalt wool cylinders with aluminium foil coating FLEXOROCK b=40 mm DN 32 ROCKWOOL |  |  | m |  | 10.000 |  |  |  |  |  |
| 25 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm ф20 ПЭП |  |  | m |  | 46.000 |  |  |  |  |  |
| 26 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm, 25 mm diameter PEP |  |  | m |  | 171.000 |  |  |  |  |  |
| 27 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm, 40 mm diameter PEP |  |  | m |  | 16.000 |  |  |  |  |  |
| 28 |  | IA52A |  | Metal box for protecting the gas meter (Insulated metal box 600x1000) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 29 |  | IC42A |  | Brackets and fasteners for supporting pipes, boilers, appliances and containers weighing up to 2 kg per item ( Steel for securing pipelines St 3) |  |  | kg |  | 130.000 |  |  |  |  |  |
| 30 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 6.760 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Steel radiators, monobloc with a length of up to 1000 mm, including side-connected steel panel radiators, complete with wall mounting brackets and a built-in air vent. 15 units, Kermi PROFIL - K, including: Including labour |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Ventilation PV 1 |  |  |  |  |  |  |  |  |  |  |
| 31 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised sheet metal or aluminium with a perimeter of 600 mm. - Galvanised sheet metal air ducts 100x100 mm, b=0.5 mm |  |  | m2 |  | 3.000 |  |  |  |  |  |
| 32 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet metal air ducts 200x100 mm , thickness = 0.5 mm |  |  | m2 |  | 6.300 |  |  |  |  |  |
| 33 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Galvanised sheet metal air ducts 200x200 mm, thickness 0.5 mm |  |  | m2 |  | 18.800 |  |  |  |  |  |
| 34 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 250x100 mm, thickness 0.5 mm |  |  | m2 |  | 0.350 |  |  |  |  |  |
| 35 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of up to 1200 mm. - Air ducts made of 300x200 mm galvanised sheet metal, thickness 0.7 mm |  |  | m² |  | 13.000 |  |  |  |  |  |
| 36 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x250 mm, thickness 0.7 mm |  |  | m2 |  | 1.100 |  |  |  |  |  |
| 37 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 300x300 mm, thickness 0.7 mm |  |  | m2 |  | 0.500 |  |  |  |  |  |
| 38 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: from 1800 mm. - Air ducts made of 500x150 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 1.950 |  |  |  |  |  |
| 39 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Galvanised sheet air ducts 500x200 mm, thickness 0.7 mm |  |  | m2 |  | 0.700 |  |  |  |  |  |
| 40 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Air ducts made of 600x150 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 0.750 |  |  |  |  |  |
| 41 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Air ducts made of 600x200 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 1.600 |  |  |  |  |  |
| 42 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet metal air ducts 600x500 mm, thickness 0.7 mm |  |  | m2 |  | 2.200 |  |  |  |  |  |
| 43 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 9.000 |  |  |  |  |  |
| 44 |  | Company price |  | R400x100 supply air control grille |  |  | pc |  | 1.000 |  |  |  |  |  |
| 45 |  | Company price |  | P500x150 supply air control grille |  |  | pc |  | 2.000 |  |  |  |  |  |
| 46 |  | Company price |  | P600x150 supply air control grille |  |  | pc |  | 6.000 |  |  |  |  |  |
| 47 |  | CC09A |  | Installation of rabbit mesh for plaster reinforcement on ceilings and walls, thermal insulation protection, and duct concealment, applied to flat ceilings on a 10×10 mm steel mesh |  |  | m² |  | 0.100 |  |  |  |  |  |
| 48 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 9.000 |  |  |  |  |  |
| 49 |  | Company price |  | R250x100 adjustable exhaust grille |  |  | pc |  | 1.000 |  |  |  |  |  |
| 50 |  | Company price |  | P500x150 adjustable exhaust grille |  |  | pc |  | 7.000 |  |  |  |  |  |
| 51 |  | Company price |  | air intake grille P600x500 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 52 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry ( -type damper) |  |  | pc |  | 11.000 |  |  |  |  |  |
| 53 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 54 |  | Company price |  | Throttle valve 200x200 |  |  | pc |  | 6.000 |  |  |  |  |  |
| 55 |  | Company price |  | Throttle valve 500×150 |  |  | pc |  | 3.000 |  |  |  |  |  |
| 56 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 2.000 |  |  |  |  |  |
| 57 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 2.000 |  |  |  |  |  |
| 58 |  | VB19A |  | Air outlet, for Type I air shower - -type hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 59 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 60 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with aluminium foil protective coating (foil-coated), b=25 mm |  |  | m² |  | 50.250 |  |  |  |  |  |
| 61 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 62 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg each (Steel for securing pipelines St 3) |  |  | kg |  | 33.000 |  |  |  |  |  |
| 63 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 1.720 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation PV 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Ventilation P 2 |  |  |  |  |  |  |  |  |  |  |
| 64 |  | VA22C |  | Installation of ventilation ducts at a height of 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of 1800 mm. - Galvanised sheet metal air ducts 500x150 mm, thickness 0.7 mm |  |  | m2 |  | 1.950 |  |  |  |  |  |
| 65 |  | VA22C |  | Installation of ventilation ducts at a height of 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Air ducts made of 500x250 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 2.250 |  |  |  |  |  |
| 66 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Galvanised sheet air ducts 500x300 mm, thickness 0.7 mm |  |  | m2 |  | 5.600 |  |  |  |  |  |
| 67 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 1800 mm. - Galvanised sheet air ducts 600x300 mm, thickness 0.7 mm |  |  | m2 |  | 0.900 |  |  |  |  |  |
| 68 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet air ducts 800x400 mm, thickness 0.7 mm |  |  | m2 |  | 1.200 |  |  |  |  |  |
| 69 |  | VA22E |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: 3600 mm. Air ducts made of 800x600 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 2.800 |  |  |  |  |  |
| 70 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 8.000 |  |  |  |  |  |
| 71 |  | Company price |  | R800x400 supply air control grille |  |  | pc |  | 4.000 |  |  |  |  |  |
| 72 |  | Company price |  | P800x600 supply air control grille |  |  | pc |  | 4.000 |  |  |  |  |  |
| 73 |  | VB19A |  | Outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 74 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 75 |  | VB19A |  | Air outlet, for Type I air shower - -type hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 76 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 77 |  | IzH22A |  | Insulation of ducts with ready-made mineral wool sleeves, thicknesses 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 14.000 |  |  |  |  |  |
| 78 |  | Company price |  | Aluminium adhesive tape for REK and KIMJAL joint sealing (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 79 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 4.000 |  |  |  |  |  |
| 80 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.210 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation P 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.5. Ventilation P 3 |  |  |  |  |  |  |  |  |  |  |
| 81 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium 0.7 mm thick, with a circular cross-section diameter of 500–560 mm. (circular cross-section) Air ducts made of galvanised sheet metal du 250 mm , б=0.7 mm |  |  | m² |  | 2.750 |  |  |  |  |  |
| 82 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium 0.7 mm thick with a perimeter of up to 1200 mm. - Air ducts made of galvanised sheet metal 300x300 mm, thickness 0.7 mm |  |  | m2 |  | 1.800 |  |  |  |  |  |
| 83 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Air ducts made of 400x300 mm galvanised sheet metal, thickness 0.7 mm |  |  | m2 |  | 4.900 |  |  |  |  |  |
| 84 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: from 1800 mm. - Air ducts made of 600x300 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 0.900 |  |  |  |  |  |
| 85 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of 2000 mm. - Galvanised sheet air ducts 800x250 mm, thickness 0.7 mm |  |  | m2 |  | 1.050 |  |  |  |  |  |
| 86 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal with a perimeter of 2000 mm. – Air ducts made of galvanised sheet metal, 800 x 500 mm, thickness 0.7 mm |  |  | m² |  | 2.600 |  |  |  |  |  |
| 87 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 5.000 |  |  |  |  |  |
| 88 |  | Company price |  | R800x250 supply air control grille |  |  | pc |  | 4.000 |  |  |  |  |  |
| 89 |  | Company price |  | air intake grille P800x500 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 90 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry (Throttle valve) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 91 |  | Company price |  | Throttle valve 800x250 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 92 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 93 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 94 |  | VB19A |  | Air outlet, for Type I air shower - access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 95 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 96 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 15.000 |  |  |  |  |  |
| 97 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 98 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 4.000 |  |  |  |  |  |
| 99 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.210 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation P 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.6. B2 |  |  |  |  |  |  |  |  |  |  |
| 100 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet, with a circular cross-section diameter of 500–560 mm. (circular cross-section) Air ducts made of galvanised sheet du 315 mm , б=0.7 mm |  |  | m2 |  | 0.490 |  |  |  |  |  |
| 101 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of galvanised sheet 250x250 mm, thickness 0.7 mm |  |  | m2 |  | 0.500 |  |  |  |  |  |
| 102 |  | VA22C |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of 1800 mm. - Air ducts made of 500x300 mm galvanised sheet metal, thickness 0.7 mm |  |  | m² |  | 8.800 |  |  |  |  |  |
| 103 |  | VB30A |  | General-purpose hood, types A, B, C, made from 0.5 mm thick black sheet metal (Hood 1700x1100x500h, 1 pc) |  |  | m2 |  | 3.470 |  |  |  |  |  |
| 104 |  | VB19A |  | Air outlet, for type I air duct – PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 105 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 106 |  | VB19A |  | Air outlet, for Type I air shower - -type access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 107 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 108 |  | CC09A |  | Installation of rabbit mesh for plaster reinforcement on ceilings and walls, thermal insulation protection, and duct concealment, applied to flat ceilings, on a 10×10 mm steel mesh |  |  | m2 |  | 0.150 |  |  |  |  |  |
| 109 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 9.000 |  |  |  |  |  |
| 110 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 111 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 3.000 |  |  |  |  |  |
| 112 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.160 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.7. B3 |  |  |  |  |  |  |  |  |  |  |
| 113 |  | VA20A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet, with a circular cross-section diameter of 500–560 mm. (circular cross-section) Air ducts made of galvanised sheet du 315 mm , б=0.7 mm |  |  | m2 |  | 0.670 |  |  |  |  |  |
| 114 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of galvanised sheet metal 250x250 mm, thickness 0.7 mm |  |  | m2 |  | 0.500 |  |  |  |  |  |
| 115 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet metal, with a perimeter of up to 1200 mm. - Air ducts made of 300x200 mm galvanised sheet metal, thickness 0.7 mm |  |  | m² |  | 2.000 |  |  |  |  |  |
| 116 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet air ducts 300x300 mm, thickness 0.7 mm |  |  | m2 |  | 1.800 |  |  |  |  |  |
| 117 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x300 mm, thickness 0.7 mm |  |  | m2 |  | 5.600 |  |  |  |  |  |
| 118 |  | VB30A |  | General-purpose hoods, types A, B, C, made from 0.5 mm thick black sheet metal (Hood 800x700x350h, 3 pcs) |  |  | m2 |  | 3.450 |  |  |  |  |  |
| 119 |  | VB19A |  | Air outlet, for type I air duct – PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 120 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 121 |  | VB19A |  | Discharge outlet, for Type I air shower - access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 122 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 123 |  | CC09A |  | Installation of rabbit mesh for plaster reinforcement on ceilings and walls, thermal insulation protection, and duct concealment, applied to flat ceilings on a 10×10 mm steel mesh |  |  | m² |  | 0.150 |  |  |  |  |  |
| 124 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 9.000 |  |  |  |  |  |
| 125 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 126 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 3.000 |  |  |  |  |  |
| 127 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.160 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.8. BE 1 |  |  |  |  |  |  |  |  |  |  |
| 128 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 800–1000 mm. - Air ducts made of 250x250 mm galvanised sheet metal, thickness 0.5 mm |  |  | m2 |  | 5.000 |  |  |  |  |  |
| 129 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | each |  | 1.000 |  |  |  |  |  |
| 130 |  | Company price |  | R500x200 adjustable exhaust grille |  |  | pc |  | 1.000 |  |  |  |  |  |
| 131 |  | VB30A |  | General-purpose hood, types A, B, C, made from 0.5 mm thick black sheet metal (Umbrella 350x350, 1 pc) |  |  | m² |  | 0.420 |  |  |  |  |  |
| 132 |  | IzH22A |  | Insulation of pipes with ready-made mineral wool sleeves, 20; 30; 40; 50; 60; 70 and 80 mm thick - Thermal insulation with mineral wool boards on a synthetic binder, grade 125, b=60 mm, with a foil covering layer |  |  | m² |  | 5.000 |  |  |  |  |  |
| 133 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 134 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 2.000 |  |  |  |  |  |
| 135 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.100 |  |  |  |  |  |
| 136 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry Supply air grille in doors PR300x150 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 137 |  | Company price |  | PR300x150 door air intake grille |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total BE 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.9. Cold storage |  |  |  |  |  |  |  |  |  |  |
| 138 |  | IC31A |  | Copper pipe, welded, for connecting heating units and appliances in central heating systems, with an external diameter of up to 15.0 mm inclusive (Freon pipes D 6.35) |  |  | m |  | 9.000 |  |  |  |  |  |
| 139 |  | IC31A |  | Copper pipe, fitted by welding, for connecting radiators and heating appliances in central heating systems, with an external diameter of up to 15.0 mm inclusive (Freon pipes D 12.7) |  |  | m |  | 9.000 |  |  |  |  |  |
| 140 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes, b=13mm, for freon pipes ф6.35 |  |  | m |  | 9.000 |  |  |  |  |  |
| 141 |  | RpIF09C |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Insulation in Armaflex foam rubber tubes b=13mm for freon pipes ф12.7 |  |  | m |  | 9.000 |  |  |  |  |  |
| 142 |  | IC36C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed in columns in central heating systems, with an external diameter of 32.0 mm. Polypropylene pipe . ф32х3.0 |  |  | m |  | 18.000 |  |  |  |  |  |
| 143 |  | SF01C |  | Carrying out a pressure leak test on a hot or cold water system made of heavy-duty polyvinyl chloride (PVC) pipe or plastic, with a diameter of 16–110 mm |  |  | m |  | 18.000 |  |  |  |  |  |
| 144 |  | RpIF09C |  | Insulation of pipes using special insulating sleeves fitted onto the pipes - 9 mm porous polyethylene hose, 32 mm diameter, PEP |  |  | m |  | 18.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cold Supply Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.10. Dismantling |  |  |  |  |  |  |  |  |  |  |
| 145 |  | RpIB11A |  | Removal of radiators for decommissioning (Removal of radiators for decommissioning (10 sections, 15 installation points) |  |  | m2 |  | 44.850 |  |  |  |  |  |
| 146 |  | RpIC19C |  | Removal of black pipes for installations, fitted to central heating systems by welding, in residential and social-cultural buildings, at connection of appliances, distribution columns, etc., with a diameter of 1 1/2"–2" |  |  | m |  | 100.000 |  |  |  |  |  |
| 147 |  | CL20A |  | Ready-made ventilation grilles, dismantling and installation, K0.8 |  |  | pcs |  | 1.000 |  |  |  |  |  |
| 148 |  | RpVA35B |  | Dismantling of ventilation ducts made of black, galvanised or aluminium sheet metal, with a rectangular or circular cross-section perimeter of 700–1600 mm (Dismantling of ventilation ducts made of black, galvanised or aluminium sheet metal, with a rectangular or circular cross-section perimeter of 700–1600 mm) |  |  | m² |  | 28.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Dismantling Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social and health insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Semi-finished product and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 149 |  | ID04A |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1/2"-1" ASV-PV - Danfoss automatic balancing valve, 20 mm |  |  | pc |  | 4.000 |  |  |  |  |  |
| 150 |  | SE56A1 |  | Drinking water filter with flanges for pipe mounting, size 1"–2" – Danfoss strainer valve, 20 mm |  |  | pc |  | 4.000 |  |  |  |  |  |
| 151 |  | IA18B |  | Fine fittings for central heating boilers: Pressure gauge |  |  | pc |  | 2.000 |  |  |  |  |  |
| 152 |  | IA18A |  | Precision fittings for central heating boilers: Thermometer |  |  | pc |  | 2.000 |  |  |  |  |  |
| 153 |  | IA18J |  | Precision fittings for central heating boilers: fitting with control valve for fittings Fitting for thermomanometer |  |  | pc |  | 8.000 |  |  |  |  |  |
| 154 |  | IA18A |  | Precision fittings for central heating boilers: thermomanometer |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. PV1 |  |  |  |  |  |  |  |  |  |  |
| 155 |  | VC16A |  | Installation of a partial air handling unit, type ALTPA, within the premises, comprising suction, filtration, ventilation, heating, cooling and discharge, with a treated air flow rate of 3,000–7,500 m³/h ( PV1 Supply and exhaust unit with heat recovery included: type Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.000 |  |  |  |  |  |
| 156 |  | VB18A |  | Rectangular noise attenuator with dimensions of 1000–2000 mm – Noise attenuator 300×300 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 157 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 300×300 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 158 |  | AcB09A |  | Installation of electric actuators for dampers, with a diameter of 400–600 mm - Electric actuator for air damper |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. П2 |  |  |  |  |  |  |  |  |  |  |
| 159 |  | VC16A |  | Installation of an indoor partial air treatment unit, type ALTPA, comprising suction, filtration, ventilation, heating, cooling and discharge, with a treated air flow rate of 3,000–7,500 m³/h - П2 Supply air unit as a complete set: : МПА 3200 Е3 ВЕНТС |  |  | pc |  | 1.000 |  |  |  |  |  |
| 160 |  | VB18A |  | Rectangular noise attenuator with a perimeter of 1,000–2,000 mm – Noise attenuator 500×300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 161 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 600×300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 162 |  | AcB09A |  | Installation of electric actuators for valves with a diameter of 400–600 mm - Electric actuator for an air valve |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total П2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. П3 |  |  |  |  |  |  |  |  |  |  |
| 163 |  | VC16A |  | Installation of an indoor partial air treatment unit, type ALTPA, comprising suction, filtration, ventilation, heating, cooling and discharge, with a treated air flow rate of 3,000–7,500 m³/h - П2 Supply air unit as a complete set: : МПА 3200 Е3 ВЕНТС |  |  | pc |  | 1.000 |  |  |  |  |  |
| 164 |  | VB18A |  | Rectangular noise attenuator with a perimeter of 1,000–2,000 mm – Noise attenuator 400×300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 165 |  | VB04A |  | Counterweight damper, with perimeter 800–1600 mm -Servo-operated air damper 600×300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 166 |  | AcB09A |  | Installation of electric actuators for dampers, with a diameter of 400–600 mm - Electric actuator for air damper |  |  | pcs |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total П3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. В2 |  |  |  |  |  |  |  |  |  |  |
| 167 |  | VC01B |  | Installation of centrifugal fans, with a total weight of 50–400 kg, mounted on a metal frame – Centrifugal exhaust fan type DWEY 315Х143-2.2-6 ВЕНТС |  |  | pc |  | 1.000 |  |  |  |  |  |
| 168 |  | VC24A |  | Elastic fan support, with rubber elastic mounts - Soft insert ф315 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 169 |  | VC24A |  | Elastic fan support, with rubber elastic mounts - Soft insert ф250х250 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.7. В3 |  |  |  |  |  |  |  |  |  |  |
| 170 |  | VC01B |  | Installation of centrifugal fans, with a total weight of 50–400 kg, mounted on a metal frame – B2 Centrifugal exhaust fan , type DWEY 315Х143-2.2-6 VENTS |  |  | pc |  | 1.000 |  |  |  |  |  |
| 171 |  | VC24A |  | Elastic fan support, with rubber elastic mounts - Soft insert ф315 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 172 |  | VC24A |  | Elastic fan support, with rubber elastic mounts - Soft insert ф250х250 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 173 |  | VB18A |  | Rectangular noise attenuator with perimeter 1000–2000 mm - Noise attenuator 400×300 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.8. BE 1 |  |  |  |  |  |  |  |  |  |  |
| 174 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry (Hydraulically controlled air intake device with sound insulation in the upper frame of the E-EMM2 double-glazed unit) |  |  | pc |  | 16.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total BE 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.9. Cold storage |  |  |  |  |  |  |  |  |  |  |
| 175 |  | VC37A |  | Installation of domestic air conditioning units (split-system) with a motor power of up to 4.5 kW, on the staircase ( SMART Gree GWH24QD-K6DNB61 compressor-condenser outdoor unit ( "GREE") |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cold Supply Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Costs of semi-finished products and storage |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Cost of equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 176 |  | Company price |  | Automatic balancing valve DN 20 Type ASV-PV-20 "Danfoss" Austria |  |  | pc |  | 4.000 |  |  |  |  |  |
| 177 |  | Company price |  | Danfoss F20 strainer |  |  | pc |  | 4.000 |  |  |  |  |  |
| 178 |  | Company price |  | Danfoss 32 mm mesh filter |  |  | pc |  | 1.000 |  |  |  |  |  |
| 179 |  | Company price |  | Pressure gauge |  |  | pc |  | 2.000 |  |  |  |  |  |
| 180 |  | Company price |  | Thermometer |  |  | pc |  | 2.000 |  |  |  |  |  |
| 181 |  | Company price |  | thermomanometer |  |  | pc |  | 2.000 |  |  |  |  |  |
| 182 |  | Company price |  | Fitting for the thermomanometer |  |  | pc |  | 8.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. PV1 |  |  |  |  |  |  |  |  |  |  |
| 183 |  | Company price |  | PV1 Supply and exhaust unit with heat recovery included: Supply fan L=1300 m³/h; N=0.41 kW, 50 Hz, U=230 V: Exhaust fan L=1200 m³/h. N=0.28 kW, 50 Hz, U=230 V, 4 soft inserts, F7 filter. M5 filter. Counter-flow heat recovery unit, 1.8 kW electric heater, direct evaporation air heater/cooler, condensate collection tray type , Duplex 1500 Multi "ARTREA" |  |  | set |  | 1.000 |  |  |  |  |  |
| 184 |  | Company price |  | Silencer 300x300 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 185 |  | Company price |  | Servo-operated air damper 300x300 |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PV1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.5. П2 |  |  |  |  |  |  |  |  |  |  |
| 186 |  | Company price |  | P 2 Supply and exhaust unit with heat recovery included: Supply unit included: Supply fan L=2100 m³/h; N=1.27 kW, 50 Hz, U=400 V: 2 soft inserts, G4 filter. Counter-flow heat recovery unit, 25.2 kW electric heater, MPA 3200 E3 |  |  | set |  | 1.000 |  |  |  |  |  |
| 187 |  | Company price |  | Silencer 500x300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 188 |  | Company price |  | Servo-operated air damper 600x300 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total П2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.6. P3 |  |  |  |  |  |  |  |  |  |  |
| 189 |  | Company price |  | P 3 Supply and exhaust unit with heat recovery included: Supply unit included: Supply fan L=1800 m³/h; N=1.27 kW, 50 Hz, U=400 V: 2 soft inserts, G4 filter. Counter-flow heat recovery unit, 25.2 kW electric heater, MPA 3200 E3 |  |  | set |  | 1.000 |  |  |  |  |  |
| 190 |  | Company price |  | Silencer 400x300 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 191 |  | Company price |  | Servo-operated air damper 600x300 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total P3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.7. В2 |  |  |  |  |  |  |  |  |  |  |
| 192 |  | Company price |  | B2 Centrifugal exhaust fan; L= 2100 m³/h. N=2.2 kW. U=400 V, 50 Hz DWEY 315×143-2.2-6 vents |  |  | pc |  | 1.000 |  |  |  |  |  |
| 193 |  | Company price |  | Soft- e insert f315 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 194 |  | Company price |  | Soft e insert 250x250 |  |  | each |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.8. В3 |  |  |  |  |  |  |  |  |  |  |
| 195 |  | Company price |  | B3 Centrifugal exhaust fan; L= 1800 m³/h. N=2.2 kW. U=400 V, 50 Hz DWEY 315×143-2.2-6 vents |  |  | pc |  | 1.000 |  |  |  |  |  |
| 196 |  | Company price |  | Soft- e insert f315 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 197 |  | Company price |  | Soft e insert 250x250 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 198 |  | Company price |  | Sound absorber 400x300 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total В3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.9. ВЕ 1 |  |  |  |  |  |  |  |  |  |  |
| 199 |  | Company price |  | Hydraulically adjustable air intake device with sound absorption, in the upper frame of the E-EMM2 double-glazed unit |  |  | pc |  | 16.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total BE 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.10. Cold storage |  |  |  |  |  |  |  |  |  |  |
| 200 |  | Company price |  | SMART Gree GWH24QD-K6DNB61 outdoor compressor-condenser unit Maximum cooling capacity Qcool=7.1k, W, Maximum heating capacity heat=7.8kW, Maximum power consumption Nel=2.0 kW, U=230V 50Hz "GREE" |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cooling Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total value of the equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |