| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

QUOTATION  LOCAL No. 2-2-5

Workshop.  Fire and   al signalling

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 10-08-001-04 |  | Receiving devices: "PS" device for: 4 beams ( Fire detection and control device, 4/36, SmartLina 036/4 ) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 2 |  | 10-08-001-04 |  | Receiving devices: "PS" device for: 4 beams (Fire backup device (repeater) SmartLetUSee/LC) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 3 |  | 10-08-002-03 |  | Automatic detectors: smoke ID 100 (Automatic fire detector ID 100) |  |  | pc |  | 16.0000 |  |  |  |  |  |
| 4 |  | 10-04-066-05 apl |  | Wall-mounted device: Manual fire alarm (Manual fire alarm, IC0020) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 5 |  | 10-04-066-05 |  | Wall-mounted device: Alarm (Visual and audible fire alarm, IS0020RE) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 6 |  | 10-04-066-05 |  | Wall-mounted device: Alarm (Remote optical signalling device, IL0010) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 7 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting bracket (Loop monitoring terminal, EN54-13, UK-2) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 8 |  | 08-01-121-1 |  | Stationary lead-acid battery, type: CB 7.0-12 (12V battery, 7 Ah) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 9 |  | 08-02-412-1 |  | Insertion of conductors into installed metal pipes and conduits: first single-core or multi-core conductor in a common braid, cross-section up to 2.5 mm² |  |  | 100 m |  | 3.2400 |  |  |  |  |  |
| 10 |  | Supplier price |  | Cat. 3 twisted-pair copper-core communication cable, shielded, fire-resistant JE-H(St)H FE180 1×2×0.8 |  |  | m |  | 360.0000 |  |  |  |  |  |
| 11 |  | 08-02-409-1 |  | Vinyl-plastic conduit for installed structures, on walls and columns, clamped, diameter 25 mm |  |  | 100 m |  | 1.6800 |  |  |  |  |  |
| 12 |  | Supplier price |  | , non-flammable, non-smoking, non-toxic, black HDPE corrugated pipe, D20 mm |  |  | m |  | 168.0000 |  |  |  |  |  |
| 13 |  | 08-02-409-2 |  | Vinyl-plastic pipe for installed structures, on walls and columns, clamped, diameter up to 50 mm |  |  | 100 m |  | 1.5000 |  |  |  |  |  |
| 14 |  | Supplier price |  | Blue double-layer corrugated HDPE pipe, D40 mm |  |  | m |  | 150.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Equipment |  |  |  |  |  |  |  |  |  |  |
| 15 |  | Supplier price |  | Fire detection and control panel, 4/36 loops, ~220V, EN54-2, EN54-4 SmartLine036/4 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 16 |  | Supplier price |  | Fire backup device (repeater), EN54-2, EN54-4, SmartLetUSee/LCD |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 17 |  | Supplier price |  | Automatic smoke fire detector, EN54-7, ID 100 |  |  | pc |  | 18.0000 |  |  |  |  |  |
| 18 |  | Supplier price |  | Manual fire alarm, EN54 IC0020 |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 19 |  | Supplier price |  | Fire alarm strobe and sounder, red, 24V 0.2mA 95dBA, EN54-3, IS0020RE |  |  | pc |  | 3.00000 |  |  |  |  |  |
| 20 |  | Supplier price |  | Remote optical signalling device, IL0010 |  |  | pc |  | 3.000 |  |  |  |  |  |
| 21 |  | Supplier price |  | Terminal device for monitoring alarm loop parameters, UK-2 Arton |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 22 |  | Supplier price |  | Rechargeable battery, 12V 7Ah |  |  | pc |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |