| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-3-1

Boiler room. SAC

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Demolition |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (sheet 5) |  |  | m2 |  | 7.7500 |  |  |  |  |  |
| 2 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (sheet 5) |  |  | m2 |  | 6.1200 |  |  |  |  |  |
| 3 |  | RpCJ35A |  | Removal of cracked external plaster from walls or ceilings (sheet 14) |  |  | m2 |  | 281.2000 |  |  |  |  |  |
| 4 |  | RpCI42B |  | Removal of roof elements – sheet metal, asbestos cement, PVC, cardboard, felt, thatch, etc., including cutting of recoverable sheet metal (sheet 14) |  |  | m2 |  | 215.3300 |  |  |  |  |  |
| 5 |  | RpCU09C |  | Manual transport by carrying directly over a distance of 60 m with a load of up to 50 kg (for every 1 m transported vertically, 10 m horizontally is considered) |  |  | t |  | 19.4100 |  |  |  |  |  |
| 6 |  | TrI1AA01C2 |  | Loading of Group A materials – heavy and small items by throwing – from a ramp or ground into a Category 2 vehicle (construction waste) |  |  | t |  | 19.4100 |  |  |  |  |  |
| 7 |  | TsI51B5 |  | Transport of construction waste by 10-tonne tipper lorry over a distance of: 15 km |  |  | t |  | 19.4100 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Demolition Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Foundation under walls partitions |  |  |  |  |  |  |  |  |  |  |
| 8 |  | TsA02B |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in non-cohesive or weakly cohesive soil, depth < 0.75 m, medium ground |  |  | m3 |  | 7.0000 |  |  |  |  |  |
| 9 |  | CA03E2 |  | Cast concrete in foundations, plinths, retaining walls, walls below ground level, mixed on-site using a concrete mixer and poured using conventional methods; reinforced concrete class C 20/16 (Bc 20/B 250) (sheet 32) |  |  | m³ |  | 7.0000 |  |  |  |  |  |
| 10 |  | CC01C |  | OB 37 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diameter 6 A240) |  |  | kg |  | 52.0000 |  |  |  |  |  |
| 11 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 12 B500B) |  |  | kg |  | 19.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Foundation under partition walls Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Walls |  |  |  |  |  |  |  |  |  |  |
| 12 |  | CD60С |  | Brickwork, 250 x 120 x 65 mm bricks with M-50 mortar prepared on-site for reinforced partition walls, 1/2 brick thick, height up to 4 m (sheet 6, 15.45 m³/0.12 m = 128.75 m²) |  |  | 100 m² |  | 1.2900 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Walls Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Thermal insulation of wall P-2 (walls below the floor, S=64.2 m², panel 10, panel 15) |  |  |  |  |  |  |  |  |  |  |
| 13 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 77.0000 |  |  |  |  |  |
| 14 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, soil originating from medium soil |  |  | m3 |  | 67.8000 |  |  |  |  |  |
| 15 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m3 |  | 0.6800 |  |  |  |  |  |
| 16 |  | IzF04G |  | Hot-applied waterproofing layer on terraces, roofs, foundations and ground slabs, in areas without groundwater, including the flashings and valleys in the standard waterproofing on surfaces with a gradient of over 40% or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or bitumen with rubber additive (base, waterproofing layer in 2 layers, K=2 ) |  |  | m² |  | 64.2000 |  |  |  |  |  |
| 17 |  | IzF10C |  | Thermal insulation layer for terraces, roofs and floors, made of cellular polystyrene boards, jute felt, PEL, porous, stabilised, etc., bonded with bitumen mastic to surfaces with a slope of over 40% or vertical surfaces (extruded polystyrene, thickness 100 mm, using Technonicol N27 adhesive) |  |  | m² |  | 64.2000 |  |  |  |  |  |
| 18 |  | IzF04G |  | Hot-applied waterproofing layer on terraces, roofs, foundations and ground slabs, in areas without groundwater, including the troughs and valleys in the standard waterproofing on surfaces with a gradient of over 40% or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or bitumen with rubber additive (Izoflex membrane) |  |  | m² |  | 64.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Thermal insulation of wall P-2 (walls below the floor, S=64.2 m², panel 10, panel 15) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Facade (sheet 15) |  |  |  |  |  |  |  |  |  |  |
| 19 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement panels with visible fixing, with architectural detailing covering up to 30% of the total wall area (insulation panels – mineral wool, 150 mm thick, RAL-1013) |  |  | m² |  | 295.4400 |  |  |  |  |  |
| 20 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement panels and visible fixings, with architectural detailing covering up to 30% of the total wall area (insulation boards – mineral wool, 150 mm thick, RAL-9016) |  |  | m² |  | 8.9700 |  |  |  |  |  |
| 21 |  | CN54B |  | Manual application of a single coat of ‘Gleta’ quartz-based primer to external walls on the facades |  |  | m² |  | 304.4000 |  |  |  |  |  |
| 22 |  | CN11A1 |  | Exterior painting with water-based acrylic polymer paint, applied in 3 coats to façades on existing plaster, applied mechanically (RAL-1013) |  |  | m² |  | 295.4400 |  |  |  |  |  |
| 23 |  | CN11A1 |  | Exterior painting with water-based acrylic polymer paint, applied in 3 layers to facades on existing plaster, applied mechanically (RAL-9016) |  |  | m2 |  | 8.9700 |  |  |  |  |  |
| 24 |  | RpCH31A |  | Erection and dismantling of scaffolding for construction works – tubular steel scaffolding for interior and exterior works up to 7 m in height, including the materials required for the construction of working platforms, PFL guardrails or safety netting |  |  | m² |  | 318.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Facade (sheet 15) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Roof (sheet 10, sheet 23) |  |  |  |  |  |  |  |  |  |  |
| 25 |  | RpCH06B |  | Repairs to damaged roof structure elements |  |  | m² |  | 10.5500 |  |  |  |  |  |
| 26 |  | CE17A |  | Additional Ondutiss-type polymeric layer installed beneath the roof covering of tiles, corrugated sheets or embossed sheets |  |  | m² |  | 210.9000 |  |  |  |  |  |
| 27 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or ribbed, installed on metal panels, carried out on areas larger than 40 m² using profiled sheet metal sheets fastened with special clips and mechanical screws from the upper surface, including the installation of valleys, flashings, connections to chimneys, etc. |  |  | m² |  | 210.9000 |  |  |  |  |  |
| 28 |  | CE20A |  | Brass-type guttering systems made of corrosion-resistant sheet metal |  |  | m |  | 52.4000 |  |  |  |  |  |
| 29 |  | CE22A |  | Brass-style downpipe systems made from corrosion-resistant sheet metal |  |  | m |  | 29.8200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Roof (sheet 10, sheet 23) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. Doors |  |  |  |  |  |  |  |  |  |  |
| 30 |  | CK21D |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any type, in buildings up to and including 35 m in height, double-leaf, with a frame area up to 7 m² inclusive (Ue-1 Thermal insulation doors comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium, door with threshold) |  |  | m² |  | 4.5000 |  |  |  |  |  |
| 31 |  | CK03A |  | Single-leaf interior wooden doors, including balcony doors, fitted with waterproof and thermal insulation in the door frame, installed on existing door frames in buildings up to 35 m high ( wooden/MDF doors Ui-1, Ui-2) |  |  | m² |  | 4.2000 |  |  |  |  |  |
| 32 |  | CK03A |  | Single-leaf interior wooden doors, for cladding and balcony doors, including waterproof and thermal insulation of the frame, fitted to existing door frames in buildings up to 35 m high ( , wooden/MDF doors Ui-3, Ui-4) |  |  | m2 |  | 3.3600 |  |  |  |  |  |
| 33 |  | CK25A |  | Doors made from plastic profiles, including the necessary fittings and accessories, for doors installed in masonry of any kind in buildings up to and including 35 m in height, single-leaf, with a frame area up to and including 7 m² (door Ui-5, Ui-6) |  |  | m² |  | 5.0400 |  |  |  |  |  |
| 34 |  | CK27A |  | Plastic profile glazing in buildings up to 35 m high, comprising fixed panels mounted on the parapet (wall , partitions , for shower) |  |  | m2 |  | 8.1700 |  |  |  |  |  |
| 35 |  | RpCJ06B |  | Repairs to interior plasterwork around door and window frames and sills, 2 cm thick, with a rough finish, carried out using 25 T grade cement-lime mortar, with straight edges, between 15 and 25 cm wide |  |  | m |  | 44.4000 |  |  |  |  |  |
| 36 |  | Supplier price |  | -aluminium angle profile |  |  | m |  | 44.4000 |  |  |  |  |  |
| 37 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 38 |  | CF61A |  | Continuous skimming of the surface (single-layer plaster) with dry plaster mix: window sills and flat doors |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 39 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast reinforced concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 40 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 41 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 8.9000 |  |  |  |  |  |
| 42 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 43 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m² |  | 8.9000 |  |  |  |  |  |
| 44 |  | CK07B |  | Wooden mouldings fitted to doors or windows |  |  | m |  | 20.4000 |  |  |  |  |  |
| 45 |  | CK26B |  | Window sills fitted to aluminium doors |  |  | m |  | 6.8000 |  |  |  |  |  |
| 46 |  | CK26A |  | Window sills fitted to plastic windows, for windows and doors |  |  | m |  | 17.2000 |  |  |  |  |  |
| 47 |  | CK33C |  | Yalle system surface-mounted lock |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 48 |  | CK33B |  | Chrome-plated metal fittings for metal entrance doors in buildings |  |  | pc |  | 9.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Doors Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Windows |  |  |  |  |  |  |  |  |  |  |
| 49 |  | CK19B |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of between 3.00 and 6.00 m² inclusive (Thermal insulation glazing comprising 3 layers of glass separated by a space filled with inert gas. frames made of 5-chamber aluminium) |  |  | m² |  | 6.1200 |  |  |  |  |  |
| 50 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (Thermal insulation glazing comprising 3 layers of glass separated by a space filled with inert gas. Frames made of 5-chamber aluminium) |  |  | m² |  | 3.0600 |  |  |  |  |  |
| 51 |  | RpCJ06B |  | Repairs to interior plasterwork, around frames and sills, on doors and windows, 2 cm thick, rough-cast, executed with 25 T grade cement-lime mortar, with straight edges, between 15–25 cm wide |  |  | m |  | 19.0000 |  |  |  |  |  |
| 52 |  | Supplier price |  | aluminium angle profile |  |  | m |  | 19.0000 |  |  |  |  |  |
| 53 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 54 |  | CF61A |  | Continuous skimming of the surface (single-layer plaster) with dry plaster mix: window sills and flat doors |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 55 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast reinforced concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 56 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 57 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 3.8000 |  |  |  |  |  |
| 58 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 59 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m² |  | 3.8000 |  |  |  |  |  |
| 60 |  | CK26D |  | Window or door sills made of aluminium (drip mouldings) (width 200 mm) |  |  | m |  | 9.8000 |  |  |  |  |  |
| 61 |  | CK26C |  | Sills fitted to plastic windows or doors (width 350 mm) |  |  | m |  | 9.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Windows Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9. Ventilation and ducts |  |  |  |  |  |  |  |  |  |  |
| 62 |  | CD50I |  | Brickwork, 250 x 120 x 65 mm, on the walls of the discharge pits and ducts (4.14 m²) |  |  | m3 |  | 0.5000 |  |  |  |  |  |
| 63 |  | CL55A apl |  | Installation of a ventilated façade system with fibre cement panels with visible fixing, with architectural detailing covering up to 30% of the total wall area (insulation panels – 50 mm thick mineral wool, RAL-1013) |  |  | m2 |  | 4.1400 |  |  |  |  |  |
| 64 |  | CN54B |  | Manual application of a single coat of ‘Gleta’ quartz-based primer to external walls on the façade |  |  | m2 |  | 4.1400 |  |  |  |  |  |
| 65 |  | CN11A1 |  | Exterior painting with water-dispersed acrylic polymer-based paint, applied in 3 coats to façades on existing plaster, applied mechanically (RAL-1013) |  |  | m² |  | 4.1400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total ventilation ducts Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10. Cover (sheet 31) |  |  |  |  |  |  |  |  |  |  |
| 66 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 0.7500 |  |  |  |  |  |
| 67 |  | TsD01B |  | Spreading of compacted soil by shovel, in uniform layers , 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, soil originating from medium soil |  |  | m³ |  | 0.2500 |  |  |  |  |  |
| 68 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill material being non-cohesive soil |  |  | 100 m³ |  | 0.0100 |  |  |  |  |  |
| 69 |  | CA04F |  | Concrete cast into slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with item CA01, and cast using conventional methods (C 16/20 concrete) |  |  | m3 |  | 0.5000 |  |  |  |  |  |
| 70 |  | CL18A |  | Various metal structures made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete |  |  | kg |  | 20.0000 |  |  |  |  |  |
| 71 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in lightweight construction |  |  | t |  | 0.5400 |  |  |  |  |  |
| 72 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.5400 |  |  |  |  |  |
| 73 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.5400 |  |  |  |  |  |
| 74 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.5400 |  |  |  |  |  |
| 75 |  | CE06A |  | Roofing made of corrosion-protected profiled sheet metal, corrugated or -cut, mounted on metal panels, installed on areas larger than 40 m² using profiled sheet metal sheets fastened with special clips and mechanical screws on the upper surface, including the installation of valleys, flashings, connections to chimneys, etc. |  |  | m² |  | 11.0000 |  |  |  |  |  |
| 76 |  | CL54B |  | Ventilated façade system with metal cladding (external panelling) without thermal insulation, with architectural details covering up to 30% of the total wall area |  |  | m2 |  | 5.5200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cover (sheet 31) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 's quotation service |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total quote: Including salary |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |