| Major repair and modernisation of the Public Institution "Mihai Eminescu", Telenesti District, 8 Codrilor Street, cadastral number 8901224. 151. 02 No. 106-2025-3-IVC | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-3-3

Changing room block         Heating .   ventilation       (brand   IVC)

| No. No. | Standard code and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Heating |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Steel radiators, monobloc, up to 1000 mm in length, including steel panel radiators with side connections, complete with wall mounting brackets and a built-in air vent. 15 units, Kermi PROFIL - K, including: |  |  |  |  |  |  |  |  |  |  |
| 1 |  | IB06A |  | Steel radiators, monobloc with a length of up to 1000 mm, including Steel panel radiators with side connection TIP 33K 800X500 Kermi PROFIL - K |  |  | Pc |  | 7.000 |  |  |  |  |  |
| 2 |  | IB06C |  | Steel radiators, monobloc with a length of 1501–2000 mm - Steel panel radiators with side connection TYPE 33K 1600X500 Kermi PROFIL - K |  |  | pc |  | 2.000 |  |  |  |  |  |
| 3 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" – RLV angle shut-off valve Db=15 . DANFOSS |  |  | pc |  | 9.000 |  |  |  |  |  |
| 4 |  | ID04A |  | Through or check valve with connectors for central heating systems, with a nominal diameter of 1/2" - 1" - Direct thermostatic valve Dn15 Type RA-N ("Danfoss" Austria) |  |  | pc |  | 9.000 |  |  |  |  |  |
| 5 |  | ID04A |  | Gate or check valve with sockets for central heating systems, Thermostatic head Dn15 P=10 bar. Type RA2991 ("Danfoss" Austria) |  |  | pc |  | 4.000 |  |  |  |  |  |
| 6 |  | ID04A |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1/2"–1" Ball valve φ20 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 7 |  | ID04A |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1/2"–1" Ball valve, diameter 25 mm |  |  | pc |  | 2.000 |  |  |  |  |  |
| 8 |  | ID04A |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1/2" - 1" (Shut-off and measuring valve dв=20. ASV-M 20 Danfoss) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 9 |  | IC46A |  | Manifold for heating points and substations, mounted on a ready-made support 65–100 mm ( , φ32, L=250 mm) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 10 |  | ID06A.F |  | Air vent valve with ball valve for central heating systems, (Automatic air vent valve with ball valve, 15 mm) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 11 |  | ID06A.F |  | Air vent valve with movable key for central heating systems, ( -type drain valve, , 15 mm) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 12 |  | IC11D |  | , black steel pipe, longitudinally welded for plumbing systems, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings; pipe diameter 1 1/4" - Light-gauge steel water and gas pipe, diameter 25 mm, wall thickness 2.8 mm, GOST 3262-75* |  |  | m |  | 33.000 |  |  |  |  |  |
| 13 |  | IzA06D |  | Anti-corrosive paintwork on metal joinery, technological equipment and metal structures with alkyd enamel (one coat of GF-21 primer and two coats of BT-177l enamel) |  |  | m² |  | 3.960 |  |  |  |  |  |
| 14 |  | IE03B |  | Carrying out pressure leak tests on supply pipes for heating appliances (air heaters, convector heaters, baseboard convectors, etc.) with a diameter of 1 1/4" ... 2" |  |  | m |  | 33.000 |  |  |  |  |  |
| 15 |  | IC35B |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed at the connection points of heating units or appliances in central heating systems, Polypropylene pipe . ф20х3.4 PP-R 20 |  |  | m |  | 51.000 |  |  |  |  |  |
| 16 |  | IC37B |  | Polyethylene fittings with two joints, press-fitted with a threaded connection to the reinforced polyethylene pipe, with an external diameter of 20.0 mm - Fittings for polypropylene pipe account for 60% of the pipe length |  |  | pc |  | 31.000 |  |  |  |  |  |
| 17 |  | IC35C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed for connecting heating units or appliances in central heating systems, Polypropylene pipe . ф25х4.2 PP-R 25 |  |  | m |  | 29.000 |  |  |  |  |  |
| 18 |  | IC37C |  | Two-joint polyethylene fittings, press-fitted with threaded connections to reinforced polyethylene pipes, with an external diameter of 25.0 mm - Fittings for polypropylene pipes account for 60% of the pipe length |  |  | pc |  | 17.000 |  |  |  |  |  |
| 19 |  | SF01C |  | Carrying out a pressure leak test on hot or cold water installations made of heavy-duty polyvinyl chloride (PVC) pipe or plastic, with a diameter of 16–110 mm |  |  | m |  | 72.000 |  |  |  |  |  |
| 20 |  | RpIF09C |  | Insulation of pipes using special insulation sleeves fitted onto the pipes Basalt wool cylinders with aluminium foil coating FLEXOROCK b=40 mm DN 25 ROCKWOOL |  |  | m |  | 33.000 |  |  |  |  |  |
| 21 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm ф20 ПЭП |  |  | m |  | 51.000 |  |  |  |  |  |
| 22 |  | RpIF09B |  | Insulation of pipes with special insulation sleeves, fitted onto pipes - Porous polyethylene hose 9 mm, 25 mm diameter PEP |  |  | m |  | 29.000 |  |  |  |  |  |
| 23 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines St 3) |  |  | kg |  | 7.000 |  |  |  |  |  |
| 24 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 0.360 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total: Steel radiators, monobloc, with lengths ranging from to 1000 mm, including steel panel radiators with side connections, complete with wall mounting brackets and a built-in air vent. 15 units, Kermi PROFIL - K, including: Including labour |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Ventilation P 1 |  |  |  |  |  |  |  |  |  |  |
| 25 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 250x150 mm, b=0.7 mm |  |  | m2 |  | 2.400 |  |  |  |  |  |
| 26 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Galvanised sheet air ducts 250x200 mm, thickness 0.7 mm |  |  | m2 |  | 1.350 |  |  |  |  |  |
| 27 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of up to 1200 mm. - Air ducts made of 400x200 mm galvanised sheet, thickness 0.7 mm |  |  | m2 |  | 3.600 |  |  |  |  |  |
| 28 |  | VA22B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised sheet metal or aluminium, with a perimeter of up to 1200 mm. - Galvanised sheet metal air ducts 400x300 mm, thickness 0.7 mm |  |  | m2 |  | 0.700 |  |  |  |  |  |
| 29 |  | VA22D |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.7 mm thick galvanised or aluminium sheet with a perimeter of: 2000 mm. - Galvanised sheet air ducts 800x200 mm, thickness 0.7 mm |  |  | m2 |  | 1.000 |  |  |  |  |  |
| 30 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and fitted into masonry |  |  | pc |  | 3.000 |  |  |  |  |  |
| 31 |  | Company price |  | P800x200 supply air control grille |  |  | pc |  | 2.000 |  |  |  |  |  |
| 32 |  | Company price |  | , P400x300 exhaust grille |  |  | pc |  | 1.000 |  |  |  |  |  |
| 33 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 3.000 |  |  |  |  |  |
| 34 |  | Company price |  | Throttle valve 150x250 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 35 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 36 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 37 |  | VB19A |  | Air outlet, for Type I air shower - -type hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 38 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 39 |  | CC09A |  | Installation of mesh for plaster reinforcement on ceilings and walls, thermal insulation protection, and duct concealment, applied to flat ceilings on a 10×10 mm steel mesh |  |  | m² |  | 1.000 |  |  |  |  |  |
| 40 |  | IzH22A |  | Insulation of pipes using ready-made mineral wool sleeves, with thicknesses of 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 8.000 |  |  |  |  |  |
| 41 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 42 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers, with a weight of up to 2 kg per piece (Steel for securing pipelines St 3) |  |  | kg |  | 4.000 |  |  |  |  |  |
| 43 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer. Painting of fixings |  |  | m² |  | 0.210 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation P 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. B1 |  |  |  |  |  |  |  |  |  |  |
| 44 |  | VA19A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium, 0.5 mm thick, with a circular cross-section diameter of 200 mm. (circular cross-section) Air ducts made of galvanised sheet metal 200 mm , б=0.5 mm |  |  | m2 |  | 0.314 |  |  |  |  |  |
| 45 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of galvanised sheet metal or aluminium, 0.5 mm thick, with a perimeter of 600 mm. - Air ducts made of galvanised sheet metal 100x100 mm , б=0.5 mm |  |  | m2 |  | 3.200 |  |  |  |  |  |
| 46 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 600 mm. - Galvanised sheet metal air ducts 150x100 mm , thickness = 0.5 mm |  |  | m2 |  | 1.200 |  |  |  |  |  |
| 47 |  | VA21A |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet metal with a perimeter of 600 mm. - Galvanised sheet metal air ducts 200x150 mm , thickness = 0.5 mm |  |  | m² |  | 0.300 |  |  |  |  |  |
| 48 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of galvanised sheet metal 200x200 mm, thickness 0.5 mm |  |  | m² |  | 1.750 |  |  |  |  |  |
| 49 |  | VA21B |  | Installation of ventilation ducts at a height of up to 3 m from the floor, made of 0.5 mm thick galvanised or aluminium sheet with a perimeter of 800–1000 mm. - Air ducts made of 300x100 mm galvanised sheet, thickness 0.5 mm |  |  | m2 |  | 0.400 |  |  |  |  |  |
| 50 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry |  |  | pc |  | 8.000 |  |  |  |  |  |
| 51 |  | Company price |  | R200x100 adjustable exhaust grille |  |  | pc |  | 4.000 |  |  |  |  |  |
| 52 |  | Company price |  | P300x100 adjustable exhaust grille |  |  | pc |  | 4.000 |  |  |  |  |  |
| 53 |  | CL20A |  | Ready-made ventilation grilles made of black sheet metal, with manually adjustable louvers, painted and installed in masonry (Throttle valve) |  |  | pc |  | 6.000 |  |  |  |  |  |
| 54 |  | Company price |  | Throttle valve 100x100 |  |  | pc |  | 4.000 |  |  |  |  |  |
| 55 |  | Company price |  | Throttle valve 150x150 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 56 |  | VB19A |  | Discharge outlet, for Type I air shower - PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 57 |  | Company price |  | PT 614 L35 pitot tube |  |  | pc |  | 1.000 |  |  |  |  |  |
| 58 |  | VB19A |  | Air outlet, for Type I air shower - Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 59 |  | Company price |  | Access hatch for cleaning air ducts PT 614 L36 L37 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 60 |  | CC09A |  | Installation of mesh for plaster reinforcement on ceilings and walls, thermal insulation protection, and duct concealment, applied to flat ceilings on a 10×10 mm steel mesh |  |  | m² |  | 0.100 |  |  |  |  |  |
| 61 |  | IzH22A |  | Insulation of pipes with ready-made mineral wool sleeves, thickness 20; 30; 40; 50; 60; 70 and 80 mm – Mineral wool insulation, thickness 25 mm K-FLEX AIR METAL self-adhesive roll insulation with a protective aluminium foil coating (foil-faced), b=25 mm |  |  | m² |  | 11.000 |  |  |  |  |  |
| 62 |  | Company price |  | Aluminium adhesive tape for sealing joints REK and KIMJAL (48 m) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 63 |  | IC42A |  | Supports and fasteners for securing pipes, boilers, appliances and containers weighing up to 2 kg per item (Steel for securing pipelines Grade 3) |  |  | kg |  | 5.000 |  |  |  |  |  |
| 64 |  | CN20B |  | Interior or exterior paint applied to metal joinery with alkyd enamel in 2 coats, including primer Painting of fixings |  |  | m² |  | 0.420 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total B1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social and health insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Semi-finished product and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 65 |  | ID04A |  | Gate or check valve with sockets for central heating systems, with a nominal diameter of 1/2"-1" ASV-PV - Danfoss automatic balancing valve, 20 mm |  |  | pc |  | 2.000 |  |  |  |  |  |
| 66 |  | SE56A1 |  | Drinking water filter with flanges for pipe mounting, size 1"–2" – Danfoss mesh filter ф20 |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. P 1 |  |  |  |  |  |  |  |  |  |  |
| 67 |  | VC16A |  | Installation of an ALTPA-type indoor partial air handling unit, comprising air intake, filtration, ventilation, heating, cooling and air discharge, with a treated air flow rate of 3,000–7,500 m³/h – Part 1: Supply air unit as a complete set: Supply air fan L=500 m³/h; N=0.41 kW, 50 Hz, U=400 V: , 2 soft inserts, G4 filter. 9.9 kW electric heater, MPA 1200, E3 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 68 |  | VB18A |  | Noise enclosure, rectangular with perimeter 1000–2000 mm, - Silencer 400×200 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 69 |  | VB04A |  | Counterweight damper, with a perimeter of 800–1600 mm - Servo-operated air damper 400×200 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 70 |  | AcB09A |  | Installation of electric actuators for dampers, dampers with a diameter of 400–600 mm - Servo-actuator |  |  | pcs |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total П 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. В1 |  |  |  |  |  |  |  |  |  |  |
| 71 |  | VC03B |  | Installation of standardised, silent, single-inlet radial fans, belt-driven, with an air flow rate of 900–11,200 m³/h and an electric motor of 0.37–7.5 kW -В1 Duct fan, centrifugal exhaust fan; L= 500 m³/h. N=0.16 kW. U=230 V, 50 Hz VKMts 200 VENTS |  |  | pc |  | 1.000 |  |  |  |  |  |
| 72 |  | VC24A |  | Elastic fan mounting, with rubber elastic mounts - Soft insert Ф200 |  |  | pc |  | 2.000 |  |  |  |  |  |
| 73 |  | VB17A |  | Noise attenuator, circular type AZC with a circumference of 800–1600 mm - Noise attenuator ф200 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 74 |  | VB01A |  | Control damper, butterfly, mounted on circular ducts type CFC-I, CFC-II, with a circumference of 800–1600 mm - Servo-operated air damper ф200 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 75 |  | AcB09A |  | Installation of electric actuators for dampers, dampers with a diameter of 400–600 mm - Servo drive |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total В1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social security |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Costs of semi-finished products and storage |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Quotation profit |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Cost of equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Heating |  |  |  |  |  |  |  |  |  |  |
| 76 |  | Company price |  | Automatic balancing valve DN 20 Type ASV-PV-20 "Danfoss" Austria |  |  | pc |  | 2.000 |  |  |  |  |  |
| 77 |  | Company price |  | Danfoss F20 strainer |  |  | pc |  | 2.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Ventilation |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. P1 |  |  |  |  |  |  |  |  |  |  |
| 78 |  | Company price |  | P1 Supply air unit complete with: Supply air fan L=500 m³/h; N=0.41 kW, 50 Hz, U=400 V: 2 soft inserts, G4 filter. 9.9 kW electric heater, MPA 1200 E3 |  |  | pc |  | 1000 |  |  |  |  |  |
| 79 |  | Company price |  | Silencer 400x200 |  |  | pc |  | 1.000 |  |  |  |  |  |
| 80 |  | Company price |  | Servo-operated air damper 400x200 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total П1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. В1 |  |  |  |  |  |  |  |  |  |  |
| 81 |  | Company price |  | B1 Duct fan, centrifugal exhaust fan; L = 500 m³/h; N = 0.16 kW; U = 230 V, 50 Hz; VKMts 200 VENTS |  |  | pc |  | 1.000 |  |  |  |  |  |
| 82 |  | Company price |  | Soft- e insert, diameter 200 mm |  |  | pc |  | 2.000 |  |  |  |  |  |
| 83 |  | Company price |  | Silencer, 200 mm |  |  | pc |  | 1.000 |  |  |  |  |  |
| 84 |  | Company price |  | Servo-operated air damper ф200 |  |  | pc |  | 1.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total В1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Value of equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |