| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-3-4

Boiler room. Networks:   Electrical:   Interior

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and |  |  |  |  |  |  |  |  |  |  |
| 1 |  | TsA16A1 |  | Manual excavation of soil, in confined spaces, in trenches up to 4 m deep, for high-voltage electrical cables, in soil with natural moisture without shoring, width < 1 m, depth < 1.5 m, soft ground |  |  | m3 |  | 2.1000 |  |  |  |  |  |
| 2 |  | TsD01B |  | Spreading of sifted soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, soil originating from medium-grade terrain |  |  | m3 |  | 2.1000 |  |  |  |  |  |
| 3 |  | TsD04A |  | Compaction with a hand tamper of backfill in horizontal or 1/4-inclined trenches, including wetting each individual layer of soil, 10 cm thick, non-cohesive soil |  |  | m3 |  | 2.1000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Completion of the existing boiler room design |  |  |  |  |  |  |  |  |  |  |
| 4 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/1P/C25A circuit breaker) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 5 |  | Supplier price |  | Y-134 cable label |  |  | pc |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Completion of existing boiler room Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. PDF5 |  |  |  |  |  |  |  |  |  |  |
| 6 |  | 08-03-572-3 |  | Control panel, cabinet type or distribution point (cabinet), wall-mounted, height and width, mm, up to 540×310×120 (Metal distribution board ЩРн-36з-0 36 УХЛ3 IP31) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 7 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load-breaking switch QS/3P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 8 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (QF/3P/B25A circuit breaker) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 9 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (automatic) (QF/1P/C4A circuit breaker) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 10 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker QF/1P/B10A) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 11 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic switches ) (automatic switch QF/1P/B6A) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 12 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (circuit breakers) (residual current circuit breaker АВДТ/2P/B10A/30mА) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 13 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/C16A/10mА) |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 14 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic differential circuit breaker АВДТ/2P/C16A/30mА) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 15 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM/4P/25A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 16 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM/2P/25A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 17 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting preparation (2-channel time relay РЭВ-201) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 18 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (RBD 80A rail-mounted distribution block) |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 19 |  | Supplier price |  | Rail <N>, 10 holes |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 20 |  | Supplier price |  | <PE> rail, 10 holes |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 21 |  | Supplier price |  | Y-134 cable label |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDF5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Set of cables and pipes |  |  |  |  |  |  |  |  |  |  |
| 22 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and junction boxes, weight per 1 m: up to 1 kg |  |  | 100 m |  | 3.4000 |  |  |  |  |  |
| 23 |  | Market price |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products, TU 16-705.496-2011, GOST 31996-2012: VVGng-LSLTx 2x1.5 mm² - 0.6 kV |  |  | m |  | 20.0000 |  |  |  |  |  |
| 24 |  | Price of on the market |  | Fire-retardant copper cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 3x1.5 mm² - 0.6 kV |  |  | m |  | 230.0000 |  |  |  |  |  |
| 25 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 3x2.5 mm² - 0.6 kV |  |  | m |  | 45.0000 |  |  |  |  |  |
| 26 |  | Market price for |  | Fire-retardant copper-core cable with low smoke and gas emission and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-LSLTx 5x6.0 mm² - 0.6 kV |  |  | m |  | 45.0000 |  |  |  |  |  |
| 27 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and pre-installed boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 0.5500 |  |  |  |  |  |
| 28 |  | Market price |  | Fire-resistant copper-core cable that does not propagate fire, with low smoke and gas emission and low toxicity of combustion products; TU 16-705.496-2011, GOST 31996-2012: VVGng-FRLSLTx 2x1.5 mm² - 0.6 kV |  |  | m |  | 10.0000 |  |  |  |  |  |
| 29 |  | Market price for |  | Fire-resistant copper-core cable that does not propagate fire, with low smoke and gas emission, and low toxicity of combustion products TU 16-705.496-2011 GOST 31996-2012: VVGng-FRLSLTx 3x1.5 mm² - 0.6 kV |  |  | m |  | 45.0000 |  |  |  |  |  |
| 30 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and pre-installed boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 0.1000 |  |  |  |  |  |
| 31 |  | Market price |  | Fire-retardant control cable with copper conductors, low smoke and gas emission, GOST 31996-2012 КВВГнг-LS, cross-section 4x1.5mm²-0.66V |  |  | m |  | 10.0000 |  |  |  |  |  |
| 32 |  | 08-02-405-1 |  | Conductor for steel structures and mounted panels, cross-section up to 16 mm² |  |  | 100 m |  | 0.4000 |  |  |  |  |  |
| 33 |  | Market price |  | Copper-wired cable, PVC-insulated, GOST 6323-79, cross-section 1x6mm² ПВ-3 |  |  | m |  | 30.0000 |  |  |  |  |  |
| 34 |  | Price from market |  | Copper-wired cable, PVC-insulated, GOST 6323-79, cross-section 1x16mm² ПВ-3 |  |  | m |  | 10,0000 |  |  |  |  |  |
| 35 |  | 08-02-409-1 |  | Vinyl-plastic pipe for building installations, on walls and columns, clamped in place, diameter up to 25 mm |  |  | 100 m |  | 2.2500 |  |  |  |  |  |
| 36 |  | Market price |  | PVC pipe, TU 2248-002-14369938-2008, diameter D16 mm |  |  | m |  | 200.0000 |  |  |  |  |  |
| 37 |  | Price from market |  | PVC pipe, TU 2248-002-14369938-2008 with diameter, D25mm |  |  | m |  | 25.0000 |  |  | 0 |  |  |
| 38 |  | 08-02-411-1 |  | Metal hose, outer diameter up to 48 mm |  |  | 100 m |  | 1.2000 |  |  |  |  |  |
| 39 |  | Market price |  | PVC-insulated metal hose, РЗ-ЦП: with a diameter of 18 mm |  |  | m |  | 100.0000 |  |  |  |  |  |
| 40 |  | Price from market |  | PVC-insulated metal hose, РЗ-ЦП: with diameter, D25mm |  |  | m |  | 20,0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Set of cables and pipes Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Lighting fixtures |  |  |  |  |  |  |  |  |  |  |
| 41 |  | 08-03-594-17 |  | Lighting fixture with fluorescent lamps in suspended ceilings, fitted into recessed units; number of lamps in the fixture: 4 |  |  | 100 pcs |  | 0.1000 |  |  |  |  |  |
| 42 |  | Supplier price |  | LED luminaire (recessed) 600x600mm, 4000K, 3400lm, 28W , IP20 RC065B G5 34S/840 PSD W60L60 OC SC |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 43 |  | Supplier price |  | LED luminaire (recessed) 595x595mm, 4000K, 3800lm, 34W, IP54 OWP OPTIMA LED 595 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 44 |  | 08-03-593-6 |  | Lighting fixture for recessed ceiling or wall lamps with screw mounting for rooms with normal environmental conditions, single-lamp |  |  | 100 pcs |  | 0.0800 |  |  |  |  |  |
| 45 |  | Supplier price |  | Round LED luminaire (recessed) 4000K, 1350lm, 12W, IP44 Eglo Fueva 5 |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 46 |  | 08-03-594-6 |  | Lighting fixture with fluorescent lamps mounted separately on suspension arms (bars), number of lamps in the fixture, 1 |  |  | 100 pcs |  | 0.0400 |  |  |  |  |  |
| 47 |  | Supplier price |  | LED luminaire (surface-mounted) 1000mm, 3000K, 5000lm, 40W, IP66 PowMr LBLWLW040 |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 48 |  | 08-03-593-6 |  | Lighting fixture for recessed ceiling or wall lamps with screw mounting for rooms with normal environmental conditions, single-lamp |  |  | 100 pcs |  | 0.0800 |  |  |  |  |  |
| 49 |  | Supplier price |  | LED luminaire (surface-mounted) 4000K, 1800lm, 18W, IP65, PL-13272P |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 50 |  | 08-03-593-10 wall-mounted |  | Wall-mounted lighting indicators (emergency lighting fixtures) |  |  | 100 pcs |  | 0.0300 |  |  |  |  |  |
| 51 |  | Supplier price |  | 8W exit sign, IP20 |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Lighting fixtures Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Sockets, sockets and switches |  |  |  |  |  |  |  |  |  |  |
| 52 |  | 08-03-591-2 |  | Single-pole switch, flush-mounted type, for concealed installation |  |  | 100 pcs |  | 0.0700 |  |  |  |  |  |
| 53 |  | Supplier price |  | Single-pole switch 10A, 250V for flush mounting, IP20 |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 54 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (Motion sensor) |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 55 |  | Supplier price |  | IP20 motion sensor |  |  | pc |  | 9.0000 |  |  |  |  |  |
| 56 |  | 08-03-532-4 |  | Control panel with buttons, general purpose, wall or column mounted, up to 3 panel elements |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 57 |  | Supplier price |  | Control panel with two "START-STOP" buttons ПКЕ 722-2У2 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 58 |  | 08-03-591-9 |  | Surface-mounted socket, for flush-mounted installation |  |  | 100 pcs |  | 0.0300 |  |  |  |  |  |
| 59 |  | Supplier price |  | Socket for flush-mounted installation, Unom=250V, Inom=16A, protection rating IP20 16A / 230V / 2P+PE IP20 |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 60 |  | 08-03-591-8 |  | Surface-mounted plug socket, for open installation |  |  | 100 pcs |  | 0.0800 |  |  |  |  |  |
| 61 |  | Supplier price |  | Socket for open installation, Unom=250V, Inom=16A, protection rating IP54 16A / 230V / 2P+PE IP54 |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 62 |  | Supplier price |  | Junction box for concealed wiring У -197 УХЛ4 |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 63 |  | Supplier price |  | Box for mounting switches and sockets У -196 УХЛ4 |  |  | pc |  | 8.0000 |  |  |  |  |  |
| 64 |  | Supplier price |  | Junction box for open wiring 80x80x40mm, IP54 |  |  | pc |  | 8.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sockets and Switches Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. Earth socket |  |  |  |  |  |  |  |  |  |  |
| 65 |  | 08-02-471-4 |  | Earth terminal, vertical, made of round steel, diameter 16 mm |  |  | 10 pcs |  | 0.3000 |  |  |  |  |  |
| 66 |  | 08-02-472-6 |  | Open earthing conductor, on building supports, made of split steel, cross-section 100 mm² |  |  | 100 m |  | 0.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Earth connection Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | 10% |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Completion of PDG for existing boiler room |  |  |  |  |  |  |  |  |  |  |
| 67 |  | Supplier price |  | Circuit breaker, 1P, characteristic C, In=25A, Icn=6kA QF/1P/C25A |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Completion of existing boiler house Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. PDF5 |  |  |  |  |  |  |  |  |  |  |
| 68 |  | Supplier price |  | Surface-mounted metal distribution board for the installation of modular units (up to 36 modules), IP31, ЩРн-36з-0 36 УХЛ3 IP31 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 69 |  | Supplier price |  | -load separator 3P, In=40A, Un=400V QS/3P/40A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 70 |  | Supplier price |  | Circuit breaker, 3P, B-type, In=25A, Icn=6kA QF/3P/B25A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 71 |  | Supplier price |  | Circuit breaker, 1P, characteristic C, In=4A, Icn=6kA QF/1P/C4A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 72 |  | Supplier price |  | Circuit breaker, 1P, characteristic B, In=10A, Icn=6kA QF/1P/B10A |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 73 |  | Supplier price |  | Circuit breaker, 1P, B-type, In=6A, Icn=6kA QF/1P/B6A |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 74 |  | Supplier price |  | Residual current circuit breaker, type B, 2P, Inom=10A, Idif=30mA. AVDT/2P/B10A/30mA |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 75 |  | Supplier price |  | Residual current circuit breaker, type B, 2-pole, rated current = 16A, residual current = 10mA. RCCB/2P/C16A/10mA |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 76 |  | Supplier price |  | Residual current circuit breaker, type B, 2P, Rated current=16A, RCD current=30mA. RCCB/2P/C16A/30mA |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 77 |  | Supplier price |  | Modular contactor, 4P, In=25A KM/4P/25A |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 78 |  | Supplier price |  | Modular contactor, 2P, In=25A KM/2P/25A |  |  | pc |  | 3.0000 |  |  |  |  |  |
| 79 |  | Supplier price |  | 2-channel time relay, 230V/16A РЭВ-201 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 80 |  | Supplier price |  | DIN rail distribution block (2.7) РБД 80А |  |  | pc |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDF5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |