| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-3-6

Boiler room. Design

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Ceilings |  |  |  |  |  |  |  |  |  |  |
| 1 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, with a height of up to 4 m: ceilings without insulation (moisture-resistant plasterboard -GTS for walls and GTS for ceilings) |  |  | m² |  | 11.2000 |  |  |  |  |  |
| 2 |  | СF59D |  | Cladding of surfaces with a layer of PGC with the installation of a simple flat metal frame, up to 4 m high: ceilings without insulation (moisture-resistant plasterboard – shower for women and shower for men) |  |  | m² |  | 15.3000 |  |  |  |  |  |
| 3 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 26.5000 |  |  |  |  |  |
| 4 |  | CF52B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture on the ceiling, with mortar prepared by hand |  |  | m² |  | 26.5000 |  |  |  |  |  |
| 5 |  | CF53B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture, on the ceiling, with mortar prepared by hand. Additional or reduced amount for every 1.0 mm (deduct 3 mm from item CF52, K=3) |  |  | m² |  | -26.5000 |  |  |  |  |  |
| 6 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 26.5000 |  |  |  |  |  |
| 7 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 26.5000 |  |  |  |  |  |
| 8 |  | CN53A |  | Priming of interior ceiling surfaces |  |  | m² |  | 26.5000 |  |  |  |  |  |
| 9 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing plaster, carried out manually |  |  | m2 |  | 26.5000 |  |  |  |  |  |
| 10 |  | CK29F apl |  | Suspended ceilings made of prefabricated panels, including the grid system (Rockfon Artic E25 painted ceiling tile 600x600x15 mm) |  |  | m2 |  | 22.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ceilings Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Floors |  |  |  |  |  |  |  |  |  |  |
| 11 |  | IzF53A |  | Manual installation of floor underlay with thermal insulation layer made of extruded polystyrene foam boards, 40 mm thick, in a single layer |  |  | m2 |  | 49.3000 |  |  |  |  |  |
| 12 |  | CG01A |  | Floor screed made of 3 cm thick M 100-T cement mortar with a finely trowelled finish |  |  | m2 |  | 49.3000 |  |  |  |  |  |
| 13 |  | CG01A1 |  | Sub-base layer for flooring made of 3 cm thick M 100-T cement mortar with a finely trowelled finish. For every 0.5 cm deviation (either way) from the 3 cm M 100-T mortar sub-base layer, add 2 cm, K=4 |  |  | m² |  | 49.3000 |  |  |  |  |  |
| 14 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 100x100x4 - 1.75 kg/m²) |  |  | kg |  | 86.3000 |  |  |  |  |  |
| 15 |  | IzF04A |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including flashings and valleys in the existing waterproofing on horizontal or sloping surfaces up to 40% gradient, flat or curved, using bitumen mastic applied with a brush or rubber trowel (trowel) |  |  | m² |  | 49.3000 |  |  |  |  |  |
| 16 |  | CG47D |  | Ceramic tile flooring, including the adhesive backing layer (dry mix), tile dimensions: 600 x 600 mm (6 mm porcelain tile underlay; tile dimensions 600 x 600 mm (Bascal Bone porcelain tiles) |  |  | m² |  | 49.3000 |  |  |  |  |  |
| 17 |  | CI14A |  | Linear elements made of ceramic tiles applied with adhesive |  |  | m |  | 32.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Flooring Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Walls |  |  |  |  |  |  |  |  |  |  |
| 18 |  | CF05A |  | 3 cm thick interior plaster, applied over wire mesh, trowelled, using M 100-T cement-lime mortar for the scratch coat, M 50-T cement-lime mortar for the base coat and M 10-T lime-cement mortar for the finish coat, applied by hand to flat surfaces, including the installation of steel reinforcement and the wire mesh, applied to walls |  |  | m² |  | 90.4000 |  |  |  |  |  |
| 19 |  | CF17B |  | Miscellaneous works – thickness variation, ±5 mm for the primer coat applied to walls, executed with mortar (add 10 mm, K=2) |  |  | m² |  | 90.4000 |  |  |  |  |  |
| 20 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 90.4000 |  |  |  |  |  |
| 21 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mixture, on walls and partition walls, with mortar prepared by hand. |  |  | m2 |  | 90.4000 |  |  |  |  |  |
| 22 |  | CF51B |  | 5 mm thick interior plastering, applied by hand, using a dry gypsum-based mixture, on walls and partition walls, with mortar prepared by hand. Difference in excess or shortfall for every 1.0 mm (2 mm is deducted from item CF50, K=2) |  |  | m² |  | -90.4000 |  |  |  |  |  |
| 23 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 90.4000 |  |  |  |  |  |
| 24 |  | CF57A |  | Manual application of 1.0 mm thick "Eurofin" gypsum-based plaster to the surfaces of walls, columns and ceilings |  |  | m2 |  | 90.4000 |  |  |  |  |  |
| 25 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in 2 coats to existing plaster, carried out manually (washable paint applied in 2 coats, RAL 7040) |  |  | m² |  | 90.4000 |  |  |  |  |  |
| 26 |  | IzF04F |  | Hot-applied waterproofing layer on terraces, roofs, foundations and ground slabs, in areas without groundwater, including the troughs and valleys in the existing waterproofing on surfaces with a slope of over 40% or on vertical, flat or curved surfaces, using bitumen mastic or bitumen with added rubber, applied with a brush or rubber trowel (trowel) |  |  | m² |  | 21.3600 |  |  |  |  |  |
| 27 |  | CF02A |  | 2 cm thick interior plaster, rough-textured, applied by hand to walls or pillars, on flat surfaces using M 25-T lime-cement mortar for the base coat and M 10-T mortar for the primer and finish coat, on brickwork or small concrete blocks |  |  | m2 |  | 84.0000 |  |  |  |  |  |
| 28 |  | CF17B |  | Miscellaneous works – thickness variation, ±5 mm for the primer coat applied to walls, executed with mortar (add 10 mm, K=2) |  |  | m2 |  | 84.0000 |  |  |  |  |  |
| 29 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 84.0000 |  |  |  |  |  |
| 30 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pilasters and window sills) fixed with adhesive (dry mix), tile dimensions: up to 200 x 200 mm (Eleganza Saggio porcelain tiles) |  |  | m2 |  | 42.0000 |  |  |  |  |  |
| 31 |  | CI22B |  | Ceramic tile cladding (on walls, columns, pilasters and window sills) fixed with adhesive (dry mix), tile dimensions: up to 600 x 1200 mm (ROUGE POL.RECT 60x120 tile) |  |  | m2 |  | 42.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Walls Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Sanitary engineering ( ) |  |  |  |  |  |  |  |  |  |  |
| 32 |  | SC13A |  | Semi-crystal bathroom mirror with polished edges (Mirror dimensions: 2000x800mm) |  |  | pc |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sanitary Engineering Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation work |  |  |  |  |  |  |  |  |  |  |
| 33 |  | SC11B apl |  | Soap dispenser, Esenia stainless steel, 1000ml |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 34 |  | SC11B wall-mounted |  | Folded Towel Dispenser Dimensions: 260 x 280 x 110 mm Material: satin-finish stainless steel |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 35 |  | SC11B wall-mounted |  | Toilet Paper Dispenser Satin-Finish Stainless Steel Dimensions: D260 x 120 mm Material: satin-finish stainless steel |  |  | pc |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Sanitary plumbing equipment |  |  |  |  |  |  |  |  |  |  |
| 36 |  | Supplier price |  | Soap dispenser, Esenia stainless steel, 800ml |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 37 |  | Supplier price |  | Folded Towel Dispenser Dimensions: 260 x 280 x 110 mm Material: satin-finish stainless steel |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 38 |  | Supplier price |  | Toilet Paper Dispenser Satin-Finish Stainless Steel Dimensions: D260 x 120 mm Material: satin-finish stainless steel |  |  | pc |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sanitary equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |