| Major repair and modernisation of the public institution ‘Mihai Eminescu’ Secondary School, Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.03 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 4-1-3

Outdoor Electrical Lighting

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Quotation value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. T2 trenches |  |  |  |  |  |  |  |  |  |  |
| 1 |  | TsC03B1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading at a Category II landfill site |  |  | 100 m³ |  | 3.1200 |  |  |  |  |  |
| 2 |  | TsA20B |  | Manual earth excavation, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade soil |  |  | m3 |  | 31.1600 |  |  |  |  |  |
| 3 |  | AcF03A |  | Backfill in trenches for water supply or sewerage pipes, as a substrate, protective layer, insulation layer or filter layer ( ) for drainage pipes, carried out with sand |  |  | m3 |  | 69.2400 |  |  |  |  |  |
| 4 |  | 34-02-003-1 |  | Installation of polyethylene pipes: up to 2 holes (corrugated PE pipe, 50 mm diameter) |  |  | 1 km |  | 1.1540 |  |  |  |  |  |
| 5 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp crawler tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 2.4200 |  |  |  |  |  |
| 6 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill material being non-cohesive soil |  |  | 100 m3 |  | 2.4200 |  |  |  |  |  |
| 7 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading into category II off-road vehicles |  |  | 100 m3 |  | 0.6900 |  |  |  |  |  |
| 8 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 114.2500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total T2 Trench Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. T3, T4 brackets |  |  |  |  |  |  |  |  |  |  |
| 9 |  | TsC03B1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading at a Category II landfill site |  |  | 100 m³ |  | 0.0600 |  |  |  |  |  |
| 10 |  | TsA20B |  | Manual earth excavation, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade soil |  |  | m3 |  | 0.5600 |  |  |  |  |  |
| 11 |  | AcF03A |  | Backfilling in trenches for water supply or sewerage pipes, as a substrate, protective layer, insulation layer or filter layer for drainage pipes, carried out with sand |  |  | m³ |  | 1.2400 |  |  |  |  |  |
| 12 |  | 34-02-003-1 |  | Installation of polyethylene pipes: up to 2 holes ( PE corrugated pipe, 50 mm diameter) |  |  | 1 km |  | 0.0130 |  |  |  |  |  |
| 13 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp crawler tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.0400 |  |  |  |  |  |
| 14 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill material being non-cohesive soil |  |  | 100 m3 |  | 0.0400 |  |  |  |  |  |
| 15 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading into category II off-road vehicles |  |  | 100 m3 |  | 0.0120 |  |  |  |  |  |
| 16 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 2.0500 |  |  |  |  |  |
| 17 |  | AcA16A1 |  | For each cable, duct, telephone line, etc. crossing, add |  |  | pcs |  | 17.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total T3, T4 trenches Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Outdoor lighting system |  |  |  |  |  |  |  |  |  |  |
| 18 |  | TsA02B |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in non-cohesive or weakly cohesive soil, depth < 0.75 m, medium ground |  |  | m³ |  | 13.5200 |  |  |  |  |  |
| 19 |  | TsC54C |  | Gravel foundation layer |  |  | m3 |  | 2.7200 |  |  |  |  |  |
| 20 |  | CA03E2 |  | Concrete cast in foundations, plinths, retaining walls and walls below ground level, mixed on-site using a concrete mixer and poured using conventional methods; reinforced concrete, grade C 20/16 (Bc 20/B 250) |  |  | m³ |  | 10.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Outdoor lighting system Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Completion of existing general distribution panel |  |  |  |  |  |  |  |  |  |  |
| 21 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker ВА 47-29/3P/C/16A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 22 |  | 08-02-163-2 |  | Terminal cap with polyethylene heat-shrink sleeves for 3-4 conductor cable with paper insulation, voltage up to 1 kV, conductor cross-section up to: 120 mm² ( heat-shrink sleeve 5ТПИ-4/25 mm²) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 23 |  | Supplier price |  | Heat-shrinkable sleeve 4/25mm², 5 outlets, black 5ТПИ-4/25mm² |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 24 |  | Supplier price |  | Heat-shrink tubing 16/8mm |  |  | m |  | 3.0000 |  |  |  |  |  |
| 25 |  | Supplier price |  | Cable label |  |  | pc |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Completion of existing general distribution panel Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. PDIE Distribution Panel |  |  |  |  |  |  |  |  |  |  |
| 26 |  | 08-03-572-3 |  | Cabinet-type control unit or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Metal distribution board 650(h)×500×220 mm ЩМП-3-0 36 УХЛ3 IP31 ) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 27 |  | 08-01-066-1 |  | Surge arrester, voltage up to 10 kV (3P surge arrester, class B In=30kA, ОПС1-B 3P/30kA) |  |  | set |  | 1.0000 |  |  |  |  |  |
| 28 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (load break switch BH 32/3P/40A) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 29 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker BA47-29/3P/B/10A) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 30 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic circuit breaker BA47-29/1P/B/6A) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 31 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (modular contactor KM20-20/2P/20A) |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 32 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting preparation (Time relay ТЭ15/16А) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 33 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting preparation (Modular control button КМУ11) |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 34 |  | Supplier price |  | Galvanised bar 25x4mm |  |  | m |  | 0.5000 |  |  |  |  |  |
| 35 |  | Supplier price |  | SM-30 bushing insulator |  |  | pc |  | 2.0000 |  |  |  |  |  |
| 36 |  | 08-02-163-2 |  | Terminal cap with polyethylene heat-shrink sleeves for 3-4 conductor cable with paper insulation, voltage up to 1 kV, conductor cross-section up to: 120 mm² ( heat-shrink sleeve 5ТПИ-4/25 mm²) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 37 |  | Supplier price |  | Heat-shrinkable sleeve 4/25 mm², 5 outlets, black 5ТПИ-4/25mm² |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 38 |  | 08-02-165-1 |  | Terminal sleeve, made of synthetic resins (epoxides), for 3-core cable, voltage up to 1 kV, conductor cross-section up to: 35 mm² |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 39 |  | Supplier price |  | Manson terminal 5x16/25mm² 5ПКТп(б)-1-16/25(б) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 40 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (DIN mounting rail 35x1mm 1 m) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 41 |  | Supplier price |  | DIN mounting rail 35x1mm |  |  | m |  | 0.5000 |  |  |  |  |  |
| 42 |  | Supplier price |  | Heat-shrink tubing 16/8mm |  |  | m |  | 10.0000 |  |  |  |  |  |
| 43 |  | 08-02-411-4 |  | Flexible connection, outer diameter of metal hose up to 27 mm (PG29 cable gland (18–24) mm IP54) |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 44 |  | 08-02-411-4 |  | Flexible connection, outer diameter of metal hose up to 27 mm (PG36 connector, 24–32 mm, IP54) |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 45 |  | Supplier price |  | Cable label |  |  | pc |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDIE Distribution Panel Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Outdoor lighting system |  |  |  |  |  |  |  |  |  |  |
| 46 |  | RpEN05A |  | Installation of poles (Elmos AD40 galvanised steel pole 4000x105/60mm) |  |  | pc |  | 50.0000 |  |  |  |  |  |
| 47 |  | RpEN05A |  | Installation of posts (Elmos AD90 galvanised steel post 9000x153/60mm) |  |  | pc |  | 18.0000 |  |  |  |  |  |
| 48 |  | RpEN05A |  | Installation of posts (Elmos AD120 galvanised steel post 12000x153/60mm) |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 49 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (automatic) (automatic switch, 1P, In=2A, BA47-29/1P/B2A) |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 50 |  | Supplier price |  | Circuit breaker, 1P, In=2A, BA47-29/1P/B2A |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 51 |  | RpEQ04A apl |  | Installation of the HBOXCON Fumagalli box |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 52 |  | Supplier price |  | Junction box, Class II, IP44 HBOXCON Fumagalli |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 53 |  | Supplier price |  | Galvanised components for fixing the column to the foundation |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 54 |  | Supplier price |  | Galvanised anchor bolts M16x100 |  |  | pc |  | 296.0000 |  |  |  |  |  |
| 55 |  | Supplier price |  | Galvanised reinforced washers DIN 125 |  |  | pc |  | 296.0000 |  |  |  |  |  |
| 56 |  | Supplier price |  | Galvanised hexagon nuts M16 (DIN 934) |  |  | pc |  | 296.0000 |  |  |  |  |  |
| 57 |  | RpEO02A |  | Mounting of single-arm bracket |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 58 |  | Supplier price |  | Galvanised single bracket 1000x60mm |  |  | pc |  | 74.0000 |  |  |  |  |  |
| 59 |  | 08-03-595-1 |  | Lighting fixture with mercury lamps on brackets, including ARD mounting |  |  | 100 pcs |  | 0.7400 |  |  |  |  |  |
| 60 |  | Supplier price |  | 330W LED street light, 5700K, 45500lm, IP66, IK10 Philips BRP370 LED455 CW SLC S1 |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 61 |  | Supplier price |  | 37W LED street light, 4000K, 5162lm, IP65 Philips Malaga BRP102 LED58/740 |  |  | pc |  | 50.0000 |  |  |  |  |  |
| 62 |  | Supplier price |  | 107W LED street light, 4000K, 13940lm, IP66, IK 10, Philips Iridium gen4 BGP502 LED180-4S/740 |  |  | pc |  | 18.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Outdoor lighting system Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Cable and pipe production |  |  |  |  |  |  |  |  |  |  |
| 63 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and pre-installed boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 0.1000 |  |  |  |  |  |
| 64 |  | Market price |  | Fire-retardant aluminium-core cable with low smoke and gas emission and low toxicity of combustion products, TU 16-705.496-2011, GOST 31996-2012, cross-section 4x16mm² -0.66/1.V AVVGmр |  |  | m |  | 10.0000 |  |  |  |  |  |
| 65 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 16.0000 |  |  |  |  |  |
| 66 |  | Supplier price |  | APvBbShp cable 5×16 mm² |  |  | m |  | 1,600.00 |  |  |  |  |  |
| 67 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and installed boxes, weight 1 m up to: 1 kg |  |  | 100 m |  | 7.0000 |  |  |  |  |  |
| 68 |  | Market price |  | Fire-retardant copper-core cable, GOST 31996-2012, VVGng 3x1.5 mm² |  |  | m |  | 700,000 |  |  |  |  |  |
| 69 |  | 08-02-409-2 |  | Vinyl-plastic pipe on existing structures, on walls and columns, clamped in place, diameter up to 50 mm (difference 1300 - 1167 = 133 m) |  |  | 100 m |  | 1.3300 |  |  |  |  |  |
| 70 |  | Supplier price |  | Corrugated pipe with PE polyethylene insulation, 750N, TU 2248-015-47022248-2006, diameter 50 mm |  |  | m |  | 133.0000 |  |  |  |  |  |
| 71 |  | 08-02-409-3 |  | Vinyl-plastic pipe for installed structures, on walls and columns, clamped, diameter up to 63 mm |  |  | 100 m |  | 0.0500 |  |  |  |  |  |
| 72 |  | Supplier price |  | PE pipe - GOST 18599-2001 with a diameter of 63 mm |  |  | m |  | 5.0000 |  |  |  |  |  |
| 73 |  | 08-02-397-1 |  | Perforated mounting profile, length 2 m |  |  | 100 pcs |  | 0.0450 |  |  |  |  |  |
| 74 |  | Supplier price |  | Perforated metal channel with cover 50x50x3000mm |  |  | m |  | 9.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cable and pipe production Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Accessories |  |  |  |  |  |  |  |  |  |  |
| 75 |  | Supplier price |  | Anti-inflammatory foam 750ml |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 76 |  | Supplier price |  | KKX wall plug 12x100mm |  |  | pc |  | 10.0000 |  |  |  |  |  |
| 77 |  | Supplier price |  | Cable ties (100 pcs) REXANT 200x4.8mm |  |  | set |  | 2.0000 |  |  |  |  |  |
| 78 |  | Supplier price |  | Universal welding electrode, 3 mm |  |  | kg |  | 5.0000 |  |  |  |  |  |
| 79 |  | Supplier price |  | Black paint PF-115P 3L |  |  | kg |  | 5.0000 |  |  |  |  |  |
| 80 |  | Supplier price |  | "CABLE WARNING" warning tape |  |  | m |  | 1,200.0000 |  |  |  |  |  |
| 81 |  | Supplier price |  | Quick-fit wall plug 6x40mm |  |  | pc |  | 100.0000 |  |  |  |  |  |
| 82 |  | 08-02-163-2 |  | Terminal cap with polyethylene heat-shrink sleeves for 3-4 conductor cables with paper insulation, voltage up to 1 kV, conductor cross-section ( ) up to: 120 mm² (Heat-shrink sleeve 5ТПИ-4/25 mm²) |  |  | pc |  | 148.0000 |  |  |  |  |  |
| 83 |  | Supplier price |  | Heat-shrinkable sleeve 4/25 mm², 5 outlets, black 5ТПИ-4/25mm2 |  |  | pc |  | 148.0000 |  |  |  |  |  |
| 84 |  | Supplier price |  | Heat-shrink tubing 16/8mm |  |  | m |  | 150.000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Accessories Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. The ground-to- -to-ground connection system |  |  |  |  |  |  |  |  |  |  |
| 85 |  | 08-02-471-4 |  | Ground connection, vertical, made of round steel, diameter 16 mm |  |  | 10 pcs |  | 7.4000 |  |  |  |  |  |
| 86 |  | 08-02-472-6 |  | Open earthing conductor, on building supports, made of split steel, cross-section 100 mm² |  |  | 100 m |  | 1.1200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Earthing system Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Completion of existing general distribution panel |  |  |  |  |  |  |  |  |  |  |
| 87 |  | Supplier price |  | Circuit breaker, 3P, type C, In=16A, Icn=4.5kA BA 47-29/3P/C16A |  |  | pcs |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Completion of existing main distribution board Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. PDIE Distribution Panel |  |  |  |  |  |  |  |  |  |  |
| 88 |  | Supplier price |  | Metal cabinet with mounting panel, protection rating IP65, dimensions 650(h)×500×250 mm ЩМП-3-0 36 УХЛ3 IP31 |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 89 |  | Supplier price |  | 3P surge arrester, class B In=30kA ОПС1-B 3P/30kA |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 90 |  | Supplier price |  | BH 32/3P/40A load break switch |  |  | pc |  | 1.0000 |  |  |  |  |  |
| 91 |  | Supplier price |  | BA47-29/3P/B/10A circuit breaker |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 92 |  | Supplier price |  | BA47-29/1P/B/6A circuit breaker |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 93 |  | Supplier price |  | Modular contactor, 2P, In=20A KM20-20/2P/20A |  |  | pc |  | 12.0000 |  |  |  |  |  |
| 94 |  | Supplier price |  | Time relay, 230V/16A ТЭ15/16А |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 95 |  | Supplier price |  | Modular control button КМУ11 |  |  | pc |  | 12.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total PDIE Distribution Panel Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Outdoor lighting system |  |  |  |  |  |  |  |  |  |  |
| 96 |  | Supplier price |  | Elmos AD40 galvanised steel pole 4000x105/60mm with sealed access hatch |  |  | pc |  | 50.0000 |  |  |  |  |  |
| 97 |  | Supplier price |  | Elmos AD90 galvanised steel post 9000x153/60mm with sealed access hatch |  |  | pc |  | 18.0000 |  |  |  |  |  |
| 98 |  | Supplier price |  | Elmos AD120 galvanised steel post 12000x153/60mm with sealed access hatch |  |  | pc |  | 6.0000 |  |  |  |  |  |
| 99 |  | Supplier price |  | 107W LED street light, 4000K, 13940lm, IP66, IK10 Philips Iridium gen4 BGP502 LED180-4S/740 |  |  | pc |  | 18.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Outdoor lighting system Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | nd storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Equipment Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |