| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 7-1-1

General plan. Structural

| No. No. | Symbol, standards and resource code | Works and expenses | Unit | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Support wall |  |  |  |  |  |  |  |  |  |  |
| 1 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading into category II off-road vehicles |  |  | 100 m³ |  | 2.3900 |  |  |  |  |  |
| 2 |  | TsA20B |  | Manual earth excavation, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade terrain |  |  | m3 |  | 12.5700 |  |  |  |  |  |
| 3 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 394.2000 |  |  |  |  |  |
| 4 |  | TsC50B |  | Repair and maintenance of natural roads during earth transport, for every 0.5 km, category II terrain (K=2, per 1 km) |  |  | 100 m³ |  | 2.3900 |  |  |  |  |  |
| 5 |  | TsC03F1 |  | Mechanical excavation with a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture content , unloading into vehicles, category II terrain (in quarry for backfilling of trenches) |  |  | 100 m³ |  | 1.5000 |  |  |  |  |  |
| 6 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 248.0600 |  |  |  |  |  |
| 7 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 16.3000 |  |  |  |  |  |
| 8 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp tracked tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 1.4600 |  |  |  |  |  |
| 9 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 1.6300 |  |  |  |  |  |
| 10 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 6.5000 |  |  |  |  |  |
| 11 |  | CA03G |  | Reinforced concrete cast using conventional methods, in foundations, plinths, retaining walls and walls below ground level, prepared at a concrete batching plant or supplied as ready-mixed concrete in accordance with Article CA01, cast using conventional methods, reinforced concrete class... (C 20/25 concrete) |  |  | m³ |  | 75.0000 |  |  |  |  |  |
| 12 |  | CB03B |  | Formwork made of reusable panels, with 15 mm plywood for casting concrete in elevations, straight walls up to and including 6 m in height, with supports included |  |  | m2 |  | 386.4500 |  |  |  |  |  |
| 13 |  | CC02I |  | OB 37 concrete steel reinforcement, cut to size in site workshops, with bar diameters up to and including 8 mm, and installed in walls and diaphragms, at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 6 A240) |  |  | kg |  | 10.6500 |  |  |  |  |  |
| 14 |  | CC02I |  | OB 37 reinforced concrete steel bars, cut to size in site workshops, with bar diameters up to and including 8 mm, and installed in walls and diaphragms, at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 8 A240) |  |  | kg |  | 61.5600 |  |  |  |  |  |
| 15 |  | CC02J2 |  | PC 52 reinforced concrete steel bars cut on-site, with bar diameters exceeding 8 mm, in walls and diaphragms at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 10 A500c) |  |  | kg |  | 1 349.7900 |  |  |  |  |  |
| 16 |  | CC02J2 |  | PC 52 reinforced concrete steel bars cut on-site, with a bar diameter exceeding 8 mm, in walls and diaphragms at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 12 A500c) |  |  | kg |  | 843.4200 |  |  |  |  |  |
| 17 |  | CC02J2 |  | PC 52 concrete steel reinforcement cut on-site, with bar diameters over 8 mm, in walls and diaphragms at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 14 A500c) |  |  | kg |  | 661.2400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Retaining wall Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Support wall 2 |  |  |  |  |  |  |  |  |  |  |
| 18 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into category II off-road vehicles |  |  | 100 m³ |  | 1.1900 |  |  |  |  |  |
| 19 |  | TsA20B |  | Manual earth excavation, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium terrain |  |  | m3 |  | 6.2500 |  |  |  |  |  |
| 20 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 195.9000 |  |  |  |  |  |
| 21 |  | TsC50B |  | Repair and maintenance of natural roads during soil transport, for every 0.5 km, category II terrain (K=2, per 1 km) |  |  | 100 m³ |  | 1.1900 |  |  |  |  |  |
| 22 |  | TsC03F1 |  | Mechanical excavation with a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, category II terrain (in quarry for backfilling of trenches) |  |  | 100 m³ |  | 0.8800 |  |  |  |  |  |
| 23 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 144.8000 |  |  |  |  |  |
| 24 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 9.4000 |  |  |  |  |  |
| 25 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp tracked tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.8500 |  |  |  |  |  |
| 26 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 0.9400 |  |  |  |  |  |
| 27 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 3.5000 |  |  |  |  |  |
| 28 |  | CA03G |  | Reinforced concrete cast using conventional methods, in foundations, plinths, retaining walls and walls below ground level, prepared at a concrete batching plant or supplied as ready-mixed concrete in accordance with Article CA01, cast using conventional methods, reinforced concrete class... (C 20/25 concrete) |  |  | m³ |  | 56.0000 |  |  |  |  |  |
| 29 |  | CB03B |  | Formwork made of reusable panels, with 15 mm plywood for concrete casting in elevations, straight walls up to and including 6 m in height, with supports included |  |  | m2 |  | 187.3100 |  |  |  |  |  |
| 30 |  | CC02I |  | OB 37 concrete steel reinforcement, cut to size in site workshops, with bar diameters up to and including 8 mm, and installed in walls and diaphragms, at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 8 A240) |  |  | kg |  | 61.5600 |  |  |  |  |  |
| 31 |  | CC02J2 |  | PC 52 reinforced concrete steel bars cut on site, with bar diameters exceeding 8 mm, in walls and diaphragms at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 10 A500c) |  |  | kg |  | 1 590.6700 |  |  |  |  |  |
| 32 |  | CC02J2 |  | PC 52 reinforced concrete steel bars cut on-site, with a bar diameter exceeding 8 mm, in walls and diaphragms at heights less than or equal to 35 m, excluding structures constructed using sliding formwork (diam. 14 A500c) |  |  | kg |  | 661.2400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Retaining wall 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Staircase Ex 1 |  |  |  |  |  |  |  |  |  |  |
| 33 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 34 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil originating from medium soil |  |  | m3 |  | 7.7300 |  |  |  |  |  |
| 35 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m³ |  | 0.0800 |  |  |  |  |  |
| 36 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 1.1000 |  |  |  |  |  |
| 37 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 6.0000 |  |  |  |  |  |
| 38 |  | CB03E |  | Reusable panel formwork, with 15 mm plywood, for casting concrete slabs and beams in buildings up to and including 20 m in height, excluding supports |  |  | m² |  | 24.0000 |  |  |  |  |  |
| 39 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 22.7300 |  |  |  |  |  |
| 40 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on-site and installed in accordance with the ‘ ’ standard, with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 41 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 194.4700 |  |  |  |  |  |
| 42 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 10 A500c) |  |  | kg |  | 232.2500 |  |  |  |  |  |
| 43 |  | CI24A apl |  | Cladding of steps with 20 mm thick bonded granite slabs (PG section) |  |  | m² |  | 38.3600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Scale Ex 2 |  |  |  |  |  |  |  |  |  |  |
| 44 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 20.7400 |  |  |  |  |  |
| 45 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, soil category II |  |  | 100 m³ |  | 0.5000 |  |  |  |  |  |
| 46 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 82.5000 |  |  |  |  |  |
| 47 |  | TsC50B |  | Repair and maintenance of natural roads during earth transport, for every 0.5 km, category II terrain (K=2, at 1 km) |  |  | 100 m³ |  | 0.5000 |  |  |  |  |  |
| 48 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading into category II off-road vehicles (in , quarry for , backfilling of trenches) |  |  | 100 m³ |  | 0.2000 |  |  |  |  |  |
| 49 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 33.0000 |  |  |  |  |  |
| 50 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 25.7400 |  |  |  |  |  |
| 51 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a bulldozer on a 65–80 hp tracked tractor, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.1500 |  |  |  |  |  |
| 52 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 0.4100 |  |  |  |  |  |
| 53 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 5.0000 |  |  |  |  |  |
| 54 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 59.0000 |  |  |  |  |  |
| 55 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for casting concrete into slabs and beams in structures with a height of up to and including 20 m, excluding supports |  |  | m2 |  | 236.0000 |  |  |  |  |  |
| 56 |  | CC01C |  | OB 37 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diameter 6 A240) |  |  | kg |  | 42.6200 |  |  |  |  |  |
| 57 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 58 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 846.0900 |  |  |  |  |  |
| 59 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 10 A500c) |  |  | kg |  | 739.8500 |  |  |  |  |  |
| 60 |  | CI24A apl |  | Cladding of steps with 20 mm thick bonded granite slabs (PG section) |  |  | m² |  | 165.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Scale Ex 3 |  |  |  |  |  |  |  |  |  |  |
| 61 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 62 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including the breaking up of clods , the soil originating from medium-cohesive soil |  |  | m³ |  | 7.7300 |  |  |  |  |  |
| 63 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill material being non-cohesive soil |  |  | 100 m³ |  | 0.0800 |  |  |  |  |  |
| 64 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 0.4000 |  |  |  |  |  |
| 65 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 8.2000 |  |  |  |  |  |
| 66 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures up to and including 20 m in height, excluding supports |  |  | m2 |  | 32.8000 |  |  |  |  |  |
| 67 |  | CC01C |  | OB 37 concrete steel reinforcement, cut to size in site workshops and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 14.2100 |  |  |  |  |  |
| 68 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 69 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diam. 8 A500c) |  |  | kg |  | 320.4200 |  |  |  |  |  |
| 70 |  | CC01F1 |  | PC 52 concrete steel reinforcement bars, cut on site and installed with bar diameters of 8 mm and above in continuous and raft foundations ( , diam. 10 A500c) |  |  | kg |  | 232.2500 |  |  |  |  |  |
| 71 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG compartment) |  |  | m² |  | 40.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Scale Ex 4 |  |  |  |  |  |  |  |  |  |  |
| 72 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 73 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil originating from medium soil |  |  | m3 |  | 7.7300 |  |  |  |  |  |
| 74 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m³ |  | 0.0800 |  |  |  |  |  |
| 75 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 0.4000 |  |  |  |  |  |
| 76 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 6.0000 |  |  |  |  |  |
| 77 |  | CB03E |  | Reusable panel formwork, with 15 mm plywood for casting concrete slabs and beams in buildings up to and including 20 m in height, excluding supports |  |  | m² |  | 24.0000 |  |  |  |  |  |
| 78 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 25.5700 |  |  |  |  |  |
| 79 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 80 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 179.0400 |  |  |  |  |  |
| 81 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 10 A500c) |  |  | kg |  | 209.3200 |  |  |  |  |  |
| 82 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG section) |  |  | m² |  | 35.6000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. Scale Ex 5 |  |  |  |  |  |  |  |  |  |  |
| 83 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 84 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, soil originating from medium-cohesive soil |  |  | m³ |  | 7.7300 |  |  |  |  |  |
| 85 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill material being non-cohesive soil |  |  | 100 m³ |  | 0.0800 |  |  |  |  |  |
| 86 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 0.4000 |  |  |  |  |  |
| 87 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 6.0000 |  |  |  |  |  |
| 88 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures up to and including 20 m in height, excluding supports |  |  | m2 |  | 24.0000 |  |  |  |  |  |
| 89 |  | CC01C |  | OB 37 concrete steel reinforcement, cut to size in site workshops and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 28.6300 |  |  |  |  |  |
| 90 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 91 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 8 A500c) |  |  | kg |  | 179.0400 |  |  |  |  |  |
| 92 |  | CC01F1 |  | PC 52 concrete steel reinforcement bars, cut on-site and installed in a continuous and raft foundation (diam. 10 A500c), with bar diameters of 8 mm and above |  |  | kg |  | 209.3200 |  |  |  |  |  |
| 93 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG section) |  |  | m² |  | 43.9800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 5 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Scale Ex 6 |  |  |  |  |  |  |  |  |  |  |
| 94 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 13.8200 |  |  |  |  |  |
| 95 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, soil category II |  |  | 100 m³ |  | 0.3900 |  |  |  |  |  |
| 96 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 63.6000 |  |  |  |  |  |
| 97 |  | TsC50B |  | Repair and maintenance of natural roads during earth transport, for every 0.5 km, category II terrain (K=2, at 1 km) |  |  | 100 m³ |  | 0.3900 |  |  |  |  |  |
| 98 |  | TsC03F1 |  | Mechanical excavation with a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, category II terrain (in quarry for backfilling of trenches) |  |  | 100 m³ |  | 0.1600 |  |  |  |  |  |
| 99 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 26.0000 |  |  |  |  |  |
| 100 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 3.0000 |  |  |  |  |  |
| 101 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp tracked tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.2600 |  |  |  |  |  |
| 102 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 0.3000 |  |  |  |  |  |
| 103 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 5.0000 |  |  |  |  |  |
| 104 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 59.0000 |  |  |  |  |  |
| 105 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures up to and including 20 m in height, excluding supports |  |  | m2 |  | 236.0000 |  |  |  |  |  |
| 106 |  | CC01C |  | OB 37 reinforced steel bars, cut to size in site workshops and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 42.6200 |  |  |  |  |  |
| 107 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 108 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 8 A500c) |  |  | kg |  | 846.0900 |  |  |  |  |  |
| 109 |  | CC01F1 |  | PC 52 concrete steel reinforcement, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 10 A500c) |  |  | kg |  | 739.8500 |  |  |  |  |  |
| 110 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG section) |  |  | m² |  | 190.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 6 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9. Scale Ex 7 |  |  |  |  |  |  |  |  |  |  |
| 111 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 112 |  | TsD01B |  | Spreading of fine-grained soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil originating from medium-cohesive soil |  |  | m3 |  | 7.7300 |  |  |  |  |  |
| 113 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m³ |  | 0.0800 |  |  |  |  |  |
| 114 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared using a concrete batching plant ( ) in accordance with CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 0.4000 |  |  |  |  |  |
| 115 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared using a concrete batching plant or ready-mixed concrete in accordance with Art. CA01 and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 6.0000 |  |  |  |  |  |
| 116 |  | CB03E |  | Reusable panel formwork, with 15 mm plywood, for casting concrete slabs and beams in buildings up to and including 20 m in height, excluding supports |  |  | m² |  | 24.0000 |  |  |  |  |  |
| 117 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 25.5700 |  |  |  |  |  |
| 118 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 119 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 179.0400 |  |  |  |  |  |
| 120 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 10 A500c) |  |  | kg |  | 209.3200 |  |  |  |  |  |
| 121 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG section) |  |  | m² |  | 36.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 7 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10. Scale Ex 8 |  |  |  |  |  |  |  |  |  |  |
| 122 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 10.3700 |  |  |  |  |  |
| 123 |  | TsD01B |  | Spreading of fine-grained soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil originating from medium-cohesive soil |  |  | m3 |  | 7.7300 |  |  |  |  |  |
| 124 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m3 |  | 0.0800 |  |  |  |  |  |
| 125 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 0.4000 |  |  |  |  |  |
| 126 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 6.0000 |  |  |  |  |  |
| 127 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures up to and including 20 m in height, excluding supports |  |  | m2 |  | 24.0000 |  |  |  |  |  |
| 128 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 25.5700 |  |  |  |  |  |
| 129 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on-site and installed in accordance with the ‘ ’ standard, with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 130 |  | CC01F1 |  | PC 52 concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diam. 8 A500c) |  |  | kg |  | 179.0400 |  |  |  |  |  |
| 131 |  | CC01F1 |  | PC 52 concrete steel reinforcement, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 10 A500c) |  |  | kg |  | 209.3200 |  |  |  |  |  |
| 132 |  | CI24A apl |  | Cladding of steps with bonded granite slabs, 20 mm thick (PG section) |  |  | m² |  | 44.6000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 8 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11. Scale Ex 9 |  |  |  |  |  |  |  |  |  |  |
| 133 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 13.8200 |  |  |  |  |  |
| 134 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, soil category II |  |  | 100 m³ |  | 0.3900 |  |  |  |  |  |
| 135 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 63.6000 |  |  |  |  |  |
| 136 |  | TsC50B |  | Repair and maintenance of natural roads during earth transport, for every 0.5 km, category II terrain (K=2, at 1 km) |  |  | 100 m³ |  | 0.3900 |  |  |  |  |  |
| 137 |  | TsC03F1 |  | Mechanical excavation with a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, category II terrain (in quarry for backfilling of trenches) |  |  | 100 m³ |  | 0.1600 |  |  |  |  |  |
| 138 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 26.0000 |  |  |  |  |  |
| 139 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 3.0000 |  |  |  |  |  |
| 140 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a 65–80 hp tracked tractor-mounted bulldozer, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.2600 |  |  |  |  |  |
| 141 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 0.3000 |  |  |  |  |  |
| 142 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 5.0000 |  |  |  |  |  |
| 143 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 50.0000 |  |  |  |  |  |
| 144 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures with a height of up to and including 20 m, excluding supports |  |  | m2 |  | 200.0000 |  |  |  |  |  |
| 145 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 42.6200 |  |  |  |  |  |
| 146 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 5.6800 |  |  |  |  |  |
| 147 |  | CC01F1 |  | PC 52 concrete reinforcement bars cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diam. 8 A500c) |  |  | kg |  | 846.0900 |  |  |  |  |  |
| 148 |  | CC01F1 |  | PC 52 concrete steel reinforcement bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 10 A500c) |  |  | kg |  | 739.8500 |  |  |  |  |  |
| 149 |  | CI24A apl |  | Cladding of steps with 20 mm thick bonded granite slabs (PG section) |  |  | m² |  | 140.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Staircase Ex 9 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12. Gazebo - 3 units |  |  |  |  |  |  |  |  |  |  |
| 150 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 28.2000 |  |  |  |  |  |
| 151 |  | TsD01B |  | Spreading of refined soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including the breaking up of clods ( ), the soil originating from medium-cohesive soil |  |  | m³ |  | 15.1500 |  |  |  |  |  |
| 152 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m3 |  | 0.1500 |  |  |  |  |  |
| 153 |  | CA02C |  | Plain concrete cast in levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m3 |  | 7.3500 |  |  |  |  |  |
| 154 |  | CA03G |  | Reinforced concrete cast using conventional methods, in foundations, plinths, retaining walls and walls below ground level, prepared at a concrete batching plant or supplied as ready-mixed concrete in accordance with Article CA01, cast using conventional methods, reinforced concrete class... (C 20/25 concrete) |  |  | m³ |  | 16.5000 |  |  |  |  |  |
| 155 |  | CB03B |  | Formwork made of reusable panels, with 15 mm plywood for casting concrete in elevations, straight walls up to and including 6 m in height, with supports included |  |  | m2 |  | 79.1100 |  |  |  |  |  |
| 156 |  | CC01F1 |  | PC 52 reinforced concrete steel reinforcement, cut to size in site workshops and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 61.6500 |  |  |  |  |  |
| 157 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 10 A500c) |  |  | kg |  | 1,034.1000 |  |  |  |  |  |
| 158 |  | CL18A |  | Miscellaneous metalwork made from rolled sections, sheet metal, corrugated sheet, reinforcing steel, pipes for supports or coverings, fully or partially embedded in concrete ( -embedded items) |  |  | kg |  | 215.2000 |  |  |  |  |  |
| 159 |  | IzF04A |  | Hot-applied waterproofing layer on terraces, roofs, foundations and ground slabs, in areas without groundwater, including the flashings and valleys in the standard waterproofing system on horizontal or sloping surfaces up to 40% gradient, whether flat or curved, using bitumen mastic applied with a brush or rubber trowel |  |  | m² |  | 39.6000 |  |  |  |  |  |
| 160 |  | IzF50A |  | Waterproofing carried out with cement mortar containing soluble glass on foundations and walls, applied to horizontal surfaces |  |  | m2 |  | 37.6800 |  |  |  |  |  |
| 161 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in the lightweight construction structure |  |  | t |  | 5.7630 |  |  |  |  |  |
| 162 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 5.7630 |  |  |  |  |  |
| 163 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral brush |  |  | t |  | 5.7630 |  |  |  |  |  |
| 164 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 5.7630 |  |  |  |  |  |
| 165 |  | CE35A apl |  | Covering with fabric (solar sails) (PG, f. 41, https://prom.md/Tkani-dlya-markiz.html) |  |  | m² |  | 124.8000 |  |  |  |  |  |
| 166 |  | CE30B |  | Roof battens or battens for roofing tiles, fibre cement sheets, etc., made from rough-sawn softwood planks (24 mm thick) planed on one side, for standard constructions (Wooden batten 25x100x6000 mm) |  |  | m2 |  | 63.00 |  |  |  |  |  |
| 167 |  | CN50A |  | Fireproofing of timberwork; trusses, arches, beams, rafters, purlins. |  |  | m³ |  | 1.5800 |  |  |  |  |  |
| 168 |  | CN51B |  | Antiseptic treatment of timber, on concealed surfaces with antiseptic pastes: timber frames. |  |  | m3 |  | 1.5800 |  |  |  |  |  |
| 169 |  | CN16D |  | Painters applying oil-based varnishes and paints to wooden joinery, finished with 2 coats of enamel paint on the floorboards |  |  | m2 |  | 63.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Gazebo - 3 units Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13. relaxation area |  |  |  |  |  |  |  |  |  |  |
| 170 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles on category II terrain |  |  | 100 m³ |  | 0.9600 |  |  |  |  |  |
| 171 |  | TsA20B |  | Manual earth excavation, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade terrain |  |  | m3 |  | 9.5000 |  |  |  |  |  |
| 172 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 158.0000 |  |  |  |  |  |
| 173 |  | TsC50B |  | Repair and maintenance of natural roads during earth transport, for every 0.5 km, category II terrain (K=2, per 1 km) |  |  | 100 m³ |  | 0.9600 |  |  |  |  |  |
| 174 |  | TsC03F1 |  | Mechanical excavation with a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles, category II terrain (in quarry for backfilling of trenches) |  |  | 100 m³ |  | 0.6700 |  |  |  |  |  |
| 175 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 110.5500 |  |  |  |  |  |
| 176 |  | TsD01B |  | Spreading of screened soil with a shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from the piles, including breaking up clods; soil sourced from medium-grade land |  |  | m³ |  | 7.7000 |  |  |  |  |  |
| 177 |  | TsD02B1 |  | Spreading of screened soil from Category I or II land, carried out using a bulldozer on a 65–80 hp tracked tractor, in layers 21–30 cm thick |  |  | 100 m³ |  | 0.7000 |  |  |  |  |  |
| 178 |  | TsD08B1 |  | Mechanical compaction of fill using a 10.1–16 t self-propelled static tyre-mounted roller, in successive layers 15–25 cm thick after compaction, excluding the wetting of each individual layer; the fill is carried out using cohesive soil |  |  | 100 m3 |  | 0.7000 |  |  |  |  |  |
| 179 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with CA01 or ready-mixed concrete, cast using conventional methods (C 8/10 concrete) |  |  | m³ |  | 5.0000 |  |  |  |  |  |
| 180 |  | CA04F |  | Concrete cast in slabs, beams and columns, prepared at a concrete batching plant or using ready-mixed concrete in accordance with Article CA01, and cast using conventional methods (C 20/25 concrete) |  |  | m3 |  | 65.0000 |  |  |  |  |  |
| 181 |  | CB03E |  | Formwork made of reusable panels, with 15 mm plywood for the casting of concrete into slabs and beams in structures up to and including 20 m in height, excluding supports |  |  | m2 |  | 234.0000 |  |  |  |  |  |
| 182 |  | CC01C |  | OB 37 reinforced steel bars, cut to size on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 6 A240) |  |  | kg |  | 78.7800 |  |  |  |  |  |
| 183 |  | CC01C |  | OB 37 reinforced concrete steel bars, cut on site and installed with bar diameters up to and including 8 mm in isolated foundations (diam. 8 A240) |  |  | kg |  | 65.3400 |  |  |  |  |  |
| 184 |  | CC01F1 |  | PC 52 reinforced concrete steel reinforcement, cut to size on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 8 A500c) |  |  | kg |  | 6,980.5800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Relaxation area Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total quote: Including salary |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |