| Major repair and modernisation of the Public Institution "Mihai Eminescu" Telenesti district, 8 Codrilor Street, cadastral number 8901224.151.02 | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 7-1-2

General plan

| No. No. | Symbol, standards and resource code | Works and expenditure | Unit of measurement | Quantity according to project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Installation and dismantling work |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCB18G |  | Demolition of old concrete using mechanical means, reinforced concrete |  |  | m3 |  | 80.0000 |  |  |  |  |  |
| 2 |  | RpCS19B |  | Dismantling: precast concrete slabs |  |  | m2 |  | 600.0000 |  |  |  |  |  |
| 3 |  | RpCP45D |  | Dismantling of railings, grilles, parapets and metal fencing, etc. |  |  | kg |  | 6,540.0000 |  |  |  |  |  |
| 4 |  | RpCG29A |  | Demolition of brick or aerated concrete walls 6–8 cm thick (demolition of existing structures) |  |  | m² |  | 36.5000 |  |  |  |  |  |
| 5 |  | RpCK41C apl |  | Dismantling of existing sports ground |  |  | m² |  | 660.0000 |  |  |  |  |  |
| 6 |  | RpCP45D app |  | Dismantling of existing metal sports equipment |  |  | kg |  | 5,700.0000 |  |  |  |  |  |
| 7 |  | RpCB18G |  | Demolition of old concrete using mechanical means, reinforced concrete (existing retaining wall) |  |  | m3 |  | 27.0000 |  |  |  |  |  |
| 8 |  | TsG05B |  | Manual felling of coniferous trees, including manual transport of the material into piles, outside or within the works area, trees having a diameter of 31–50 cm |  |  | pc |  | 15.0000 |  |  |  |  |  |
| 9 |  | TsG06B |  | The manual removal of stumps from coniferous trees (excluding fir) or softwood trees, and the transport of the timber to storage areas, either off-site or within the work area, with transport carried out by hand; the diameter of the stumps or roots is 31–50 cm |  |  | pc |  | 15.0000 |  |  |  |  |  |
| 10 |  | TsC35B12 |  | Excavation and transport using a front-loader, at loading distances of 0.5–0.99 m³ to a tracked front-loader vehicle, soil from Category II land at a distance of 21–30 m (construction waste ) |  |  | 100 m³ |  | 1.7500 |  |  |  |  |  |
| 11 |  | TsI51B5 |  | Transport of construction waste by 10-tonne tipper lorry over a distance of: 15 km |  |  | t |  | 386.0500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total dismantling works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Greening |  |  |  |  |  |  |  |  |  |  |
| 12 |  | TsH25B |  | Transplanting with soil balls, carried out manually, for deciduous and coniferous trees with a diameter of up to 10 cm |  |  | pc |  | 31.0000 |  |  |  |  |  |
| 13 |  | TsH238E |  | Manual digging of planting pits for trees and shrubs in unfrozen soil, depth 0.4–0.7 m: – medium soil |  |  | pc |  | 39.0000 |  |  |  |  |  |
| 14 |  | TsH110B |  | Planting trees in pre-dug holes: coniferous trees |  |  | pc |  | 39.0000 |  |  |  |  |  |
| 15 |  | Supplier price |  | Thuja trees |  |  | pc |  | 35.0000 |  |  |  |  |  |
| 16 |  | Supplier price |  | -new |  |  | pc |  | 4,0000 |  |  |  |  |  |
| 17 |  | TsH238B |  | Manual digging of holes for planting trees and shrubs in unfrozen soil, depth up to 0.4 m: - medium soil |  |  | pc |  | 16.0000 |  |  |  |  |  |
| 18 |  | TsH24A |  | Planting of shrubs without root balls |  |  | pc |  | 16.0000 |  |  |  |  |  |
| 19 |  | Supplier price |  | e trees, new |  |  | pc |  | 16.0000 |  |  |  |  |  |
| 20 |  | TsH238B |  | Manual digging of holes for planting trees and shrubs in unfrozen soil, depth up to 0.4 m: - medium soil |  |  | pc |  | 42.0000 |  |  |  |  |  |
| 21 |  | TsH30B |  | Planting of flowers and perennials over 15 cm in height in the ground |  |  | 100 pcs |  | 0.4200 |  |  |  |  |  |
| 22 |  | Supplier price |  | Planting of annual flowers (4 per 1 m²) |  |  | pc |  | 42.0000 |  |  |  |  |  |
| 23 |  | TsE01A |  | Manual levelling of land and platforms, with unevenness of 10–20 cm, on soft ground |  |  | pc |  | 27.5200 |  |  |  |  |  |
| 24 |  | TsH09A |  | Sowing of lawns on horizontal surfaces or slopes of less than 30% |  |  | 100m² |  | 27.5200 |  |  |  |  |  |
| 25 |  | TsH12B |  | Watering surfaces with a hose from a water tank |  |  | 100m2 |  | 27.5200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Greening Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Paving type - 1 carriageway S=6,522 m² |  |  |  |  |  |  |  |  |  |  |
| 26 |  | TsD05B |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer; the fill is to be made of cohesive soil |  |  | 100 m³ |  | 3.8400 |  |  |  |  |  |
| 27 |  | DA12C |  | Foundation or reprofiling layer of crushed stone for roads, mechanically laid, without grading or screening (particle size 20–40 mm, grain size 150 mm) |  |  | m3 |  | 978.3000 |  |  |  |  |  |
| 28 |  | DE17B |  | Paving made of precast concrete paving slabs laid on a layer of dry cement and sand mix, in a 1:3 ratio, grouted with dry cement and sand mix, layer thickness 10 cm (paving: 60 mm vibro-pressed concrete slabs ) |  |  | m² |  | 6,522.0000 |  |  |  |  |  |
| 29 |  | DE10A |  | Precast concrete kerbs for pavements, 20x10 cm, on a 30x15 cm concrete base (BR 100.30.15 kerb) |  |  | m |  | 2,158.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Paving type - 1 carriageway S=6,522 m² Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Paving type - 2 pedestrian S=262 m² |  |  |  |  |  |  |  |  |  |  |
| 30 |  | TsD05B |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made of cohesive soil |  |  | 100 m3 |  | 0.3900 |  |  |  |  |  |
| 31 |  | TsC54B |  | Crushed stone foundation layer (size 20–40 mm, 150 mm thick) |  |  | m3 |  | 39.3000 |  |  |  |  |  |
| 32 |  | DE17B |  | Paving laid with prefabricated concrete paving slabs set on a layer of dry cement and sand mix in a 1:3 ratio, grouted with dry cement and sand mix, layer thickness 10 cm ( e concrete paving, 40 mm thick) |  |  | m2 |  | 262.0000 |  |  |  |  |  |
| 33 |  | DE10A |  | Precast concrete kerbs for pavements, 20x10 cm, on a 30x15 cm concrete foundation (BR 100.20.10 kerb) |  |  | m |  | 300.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Paving type - 2 pedestrian S=262 m² Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Artificial turf: 1,660 m² |  |  |  |  |  |  |  |  |  |  |
| 34 |  | TsC54B |  | Crushed stone sub-base layer M400 (particle size 40–70 mm, thickness 150 mm) |  |  | m³ |  | 249.0000 |  |  |  |  |  |
| 35 |  | TsC54B |  | M400 crushed stone foundation layer (particle size 20–40 mm, thickness 100 mm) |  |  | m3 |  | 166.0000 |  |  |  |  |  |
| 36 |  | TsC54B |  | Crushed stone foundation layer M400 (particle size 5–20 mm, thickness 50 mm) |  |  | m3 |  | 83.0000 |  |  |  |  |  |
| 37 |  | TsC54B |  | Crushed stone foundation layer (granite chippings, size 1–5 mm, thickness 30 mm) |  |  | m3 |  | 49.8000 |  |  |  |  |  |
| 38 |  | TsH43A |  | Laying of artificial turf for football pitches: FLD Champion Supra 40 type turf, pile height 40 mm, covered with sand and crumb rubber |  |  | m2 |  | 1,660.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Artificial turf S=1,660 m² Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Rubber- e and synthetic e surfacing, Tartan (S= 3,690.3 + 1,350 + 660.2 + 800 = 6,500.5 m²) |  |  |  |  |  |  |  |  |  |  |
| 39 |  | TsD05B |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers of 20–30 cm thickness, excluding the wetting of each individual layer; the fill is to be made of cohesive soil |  |  | 100 m³ |  | 9.2100 |  |  |  |  |  |
| 40 |  | DA12C |  | Foundation or reprofiling layer of crushed stone for roads, mechanically laid, executed without compaction, without levelling |  |  | m3 |  | 975.0800 |  |  |  |  |  |
| 41 |  | CA03G |  | Reinforced concrete cast using conventional methods, in foundations, plinths, retaining walls, walls below ground level, prepared at a concrete plant or using ready-mixed concrete in accordance with Art. CA01, casting using conventional methods, C 20/25 concrete |  |  | m3 |  | 650.0500 |  |  |  |  |  |
| 42 |  | CG25B |  | Sports surfaces made from SBR rubber granules (0.5–2.5 mm) with a thickness of 8 mm and EPDM granules (1.0–3.5 mm, yellow) with a thickness of 8 mm (consumption of new materials: SBR rubber granules 0.5–2.5 mm = 5.6 kg/m², EPDM rubber granules, 1.0–3.5 mm in size = 5.6 kg/m², specification for granules – 2.46 kg/m², pigment – 0.5 kg/m²) (for application purposes, the following resources are excluded: gravel (sand), petrol, graded sand, firewood, road bitumen, limestone filler); the resource "Bitumen melter" is changed to " -type 150 L mortar mixer powered by a combustion engine" |  |  | m² |  | 6,500.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total flooring with rubber- e synthetic e, Tartan (S = 3,690.3 + 1,350 + 660.2 + 800 = 6,500.5 m²) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. New asphalt S=350 m² |  |  |  |  |  |  |  |  |  |  |
| 43 |  | DA06B2 |  | Layer of rolled natural aggregates, serving as a filter, insulation, aeration, frost protection and anti-capillary layer, mechanically laid, with sand (150 mm) |  |  | m3 |  | 52.5000 |  |  |  |  |  |
| 44 |  | DA12C |  | Crushed stone foundation or sub-base layer for roads, mechanically laid, executed in a e without compaction or consolidation (particle size 0–40 mm, thickness 240 mm) |  |  | m3 |  | 84.0000 |  |  |  |  |  |
| 45 |  | Dl107 |  | Priming of base course surfaces prior to the application of an asphalt concrete layer (bitumen spreading) 0.8 litres/m² |  |  | t |  | 0.2800 |  |  |  |  |  |
| 46 |  | DB19G |  | Hot-laid asphalt concrete surface course with coarse aggregate, 6.0 cm thick, mechanically laid |  |  | m² |  | 350.0000 |  |  |  |  |  |
| 47 |  | Dl107 |  | Priming of base course surfaces prior to the application of an asphalt concrete layer (bitumen spreading) 0.3 litres/m² |  |  | t |  | 0.1050 |  |  |  |  |  |
| 48 |  | DB16H |  | Hot-mix asphalt surface course with fine aggregates, 4.0 cm thick, mechanically laid (50 mm grade, K=1.25) |  |  | m² |  | 300.4000 |  |  |  |  |  |
| 49 |  | DE10A |  | Precast concrete kerbs for pavements, 20x10 cm, on a 30x15 cm concrete foundation (BR 100.30.15 kerb) |  |  | m |  | 164.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total new asphalt S=350 m² Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Widening of existing access roads S=600 m² (paving type 2) |  |  |  |  |  |  |  |  |  |  |
| 50 |  | TsD05B |  | Compaction using a 150–200 kg mechanical tamper of fill material in successive layers 20–30 cm thick, excluding the wetting of each individual layer; the fill material consists of cohesive soil |  |  | 100 m³ |  | 1.5000 |  |  |  |  |  |
| 51 |  | TsC54B |  | Crushed stone foundation layer (size 20–40 mm, 150 mm thick) |  |  | m3 |  | 90.0000 |  |  |  |  |  |
| 52 |  | DE17B |  | Paving laid using precast concrete paving slabs set on a bed of dry cement and sand mix in a 1:3 ratio, grouted with a dry cement and sand mix, with a layer thickness of 10 cm ( , 40 mm thick concrete paving slabs) |  |  | m² |  | 600.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Widening of existing accesses S=600 m² (paving type 2) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9. Type-1 gutter |  |  |  |  |  |  |  |  |  |  |
| 53 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with internal combustion engine and hydraulic control, in soil with natural moisture, unloading into vehicles on Category II terrain |  |  | 100 m³ |  | 2.9700 |  |  |  |  |  |
| 54 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 490.0000 |  |  |  |  |  |
| 55 |  | TsC54C |  | Gravel foundation layer |  |  | m³ |  | 133.9000 |  |  |  |  |  |
| 56 |  | TsC54A |  | Sand foundation layer |  |  | m3 |  | 58.4000 |  |  |  |  |  |
| 57 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with CA01 or ready-mixed concrete, cast using conventional methods |  |  | m3 |  | 48.7500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type-1 Gutter Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10. Type-2 Gutter |  |  |  |  |  |  |  |  |  |  |
| 58 |  | TsC03F1 |  | Mechanical excavation using a 0.40–0.70 m³ excavator, with an internal combustion engine and hydraulic control, in soil with natural moisture content, unloading into category II off-road vehicles |  |  | 100 m³ |  | 0.8600 |  |  |  |  |  |
| 59 |  | TsI51A5 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 5 km |  |  | t |  | 141.25 |  |  |  |  |  |
| 60 |  | TsC54C |  | Gravel foundation layer |  |  | m³ |  | 34.9000 |  |  |  |  |  |
| 61 |  | TsC54A |  | Sand foundation layer |  |  | m3 |  | 22.2000 |  |  |  |  |  |
| 62 |  | DE16A |  | Installation of precast reinforced concrete elements in trenches, gutters, etc., up to and including 0.02 m³ per unit (fibre-reinforced concrete gutter with grating) |  |  | pcs |  | 317.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type-2 Gutter Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11. Small- s |  |  |  |  |  |  |  |  |  |  |
| 63 |  | TsH91A |  | Installation of bench on 2 legs |  |  | each |  | 16.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Small Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12. Fence |  |  |  |  |  |  |  |  |  |  |
| 64 |  | CO06B3 |  | Wire mesh fencing with steel-framed fence panels fixed to prefabricated reinforced concrete posts installed 2 m apart, with a centre-to-centre distance, using crushed stone backfill, with a ridge height of 2.05 m, for use with metal posts fixed in concrete cast on site |  |  | m |  | 389.5000 |  |  |  |  |  |
| 65 |  | Supplier price |  | Edged, painted fence panel, h-1.8m |  |  | pc |  | 155.0000 |  |  |  |  |  |
| 66 |  | Supplier price |  | "ECO" post, 61x35x1mm |  |  | pc |  | 159.0000 |  |  |  |  |  |
| 67 |  | Supplier price |  | Caps for posts |  |  | pc |  | 159.0000 |  |  |  |  |  |
| 68 |  | Supplier price |  | M6x30 screw |  |  | pc |  | 795.0000 |  |  |  |  |  |
| 69 |  | Supplier price |  | Fixing clip |  |  | pc |  | 795.0000 |  |  |  |  |  |
| 70 |  | Supplier price |  | Concrete C 16/20 |  |  | m³ |  | 11.4500 |  |  |  |  |  |
| 71 |  | CK14A |  | Metal gates with frames made from prefabricated round steel sections, including all necessary fittings, mounted on reinforced concrete posts (gate L=2.25 m, H=1.8 m – 4 units) |  |  | m² |  | 8.1000 |  |  |  |  |  |
| 72 |  | Supplier price |  | Edged, painted gate panel, h-1.8m |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 73 |  | Supplier price |  | "ECO" post, 100x100mm tube |  |  | pc |  | 4.0000 |  |  |  |  |  |
| 74 |  | Supplier price |  | Concrete C 16/20 |  |  | m³ |  | 0.5100 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Fence Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13. Stands |  |  |  |  |  |  |  |  |  |  |
| 75 |  | CI11A |  | Marble and travertine cladding up to 5 cm thick, applied to flat surfaces on walls and columns, excluding the installation of window sills, fixed with steel staples and M 100-T cement mortar with added lime, 3 cm thick, grouted with white cement mortar and marble dust, or travertine, for slab sizes up to 0.2 m² per piece inclusive |  |  | m² |  | 176.2000 |  |  |  |  |  |
| 76 |  | Supplier price |  | Seat with backrest for grandstand with three fixing points |  |  | pc |  | 132.0000 |  |  |  |  |  |
|  |  |  |  |  |  |  | pc |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Grandstand Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14. relaxation area |  |  |  |  |  |  |  |  |  |  |
| 77 |  | CL57A |  | Installation and fixing of embedded parts in monolithic reinforced concrete: weighing less than 4 kg (30x40x2 metal structural profile, L=300) |  |  | kg |  | 22.4200 |  |  |  |  |  |
| 78 |  | CK17A1 apl |  | Wall panelling, installed on site in quantities of up to 10 m² inclusive, made from softwood timber, in the form of panels and friezes or slats ( -bar timber 420.6 m x 0.06 m) |  |  | m² |  | 25.2400 |  |  |  |  |  |
| 79 |  | CN50A |  | Fireproofing of timberwork; trusses, arches, beams, rafters, purlins. |  |  | m³ |  | 1.0100 |  |  |  |  |  |
| 80 |  | CN51B |  | Antiseptic treatment of timber, on concealed surfaces with antiseptic pastes: timber frames. |  |  | m3 |  | 1.0100 |  |  |  |  |  |
| 81 |  | CN16D |  | Painters applying oil-based varnishes and paints to wooden joinery, finished with 2 coats of enamel paint on the floorboards |  |  | m2 |  | 84.1200 |  |  |  |  |  |
| 82 |  | CK35A apl |  | Self-tapping screw, 6.3x60mm |  |  | pcs |  | 148.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Relaxation area Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 15. Balustrade (amphitheatre) |  |  |  |  |  |  |  |  |  |  |
| 83 |  | CL17B |  | Various exposed metalwork: balustrade, grilles, coping, snow guards, grates |  |  | kg |  | 135.7700 |  |  |  |  |  |
| 84 |  | RpIzE16B |  | Cleaning of metalwork and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.1360 |  |  |  |  |  |
| 85 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.1360 |  |  |  |  |  |
| 86 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.1360 |  |  |  |  |  |
| 87 |  | CK35B |  | Metal wall plugs for reinforced concrete walls (20 mm metal wall plug) |  |  | each |  | 50.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Balustrade (amphitheatre) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 16. Balustrade (stairs) |  |  |  |  |  |  |  |  |  |  |
| 88 |  | CL17B |  | Various exposed metalwork: balustrades, grilles, handrails, snow guards, grates |  |  | kg |  | 378.3000 |  |  |  |  |  |
| 89 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.3780 |  |  |  |  |  |
| 90 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.3780 |  |  |  |  |  |
| 91 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.3780 |  |  |  |  |  |
| 92 |  | CK35B |  | Metal wall plugs fixed in reinforced concrete walls (Metal wall plug with D20mm) |  |  | pcs |  | 194.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Balustrade (stairs) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17. Movable handrails |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.1. Type 1 (24 units) |  |  |  |  |  |  |  |  |  |  |
| 93 |  | CL17B |  | Various metalwork, surface-mounted: balustrades, grilles, copings, snow guards, grates |  |  | kg |  | 727.2000 |  |  |  |  |  |
| 94 |  | CD07A |  | Walls made of corrugated galvanised sheet metal panels fixed by riveting to the metal framework, installed at a height of up to and including 6 m (2 mm thick painted sheet metal) |  |  | m2 |  | 48.0000 |  |  |  |  |  |
| 95 |  | CE40A |  | Installation of frame elements made of beams (bars) with antiseptic treatment (wooden bar 40x30x23000) |  |  | m³ |  | 0.7200 |  |  |  |  |  |
| 96 |  | CL57A |  | Installation and fixing of components embedded in monolithic reinforced concrete: weighing less than 4 kg ( -metal plate 100x100x2) |  |  | kg |  | 15.0700 |  |  |  |  |  |
| 97 |  | Supplier price |  | Swivel castor with brake d=80mm |  |  | pc |  | 96.0000 |  |  |  |  |  |
|  |  |  |  |  |  |  | pc |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type-1 (24 pcs) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.2. Type 2 (2 items) |  |  |  |  |  |  |  |  |  |  |
| 98 |  | CL17B |  | Various exposed metalwork: balustrades, grilles, , snow guards, grates |  |  | kg |  | 92.9200 |  |  |  |  |  |
| 99 |  | CD07A |  | Walls made of corrugated galvanised sheet metal panels fixed by riveting to the metal framework, installed at a height of up to and including 6 m (2 mm thick painted sheet metal) |  |  | m² |  | 4.0000 |  |  |  |  |  |
| 100 |  | CE40A |  | Installation of frame elements made of beams (bars) with antiseptic treatment (wooden bar 40x30x9400) |  |  | m³ |  | 0.0200 |  |  |  |  |  |
| 101 |  | CL57A |  | Installation and fixing of embedded parts in monolithic reinforced concrete: weighing less than 4 kg (100x100x2 mm metal plate ) |  |  | kg |  | 1.2600 |  |  |  |  |  |
| 102 |  | Supplier price |  | Swivel castor with brake d=80mm |  |  | pc |  | 8.0000 |  |  |  |  |  |
|  |  |  |  |  |  |  | pc |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type-2 (2 pcs) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Mobile tumblers Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18. Drainage |  |  |  |  |  |  |  |  |  |  |
| 103 |  | TsC59B |  | Mechanical excavation of soil using an excavator with a 0.15 m³ bucket, with discharge into a tipper lorry: Category 2 land |  |  | 100 m³ |  | 1.2000 |  |  |  |  |  |
| 104 |  | TsA20B |  | Manual excavation of soil, in slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade soil |  |  | m3 |  | 13.3100 |  |  |  |  |  |
| 105 |  | TsD01B |  | Spreading of fine-grained soil by shovel, in uniform layers 10–30 cm thick, by throwing up to 3 m from piles, including breaking up clods, the soil being from medium-grade terrain |  |  | m3 |  | 94.7600 |  |  |  |  |  |
| 106 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of each individual layer, the fill being made from non-cohesive soil |  |  | 100 m3 |  | 0.9500 |  |  |  |  |  |
| 107 |  | AcD18B |  | Installation of M-3 type PVC drainage pipes, joined by welding, outside buildings, buried to a depth of up to 2 m, with a diameter of 110 mm (drainage pipe with perforated geotextile) |  |  | m |  | 185.6000 |  |  |  |  |  |
| 108 |  | AcD18D apl |  | Installation of M-3 type PVC drainage pipes, joined by welding, outside buildings, buried to a depth of up to 2 m, with a diameter of 300 mm (drainage pipe) |  |  | m |  | 79.4000 |  |  |  |  |  |
| 109 |  | AcF03C |  | Backfill in trenches for water supply or sewerage pipes, as a substrate, protective layer, insulation layer or filter layer for drainage pipes, made of crushed stone |  |  | m³ |  | 16.3300 |  |  |  |  |  |
| 110 |  | AcF03A |  | Backfill in trenches for water supply or sewerage pipes, as a base, protective layer, insulation layer or filter layer for drainage pipes, made with sand |  |  | m3 |  | 2.2300 |  |  |  |  |  |
| 111 |  | RpDD27B |  | Reinforcement of weak or unstable foundation soils in single or multiple layers (for roads, platforms, asphalt pavements, embankments, behind retaining walls, etc.), subject to high and permanent loads, constructed using Basetex-type geotextiles (membrane tensioned between bored piles at the base of the foundations) |  |  | m² |  | 50.1100 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Drainage Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19. Marking |  |  |  |  |  |  |  |  |  |  |
| 112 |  | RpDE10B |  | Longitudinal road markings, single or double, with interrupted or continuous lines, applied mechanically using enamel paint on road surfaces without glass beads |  |  | m |  | 1,280.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Mark Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 20. Fencing for mini-football pitch |  |  |  |  |  |  |  |  |  |  |
| 113 |  | TsA02E |  | Manual excavation of soil in confined spaces, less than 1.00 m or more than 1.00 m in width, carried out without shoring, with vertical slopes, for foundations, trenches, basements, drains, and access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium ground |  |  | m³ |  | 14.8000 |  |  |  |  |  |
| 114 |  | CA03E |  | Cast concrete in foundations, plinths, retaining walls, walls below ground level, prepared at a concrete batching plant and cast using conventional methods, reinforced concrete class C 20/16 (Bc 20/B 250) |  |  | m3 |  | 14.8000 |  |  |  |  |  |
| 115 |  | CC01E |  | OB 37 steel reinforcement bars, cut to size on site and installed with bar diameters up to and including 8 mm in continuous foundations and raft foundations (diam. 6 AI) |  |  | kg |  | 83.0000 |  |  |  |  |  |
| 116 |  | CC01F1 |  | PC 52 reinforced concrete reinforcement bars, cut to size on site and installed with bar diameters of 8 mm and above in continuous footings and raft foundations (diameter 12 A III) |  |  | kg |  | 182.3000 |  |  |  |  |  |
| 117 |  | CC01F1 |  | PC 52 concrete steel reinforcement, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 16 A III) |  |  | kg |  | 70.1000 |  |  |  |  |  |
| 118 |  | CL08A |  | Prefabricated metal elements (columns, beams, trusses), delivered fully assembled, installed on site, in lightweight construction |  |  | t |  | 2.4100 |  |  |  |  |  |
| 119 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 2.4100 |  |  |  |  |  |
| 120 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 2.4100 |  |  |  |  |  |
| 121 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 2.4100 |  |  |  |  |  |
| 122 |  | CE35A apl |  | Walls made of wire mesh plaster |  |  | m² |  | 384.4000 |  |  |  |  |  |
| 123 |  | CE35A |  | Safety net, reusable, used in the construction of building cladding |  |  | m² |  | 235.6000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Fence for mini-football pitch Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 21. Fencing for the ’s multi-purpose pitch |  |  |  |  |  |  |  |  |  |  |
| 124 |  | TsA02E |  | Manual earth excavation in confined spaces, less than 1.00 m or more than 1.00 m wide, carried out without shoring, with vertical slopes, for foundations, channels, basements, drains, access steps, in medium-cohesive or highly cohesive soil, depth < 1.5 m, medium soil |  |  | m³ |  | 6.6000 |  |  |  |  |  |
| 125 |  | CA03E |  | Cast concrete in foundations, plinths, retaining walls, walls below ground level, prepared at a concrete batching plant and cast using conventional methods; reinforced concrete class C 20/16 (Bc 20/B 250) |  |  | m3 |  | 6.6000 |  |  |  |  |  |
| 126 |  | CC01E |  | OB 37 reinforced concrete steel bars, cut to length on site and installed with bar diameters up to and including 8 mm in continuous foundations and raft foundations (diameter 6 AI) |  |  | kg |  | 37.0500 |  |  |  |  |  |
| 127 |  | CC01F1 |  | PC 52 reinforced concrete steel bars, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 12 A III) |  |  | kg |  | 81.4000 |  |  |  |  |  |
| 128 |  | CC01F1 |  | PC 52 concrete steel reinforcement, cut on site and installed with bar diameters of 8 mm and above in continuous foundations and raft foundations (diam. 16 A III) |  |  | kg |  | 31.3000 |  |  |  |  |  |
| 129 |  | CL08A |  | Prefabricated metal components (columns, beams, trusses), delivered fully assembled and installed on site as part of a lightweight construction |  |  | t |  | 1.9360 |  |  |  |  |  |
| 130 |  | RpIzE16B |  | Cleaning of metal fabrications and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 1.9360 |  |  |  |  |  |
| 131 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from profiles with thicknesses between 8 mm and 12 mm inclusive, using a mineral brush |  |  | t |  | 1.9360 |  |  |  |  |  |
| 132 |  | IzD04A |  | Painting of metal structures and fabrications with oil-based paint in 2 coats, made from profiles, with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 1.9360 |  |  |  |  |  |
| 133 |  | CE35A apl |  | Walls made of wire mesh plaster |  |  | m² |  | 282.0000 |  |  |  |  |  |
| 134 |  | CE35A |  | Safety netting, reusable, used in the construction of building cladding |  |  | m² |  | 188.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Fencing for the multi-purpose sports ground at Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24% |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total quote: Including salary |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |