| Thermal insulation of the building of the kindergarten in the village of Pirita, rl. Dubasari. |  |
| --- | --- |
| (name of the objective) |  |

Thermal insulation of the kindergarten building. Block A

| Yes Crt. | Norms Symbol and Resource Code | Works and expenses | U.M. | Quantity according to project data |
| --- | --- | --- | --- | --- |

|  |  |  | 1.1. Dismantling works. Block A |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | RpCI42A Application |  | Dismantling of exterior window sills |  | m | 60,000 |
| 2 | RpCI42A |  | Dismantling of roof elements - gutters, downspouts |  | m | 80,000 |
| 3 | RpCJ35A |  | Removal of exterior plasters to the walls |  | m2 | 115,000 |
| 4 | RpCM33A |  | Dismantling of tiles, ceramic plywood (plinth) |  | m2 | 110,000 |
| 5 | TrI1AA02F1 |  | Loading of group A materials - heavy, in Bulgarian by transport up to 10 m - from ramp or terrain, in car, category 1 |  | t | 13,780 |
| 6 | TsI51A3 |  | Transport of garbage by 10 t dump truck at a distance of: 3 km |  | t | 13,780 |
|  |  |  |  | Total Dismantling Works. Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |
|  |  |  | 3. Facade block A |  |  |  |
|  |  |  | 3.1. Facades. Block A |  |  |  |
| 7 | CB15A |  | Self-erecting metal scaffolding with 2 castles (Lmax=12 m, Hmax=31 m) for common façade works |  | m2 | 715,000 |
| 8 | CF15A |  | Hand-made slipped exterior plasters with 2 cm thick M 100-T cement mortar on concrete or brick walls with flat surfaces |  | m2 | 115,000 |
| 9 | CF17B |  | Various works - thickness difference, +-5 mm at the primer layer applied to the walls, executed with M 100-T mortar Labor coefficient = 2.000 Coefficient for materials = 2,000 Coefficient at machine=2.000 |  | m2 | 115,000 |
| 10 | CN53A |  | Priming the outer surfaces of the walls |  | m2 | 715,000 |
| 11 | IzF55C |  | External thermal insulation of the walls of buildings with fine plasters on thermal insulation (systems with rigid fixing of thermal insulation), smooth wall surface: with rigid stone wool plates, thickness 100 mm with a density of 135 kg/m3 on the walls. Aluminium angle, = 60.0 m |  | m2 | 635,000 |
| 12 | IzF55E |  | External thermal insulation of window sills of buildings with fine plasters on thermal insulation (systems with rigid fixing of thermal insulation), smooth wall surface: with rigid stone wool tiles, thickness 50 mm, 135 kg/m3 for decorative elements. ( Corner corner corner window openings, doors = 235 m) |  | m2 | 80,000 |
| 13 | CN54B |  | Manual application of "Gleta" quartz primer in a layer, to exterior walls to facades |  | m2 | 715,000 |
| 14 | CF30A |  | Exterior plasters 2-3 mm. thick, made by hand. with "TINC" mixture on the walls (white color) |  | m2 | 715,000 |
| 15 | CK26D |  | Window sills or doors of aluminium, width=220 mm |  | m | 60,000 |
| 16 | CE18A |  | Skylights made of corrugated or corrugated plates (monolithic polycarbonate 4 mm) (Expanded polystyrene plate, rubber tape, price 0.000 lei) Small material (position spacers, nut screws, wood surfer, etc.)=1,050 |  | m2 | 10,000 |
|  |  |  |  | Total Facades. Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |
|  |  |  | 3.2. Base. Block A |  |  |  |
| 17 | CK26D |  | Dripper socket profile, width=180 mm |  | m | 100,000 |
| 18 | IzF55B |  | Exterior thermal insulation of the walls of buildings with fine plasters on thermal insulation (systems with rigid fixing of thermal insulation), smooth wall surface: with XPS extruded polystyrene sheets - 100 mm. Corner profile for thermal insulation, = 20.0 m |  | m2 | 90,000 |
| 19 | CN53A |  | Priming the outer surfaces of the walls |  | m2 | 90,000 |
| 20 | CF15A |  | Hand-made slipped exterior plasters with 2 cm thick M 100-T cement mortar on concrete or brick walls with flat surfaces |  | m2 | 15,000 |
| 21 | IzF31A |  | Waterproofing of concrete surfaces (vertical) with mixture "Hidrostop waterproofing mortar" or analogous, , thickness 3 mm /Applicative, hydrostop mortar consumption, 4.5 kg/m2 |  | m2 | 90,000 |
| 22 | CF15A |  | Hand-made slipped exterior plasters with 2 cm thick M 100-T cement mortar on concrete or brick walls with flat surfaces |  | m2 | 90,000 |
| 23 | CC03A (application) |  | Mounting of raisin nets 45 x 45 x 1.8 mm (90 sq.m.) |  | kg | 90,000 |
| 24 | CI21A |  | Wall cladding with ceramic-granite tiles: dimensions up to 400 x 400 mm (frost-resistant tiles for plinth) |  | m2 | 90,000 |
|  |  |  |  | Total Soclu. Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |
|  |  |  | 3.4. Cerdac. Block A |  |  |  |
| 25 | CC03C |  | Installation of welded meshes at heights less than or equal to 35 m, on slabs (D4mm Bp1 150 x 150) |  | kg | 441,110 |
| 26 | IzF18C |  | Levelling or protective support layer for insulation, including related scales, made with ready-made cement mortar of the M100-T brand without the addition of lime, dried, on horizontal or inclined surfaces up to and including 40 %, applied in an average thickness of 3 cm |  | m2 | 326,750 |
| 27 | IzF04J1 Application |  | Waterproofing film |  | m2 | 326,750 |
| 28 | IzF10H |  | Thermal insulation layer on floors, made of stone wool slabs 135 kg/m3 - 150 mm, simply laid in dry bridges, not circulating on horizontal surfaces or inclined up to 20 % |  | m2 | 326,750 |
| 29 | IzF04J1 |  | Vapour Barrier (Film) |  | m2 | 326,750 |
|  |  |  |  | Total Cerdac. Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |
|  |  |  | 3.6. Floor plan of the basement. Block A |  |  |  |
| 30 | IzF55C |  | Interior thermal insulation of ceilings of buildings with fine plasters on thermal insulation (systems with rigid fixing of thermal insulation), smooth ceiling surface: with mineral wool tiles 150 mm, 135 kg/m3 |  | m2 | 295,860 |
| 31 | IzF31A |  | Waterproofing of concrete surfaces (ceilings) with mixture "Hydrostop waterproofing mortar" or analogous, thickness 3 mm / Applicative, hydrostop mortar consumption, 4.5 kg/m2 |  | m2 | 295,860 |
| 32 | CN53A |  | Priming the interior surfaces of the ceilings |  | m2 | 295,860 |
|  |  |  |  | Total Floor Pod Basement. Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |
|  |  |  |  | Total Facade Block A |  |  |
|  |  |  |  | Salary Inclusive |  |  |

| Prepared |  |
| --- | --- |
| (position, signature) |  |
| Verified |  |
| (position, signature) |  |