ANNEX 2: QUOTATION SUBMISSION FORM

Bidders are requested to complete this form, including the Company Profile and Bidder’s Declaration, sign it and return it as part of their quotation along with Annex 3: Technical and Financial Offer. The Bidder shall fill in this form in accordance with the instructions indicated. No alterations to its format shall be permitted and no substitutions shall be accepted.

| Name of Bidder: | Click or tap here to enter text. |  |
| --- | --- | --- |
| RFQ reference: | RFQ26/03282 | Date: Click or tap to enter a date. |

Company Profile

| Item Description |  | Detail |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Legal name of bidder or Lead entity for JVs |  | Click or tap here to enter text. |  |  |  |
| Legal Address, City, Country |  | Click or tap here to enter text. |  |  |  |
| Website |  | Click or tap here to enter text. |  |  |  |
| Year of Registration |  | Click or tap here to enter text. |  |  |  |
| Legal structure |  | Choose an item. |  |  |  |
| Are you a UNGM registered vendor? |  | ☐ Yes ☐ No If yes, insert UNGM Vendor Number |  |  |  |
| Quality Assurance Certification (e.g. ISO 9000 or Equivalent) (If yes, provide a Copy of the valid Certificate): |  | ☐ Yes ☐ No |  |  |  |
| Does your Company hold any accreditation such as ISO 14001 or ISO 14064 or equivalent related to the environment? (If yes, provide a Copy of the valid Certificate): |  | ☐ Yes ☐ No |  |  |  |
| Does your Company have a written Statement of its Environmental Policy? (If yes, provide a Copy) |  | ☐ Yes ☐ No |  |  |  |
| Does your organization demonstrate significant commitment to sustainability through some other means, for example internal company policy documents on women empowerment, renewable energies or membership of trade institutions promoting such issues (If yes, provide a Copy) |  | ☐ Yes ☐ No |  |  |  |
| Is your company a member of the UN Global Compact |  | ☐ Yes ☐ No |  |  |  |
| Bank Information |  | Bank Name: Click or tap here to enter text. Bank Address: Click or tap here to enter text. IBAN: Click or tap here to enter text. SWIFT/BIC: Click or tap here to enter text. Account Currency: Click or tap here to enter text. Bank Account Number: Click or tap here to enter text. |  |  |  |
| Previous relevant experience: List and value of 3 (three) most relevant projects/ contracts (in terms of type and volume of goods) completed for the past 5 (five) years |  |  |  |  |  |
| Name of previous contracts | Client & Reference Contact Details including e-mail |  | Contract Value | Period of activity | Types of activities undertaken |

Bidder’s Declaration

| Yes | No |  |
| --- | --- | --- |
| ☐ | ☐ | Requirements and Terms and Conditions: I/We have read and fully understand the RFQ, including the RFQ Information and Data, Schedule of Requirements, the General Conditions of Contract, and any Special Conditions of Contract. I/we confirm that the Bidder agrees to be bound by them. |
| ☐ | ☐ | I/We confirm that the Bidder has the necessary capacity, capability, and necessary licenses to fully meet or exceed the Requirements and will be available to deliver throughout the relevant Contract period. |
| ☐ | ☐ | Ethics: In submitting this Quote I/we warrant that the bidder: has not entered into any improper, illegal, collusive or anti-competitive arrangements with any Competitor; has not directly or indirectly approached any representative of the Buyer (other than the Point of Contact) to lobby or solicit information in relation to the RFQ ;has not attempted to influence, or provide any form of personal inducement, reward or benefit to any representative of the Buyer. |
| ☐ | ☐ | I/We confirm to undertake not to engage in proscribed practices, , or any other unethical practice, with the UN or any other party, and to conduct business in a manner that averts any financial, operational, reputational or other undue risk to the UN and we have read the United Nations Supplier Code of Conduct :https://www.un.org/Depts/ptd/about-us/un-supplier-code-conduct and acknowledge that it provides the minimum standards expected of suppliers to the UN. |
| ☐ | ☐ | Conflict of interest: I/We warrant that the bidder has no actual, potential, or perceived Conflict of Interest in submitting this Quote or entering a Contract to deliver the Requirements. Where a Conflict of Interest arises during the RFQ process the bidder will report it immediately to the Procuring Organisation’s Point of Contact. |
| ☐ | ☐ | Prohibitions and Sanctions: l/We hereby declare that our firm, ultimate beneficial owners, affiliates or subsidiaries or employees, including any JV/Consortium members or subcontractors or suppliers for any part of the contract is not under procurement prohibition by the United Nations, including but not limited to prohibitions derived from the Compendium of United Nations Security Council Sanctions Lists and have not been suspended, debarred, sanctioned or otherwise identified as ineligible by any UN Organization or the World Bank Group or any other international Organization. |
| ☐ | ☐ | Bankruptcy: l/We have not declared bankruptcy, are not involved in bankruptcy or receivership proceedings, and there is no judgment or pending legal action against them that could impair their operations in the foreseeable future. |
| ☐ | ☐ | Offer Validity Period: I/We confirm that this Quote, including the price, remains open for acceptance for the Offer Validity. |
| ☐ | ☐ | I/We understand and recognize that you are not bound to accept any Quotation you receive, and we certify that the goods offered in our Quotation are new and unused. |
| ☐ | ☐ | By signing this declaration, the signatory below represents, warrants and agrees that he/she has been authorised by the Organization/s to make this declaration on its/their behalf. |

Signature:

Name: 	Click or tap here to enter text.

Title:	Click or tap here to enter text.

Date:  	Click or tap to enter a date.

## ANNEX 3: TECHNICAL AND FINANCIAL OFFER

Bidders are requested to complete this form, sign it and return it as part of their bid along with Annex 2: Quotation Submission Form. The Bidder shall fill in this form in accordance with the instructions indicated. No alterations to its format shall be permitted and no substitutions shall be accepted.

| Name of Bidder: | Click or tap here to enter text. |  |
| --- | --- | --- |
| RFQ reference: | RFQ26/03282 | Date: Click or tap to enter a date. |

Currency of Quotation: MDL (Moldovan Leu) for local suppliers / USD (US Dollars) for international suppliers.

| INCOTERMS: DDP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref | Description | Q-ty | Unit price, (VAT 0%) | Transportation, insurance and other charges, (VAT 0%) | Total price (please insert currency) (VAT 0%) |
|  | COMMUNICATION MODULES FOR PRIME 1.4 DATA CONCENTRATORS | 495 |  |  |  |
|  | TRAINING OF THE DISTRIBUTORS STAFF (2 groups) | 2 |  |  |  |
| Incoterms charges, including transportation/delivery costs |  |  |  |  |  |
| Warranty |  |  |  |  |  |
| Total Final and All-inclusive Price |  |  |  |  |  |

Compliance with Requirements

|  | You Responses |  |  |
| --- | --- | --- | --- |
|  | Yes, we will comply | No, we cannot comply | If you cannot comply, pls. indicate counter - offer |
| Minimum Technical Specifications | ☐ | ☐ | Click or tap here to enter text. |
| Delivery Term (INCOTERMS) DDP | ☐ | ☐ | Click or tap here to enter text. |
| Delivery Lead Time 90 days from signature of the contract | ☐ | ☐ | Click or tap here to enter text. |
| Training on Operation and Maintenance |  |  |  |
| Warranty: minimum 36 months | ☐ | ☐ | Click or tap here to enter text. |
| Validity of Quotation – 90 days | ☐ | ☐ | Click or tap here to enter text. |
| Payment terms | ☐ | ☐ | Click or tap here to enter text. |
| Other requirements [pls. specify] | ☐ | ☐ | Click or tap here to enter text. |

| I, the undersigned, certify that I am duly authorized to sign this quotation and bind the company below in event that the quotation is accepted. |  |
| --- | --- |
| Exact name and address of company Company NameClick or tap here to enter text. Address: Click or tap here to enter text. Phone No.: Click or tap here to enter text. Email Address: Click or tap here to enter text. | Authorized Signature: Date: Click or tap here to enter text. Name: Click or tap here to enter text. Functional Title of Authorised Signatory: Click or tap here to enter text. Email Address: Click or tap here to enter text. |

ANNEX 4: TECHNICAL RESPONSIVENESS TABLE

Bidders are requested to complete this form, sign it and return it as part of their bid along with Annex 2: Quotation Submission Form and Annex 3: Technical and Financial Offer. The Bidder shall fill in this form in accordance with the instructions indicated. No alterations to its format shall be permitted and no substitutions shall be accepted.

| Name of Bidder: | Click or tap here to enter text. |  |  |
| --- | --- | --- | --- |
| RFQ reference: | RFQ26/03282 | Date: Click or tap to enter a date. |

Bidders shall supplement the information provided in the tables below with detailed description of the offered goods, including design and Technical Data Sheets (including photos).

| ID | Requirement | Evidence required | Conformity (Yes/No) | Offered Brand, Model and Exact characteristics (Technical characteristics should not be less/worse than those listed in Annex 1: Schedule of Requirements) |
| --- | --- | --- | --- | --- |
| CM-001 | The communication module shall provide secure WAN/backhaul connectivity between the PRIME 1.4 Data Concentrator and HES/MDMS. | Exact brand/model and datasheet reference. |  |  |
| CM-002 | The module shall be compatible with DC PRIME 1.4 deployments and shall not limit the DC's PRIME PLC functions, meter registration, scheduled reads, event delivery, or remote commands. | Compatibility statement with PRIME 1.4 certificated DC and HES. |  |  |
| CM-003 | Minimum 1 x WAN Ethernet port 10/100/1000Base-T, auto-negotiation, auto MDI/MDIX. | Datasheet. |  |  |
| CM-004 | Minimum 2 x LAN Ethernet ports 10/100 Mbps. | Datasheet. |  |  |
| CM-005 | Minimum 1 x RS-485 serial interface, EIA/TIA-485 compliant, 300 bps to 115200 bps or wider, ESD protection +/-15 kV. | Datasheet and pinout. |  |  |
| CM-006 | Integrated 4G LTE modem supporting European/EMEA operator bands. Minimum LTE-FDD bands B1, B3, B7, B8, B20 and B28. | Datasheet and regional modem variant. |  |  |
| CM-007 | Minimum 1 physical SIM slot. eSIM shall be accepted as equivalent/additional capability but not mandatory if physical SIM is provided. | Datasheet. |  |  |
| CM-008 | The hardware must provide double antenna industrial connectors. Supplier shall provide suitable external LTE antennas and required cables. Antenna RF cable shall be low-loss coaxial type suitable for LTE communication. Minimum RF cable length shall be 5 meters. Extension of antenna cable shall be possible without modification of the communication module. Cable shall be UV-resistant and suitable for industrial outdoor usage. Cable and connectors shall be mechanically protected against vibration and environmental impact. Connector type shall be compatible with the offered communication module/router. | Datasheet. |  |  |
| CM-009 | TCP/IP, UDP/IP, DHCP client/server as applicable, DNS, NTP, IPv4 and IPv6 support. | Datasheet. |  |  |
| CM-010 | Support static routing, NAT, stateful firewall rules, port filtering and IP/MAC filtering. | Datasheet and configuration guide. |  |  |
| CM-011 | Support secure VPN using IPsec and/or OpenVPN/WireGuard; IPsec for utility APN/VPN integration. GRE/L2/L3 tunnel supported. | Datasheet and configuration guide. |  |  |
| CM-012 | Support private APN configuration and operator-specific cellular profiles. | Configuration guide. |  |  |
| CM-013 | Web interface shall use HTTPS/TLS. Plain HTTP shall be disabled by default or disabled before handover. | Security guide. |  |  |
| CM-014 | Secure CLI/file transfer over SSH/SFTP or equivalent secure mechanism. Telnet/FTP shall be disabled by default. | Configuration guide. |  |  |
| CM-015 | Vendor shall provide central management capability or compatibility with a standard NMS/SNMP/Syslog platform for inventory, configuration backup, firmware, logs and health status. | Description. |  |  |
| CM-016 | SNMPv3 and remote Syslog export support. | Datasheet. |  |  |
| CM-017 | Built-in hardware/software watchdog and automatic recovery after cellular or WAN loss. | Datasheet. |  |  |
| CM-018 | Configurable scheduled reboot and remote reboot function. | Configuration guide. |  |  |
| CM-019 | Remote and local firmware upgrade, signed firmware or vendor-authenticated packages, rollback or recovery mechanism. | Security guide. |  |  |
| CM-020 | Role-based accounts or at minimum unique admin/service credentials, password policy, credential change at first use, no shared default production credentials. | Security guide. |  |  |
| CM-021 | Local log and remote export for login, configuration change, VPN status, cellular status, reboot, firmware update and security events. | Log sample. |  |  |
| CM-022 | NTP client support with configurable servers; local RTC. | Datasheet/ configuration. |  |  |
| CM-023 | LEDs for power, cellular network/service, signal level or quality, WAN/LAN activity and VPN/online status where available. | Datasheet. |  |  |
| CM-024 | 12 VDC or 24 VDC operation. Supplier shall include DIN-rail industrial power supply if not powered directly by the DC. Battery deep-discharge protection. | Datasheet. |  |  |
| CM-025 | Low-power industrial design. Bidder shall state idle, typical and peak consumption. Target: <=3 W idle and <=10 W typical/peak during transmission; deviations allowed if justified by integrated features such as multiple LTE modules or SFP. | Datasheet. |  |  |
| CM-026 | Industrial enclosure suitable for DIN-rail mounting; wall/panel mounting accepted only if DIN-rail adapter is included. | Datasheet. |  |  |
| CM-027 | Operating temperature at least -20 C to +60 C; -30 C to +70 C or wider. Humidity 5% to 95% non-condensing or equivalent. | Datasheet. |  |  |
| CM-028 | Industrial EMC compliance; RS-485 ESD protection +/-15 kV; CE/EMC declaration for EU/EEA market or equivalent. | Certificates. |  |  |
| CM-029 | Unused services disabled, secure default configuration, documented hardening checklist, vulnerability disclosure and security update process. | Hardening checklist. |  |  |
| CM-030 | Bidder shall provide a compatibility matrix with the offered DC PRIME 1.4 model, HES/APN/VPN setup and any required cable/power accessories. | Completed matrix. |  |  |
| CM-031 | Bidder shall provide at least one sample communication module per offered configuration for laboratory testing with the PRIME 1.4 certificated DC, if requested by the UNDP. | Sample commitment. |  |  |
| CM-032 | Minimum 24 months warranty; Bidder shall provide authorized service/warranty repair pathway. | Warranty statement and service center evidence. |  |  |
| CM-033 | Bidder shall deliver the goods in up to 90 calendar days after Contract signature. | Delivery plan. |  |  |
| CM-034 | Bidder shall provide manufacturer authorization or, if manufacturer, clear manufacturer declaration and after-sales support commitment. | Authorization letter. |  |  |
| CM-035 | Bidder shall provide datasheets, user manuals, configuration guide, security guide, certificates, wiring diagrams and recommended antenna/cable installation instructions. | Documents. |  |  |
| CM-036 | The module shall support standard IP networking and shall not impose proprietary lock-in preventing use with other PRIME-certified DCs or HES platforms, subject to interface compatibility. | Declaration. |  |  |

| I, the undersigned, certify that I am duly authorized to sign this quotation and bind the company below in event that the quotation is accepted. |  |
| --- | --- |
| Exact name and address of company Company NameClick or tap here to enter text. Address: Click or tap here to enter text. Phone No.: Click or tap here to enter text. Email Address: Click or tap here to enter text. | Authorized Signature: Date: Click or tap here to enter text. Name: Click or tap here to enter text. Functional Title of Authorised Signatory: Click or tap here to enter text. Email Address: Click or tap here to enter text. |

ANNEX 5: FORMAT FOR CV OF PROPOSED KEY PERSONNEL

| Name of Proposer: | Click or tap here to enter text. | Date: | Click or tap to enter a date. |
| --- | --- | --- | --- |
| RFQ reference: | RFQ26/03282 |  |  |

| Position (as per ToR) |  |  |
| --- | --- | --- |
| Personnel Information | Name: |  |
|  | Nationality: | Date of birth: |
|  | Language Proficiency: |  |
| Present Employment | Name of employer: | Contact: (manager or HR) |
|  | Address of employer: |  |
|  | Telephone: | Email: |
|  | Job title: | Years with present employer: |
| Education / Qualifications | Summarise college/university and other specialised education of personnel member, giving names of schools, dates attended, and degrees/qualifications obtained. |  |
| Professional Certifications | Provide details of professional certifications relevant to the scope of services including name of institution and date of certification. |  |
| References: | Provide names, addresses, phone and email contact information for two (2) references. |  |

Summarise professional experience over the last 3 years in reverse chronological order. Indicate particular technical and managerial experience relevant to the project.

| From | To | Company / Project / Position / Relevant technical and management experience |
| --- | --- | --- |

I, the undersigned, certify that, to the best of my knowledge and belief, this CV is accurate.

Signature of Personnel																Date (Day/Month/Year)