ANNEX 2.1: TECHNICAL RESPONSIVENESS TABLE

LOT 1 and LOT 2

Bidders are requested to complete only the table(s) for the LOT(s) applied. The two LOTs cover related AMI equipment, but the equipment is technically different; therefore LOT 1 and LOT 2 are assessed separately. Compliance shall be supported by manufacturer documentation, certificates, catalogues, datasheets, protocol documentation, test reports or other relevant evidence.

This table is a simplified responsiveness matrix. It does not replace the detailed requirements in Annex 1.1 and Annex 1.2. Technical characteristics shall not be less/worse than those listed in the applicable Schedule of Requirements. Cross-lot interoperability between PRIME 1.4 smart meters, PRIME 1.4 data concentrators and the HES remains a mandatory acceptance condition.

LOT 1 - Supply of 65,000 PRIME 1.4 Smart Electricity Meters

| Item no. | Generic Description | Indicate compliance (YES / NO) | Offered Specification / Response (Include the offered brand and model, where applicable, together with the relevant technical specifications or service details demonstrating compliance with the requirement.) |  |  | Deviations and Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single-phase smart meters, PRIME 1.4 | Three-phase direct connected smart meters/DC, PRIME 1.4 | Three-phase direct connected smart meters/TC, PRIME 1.4 |  |
| 1.1. | Offered Brand, Model per type of single-phase, three-phase direct and/or three-phase indirect meters in line with Annex 1.1. |  |  |  |  |  |
| 1.2 | Electrical ratings and metrology are compliant with Annex 1.1, including voltage, current range, frequency, accuracy class, power consumption, clock accuracy and IP/mechanical requirements. |  |  |  |  |  |
| 1.3 | Valid MID / CE + M metrology certification or legally acceptable local metrology approval path is provided for the exact offered meter model and connection type. |  |  |  |  |  |
| 1.4 | The offered meters have PRIME 1.4 service-node PLC communication certification/evidence for the exact model, hardware version, firmware version and communication profile. |  |  |  |  |  |
| 1.5 | The meters support PRIME 1.4 operation and interoperability with PRIME 1.4 certified base-node data concentrators. Backward compatibility requirements, if applicable, are documented. |  |  |  |  |  |
| 1.6 | DLMS/COSEM compliance is documented, including OBIS mapping, event-code map, access levels, companion profile and HES integration guide. |  |  |  |  |  |
| 1.7 | Metering functions comply with Annex 1.1, including import/export active energy, reactive energy where required, load profiles, billing profiles, tariffs, instantaneous values and power-quality/outage events. |  |  |  |  |  |
| 1.8 | Cybersecurity requirements are met, including AES-128 or stronger security where applicable, role-based access, key management, signed firmware and auditable remote commands. |  |  |  |  |  |
| 1.9 | Required local/service interfaces and physical features are provided, including optical port, display, test outputs, push button, tamper sensors and relay/remote disconnect where required. |  |  |  |  |  |
| 1.10 | Warranty, service life, firmware support, cybersecurity support and lifecycle/production availability statements are provided as required in Annex 1.1. |  |  |  |  |  |
| 1.11 | The bidder provides a clear HES / HES module / integration-tool statement if required for testing, onboarding, DLMS/COSEM mapping, firmware management and operation of the supplied meters. |  |  |  |  |  |
| 1.12 | The bidder accepts sample provision, pre-award verification and FAT requirements, where requested by UNDP, in line with Annex 1.1 and the bidding documents. |  |  |  |  |  |
| 1.13 | The bidder commits to cooperate in end-to-end interoperability testing with the awarded LOT 2 data concentrator supplier and the HES environment. |  |  |  |  |  |

LOT 2 - Supply of 495 PRIME 1.4 Data Concentrators

| Item no. | Generic Description | Indicate compliance (YES / NO) | Offered Specification / Response (Include the offered brand and model, where applicable, together with the relevant technical specifications or service details demonstrating compliance with the requirement.) | Deviations and Comments |
| --- | --- | --- | --- | --- |
| 2.1 | Offered Brand, Model per Data Concentrator in line with Annex 1.2. |  |  |  |
| 2.1 | LOT 2 - Data Concentrators: bidder confirms compliance with all applicable requirements of Annex 1.2 for the offered PRIME 1.4 Data Concentrators. |  |  |  |
| 2.2 | The offered DC is new, unused, production-grade equipment suitable for LV transformer/substation use and includes embedded PRIME OFDM PLC base-node function. |  |  |  |
| 2.3 | Electrical and environmental requirements are compliant with Annex 1.2, including voltage range, frequency, power consumption, temperature range, humidity, IP rating, EMC/safety and reliability. |  |  |  |
| 2.4 | The offered DC has PRIME 1.4 base-node certification/evidence for the exact hardware/firmware profile offered, including relevant band/channel information and limitations. |  |  |  |
| 2.5 | The DC supports PRIME 1.4 operation and interoperability/backward compatibility with PRIME service nodes/meters, where required for mixed or phased migration networks. |  |  |  |
| 2.6 | PLC network management functions are provided, including automatic discovery, registration, route/repeating mechanisms, topology/quality metrics and three-phase PLC signal injection or equivalent coverage solution. |  |  |  |
| 2.7 | The DC supports the required number of end-point devices/service nodes with suitable engineering headroom and provides documented capacity assumptions. |  |  |  |
| 2.8 | Backhaul and local communication interfaces are compliant with Annex 1.2, including Ethernet, cellular backhaul where required, local secure management and relevant antennas/accessories. |  |  |  |
| 2.9 | DLMS/COSEM and open protocol support are documented, including compatibility with meter object lists, OBIS mapping and HES/MDC integration requirements. |  |  |  |
| 2.10 | Data collection and buffering functions comply with Annex 1.2, including scheduled/on-demand/event reads, non-volatile storage and forwarding after backhaul recovery. |  |  |  |
| 2.11 | Cybersecurity requirements are met, including secure backhaul, VPN/TLS/IPSec or equivalent, AES-128 or stronger encryption where applicable, access control, audit logs, tamper events and signed firmware update. |  |  |  |
| 2.12 | Remote management, configuration, firmware update, NTP/time synchronization, event/alarm reporting, LED indications and backup power support are provided as required. |  |  |  |
| 2.13 | Warranty, service life, firmware support, cybersecurity support, spare parts and lifecycle/production availability statements are provided as required in Annex 1.2. |  |  |  |
| 2.14 | The bidder accepts sample provision, pre-award verification and FAT requirements, where requested by UNDP, in line with Annex 1.2 and the bidding documents. |  |  |  |
| 2.15 | The bidder commits to cooperate in end-to-end interoperability testing with the awarded LOT 1 smart meter supplier and the HES environment. |  |  |  |