| Major repairs and modernisation of the ‘Mihai Eminescu’ Public Lower Secondary School, cadastral number 2101101.004.01, situated in the town of Cantemir, Cantemir municipality, 10 Trandafirilor Street | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL ESTIMATE No. 2-1-1

Architectural solutions

Prepared at current prices

| No. No. | Symbol, standards and resource code | Works and expenses | Unit of measurement | Quantity as per project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Demolition of buildings |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Elevation - 1,900, - 2,900 |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, aerated concrete (BCA), ceramic or lightweight concrete blocks, and GVP bricks, excluding scaffolding and the removal of bricks (204 m² × 0.12 = 24.48 m³) |  |  | m³ |  | 24.4800 |  |  |  |  |  |
| 2 |  | RpCJ35A |  | Removal of interior or exterior plaster attached to walls (removal of interior plaster, 2–3 cm thick) |  |  | m² |  | 470.0000 |  |  |  |  |  |
| 3 |  | RpCK42A |  | Removal of cold concrete or cement mortar floors |  |  | m² |  | 762.0000 |  |  |  |  |  |
| 4 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x900 – 3 pieces) |  |  | m² |  | 5.6700 |  |  |  |  |  |
| 5 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1000 – 1 piece) |  |  | m² |  | 2.1000 |  |  |  |  |  |
| 6 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1200 – 2 pieces) |  |  | m² |  | 5.0400 |  |  |  |  |  |
| 7 |  | RpCO56A |  | Dismantling: wooden joinery (windows) |  |  | m² |  | 4.9500 |  |  |  |  |  |
| 8 |  | RpCP44B |  | Dismantling of metal structures without recovery of materials |  |  | kg |  | 790.0000 |  |  |  |  |  |
| 9 |  | RpCB18F |  | Demolition of old concrete using mechanical means, plain concrete (Removal of concrete sills, 4 cm thick, dimensions 1500 x 400 mm, 3 pieces) |  |  | m³ |  | 0.0700 |  |  |  |  |  |
| 10 |  | RpCJ35A |  | Removal of cracked interior or exterior plaster from ceilings |  |  | m² |  | 778.0000 |  |  |  |  |  |
| 11 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, aerated concrete (BCA), ceramic or lightweight concrete blocks, and GVP bricks, excluding scaffolding and the removal of bricks |  |  | m³ |  | 3.9700 |  |  |  |  |  |
| 12 |  | TsI51B10 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 20 km |  |  | t |  | 75.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Share - 1,900, - 2,900 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Odds - 0.450 |  |  |  |  |  |  |  |  |  |  |
| 13 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, aerated concrete (BCA), ceramic or lightweight concrete blocks, and GVP bricks, excluding scaffolding and the removal of bricks (318 m² × 0.12 = 38.16 m³) |  |  | m³ |  | 38.1600 |  |  |  |  |  |
| 14 |  | RpCJ35A |  | Removal of interior or exterior plaster attached to walls (removal of interior plaster, 2–3 cm thick) |  |  | m² |  | 858.0000 |  |  |  |  |  |
| 15 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, carpet, etc. |  |  | m² |  | 510.0000 |  |  |  |  |  |
| 16 |  | RpCK41A |  | Removal of floor coverings consisting of plank flooring, cupboards, etc. |  |  | m² |  | 510.0000 |  |  |  |  |  |
| 17 |  | RpCK42C |  | Removal of cold flooring made of concrete slabs, marble, stone, stoneware, ceramic tiles, etc. |  |  | m² |  | 29.0000 |  |  |  |  |  |
| 18 |  | RpCK42B |  | Removal of cold flooring made of mosaic, in a continuous surface or mosaic tiles |  |  | m² |  | 173.0000 |  |  |  |  |  |
| 19 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x900 – 11 pieces) |  |  | m² |  | 20.7900 |  |  |  |  |  |
| 20 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x700 – 5 items) |  |  | m² |  | 7.3500 |  |  |  |  |  |
| 21 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1400 – 2 pieces) |  |  | m² |  | 5.8800 |  |  |  |  |  |
| 22 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1800 – 3 pieces) |  |  | m² |  | 11.3400 |  |  |  |  |  |
| 23 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (2120x2000 – 35 items) |  |  | m² |  | 148.4000 |  |  |  |  |  |
| 24 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (820x2000 – 2 pieces) |  |  | m² |  | 3.2800 |  |  |  |  |  |
| 25 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (2450x2000 – 2 pieces) |  |  | m² |  | 4.9000 |  |  |  |  |  |
| 26 |  | RpCP44B |  | Dismantling of metal structures without material recovery |  |  | kg |  | 980.0000 |  |  |  |  |  |
| 27 |  | RpCB18F |  | Demolition of old concrete using mechanical means, plain concrete (Dismantling of concrete window sills, 4 cm thick: (2120x400) – 35 pieces, (820x400) – 2 pieces, (2450x400) – 1 piece) |  |  | m³ |  | 1.2500 |  |  |  |  |  |
| 28 |  | RpCJ35A |  | Removal of cracked interior or exterior plaster from ceilings |  |  | m² |  | 711.0000 |  |  |  |  |  |
| 29 |  | RpCP44B |  | Dismantling of metal structures without recovery of materials (16 m x 5 kg/m = 80 kg) |  |  | kg |  | 80.0000 |  |  |  |  |  |
| 30 |  | RpEF23A |  | Dismantling of lighting fixtures of any type, including poles and globes |  |  | piece |  | 136.0000 |  |  |  |  |  |
| 31 |  | RpIB12A apl |  | Removal of old radiators |  |  | piece |  | 45.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total share – 0.450 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Odds: +3,300 |  |  |  |  |  |  |  |  |  |  |
| 32 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, aerated concrete (BCA), ceramic or lightweight concrete blocks, and GVP bricks, excluding scaffolding and the removal of bricks (249 m² × 0.12 = 29.9 m³) |  |  | m³ |  | 29.9000 |  |  |  |  |  |
| 33 |  | RpCJ35A |  | Removal of interior or exterior plaster attached to walls (removal of interior plaster, 2–3 cm thick) |  |  | m² |  | 1,060.0000 |  |  |  |  |  |
| 34 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, carpet, etc. |  |  | m² |  | 516.0000 |  |  |  |  |  |
| 35 |  | RpCK41A |  | Removal of floor coverings consisting of plank flooring, cupboards, etc. |  |  | m² |  | 516.0000 |  |  |  |  |  |
| 36 |  | RpCK42C |  | Removal of cold flooring made of concrete slabs, marble, stone, stoneware, ceramic tiles, etc. |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 37 |  | RpCK42B |  | Removal of cold flooring made of mosaic, in a continuous surface or mosaic tiles |  |  | m² |  | 165.0000 |  |  |  |  |  |
| 38 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x900 – 14 pieces) |  |  | m² |  | 26.4600 |  |  |  |  |  |
| 39 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x700 – 2 pieces) |  |  | m² |  | 2.9400 |  |  |  |  |  |
| 40 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1400 – 2 pieces) |  |  | m² |  | 5.8800 |  |  |  |  |  |
| 41 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (2120x2000 – 40 items) |  |  | m² |  | 169.6000 |  |  |  |  |  |
| 42 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (820x2000 – 2 pieces) |  |  | m² |  | 3.2800 |  |  |  |  |  |
| 43 |  | RpCP44B |  | Dismantling of metal structures without material recovery |  |  | kg |  | 990.0000 |  |  |  |  |  |
| 44 |  | RpCB18F |  | Demolition of old concrete using mechanical means, plain concrete (Removal of concrete window sills, 4 cm thick: (2120 x 400) – 40 pieces, (820 x 400) – 2 pieces) |  |  | m³ |  | 1.3800 |  |  |  |  |  |
| 45 |  | RpCJ35A |  | Removal of cracked interior or exterior plaster from ceilings |  |  | m² |  | 716.0000 |  |  |  |  |  |
| 46 |  | RpCP44B |  | Dismantling of metal structures without recovery of materials (16 m x 5 kg/m = 80 kg) |  |  | kg |  | 80.0000 |  |  |  |  |  |
| 47 |  | RpEF23A |  | Dismantling of lighting fixtures of any type, including poles and globes |  |  | piece |  | 130.0000 |  |  |  |  |  |
| 48 |  | RpIB12A apl |  | Removal of old radiators |  |  | piece |  | 58.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total share + 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Share + 6,600 |  |  |  |  |  |  |  |  |  |  |
| 49 |  | RpCG29C |  | Demolition of masonry walls made of solid brick, aerated concrete blocks (BCA), ceramic or lightweight concrete blocks, and GVP bricks, excluding scaffolding and the removal of bricks (252 m² × 0.12 = 30.24 m³) |  |  | m³ |  | 30.2400 |  |  |  |  |  |
| 50 |  | RpCJ35A |  | Removal of interior or exterior plaster attached to walls (removal of interior plaster, 2–3 cm thick) |  |  | m² |  | 885.0000 |  |  |  |  |  |
| 51 |  | RpCK41C |  | Removal of PVC carpet flooring with or without a textile backing, carpet, etc. |  |  | m² |  | 517.0000 |  |  |  |  |  |
| 52 |  | RpCK41A |  | Removal of floor coverings consisting of plank flooring, cupboards, etc. |  |  | m² |  | 517.0000 |  |  |  |  |  |
| 53 |  | RpCK42C |  | Removal of cold flooring made of concrete slabs, marble, stone, stoneware, ceramic tiles, etc. |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 54 |  | RpCK42B |  | Removal of cold flooring made of mosaic, in a continuous surface or mosaic tiles |  |  | m² |  | 168.0000 |  |  |  |  |  |
| 55 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x900 – 13 pieces) |  |  | m² |  | 24.5700 |  |  |  |  |  |
| 56 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x700 – 5 items) |  |  | m² |  | 7.3500 |  |  |  |  |  |
| 57 |  | RpCO56A |  | Dismantling: wooden joinery (doors) (2100x1400 – 2 pieces) |  |  | m² |  | 5.8800 |  |  |  |  |  |
| 58 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (2120x2000 – 40 items) |  |  | m² |  | 169.6000 |  |  |  |  |  |
| 59 |  | RpCO56A |  | Dismantling: wooden joinery (windows) (820x2000 – 2 pieces) |  |  | m² |  | 3.2800 |  |  |  |  |  |
| 60 |  | RpCP44B |  | Dismantling of metal structures without material recovery |  |  | kg |  | 860.0000 |  |  |  |  |  |
| 61 |  | RpCB18F |  | Demolition of old concrete using mechanical means, plain concrete (Removal of concrete window sills, 4 cm thick: (2120 x 400) – 40 pieces, (820 x 400) – 2 pieces) |  |  | m³ |  | 1.3800 |  |  |  |  |  |
| 62 |  | RpCJ35A |  | Removal of cracked interior or exterior plaster from ceilings |  |  | m² |  | 716.0000 |  |  |  |  |  |
| 63 |  | RpCP44B |  | Dismantling of metal structures without recovery of materials (18 m × 5 kg/m = 90 kg) |  |  | kg |  | 90.0000 |  |  |  |  |  |
| 64 |  | RpEF23A |  | Dismantling of lighting fixtures of any type, including poles and globes |  |  | piece |  | 126.0000 |  |  |  |  |  |
| 65 |  | RpIB12A apl |  | Removal of old radiators |  |  | piece |  | 65.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total share + 6,600 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.5. Roof |  |  |  |  |  |  |  |  |  |  |
| 66 |  | RpCI42B |  | Dismantling of roof elements – metal tile roofing |  |  | m² |  | 1,210.0000 |  |  |  |  |  |
| 67 |  | RpCH32C |  | Removal of wooden decking and roof elements – stripping of roofing with or without recovery of materials |  |  | m² |  | 1,210.0000 |  |  |  |  |  |
| 68 |  | RpCI42A |  | Dismantling of roof elements – gutters |  |  | m |  | 156.2000 |  |  |  |  |  |
| 69 |  | RpCI42A |  | Removal of roof components – downpipes |  |  | m |  | 102.0000 |  |  |  |  |  |
| 70 |  | RpCH32E |  | Dismantling of wooden floorboards and roof elements – the framework of sheds, supports, etc. |  |  | m³ |  | 37.0000 |  |  |  |  |  |
| 71 |  | RpCH32A |  | Dismantling of wooden floors and roof elements – wooden floors, including floorboards, subfloor between joists, rubble or slag infill, and removal of resulting materials |  |  | m² |  | 110.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Roof Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.6. Waste disposal |  |  |  |  |  |  |  |  |  |  |
| 72 |  | TrB05B2-6 |  | Transport, by carrying directly, of bulky materials weighing over 25 kg, over a distance of 60 m |  |  | t |  | 1,570.0000 |  |  |  |  |  |
| 73 |  | TsH92A |  | Loading into a refuse lorry |  |  | t |  | 1,570.0000 |  |  |  |  |  |
| 74 |  | TsI51B10 |  | Transport of soil by 10-tonne tipper lorry over a distance of: 20 km |  |  | t |  | 1,570.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Waste disposal Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Demolition of buildings Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Waterproofing and thermal insulation of the plinth |  |  |  |  |  |  |  |  |  |  |
| 75 |  | TsC58B |  | Mechanical earthworks at the site using an excavator with a ‘reverse bucket’ with a bucket volume of 0.15 m³: Category 2 soil |  |  | 100 m³ |  | 4.2000 |  |  |  |  |  |
| 76 |  | TsD01B |  | Spreading of screened soil by shovel, in uniform layers 10–30 cm thick, by throwing it up to 3 m from the heaps, including the breaking up of clods; soil originating from medium-grade land |  |  | m³ |  | 503.0000 |  |  |  |  |  |
| 77 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of piece individual layer; the fill is made from non-cohesive soil |  |  | 100 m³ |  | 5.0300 |  |  |  |  |  |
| 78 |  | IzF04F |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including troughs and valleys in standard waterproofing on slopes steeper than 40 per cent or on vertical, flat or curved surfaces, using bituminous mastic or rubber-modified bitumen, applied with a brush or rubber trowel (cosoroaba) |  |  | m² |  | 530.0000 |  |  |  |  |  |
| 79 |  | RpCJ12A |  | Smoothed interior and exterior plasterwork, carried out using M 100-T cement mortar with an average thickness of 2 cm, on concrete or brick walls with flat surfaces |  |  | m² |  | 530.0000 |  |  |  |  |  |
| 80 |  | RpCJ27B |  | Miscellaneous works – thickness variation, ±5 mm in the base coat applied with mortar (add 10 mm, K=2) |  |  | m² |  | 530.0000 |  |  |  |  |  |
| 81 |  | RpCD08A |  | Installation of STM or STNB welded mesh on walls and diaphragms weighing up to 3 kg/m² (VR-4 mesh 100x100, weight 1.8 kg/m²) |  |  | kg |  | 954.0000 |  |  |  |  |  |
| 82 |  | CN53A |  | Priming of interior wall surfaces (Rapid Hydro Defence Primer bituminous primer) |  |  | m² |  | 530.0000 |  |  |  |  |  |
| 83 |  | IzF10C |  | Thermal insulation layer for terraces, roofs and floors, comprising cellular polystyrene boards, jute fibre, PEL, porous, stabilised, etc., bonded with bituminous mastic to surfaces with a slope exceeding 40 per cent or vertical surfaces (XPS thermal insulation, 150 mm, density 30–45 kg/m³) |  |  | m² |  | 530.0000 |  |  |  |  |  |
| 84 |  | IzF04G |  | Hot-applied waterproofing layer for terraces, roofs, foundations and ground slabs, in areas without groundwater, including the troughs and valleys in standard waterproofing on surfaces with a gradient exceeding 40 per cent or on vertical, flat or curved surfaces, using bituminous felt, bonded over the entire surface with bituminous mastic or rubber-modified bitumen (HDPE membrane) |  |  | m² |  | 530.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total waterproofing and thermal insulation of the plinth Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. New walls |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Basement level – 3,300 |  |  |  |  |  |  |  |  |  |  |
| 85 |  | CD51C |  | Brickwork, 250 x 120 x 65, for reinforced partition walls with a thickness of half a brick, height up to 4 m |  |  | 100 m² |  | 2.5400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Basement level - 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Ground floor share 0.000 |  |  |  |  |  |  |  |  |  |  |
| 86 |  | CD51C |  | Brickwork, 250 x 120 x 65, for reinforced partition walls with a thickness of half a brick, height up to 4 m |  |  | 100 m² |  | 2.7000 |  |  |  |  |  |
| 87 |  | CK17F1 application |  | Wall panelling, fitted on site in quantities up to and including 10 m² using plastic profiles (PVC partition walls in toilet blocks, H=1.8 m) |  |  | m² |  | 41.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground Floor share 0.000 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. First floor share + 3,300 |  |  |  |  |  |  |  |  |  |  |
| 88 |  | CD51C |  | Brickwork, 250 x 120 x 65, for reinforced partition walls with a thickness of half a brick, height up to 4 m |  |  | 100 m² |  | 1.8600 |  |  |  |  |  |
| 89 |  | CK17F1 application |  | Wall panelling, fitted on site in quantities up to and including 10 m² using plastic profiles (PVC partition walls in toilet blocks, H=1.8 m) |  |  | m² |  | 29.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor Level + 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. Floor share + 6,600 |  |  |  |  |  |  |  |  |  |  |
| 90 |  | CD51C |  | Brickwork, 250 x 120 x 65, for reinforced partition walls with a thickness of half a brick, height up to 4 m |  |  | 100 m² |  | 1.8300 |  |  |  |  |  |
| 91 |  | CK17F1 application |  | Wall panelling, fitted on site in quantities up to and including 10 m² using plastic profiles (PVC partition walls in toilet blocks, H=1.8 m) |  |  | m² |  | 29.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor Level + 6,600 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total New walls Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4. Flooring |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.1. Share - 3,300 |  |  |  |  |  |  |  |  |  |  |
| 92 |  | TsC54C |  | Gravel base course, size 20–40, thickness 100 mm |  |  | m³ |  | 73.8000 |  |  |  |  |  |
| 93 |  | IzF04J1 |  | Hot-applied waterproofing layer for terraces, roofs or foundations and screeds, in areas without groundwater, including the troughs and valleys in the standard waterproofing: separation layer associated with waterproofing works, made of plain bituminous felt, laid with unsealed overlaps |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 94 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 95 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 96 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 1,328.4000 |  |  |  |  |  |
| 97 |  | IzF53A apl |  | Manual installation of floor screed with a thermal insulation layer comprising extruded polystyrene foam boards, 100 mm thick, in a single layer (XPS polystyrene foam, 100 mm thick) |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 98 |  | IzF04J1 |  | Hot-applied waterproofing layer for terraces, roofs or foundations and screeds, in areas without groundwater, including the troughs and valleys in the standard waterproofing: separation layer associated with waterproofing works, made of plain bituminous felt, laid with unsealed overlaps |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 99 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 100 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 101 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 1,328.4000 |  |  |  |  |  |
| 102 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 738.0000 |  |  |  |  |  |
| 103 |  | CG18A |  | Horizontal skirting boards with a maximum height of 15 cm on walls, made of ceramic tiles and fixed with M 100-T cement mortar, including cleaning and washing with water, in rooms with areas larger than 16 m² |  |  | m |  | 355.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Share - 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.2. Stairwell, basement |  |  |  |  |  |  |  |  |  |  |
| 104 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 8.0000 |  |  |  |  |  |
| 105 |  | CG51A |  | Cast epoxy floors. ‘FUXRAD’ ‘FX 20’ system, industrial coating in 2 layers, layer thickness 0.5–0.7 mm on existing substrate (decorative chips added – consumption 0.2 kg/m²) |  |  | m² |  | 8.0000 |  |  |  |  |  |
| 106 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 107 |  | CF52B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix on the ceiling, with mortar prepared by hand |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 108 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 109 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 110 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 111 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 6.0000 |  |  |  |  |  |
| 112 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 6.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for the stairwell and basement Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.3. Tactile strips |  |  |  |  |  |  |  |  |  |  |
| 113 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cool-touch plastic in contrasting yellow/black, dimensions 300x600x7 mm (55 pieces x 0.6 = 33 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 33.0000 |  |  |  |  |  |
| 114 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 300 x 7 mm (5 pieces x 0.3 = 1.5 m, consumption 3.33 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 1.5000 |  |  |  |  |  |
| 115 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 600 x 7 mm (17 pieces x 0.6 = 10.2 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 10.2000 |  |  |  |  |  |
| 116 |  | CK26A apl |  | Reflective tactile warning strip (stairs) (adhesive consumption 0.05 kg/m) |  |  | m |  | 6.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total tactile strips Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4. Ground floor share 0.450 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 117 |  | RpIzC04A |  | Hot-applied waterproofing layer for terraces, roofs, foundations and ground slabs, in areas without groundwater, including the flashings and valleys of the existing waterproofing, applied to horizontal or inclined surfaces up to 40 degrees, flat or curved, using bituminous mastic applied with a brush or rubber trowel (trowel) (Flexibat 10, consumption 1.4 kg/m²) |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 118 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 119 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 120 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 82.8000 |  |  |  |  |  |
| 121 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 122 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 46.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 123 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 252.0000 |  |  |  |  |  |
| 124 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 252.0000 |  |  |  |  |  |
| 125 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 453.6000 |  |  |  |  |  |
| 126 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 252.0000 |  |  |  |  |  |
| 127 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 252.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 128 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 410.0000 |  |  |  |  |  |
| 129 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 410.0000 |  |  |  |  |  |
| 130 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 738.0000 |  |  |  |  |  |
| 131 |  | CG56B1 |  | Screed made from ‘Nivelir’ self-levelling compound (12 mm thick, K=12) |  |  | m² |  | 410.0000 |  |  |  |  |  |
| 132 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical-grade linoleum) on floors (410 m² + 29 m² (skirting boards) = 439 m²; PVC profiles and floor thresholds are excluded) |  |  | m² |  | 439.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.4.4. Skirting boards |  |  |  |  |  |  |  |  |  |  |
| 133 |  | CG18A |  | Horizontal skirting boards with a maximum height of 15 cm on walls, made of ceramic tiles and fixed with M 100-T cement mortar, including cleaning and washing with water, in rooms with areas larger than 16 m² |  |  | m |  | 208.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Skirting boards Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground floor share 0.450 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.5. Stairwell, ground floor |  |  |  |  |  |  |  |  |  |  |
| 134 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 135 |  | CG51A |  | Cast epoxy floors. ‘FUXRAD’ ‘FX 20’ system, industrial coating in 2 layers, layer thickness 0.5–0.7 mm on existing substrate (decorative chips added – consumption 0.2 kg/m²) |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 136 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 137 |  | CF52B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix on the ceiling, with mortar prepared by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 138 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 139 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 140 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 141 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 142 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for the ground floor stairwell Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.6. Tactile strips |  |  |  |  |  |  |  |  |  |  |
| 143 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cool-touch plastic in contrasting yellow/black, dimensions 300x600x7 mm (120 pieces x 0.6 = 72 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 72.0000 |  |  |  |  |  |
| 144 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 300 x 7 mm (13 pieces x 0.3 = 3.9 m, consumption 3.33 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 3.9000 |  |  |  |  |  |
| 145 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 600 x 7 mm (37 pieces x 0.6 = 22.2 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 22.2000 |  |  |  |  |  |
| 146 |  | CK26A apl |  | Reflective tactile warning strip (stairs) (adhesive consumption 0.05 kg/m) |  |  | m |  | 11.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total tactile strips Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.7. Floor level + 3,300 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.7.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 147 |  | RpIzC04A |  | Hot-applied waterproofing layer for terraces, roofs, foundations and ground slabs, in areas without groundwater, including the flashings and valleys of the existing waterproofing, applied to horizontal or inclined surfaces up to 40 degrees, flat or curved, using bituminous mastic applied with a brush or rubber trowel (trowel) (Flexibat 10, consumption 1.4 kg/m²) |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 148 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 149 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 150 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 55.8000 |  |  |  |  |  |
| 151 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 152 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 31.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.7.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 153 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 154 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 155 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 352.8000 |  |  |  |  |  |
| 156 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 157 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 196.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.7.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 158 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 486.0000 |  |  |  |  |  |
| 159 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 486.0000 |  |  |  |  |  |
| 160 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 874.8000 |  |  |  |  |  |
| 161 |  | CG56B1 |  | Screed made from ‘Nivelir’ self-levelling compound (12 mm thick, K=12) |  |  | m² |  | 486.0000 |  |  |  |  |  |
| 162 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical-grade linoleum) on floors (486 m² + 30.2 m² (skirting boards) = 516.2 m²; PVC profiles and floor thresholds are excluded) |  |  | m² |  | 516.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.7.4. Skirting boards |  |  |  |  |  |  |  |  |  |  |
| 163 |  | CG18A |  | Horizontal skirting boards with a maximum height of 15 cm on walls, made of ceramic tiles and fixed with M 100-T cement mortar, including cleaning and washing with water, in rooms with areas larger than 16 m² |  |  | m |  | 165.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Skirting boards Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 3,300 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.8. Stairwell share + 3,300 |  |  |  |  |  |  |  |  |  |  |
| 164 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 165 |  | CG51A |  | Cast epoxy floors. ‘FUXRAD’ ‘FX 20’ system, industrial coating in 2 layers, layer thickness 0.5–0.7 mm on existing substrate (decorative chips added – consumption 0.2 kg/m²) |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 166 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 167 |  | CF52B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix on the ceiling, with mortar prepared by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 168 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 169 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 170 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 171 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 172 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Stairwell, level +3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.9. Tactile strips |  |  |  |  |  |  |  |  |  |  |
| 173 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cool-touch plastic in contrasting yellow/black, dimensions 300x600x7 mm (127 pieces x 0.6 = 76.2 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 76.2000 |  |  |  |  |  |
| 174 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 300 x 7 mm (18 pieces x 0.3 = 5.4 m, consumption 3.33 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 5.4000 |  |  |  |  |  |
| 175 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 600 x 7 mm (30 pieces x 0.6 = 18 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 18.0000 |  |  |  |  |  |
| 176 |  | CK26A apl |  | Reflective tactile warning strip (stairs) (adhesive consumption 0.05 kg/m) |  |  | m |  | 11.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total tactile strips Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.10. Floor level + 6,600 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.10.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 177 |  | RpIzC04A |  | Hot-applied waterproofing layer for terraces, roofs, foundations and ground slabs, in areas without groundwater, including the flashings and valleys of the existing waterproofing, applied to horizontal or inclined surfaces up to 40 degrees, flat or curved, using bituminous mastic applied with a brush or rubber trowel (trowel) (Flexibat 10, consumption 1.4 kg/m²) |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 178 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 179 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 180 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 55.8000 |  |  |  |  |  |
| 181 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 31.0000 |  |  |  |  |  |
| 182 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 31.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.10.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 183 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 184 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 185 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 352.8000 |  |  |  |  |  |
| 186 |  | CN53A |  | Priming of interior floor surfaces |  |  | m² |  | 196.0000 |  |  |  |  |  |
| 187 |  | CG17D |  | Ceramic tile flooring, including the adhesive mortar backing layer, laid on areas: larger than 16 m² |  |  | m² |  | 196.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.10.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 188 |  | CG01A |  | Floor screed made from 3 cm thick M 150-T cement mortar with a finely trowelled finish (60 mm) |  |  | m² |  | 488.0000 |  |  |  |  |  |
| 189 |  | CG01A1 |  | Floor screed made of M 100-T cement mortar, 3 cm thick, with a finely trowelled finish. For every 0.5 cm increase or decrease in the thickness of the M 100-T mortar screed, add 30 mm, K=6 |  |  | m² |  | 488.0000 |  |  |  |  |  |
| 190 |  | CC03C |  | Installation of welded mesh at heights of 35 m or less, on slabs (mesh 4Bp 100x100, weight 1.8 kg/m²) |  |  | kg |  | 878.4000 |  |  |  |  |  |
| 191 |  | CG56B1 |  | Screed made from ‘Nivelir’ self-levelling compound (12 mm thick, K=12) |  |  | m² |  | 488.0000 |  |  |  |  |  |
| 192 |  | CG49A |  | Installation of homogeneous vinyl antibacterial flooring (medical-grade linoleum) on floors (488 m² + 29.3 m² (skirting boards) = 517.3 m²; PVC profiles and floor thresholds are excluded) |  |  | m² |  | 517.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.10.4. Skirting boards |  |  |  |  |  |  |  |  |  |  |
| 193 |  | CG18A |  | Horizontal skirting boards with a maximum height of 15 cm on walls, made of ceramic tiles and fixed with M 100-T cement mortar, including cleaning and washing with water, in rooms with areas larger than 16 m² |  |  | m |  | 164.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Skirting boards Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 6,600 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.11. Stairwell share + 6,600 |  |  |  |  |  |  |  |  |  |  |
| 194 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 195 |  | CG51A |  | Cast epoxy floors. ‘FUXRAD’ ‘FX 20’ system, industrial coating in 2 layers, layer thickness 0.5–0.7 mm on existing substrate (decorative chips added – consumption 0.2 kg/m²) |  |  | m² |  | 16.0000 |  |  |  |  |  |
| 196 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 197 |  | CF52B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix on the ceiling, with mortar prepared by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 198 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 199 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 200 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 201 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 12.0000 |  |  |  |  |  |
| 202 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 12.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Stairwell, level +6,600 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.12. Tactile strips |  |  |  |  |  |  |  |  |  |  |
| 203 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cool-touch plastic in contrasting yellow/black, dimensions 300x600x7 mm (127 pieces x 0.6 = 76.2 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 76.2000 |  |  |  |  |  |
| 204 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 300 x 7 mm (18 pieces x 0.3 = 5.4 m, consumption 3.33 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 5.4000 |  |  |  |  |  |
| 205 |  | CK26A apl |  | Tactile guidance strip for the visually impaired, made of cold-touch plastic in contrasting yellow/black, dimensions 300 x 600 x 7 mm (30 pieces x 0.6 = 18 m, consumption 1.67 pieces/m, adhesive consumption 0.05 kg/piece) |  |  | m |  | 18.0000 |  |  |  |  |  |
| 206 |  | CK26A apl |  | Reflective tactile warning strip (stairs) (adhesive consumption 0.05 kg/m) |  |  | m |  | 11.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total tactile strips Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Flooring Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5. Ceilings |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.1. Basement level – 3,300 |  |  |  |  |  |  |  |  |  |  |
| 207 |  | IzF55C app |  | Thermal insulation of building ceilings with fine plaster over thermal insulation (systems with rigid fixing of the thermal insulation), smooth ceiling surfaces: with mineral wool boards (200 mm thick, 135 kg/m³, (100 mm thick boards in 2 layers, K=2)) |  |  | m² |  | 738.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Basement level - 3.300 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.2. Ground floor share 0.450 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.2.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 208 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 209 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 210 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 211 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 212 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 46.0000 |  |  |  |  |  |
| 213 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 46.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.2.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 214 |  | CK29F |  | Suspended ceilings made of ‘Armstrong’ prefabricated panels, including the grid system (mineral wool panels treated with microbiological and hygienic properties (600 × 600 × 15 mm), class 1/C/0N in accordance with EN 13964) |  |  | m² |  | 411.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.2.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 215 |  | CK52A |  | Suspended ceilings made of ‘Griliato’ slatted panels on a metal frame, using rodless bracket suspension, in rooms with a height of up to 4 m (suspended slatted ceiling) |  |  | m² |  | 108.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.2.4. Type 4 |  |  |  |  |  |  |  |  |  |  |
| 216 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 147.0000 |  |  |  |  |  |
| 217 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 147.0000 |  |  |  |  |  |
| 218 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 147.0000 |  |  |  |  |  |
| 219 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 147.0000 |  |  |  |  |  |
| 220 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 147.0000 |  |  |  |  |  |
| 221 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 147.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground Floor share 0.450 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.3. Floor level + 3,300 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.3.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 222 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 32.0000 |  |  |  |  |  |
| 223 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 32.0000 |  |  |  |  |  |
| 224 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 32.0000 |  |  |  |  |  |
| 225 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 32.0000 |  |  |  |  |  |
| 226 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 32.0000 |  |  |  |  |  |
| 227 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 32.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.3.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 228 |  | CK29F |  | Suspended ceilings made of ‘Armstrong’ prefabricated panels, including the grid system (mineral wool panels treated with microbiological and hygienic protection (600 × 600 × 15 mm), class 1/C/0N in accordance with EN 13964) |  |  | m² |  | 486.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.3.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 229 |  | CK52A |  | Suspended ceilings made of ‘Griliato’ slatted panels on a metal frame, using rodless bracket suspension, in rooms with a height of up to 4 m (suspended slatted ceiling) |  |  | m² |  | 101.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.3.4. Type 4 |  |  |  |  |  |  |  |  |  |  |
| 230 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 231 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 232 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 233 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 234 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 235 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 98.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.4. Floor share + 6,600 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.4.1. Type 1 |  |  |  |  |  |  |  |  |  |  |
| 236 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 237 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 238 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 239 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 240 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 33.0000 |  |  |  |  |  |
| 241 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 33.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.4.2. Type 2 |  |  |  |  |  |  |  |  |  |  |
| 242 |  | CK29F |  | Suspended ceilings made of ‘Armstrong’ prefabricated panels, including the grid system (mineral wool panels treated with microbiological and hygienic properties (600 × 600 × 15 mm), class 1/C/0N in accordance with EN 13964) |  |  | m² |  | 487.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.4.3. Type 3 |  |  |  |  |  |  |  |  |  |  |
| 243 |  | CK52A |  | Suspended ceilings made of ‘Griliato’ slatted panels on a metal frame, using rodless bracket suspension, in rooms with a height of up to 4 m (suspended slatted ceiling) |  |  | m² |  | 101.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 3 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.4.4. Type 4 |  |  |  |  |  |  |  |  |  |  |
| 244 |  | СF59D |  | Cladding of surfaces with a layer of PGC, including the installation of a simple flat metal framework, up to 4 m in height: ceilings without insulation (waterproof plasterboard) |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 245 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 246 |  | CF57A K=2 |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 2.0 mm thick, to the surfaces of walls, columns and ceilings (K=2) |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 247 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 248 |  | CN53A |  | Priming of interior ceiling surfaces with primer |  |  | m² |  | 98.0000 |  |  |  |  |  |
| 249 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 98.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Type 4 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 6,600 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ceilings Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6. Wall finishes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.1. Basement level - 3,000 |  |  |  |  |  |  |  |  |  |  |
| 250 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 251 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 252 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 253 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 254 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 255 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 256 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 257 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 258 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 259 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
| 260 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 1,055.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Basement level - 3,000 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.2. Ground floor share 0.000 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.2.1. Rooms 6, 9, 20, 21 |  |  |  |  |  |  |  |  |  |  |
| 261 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 387.0000 |  |  |  |  |  |
| 262 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 387.0000 |  |  |  |  |  |
| 263 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 387.0000 |  |  |  |  |  |
| 264 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 387.0000 |  |  |  |  |  |
| 265 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 266 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 267 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 268 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 269 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 270 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 271 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 230.2000 |  |  |  |  |  |
| 272 |  | CK17F |  | Wall panelling, fitted on site in quantities exceeding 10 m² using 10 mm HPL panels, supplied as a set with the fixing system |  |  | m² |  | 156.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 6, 9, 20, 21 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.2.2. Rooms 4, 5, 18, 19 |  |  |  |  |  |  |  |  |  |  |
| 273 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 118.7000 |  |  |  |  |  |
| 274 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 118.7000 |  |  |  |  |  |
| 275 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 118.7000 |  |  |  |  |  |
| 276 |  | IzF31A |  | Waterproofing of concrete surfaces (vertical, horizontal, including ceilings) using ‘Penetron’ mixture – 2 coats: smooth surface |  |  | m² |  | 118.7000 |  |  |  |  |  |
| 277 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 118.7000 |  |  |  |  |  |
| 278 |  | CI21B |  | Wall cladding with ceramic-granite tiles: sizes up to 600 x 600 mm (600 x 600 mm ceramic tiles laid over the full height) |  |  | m² |  | 118.7000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 4, 5, 18, 19 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.2.3. Rooms 1, 2, 3, 7, 8, 10, 11 12, 13, 14, 15, 16, 17, 22, 23 |  |  |  |  |  |  |  |  |  |  |
| 279 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 280 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 281 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 282 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 283 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 284 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 285 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 286 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 287 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 288 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 854.5000 |  |  |  |  |  |
| 289 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 854.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 1, 2, 3, 7, 8, 10, 11 12, 13, 14, 15, 16, 17, 22, 23 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ground floor share 0.000 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.3. First floor share + 3,300 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.3.1. Rooms 11, 12, 18 |  |  |  |  |  |  |  |  |  |  |
| 290 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 338.0000 |  |  |  |  |  |
| 291 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 338.0000 |  |  |  |  |  |
| 292 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 338.0000 |  |  |  |  |  |
| 293 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 338.0000 |  |  |  |  |  |
| 294 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 295 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 296 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 297 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 298 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 299 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 300 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 198.8000 |  |  |  |  |  |
| 301 |  | CK17F |  | Wall panelling, fitted on site in quantities exceeding 10 m² using 10 mm HPL panels, supplied as a set with the fixing system |  |  | m² |  | 139.2000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 11, 12, 18 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.3.2. Rooms 9, 10, 17 |  |  |  |  |  |  |  |  |  |  |
| 302 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 303 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 304 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 305 |  | IzF31A |  | Waterproofing of concrete surfaces (vertical, horizontal, including ceilings) using ‘Penetron’ mixture – 2 coats: smooth surface |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 306 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 307 |  | CI21B |  | Wall cladding with ceramic-granite tiles: sizes up to 600 x 600 mm (600 x 600 mm ceramic tiles laid over the full height) |  |  | m² |  | 96.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 9, 10, 17 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.3.3. Rooms 1, 2, 3, 4, 5, 6, 7, 8, 13, 14, 15, 16, 19, 20 |  |  |  |  |  |  |  |  |  |  |
| 308 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 309 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 310 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 311 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 312 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 313 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 314 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 315 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 316 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 317 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 880.9000 |  |  |  |  |  |
| 318 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 880.9000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Rooms 1, 2, 3, 4, 5, 6, 7, 8, 13, 14, 15, 16, 19, 20 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 3,300 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.4. Floor share + 6,600 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.4.1. Rooms 10, 11, 13 |  |  |  |  |  |  |  |  |  |  |
| 319 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 339.6000 |  |  |  |  |  |
| 320 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 339.6000 |  |  |  |  |  |
| 321 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 339.6000 |  |  |  |  |  |
| 322 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 339.6000 |  |  |  |  |  |
| 323 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 324 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 325 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 326 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 327 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 328 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 329 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 199.5000 |  |  |  |  |  |
| 330 |  | CK17F |  | Wall panelling, fitted on site in quantities exceeding 10 m² using 10 mm HPL panels, supplied as a set with the fixing system |  |  | m² |  | 140.1000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 10, 11, 13 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.4.2. Rooms 8, 9, 14 |  |  |  |  |  |  |  |  |  |  |
| 331 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 332 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 333 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 334 |  | IzF31A |  | Waterproofing of concrete surfaces (vertical, horizontal, including ceilings) using ‘Penetron’ mixture – 2 coats: smooth surface |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 335 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 96.0000 |  |  |  |  |  |
| 336 |  | CI21B |  | Wall cladding with ceramic-granite tiles: sizes up to 600 x 600 mm (600 x 600 mm ceramic tiles laid over the full height) |  |  | m² |  | 96.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 8, 9, 14 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.4.3. Rooms 1, 2, 3, 4, 5, 6, 7, 15, 16, 17, 18, 19 |  |  |  |  |  |  |  |  |  |  |
| 337 |  | CN53A |  | Priming of interior wall surfaces with Betonocontact |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 338 |  | RpCJ10A |  | Repairs to interior plasterwork, 3 cm thick, carried out on wire mesh, rough-cast, on walls, ceilings and skirting boards, using 100-T grade cement-lime mortar for the rough coat, 50-T grade cement-lime mortar for the base coat and 10-T grade lime-cement mortar for the finish coat, applied to walls (Note: The galvanised trapezoidal mesh 16.0 x 0.6 x 0.5 x 1500 S 2542 is to be replaced with 4 BpI 150x150 mesh, with a standard weight of 1.5 kg/m²) (35 mm) |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 339 |  | RpCJ27B |  | Miscellaneous works – thickness variation, +/-- 5 mm in the primer coat applied with mortar |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 340 |  | CN54B |  | Manual application of a single coat of deep-penetrating primer to walls |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 341 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a dry gypsum-based mix, on walls and partition walls; mortar prepared by hand. |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 342 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 343 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 344 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 345 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 346 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 893.5000 |  |  |  |  |  |
| 347 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand (washable paint with enhanced durability) |  |  | m² |  | 893.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Rooms 1, 2, 3, 4, 5, 6, 7, 15, 16, 17, 18, 19 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Floor share + 6,600 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Wall finishing Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7. Windows |  |  |  |  |  |  |  |  |  |  |
| 348 |  | CK19B |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of between 3.00 and 6.00 m² inclusive (Fs-1 windows with thermal break, 75 mm profile, including insect screens on windows with sashes) |  |  | m² |  | 6.1200 |  |  |  |  |  |
| 349 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (Fs-2 windows with thermal break, 75 mm profile, including insect screen on windows with sashes) |  |  | m² |  | 5.9400 |  |  |  |  |  |
| 350 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (F-1 windows with thermal break, 75 mm profile, including insect screen on windows with sashes) |  |  | m² |  | 9.6000 |  |  |  |  |  |
| 351 |  | CK19B |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area between 3.00 and 6.00 m² inclusive (F-2 windows with thermal break, 75 mm profile, including insect screen on sash windows) |  |  | m² |  | 394.8000 |  |  |  |  |  |
| 352 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (F-3 windows with thermal break, 75 mm profile, including insect screen on windows with sashes) |  |  | m² |  | 17.6400 |  |  |  |  |  |
| 353 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (F-4 windows with thermal break, 75 mm profile, including insect screen on windows with sashes) |  |  | m² |  | 15.1200 |  |  |  |  |  |
| 354 |  | CK19A |  | Aluminium windows with one or more sashes for buildings up to and including 35 m in height, with a frame area of up to and including 3.00 m² (F-5 windows with thermal break, 75 mm profile, including insect screens on windows with sashes) |  |  | m² |  | 6.7200 |  |  |  |  |  |
| 355 |  | CM07C |  | Skylight – wall made of pressed glass blocks with voids of 190 x 190 x 80 mm (EI30 glass blocks, F-6 window) |  |  | m² |  | 8.4000 |  |  |  |  |  |
| 356 |  | RpCJ06C k=2 |  | Repairs to interior plasterwork around door and window frames and sills, 2 cm thick, with a rough finish, carried out using 25 T grade cement-lime mortar, with straight edges, between 25 and 35 cm wide (k=2 for a thickness of 4 cm) |  |  | m |  | 712.1000 |  |  |  |  |  |
| 357 |  | CK18C |  | Fitting of aluminium corner beads |  |  | m |  | 712.1000 |  |  |  |  |  |
| 358 |  | CF61A |  | Continuous skimming of the surface (single-coat plaster) with dry plaster mix: flat window sills and door frames |  |  | m² |  | 99.2000 |  |  |  |  |  |
| 359 |  | CF61A |  | Continuous skimming of the surface (single-coat plaster) with dry plaster mix: flat window sills and door frames |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 360 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 361 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 362 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 363 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 364 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 365 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 142.4200 |  |  |  |  |  |
| 366 |  | CK26D |  | Window or door sills fitted to aluminium frames (drip mouldings) |  |  | m |  | 247.4500 |  |  |  |  |  |
| 367 |  | CK26C |  | Sills fitted to plastic windows or doors |  |  | m |  | 247.4500 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Windows Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 8. Doors |  |  |  |  |  |  |  |  |  |  |
| 368 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m high, double-leaf, with a frame area of up to and including 7 m² (Ui-1 – 1 unit, Ui-2 – 1 unit – Opaque aluminium entrance door with Up thermal break, 1.4 W/(m²K) |  |  | m² |  | 5.0400 |  |  |  |  |  |
| 369 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m high, double-leaf, with a frame area of up to and including 7 m² (Ui-3 – 2 units - Aluminium entrance door with glazing and Up thermal break, 1.4 W/(m²K) |  |  | m² |  | 9.3600 |  |  |  |  |  |
| 370 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m high, double-leaf, with a frame area of up to and including 7 m² (Ui-4 – 2 units – Aluminium double-leaf interior door with glazing, fire resistance 30 minutes) |  |  | m² |  | 9.3600 |  |  |  |  |  |
| 371 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m high, double-leaf, with a frame area of up to and including 7 m² (U-1 – 11 units – Aluminium double-leaf interior door with glazing) |  |  | m² |  | 32.3400 |  |  |  |  |  |
| 372 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m high, double-leaf, with a frame area of up to and including 7 m² (U-2 – 5 units – Double-leaf aluminium interior door with glazing) |  |  | m² |  | 12.6000 |  |  |  |  |  |
| 373 |  | CK22C |  | Aluminium-framed glazing units for buildings up to 35 m in height, comprising fixed panels and door leaves (aluminium glazing with built-in door and rectangular stainless steel pull handles) |  |  | m² |  | 5.6500 |  |  |  |  |  |
| 374 |  | CK21A |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors fitted into masonry of any kind, for buildings up to and including 35 m in height, single-leaf, with a frame area up to and including 7 m² (U-3 – 6 units – opaque aluminium interior right-hand door) |  |  | m² |  | 11.3400 |  |  |  |  |  |
| 375 |  | CK21A |  | Doors made from aluminium profiles, including the fittings and accessories required for doors installed in masonry of any kind, in buildings up to and including 35 m in height, single-leaf, with a door frame area up to and including 7 m² (U-4 – 9 units – Right-hand opaque aluminium interior door with vertical glazing) |  |  | m² |  | 17.0100 |  |  |  |  |  |
| 376 |  | CK21A |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings up to and including 35 m in height, single-leaf, with a frame area up to and including 7 m² (U-4 – 1 unit – Right-hand aluminium interior door, opaque, with vertical glazing, fire resistance 30 minutes) |  |  | m² |  | 1.8900 |  |  |  |  |  |
| 377 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m in height, double-leaf, with a frame area of up to and including 7 m² (U-5 – 8 units, aluminium double-glazed interior door) |  |  | m² |  | 20.1600 |  |  |  |  |  |
| 378 |  | CK21D1 |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings over 35 m in height, double-leaf, with a frame area up to and including 7 m² (U-6 – 12 units – 180°-opening aluminium door with adjacent glazing) |  |  | m² |  | 30.2400 |  |  |  |  |  |
| 379 |  | CK21A |  | Doors made from aluminium profiles, including the necessary fittings and accessories for doors installed in masonry of any kind, in buildings up to and including 35 m in height, single-leaf, with a frame area up to and including 7 m² (U-7 – 30 units – opaque aluminium interior right-hand door) |  |  | m² |  | 44.1000 |  |  |  |  |  |
| 380 |  | CK22C |  | Aluminium-framed glazing units for buildings up to 35 m in height, comprising fixed panels and door leaves (aluminium glazing with built-in door and rectangular stainless steel pull handles) |  |  | m² |  | 8.1200 |  |  |  |  |  |
| 381 |  | RpCJ06C k=2 |  | Repairs to interior plasterwork around door and window frames and sills, 2 cm thick, with a rough finish, carried out using 25 T grade cement-lime mortar, with straight edges, between 25 and 35 cm wide (k=2 for a thickness of 4 cm) |  |  | m |  | 480.1500 |  |  |  |  |  |
| 382 |  | CK18C |  | Fitting of aluminium corner beads |  |  | m |  | 480.1500 |  |  |  |  |  |
| 383 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 384 |  | CF61A |  | Continuous skimming of the surface (single-coat plaster) with dry plaster mix: flat window sills and door frames |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 385 |  | CF17C |  | Miscellaneous works – layer of fibreglass mesh applied to the surface of precast aerated concrete elements bonded with adhesive, including the primer coat |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 386 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 387 |  | CF57A |  | Manual application of ‘Eurofin’ gypsum-based skim coat, 1.0 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 388 |  | CF56A |  | Manual application of ‘Sheetrock’ interior plaster, 0.5 mm thick, to the surfaces of walls, columns and ceilings |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 389 |  | CN53A |  | Priming of interior wall surfaces |  |  | m² |  | 96.0300 |  |  |  |  |  |
| 390 |  | CN06A |  | Interior painting with water-based vinyl copolymer emulsion paint, applied in two coats to existing skim coat, carried out by hand |  |  | m² |  | 96.0300 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Doors Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9. Roof |  |  |  |  |  |  |  |  |  |  |
| 391 |  | IzF04J1 |  | Hot-applied waterproofing layer for terraces, roofs or foundations and screeds, in areas without groundwater, including the troughs and valleys in the standard waterproofing: separation layer associated with waterproofing works, made of plain bituminous felt, laid with unsealed overlaps |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 392 |  | CA02C |  | Plain concrete cast for levelling, slopes and screeds at heights up to and including 35 m, prepared at a concrete batching plant in accordance with Article CA01 or as ready-mixed concrete, cast using conventional methods (concrete for a gradient of 50–100 mm) |  |  | m³ |  | 71.2300 |  |  |  |  |  |
| 393 |  | IzF10A |  | Thermal insulation layer for terraces, roofs and floor slabs, comprising thermal insulation boards made of autoclaved aerated concrete (AAC), perlite, fibre-reinforced perlite, isobeton, etc. (mineral wool, 250 mm thick, K=2.5, labour included) |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 394 |  | IzF04J1 |  | Hot-applied waterproofing layer for terraces, roofs or foundations and screeds, in areas without groundwater, including the troughs and valleys in the standard waterproofing: separation layer associated with waterproofing works, made of plain bituminous felt, laid with unsealed overlaps |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 395 |  | IzF18C |  | Levelling or protective base course for insulation, including associated screeds, made from ready-mixed M100-T cement mortar without lime additive, trowelled, on horizontal or sloping surfaces up to and including 40 per cent, applied to an average thickness of 3 cm |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 396 |  | IzF18B |  | Levelling or protective base coat for insulation, including associated battens, made from ready-mixed M100-T cement mortar without lime, trowelled, on horizontal or sloping surfaces up to and including 40 per cent, applied to an average thickness of 2 cm |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 397 |  | CE13B |  | Roofing membranes made of modified bituminous membranes, mechanically fixed in a single-layer system on a horizontal surface mounted on a continuous substrate / application, 1.5 mm thick PVC membrane / excludes ‘Accessories for mechanical fastenings’, ‘Butane gas (cylinders)’, and new material consumption ‘PVC membrane fixing elements’ 2.5 pieces/m² (765+58 m² of roof area) × 0.12% overlap = 921.76 m²) |  |  | m² |  | 949.7600 |  |  |  |  |  |
| 398 |  | CE23C |  | Window sills and canopies made of 0.5 mm thick galvanised sheet metal on a layer of bituminous cardboard, mounted on a levelling screed of M 100-T cement-lime mortar, fixed to brickwork; for lengths greater than 2 m, with a developed width between 31 and 50 cm (Paint-coated steel sheet for parapet, thickness 0.8 mm, width 450 mm) |  |  | m |  | 152.0000 |  |  |  |  |  |
| 399 |  | CE22A |  | Brass-type downpipe systems made from corrosion-resistant sheet metal, D160 mm |  |  | m |  | 24.0000 |  |  |  |  |  |
| 400 |  | SB26A |  | Drain outlet (receiver) made of enamelled cast iron, for draining water from terraces and roofs, with a diameter of 100–150 mm with a leaf guard |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 401 |  | CK18C apl |  | Fitting of strips to aluminium cladding (aluminium pressure rail for securing the PVC membrane) |  |  | m |  | 150.0000 |  |  |  |  |  |
| 402 |  | SB26A apl |  | Vent for PVC membrane |  |  | piece |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Roofing Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10. Ventilation ducts (insulation) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10.1. CV-1 – 6 units |  |  |  |  |  |  |  |  |  |  |
| 403 |  | CL18A |  | Various metal fabrications made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete (installation of the casing for ventilation ducts) |  |  | kg |  | 285.3600 |  |  |  |  |  |
| 404 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from sections with a thickness of 8–12 mm inclusive |  |  | t |  | 0.2850 |  |  |  |  |  |
| 405 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.2850 |  |  |  |  |  |
| 406 |  | IzD04A |  | Painting of metal fabrications and structures with oil-based paint in 2 coats, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.2850 |  |  |  |  |  |
| 407 |  | СF59A apl |  | Cladding of surfaces with a layer of 12.5 mm thick magnesium-reinforced fibreglass board, with the construction of a simple flat metal frame, up to 4 m in height: insulated walls (100 mm thick mineral wool) |  |  | m² |  | 24.0000 |  |  |  |  |  |
| 408 |  | CE05A1 |  | Roofing made of flat galvanised sheet metal or flat sheet metal with anti-corrosion protection, fixed with clips, with double seams in both directions, installed on areas of 40 m² or less using 1 mm thick sheet metal, including the installation of valleys, flashings, connections to chimneys, etc. ( Sheet metal, 1 mm thick, weight 7.07 kg/m² ) |  |  | m² |  | 8.6400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total CV-1 – 6 items Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10.2. CV-2 – 2 items |  |  |  |  |  |  |  |  |  |  |
| 409 |  | CL18A |  | Various metal fabrications made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete (installation of the casing for ventilation ducts) |  |  | kg |  | 121.7400 |  |  |  |  |  |
| 410 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from sections with a thickness of 8–12 mm inclusive |  |  | t |  | 0.1220 |  |  |  |  |  |
| 411 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.1220 |  |  |  |  |  |
| 412 |  | IzD04A |  | Painting of metal fabrications and structures with oil-based paint in 2 coats, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.1220 |  |  |  |  |  |
| 413 |  | СF59A apl |  | Cladding of surfaces with a layer of 12.5 mm thick magnesium-reinforced fibreglass board, with the construction of a simple flat metal frame, up to 4 m in height: insulated walls (100 mm thick mineral wool) |  |  | m² |  | 11.7600 |  |  |  |  |  |
| 414 |  | CE05A1 |  | Roofing made of flat galvanised sheet metal or flat sheet metal with anti-corrosion protection, fixed with clips, with double seams in both directions, installed on areas of 40 m² or less using 1 mm thick sheet metal, including the installation of valleys, flashings, connections to chimneys, etc. ( Sheet metal, 1 mm thick, weight 7.07 kg/m² ) |  |  | m² |  | 4.3800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total CV-2 – 2 pieces Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10.3. CV-3 – 1 item |  |  |  |  |  |  |  |  |  |  |
| 415 |  | CL18A |  | Various metal fabrications made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete (installation of the casing for ventilation ducts) |  |  | kg |  | 55.3500 |  |  |  |  |  |
| 416 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from sections with a thickness of 8–12 mm inclusive |  |  | t |  | 0.0550 |  |  |  |  |  |
| 417 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.0550 |  |  |  |  |  |
| 418 |  | IzD04A |  | Painting of metal fabrications and structures with oil-based paint in 2 coats, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.0550 |  |  |  |  |  |
| 419 |  | СF59A apl |  | Cladding of surfaces with a layer of 12.5 mm thick magnesium-reinforced fibreglass board, with the construction of a simple flat metal frame, up to 4 m in height: insulated walls (100 mm thick mineral wool) |  |  | m² |  | 5.1000 |  |  |  |  |  |
| 420 |  | CE05A1 |  | Roofing made of flat galvanised sheet metal or flat sheet metal with anti-corrosion protection, fixed with clips, with double seams in both directions, installed on areas of 40 m² or less using 1 mm thick sheet metal, including the installation of valleys, flashings, connections to chimneys, etc. ( Sheet metal, 1 mm thick, weight 7.07 kg/m² ) |  |  | m² |  | 2.2600 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total CV-3 – 1 item Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10.4. CV-4 – 1 copy |  |  |  |  |  |  |  |  |  |  |
| 421 |  | CL18A |  | Various metal fabrications made from rolled sections, sheet metal, corrugated sheet metal, reinforcing steel, and pipes for supports or cladding, fully or partially embedded in concrete (installation of the casing for ventilation ducts) |  |  | kg |  | 60.3000 |  |  |  |  |  |
| 422 |  | RpIzE16B |  | Cleaning of metal structures and fabrications to remove rust and old paint using a wire brush, made from sections with a thickness of 8–12 mm inclusive |  |  | t |  | 0.0600 |  |  |  |  |  |
| 423 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a mineral oil brush |  |  | t |  | 0.0600 |  |  |  |  |  |
| 424 |  | IzD04A |  | Painting of metal fabrications and structures with oil-based paint in 2 coats, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.0600 |  |  |  |  |  |
| 425 |  | СF59A apl |  | Cladding of surfaces with a layer of 12.5 mm thick magnesium-reinforced fibreglass board, with the construction of a simple flat metal frame, up to 4 m in height: insulated walls (100 mm thick mineral wool) |  |  | m² |  | 5.8000 |  |  |  |  |  |
| 426 |  | CE05A1 |  | Roofing made of flat galvanised sheet metal or flat sheet metal with anti-corrosion protection, fixed with clips, with double seams in both directions, installed on areas of 40 m² or less using 1 mm thick sheet metal, including the installation of valleys, flashings, connections to chimneys, etc. ( Sheet metal, 1 mm thick, weight 7.07 kg/m² ) |  |  | m² |  | 2.6400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total CV-4 – 1 item Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Ventilation ducts (insulation) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 11. Façade |  |  |  |  |  |  |  |  |  |  |
| 427 |  | CN54B |  | Manual application of a single coat of ‘Betonocontact’ quartz-based primer to external walls on the façade |  |  | m² |  | 1,586.0800 |  |  |  |  |  |
| 428 |  | CL55A application |  | Installation of a ventilated façade system using 600 x 600 mm fibre-cement panels with visible fixings, with architectural detailing covering up to 30 per cent of the total wall area (insulation boards – mineral wool, 100 mm thick, off-white) |  |  | m² |  | 89.5000 |  |  |  |  |  |
| 429 |  | IzF55C |  | External thermal insulation of building walls with fine render applied over the insulation (systems with rigid fixing of the insulation), smooth wall surfaces: using mineral wool boards (50 mm thick, p=135 kg/m³, thermal conductivity of 0.037 W/m²K) |  |  | m² |  | 1,141.8400 |  |  |  |  |  |
| 430 |  | IzF55C |  | External thermal insulation of building walls with fine render applied over the insulation (systems with rigid fixing of the insulation), smooth wall surfaces: using mineral wool boards (80 mm thick, p=135 kg/m³, thermal conductivity 0.037 W/m²K, batts) |  |  | m² |  | 149.1000 |  |  |  |  |  |
| 431 |  | CF50B |  | 5 mm thick interior plastering, applied by hand using a cement-based adhesive, on walls and partition walls; mortar prepared by hand. (Ideal Fasad type adhesive) |  |  | m² |  | 1,290.9400 |  |  |  |  |  |
| 432 |  | CN54B |  | Manual application of a single coat of ‘Gleta’ quartz-based primer to external walls on façades |  |  | m² |  | 1,290.9400 |  |  |  |  |  |
| 433 |  | CF30A |  | Exterior plastering, 2–3 mm thick, applied by hand using ‘TINC’ mix on walls (TINC consumption: 2.8 kg/m²) |  |  | m² |  | 1,290.9400 |  |  |  |  |  |
| 434 |  | IzF55B |  | External thermal insulation of building walls with fine render over thermal insulation (systems with rigid fixing of the thermal insulation), smooth wall surface: using polystyrene boards (XPS polystyrene, 150 mm thick) |  |  | m² |  | 205.6400 |  |  |  |  |  |
| 435 |  | CI21A |  | Wall cladding with ceramic-granite tiles: dimensions up to 400 x 400 mm |  |  | m² |  | 205.6400 |  |  |  |  |  |
| 436 |  | CB14A |  | Tubular metal scaffolding for work on vertical surfaces at heights up to and including 30 m, with the scaffolding secured in place for 25 days (200 hours) |  |  | m² |  | 1,586.0800 |  |  |  |  |  |
| 437 |  | CB14A1 k=30 |  | Tubular metal scaffolding for work on vertical surfaces at heights up to and including 30 m, with the scaffolding in place for 25 days (200 hours); for piece additional or fewer day(s) of scaffolding in place, mobilisation for 30 days is added |  |  | m² |  | 1,586.0800 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Facade Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12. Pits and canopies |  |  |  |  |  |  |  |  |  |  |
| 438 |  | TsC59B |  | Mechanical earthworks using an excavator with a 0.15 m³ bucket, with material tipped into a tipper lorry: Category 2 ground (for new pits) |  |  | 100 m³ |  | 0.2000 |  |  |  |  |  |
| 439 |  | TsA20B |  | Manual excavation of soil, on slopes, in trenches dug by excavator or scraper, to complete the excavation to the slope profile, in medium-grade soil |  |  | m³ |  | 2.3000 |  |  |  |  |  |
| 440 |  | TsI51A10 |  | Transport of earth by 10-tonne tipper lorry over a distance of 10 km |  |  | t |  | 33.0000 |  |  |  |  |  |
| 441 |  | TsA03E |  | Manual excavation of soil in confined spaces, less than 1.00 m wide, carried out without shoring, with a sloping face at foundations, channels, etc., in soil of medium cohesion or highly cohesive, to a depth of up to 1.5 m medium soil (for drainage pipe) |  |  | m³ |  | 6.0000 |  |  |  |  |  |
| 442 |  | TsD01B |  | Spreading of screened soil by shovel, in uniform layers 10–30 cm thick, by throwing it up to 3 m from the heaps, including the breaking up of clods; soil originating from medium-grade land |  |  | m³ |  | 12.9600 |  |  |  |  |  |
| 443 |  | TsD05A |  | Compaction using a 150–200 kg mechanical tamper of the fill in successive layers 20–30 cm thick, excluding the wetting of piece individual layer; the fill is made from non-cohesive soil |  |  | 100 m³ |  | 0.1300 |  |  |  |  |  |
| 444 |  | IzF04J1 |  | Hot-applied waterproofing layer on terraces, roofs or foundations and screeds, in areas without groundwater, including the troughs and depressions in the standard waterproofing: separation layer associated with waterproofing works, made of plain bituminous felt, laid with unsealed overlaps (geotextile) |  |  | m² |  | 35.0000 |  |  |  |  |  |
| 445 |  | TsC54C |  | Gravel base course, 40–70 mm |  |  | m³ |  | 4.0000 |  |  |  |  |  |
| 446 |  | AcA52D apl |  | Polyethylene pipe, for water supply pipes laid in a trench, with a diameter of 160 mm (drainage pipe) |  |  | m |  | 15.0000 |  |  |  |  |  |
| 447 |  | RpCB04D |  | Reinforced concrete cast in columns, ribs, slabs, landings, staircases, retaining walls, reinforced concrete bearings, floor slabs, diaphragms, stair ramps, etc., for existing buildings, prepared using a concrete mixer on site and cast using conventional methods with ready-mixed concrete supplied from batching plants (C20/25 concrete) |  |  | m³ |  | 3.0000 |  |  |  |  |  |
| 448 |  | RpCC02A |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in walls and slabs, with 15 mm plywood |  |  | m² |  | 34.4000 |  |  |  |  |  |
| 449 |  | RpCD02D1 |  | PC 52 steel-concrete reinforcement, with bar diameters exceeding 8 mm, cut to size on site and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diameter 10 AIII) |  |  | kg |  | 214.0000 |  |  |  |  |  |
| 450 |  | CE18A |  | Skylights made of corrugated or ribbed sheets of PAS, PVC, polycarbonate or polymethacrylate, fitted to the edge or within the roof covering (10 mm toughened glass roof) |  |  | m² |  | 17.1000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Pits and covers Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13. Stair railings |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13.1. Stairwell no. 2 |  |  |  |  |  |  |  |  |  |  |
| 451 |  | CH06A |  | Metal handrail mounted on 15 cm high supports, spaced at intervals of 1....1.2 m and fitted with welded washers, fixed to a brick wall or concrete parapet, made from steel pipe D=1 1/4" and rolled steel, straight |  |  | m |  | 41.8000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Staircase No. 2 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 13.2. Block No. 1 |  |  |  |  |  |  |  |  |  |  |
| 452 |  | CH06A |  | Metal handrail mounted on 15 cm high supports, spaced at intervals of 1....1.2 m and fitted with welded washers, fixed to a brick wall or concrete parapet, made from steel pipe D=1 1/4" and rolled steel, straight |  |  | m |  | 28.3000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total for Staircase No. 1 Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Stair railings Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including salary |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |