| Major repairs and modernisation of the ‘Mihai Eminescu’ Secondary School, a public institution, with cadastral number 2101101.004.01, situated in the town of Cantemir, Cantemir municipality, 10 Trandafirilor Street | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-10

Elevator

| No. No. | Symbol, standards and resource code | Works and expenditure | Unit of measurement | Quantity as per project data | Quotation value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction and al works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.1. Foundations |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCB02E1 |  | Concrete cast in formwork (on one side), for continuous or isolated foundations, low-load floor slabs, basement walls, parapets, mass walls, etc., in existing buildings; concrete mixed on-site using a concrete mixer and cast using conventional methods with ready-mixed concrete supplied from batching plants, for underpinning works (concrete bed C 8/10) |  |  | m³ |  | 0.7000 |  |  |  |  |  |
| 2 |  | RpCB02E1 |  | Concrete cast in formwork (on one side), for continuous or isolated foundations, low-load floor slabs, basement walls, parapets, solid walls, etc., in existing buildings; concrete mixed on-site using a concrete mixer and cast using conventional methods with ready-mixed concrete supplied from batching plants, for underpinning works (concrete class C 16/20) |  |  | m³ |  | 4.5000 |  |  |  |  |  |
| 3 |  | RpCC01A |  | Mixed formwork comprising reusable panels, made from 15 mm thick type P beech plywood, including supports, for concrete work at height on retaining walls, plinths, etc., for repairs to existing structures, on straight walls up to 3.00 m in height |  |  | m² |  | 23.4000 |  |  |  |  |  |
| 4 |  | RpCD02B1 |  | PC 52 steel-concrete reinforcement, with bar diameters exceeding 8 mm, cut to size in on-site workshops and installed in existing structural elements; cutting and installation of reinforcement for foundations and ground slabs (diameter 10 A500) |  |  | kg |  | 130.0000 |  |  |  |  |  |
| 5 |  | RpCD02B1 |  | PC 52 steel-concrete reinforcement, with bar diameters exceeding 8 mm, cut to size in on-site workshops and installed in existing structural elements; cutting and installation of reinforcement for foundations and floor slabs (diam. 12 A500) |  |  | kg |  | 294.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Foundation Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.2. Details of the replacement of the prefabricated slab |  |  |  |  |  |  |  |  |  |  |
| 6 |  | RpCB05A |  | Reinforced concrete, class C-25/20 Bc 25 (B 330), in small quantities, for the reinforcement of reinforced concrete elements (stirrups or section enlargements of columns, beams, etc.) or for reinforced concrete reinforcement elements, prepared manually, cast using conventional methods |  |  | m³ |  | 5.2000 |  |  |  |  |  |
| 7 |  | RpCC02B |  | Mixed formwork made from reusable panels, constructed from plywood, excluding supports, for reinforced concrete, for additions or repairs to existing structures, in beams, using 15 mm plywood |  |  | m² |  | 20.8000 |  |  |  |  |  |
| 8 |  | RpCD01D |  | OB 37 steel-concrete reinforcement, with diameters up to and including 8 mm, cut to size on site and installed in existing structural elements; cutting and installation of reinforcement in flat slabs, columns, beams, etc. (diameter 6 A240) |  |  | kg |  | 33.0000 |  |  |  |  |  |
| 9 |  | RpCD02D3 |  | PC 52 steel-concrete reinforcement bars, with bar diameters exceeding 8 mm, cut in centralised workshops and installed in existing structural elements; cutting and installation of reinforcement for flat slabs, columns, beams, etc. (diam. 8 A500C |  |  | kg |  | 23.7000 |  |  |  |  |  |
| 10 |  | RpCD02D3 |  | PC 52 steel-concrete reinforcement, with bar diameters exceeding 8 mm, cut to size in centralised workshops and fitted into existing structural elements; cutting and fitting of reinforcement for flat slabs, columns, beams, etc. (diameter 16 A500C) |  |  | kg |  | 227.5000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total detail for replacement of the precast slab Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.3. Opening in floor |  |  |  |  |  |  |  |  |  |  |
| 11 |  | RpDC04B application |  | Cutting expansion joints in the wearing course concrete using a diamond disc saw |  |  | m |  | 12.0000 |  |  |  |  |  |
| 12 |  | RpCB18G |  | Demolition of old concrete using mechanical means, reinforced concrete |  |  | m³ |  | 1.5800 |  |  |  |  |  |
| 13 |  | CL17B |  | Various exposed metalwork: balustrades, grilles, shutters, snow guards, grates |  |  | kg |  | 518.4000 |  |  |  |  |  |
| 14 |  | RpIzE16B |  | Cleaning of metalwork and structures to remove rust and old paint using a wire brush, made from profiles with a thickness of 8–12 mm inclusive |  |  | t |  | 0.5180 |  |  |  |  |  |
| 15 |  | IzD03A |  | Painting of metal structures and fabrications with a coat of lead minium paint, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a mineral brush |  |  | t |  | 0.5180 |  |  |  |  |  |
| 16 |  | IzD04A |  | Painting of metal fabrications and structures with oil-based paint in 2 coats, made from sections with thicknesses between 8 mm and 12 mm inclusive, using a brush |  |  | t |  | 0.5180 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Hollow section -floor Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 1.4. Lift |  |  |  |  |  |  |  |  |  |  |
| 17 |  | CD73A |  | Walls made of lightweight, thermally insulated ‘sandwich’ panels of profiled sheet metal, mounted on metal or reinforced concrete studs at heights below 12 m: arranged in front of the studs ( panels, 150 mm thick PIR wall) |  |  | m² |  | 100.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Lift Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport ation of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on the estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation work |  |  |  |  |  |  |  |  |  |  |
| 18 |  | MАsA01A |  | Passenger elevator with automatic sliding doors, with a car speed of up to 1.0 m/s: lifting capacity up to 400 kg, serving 9 stops, shaft height 29 m |  |  | elevator |  | 1.0000 |  |  |  |  |  |
| 19 |  | MАsA01D1 |  | For each stop, whether more or fewer than those specified in the elevator’s specifications, add to or subtract from standards A and B (correction for the number of stops) |  |  | 1 stop |  | -6.0000 |  |  |  |  |  |
| 20 |  | MАsA01E1 |  | For every metre of shaft height, whether more or less than that specified in the lift’s specifications, add or subtract from standards A and B (correction for lift shaft height; Н_shaft = 10 m, 29 – 10 = 19 m) |  |  | m |  | -19.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport ation of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on the estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Plant |  |  |  |  |  |  |  |  |  |  |
| 21 |  | Supplier price |  | Passenger lift, 130 kg, 3 stops |  |  | units |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Machinery Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |