| Major repairs and modernisation of the tpieceing block at M. Eminescu High School, Cantemir | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-4 Part 1

Heating. Part 1

| No. No. | Symbol, standards and resource code | Works and expenses | Unit of measurement | Quantity as per project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Value of the machinery |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Market price |  | Automatic balancing valve on the return line, pressure differential maintenance range S-25 PN16 d-15mm ART-R |  |  | piece |  | 3.000 |  |  |  |  |  |
| 2 |  | Market price |  | Automatic return line balancing valve, pressure differential maintenance range S-25 PN16 d-20mm ART-R |  |  | piece |  | 3.000 |  |  |  |  |  |
| 3 |  | Market price |  | Automatic return line balancing valve, pressure differential maintenance range S-25 PN16 d-25mm ART-R |  |  | piece |  | 3.000 |  |  |  |  |  |
|  |  |  |  | Direct costs |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total value of the machinery Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation work |  |  |  |  |  |  |  |  |  |  |
| 4 |  | ID01A |  | Double-adjustment valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" (ART-R automatic balancing valve, d-15mm) |  |  | piece |  | 3.000 |  |  |  |  |  |
| 5 |  | ID01B |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/4"–1" (ART-R automatic balancing valve, d-20mm) |  |  | pc |  | 3.000 |  |  |  |  |  |
| 6 |  | ID01B |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/4"–1" (ART-R automatic balancing valve, 25 mm) |  |  | piece |  | .000 |  |  |  |  |  |
|  |  |  |  | Direct costs |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social fund |  |  | 24.00% |  |  |  |  |  |  |  |
|  |  |  |  | Material transport |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Direct costs |  |  | 100.00 + |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | 100.00 + |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Plumbing works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Heating system |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1.1. Radiators and radiator fittings |  |  |  |  |  |  |  |  |  |  |
| 7 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1000 mm inclusive (‘Korado’, type 22K-300-800) |  |  | piece |  | 5.000 |  |  |  |  |  |
| 8 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1,000 mm inclusive (‘Korado’, type 22K-300-900) |  |  | piece |  | 6.000 |  |  |  |  |  |
| 9 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1,000 mm inclusive (‘Korado’, type 22K-300-1000) |  |  | piece |  | 19.000 |  |  |  |  |  |
| 10 |  | IB06B |  | Steel radiators, monobloc, with a length of 1001–1500 mm (“Korado”, type 22K-300-1100) |  |  | piece |  | 13.000 |  |  |  |  |  |
| 11 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-300-1200) |  |  | piece |  | 5.000 |  |  |  |  |  |
| 12 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-300-1300) |  |  | piece |  | 12.000 |  |  |  |  |  |
| 13 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-300-1400) |  |  | piece |  | 9.000 |  |  |  |  |  |
| 14 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-300-1500) |  |  | piece |  | 6.000 |  |  |  |  |  |
| 15 |  | IB06C |  | Steel radiators, monobloc, 1501–2000 mm in length (“Korado”, type 22K-300-1600) |  |  | piece |  | 2.000 |  |  |  |  |  |
| 16 |  | IB06C |  | Steel radiators, monobloc, with a length of 1501–2000 mm. (“Korado”, type 22K-300-1700) |  |  | piece |  | 8.000 |  |  |  |  |  |
| 17 |  | IB06C |  | Steel radiators, monobloc, with a length of 1501–2000 mm. (“Korado”, type 22K-300-1800) |  |  | piece |  | 6.000 |  |  |  |  |  |
| 18 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1,000 mm, inclusive (“Korado”, type 22K-500-400) |  |  | piece |  | 6.000 |  |  |  |  |  |
| 19 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1000 mm inclusive (‘Korado’, type 22K-500-800) |  |  | piece |  | 4.000 |  |  |  |  |  |
| 20 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1,000 mm inclusive (‘Korado’, type 22K-500-900) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 21 |  | IB06A |  | Steel radiators, monobloc, with a length of up to 1,000 mm inclusive (‘Korado’, type 22K-500-1,000) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 22 |  | IB06B |  | Steel radiators, monobloc, with a length of 1001–1500 mm (“Korado”, type 22K-500-1100) |  |  | piece |  | 3.000 |  |  |  |  |  |
| 23 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-500-1300) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 24 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 22K-500-1400) |  |  | piece |  | 12.000 |  |  |  |  |  |
| 25 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 33K 300–1300) |  |  | piece |  | 4.000 |  |  |  |  |  |
| 26 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 33K 300–1400) |  |  | piece |  | 4.000 |  |  |  |  |  |
| 27 |  | IB06B |  | Steel radiators, monobloc, 1001–1500 mm in length (“Korado”, type 33K 300–1500) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 28 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" (Danfoss RTR-N Thermostar angle valve, d-15) |  |  | piece |  | 128.000 |  |  |  |  |  |
| 29 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" (RLV corner radiator shut-off valve, return, angled, internal thread d-15 mm) |  |  | piece |  | 128.000 |  |  |  |  |  |
| 30 |  | ID07A |  | RTR 7090 thermostatic control with integrated sensor and frost protection for Danfoss heating systems |  |  | piece |  | 128.000 |  |  |  |  |  |
|  |  |  |  | Total Radiators and radiator fittings Including labour |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1.2. Pipes, insulation, fittings and supports |  |  |  |  |  |  |  |  |  |  |
| 31 |  | IC35A |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, fitted to connect heating units or appliances in central heating systems, with an external diameter of up to and including 18.0 mm. (PE-X pipe , TIGRIS ALUPEX multilayer pipe with aluminium foil and oxygen barrier, T=90*c, S-Press fittings d=16x2.0 mm) |  |  | m |  | 845.000 |  |  |  |  |  |
| 32 |  | IC35B |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed to connect heating units or appliances in central heating systems, with an external diameter of 20.0 mm (TIGRIS ALUPEX multi-layer PE-X pipe with aluminium foil, with oxygen barrier, T=90*c, S-Press fittings d=20x2.25 mm) |  |  | m |  | 640.000 |  |  |  |  |  |
| 33 |  | IC35C |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, installed to connect heating units or appliances in central heating systems, with an external diameter of 25.0 mm (TIGRIS ALUPEX multi-layer PE-X pipe with aluminium foil, with oxygen barrier, T=90*c, S-Press fittings d=25x2.5 mm) |  |  | m |  | 405.000 |  |  |  |  |  |
| 34 |  | IC35D |  | High-density reinforced polyethylene or reinforced or unreinforced polypropylene pipe, fitted to connect heating units or appliances in central heating systems, with an external diameter of 32.0 mm (TIGRIS ALUPEX multi-layer PE-X pipe with aluminium foil, with oxygen barrier, T=90*c, S-Press fittings d=32x3.0 mm) |  |  | m |  | 10.000 |  |  |  |  |  |
| 35 |  | IE03A |  | Carrying out pressure leak tests on the supply pipes of heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 3/8" … 1" |  |  | m |  | 1,890.000 |  |  |  |  |  |
| 36 |  | IE03B |  | Carrying out a pressure leak test on the supply pipes for heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 1 1/4" … 2" |  |  | m |  | 10.000 |  |  |  |  |  |
| 37 |  | IE04A |  | Carrying out expansion-contraction and operational tests on the supply pipes for heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 3/8" … 1" |  |  | m |  | 1,890.000 |  |  |  |  |  |
| 38 |  | IE04B |  | Carrying out expansion-contraction and operational tests on the supply pipes for heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 1 1/4" … 2" |  |  | m |  | 10,000 |  |  |  |  |  |
| 39 |  | RpSF50A |  | Hydraulic approval and flushing of water-based central heating systems |  |  | m |  | 1,900.000 |  |  |  |  |  |
| 40 |  | RpIF09A |  | Insulation of pipes using special insulation sleeves fitted onto the pipes, with a diameter and thickness of D=18 (Tubolit S, TL-18/9-5 insulation tubing for pipes, with protective foil against aggressive environments and mechanical damage when installed in walls and floors) |  |  | m |  | 845.000 |  |  |  |  |  |
| 41 |  | RpIF09A |  | Insulation of pipes using special insulation sleeves fitted onto the pipes, with diameters and thicknesses from D=18, (Insulation tubing for pipes with protective foil against aggressive environments and mechanical damage when installed in walls and floors – Tubolit S, TL-22/9-5) |  |  | m |  | 640.000 |  |  |  |  |  |
| 42 |  | RpIF09A |  | Insulation of pipes using special insulation sleeves fitted onto the pipes, with diameters and thicknesses from D=18 (Insulation tubing for pipes with protective foil against aggressive environments and mechanical damage when installed in walls and floors: Tubolit S, TL-28/9-5) |  |  | m |  | 405.000 |  |  |  |  |  |
| 43 |  | RpIF09A |  | Insulation of pipes using special insulation sleeves fitted onto the pipes, with a diameter and thickness of D=18, (Insulation tubing for pipes with a protective film against aggressive environments and mechanical damage when installed in walls and floors: Tubolit S, TL-35/9-5) |  |  | m |  | 10.000 |  |  |  |  |  |
| 44 |  | IC11C |  | Black steel pipe, longitudinally welded for installations, unthreaded, fitted by welding in columns, in central heating systems for residential and socio-cultural buildings; the pipe has a diameter of 1" (Steel pipe D32x2, DN25) |  |  | m |  | 15.000 |  |  |  |  |  |
| 45 |  | IC11D |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings, with a pipe diameter of 1 1/4" (Steel pipe D38x2, DN32) |  |  | m |  | 20.000 |  |  |  |  |  |
| 46 |  | IC11E |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings, with a pipe diameter of 1 1/2" (Steel pipe D48x3.5, DN40) |  |  | m |  | 25.000 |  |  |  |  |  |
| 47 |  | IC11F |  | Black steel pipe, longitudinally welded for installations, unthreaded, installed by welding in columns, in central heating systems for residential and social-cultural buildings, with a pipe diameter of 2" (Steel pipe D57x3.5, DN50) |  |  | m |  | 95.000 |  |  |  |  |  |
| 48 |  | IC12C |  | Seamless or longitudinally welded steel pipe for construction, installed by welding into distribution pipes, in central heating systems for residential and socio- cultural buildings, with an external diameter and wall thickness of 76 x 3.5 mm (Steel pipe D76x3.5, DN65) |  |  | m |  | 70.000 |  |  |  |  |  |
| 49 |  | RpIF10A |  | Insulation of pipes using mats of glass mineral wool, type 1 mineral wool or type P mineral wool on corrugated cardboard or rabbit mesh, made on site, with a thickness of 50 mm, fitted to pipes with a circumference of less than 35 cm (Mineral wool insulation, thickness 50 mm) |  |  | m² |  | 65.000 |  |  |  |  |  |
| 50 |  | Market price |  | Protective layer, with self-adhesive sealing tape |  |  | m |  | 225.000 |  |  |  |  |  |
| 51 |  | IE03B |  | Carrying out pressure leak tests on the supply pipes of heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 1 1/4" ... 2" |  |  | m |  | 155.000 |  |  |  |  |  |
| 52 |  | IE03D |  | Carrying out a pressure leak test on the supply pipes of heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 80 x 3.5 … 159 x 8 mm |  |  | m |  | 70.000 |  |  |  |  |  |
| 53 |  | IE04B |  | Carrying out expansion-contraction and operational tests on the supply pipes for heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 1 1/4" … 2" |  |  | m |  | 155.000 |  |  |  |  |  |
| 54 |  | IE04D |  | Carrying out expansion-contraction and operational tests on the supply pipes for heating appliances (air heaters, convector heaters, skirting board convectors, etc.) with a diameter of 89 x 3.5 … 159 x 8 mm |  |  | m |  | 70.000 |  |  |  |  |  |
| 55 |  | RpSF50A |  | Hydraulic approval and flushing of water-based central heating systems |  |  | m |  | 225.000 |  |  |  |  |  |
| 56 |  | IC42A |  | Supports and fastenings for securing pipes, boilers, appliances and containers weighing up to 2 kg piece |  |  | kg |  | 177.000 |  |  |  |  |  |
| 57 |  | CN20B |  | Interior or exterior paints applied to metal joinery with alkyd enamel in 2 coats, including primer (Paint for supports) |  |  | m² |  | 6.400 |  |  |  |  |  |
| 58 |  | CN20B1 |  | Interior or exterior paints applied to metal joinery with alkyd enamel in 2 coats, including the primer, for the application of one fewer coat (Primer for metal pipes) |  |  | m² |  | 41.400 |  |  |  |  |  |
| 59 |  | CN20B |  | Interior or exterior paints applied to metal joinery with alkyd enamel in 2 coats, including primer (Paint for metal pipes) |  |  | m² |  | 41.400 |  |  |  |  |  |
|  |  |  |  | Total Pipes, insulation, fittings and supports Including labour |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1.3. Distribution unit and fittings |  |  |  |  |  |  |  |  |  |  |
| 60 |  | IC39D |  | Copper manifold/collector for connecting heating units, appliances or systems to the plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall (Flow/return manifold R551d-25x3 branches, spacing 50mm, Giacomini) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 61 |  |  |  | Manhole box for manifold with 3 branches (Inspection box for manifolds) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 62 |  | IC39D |  | Copper manifold for connecting radiators, appliances or heating systems to a plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall ( R551d-32x2-way manifold, 70mm spacing, Giacomini) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 63 |  |  |  | Manhole box for manifold with 2 branches (Inspection box for manifolds) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 64 |  | IC39D |  | Copper manifold for connecting radiators, appliances or heating systems to a plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall (R551d-32x3 flow/return manifold with 3 branches, 70mm spacing, Giacomini) |  |  | piece |  | 3.000 |  |  |  |  |  |
| 65 |  |  |  | Manhole box for manifold with 3 branches (Inspection box for manifolds) |  |  | piece |  | 3.000 |  |  |  |  |  |
| 66 |  | IC39D |  | Copper manifold for connecting radiators, appliances or heating systems to a plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall (R551d-32x4-outlet flow/return manifold, 70 mm spacing, Giacomini) |  |  | piece |  | 2.000 |  |  |  |  |  |
| 67 |  |  |  | Manhole box for manifold with 4 branches (Manhole box for manifolds) |  |  | piece |  | 2.000 |  |  |  |  |  |
| 68 |  | IC39D |  | Copper manifold for connecting radiators, appliances or heating systems to a plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall (R551d-40x4 flow/return manifold, 100 mm spacing, Giacomini) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 69 |  |  |  | Manhole box for manifold with 4 branches (Inspection box for manifolds) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 70 |  | IC39D |  | copper manifold for connecting radiators, appliances or heating systems to a plastic pipework heating system, mounted on a wall or in boxes with a diameter of 1" on a concrete wall (R551d-50x4-way flow/return manifold, 100mm spacing, Giacomini) |  |  | pc |  | 1.000 |  |  |  |  |  |
| 71 |  |  |  | Manhole box for manifold with 5 branches (Inspection box for manifolds) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 72 |  | ID06A |  | Air vent valve with removable key for central heating systems, with a nominal diameter of 1/4" (Flow/return end assembly with automatic air vent and drain connection) |  |  | piece |  | 18.000 |  |  |  |  |  |
| 73 |  | IB20A |  | Support brackets for radiators, mounted on a brick wall. (Mounting brackets for manifolds) |  |  | piece |  | 18.000 |  |  |  |  |  |
| 74 |  | ID01A |  | Double-control valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" (Valve – paired with MVT-R d-15mm LF flow measuring diaphragm) |  |  | piece |  | 2.000 |  |  |  |  |  |
| 75 |  | ID01A |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/8"–1/2" (Matching valve with MVT-R d-15mm flow measuring diaphragm) |  |  | piece |  | 3.000 |  |  |  |  |  |
| 76 |  | ID01B |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 3/4"–1" (Matching valve with flow-side measuring diaphragm MVT-R d-20mm) |  |  | piece |  | 2.000 |  |  |  |  |  |
| 77 |  | ID01C |  | Double-acting valve (flow or return) for central heating systems, with a nominal diameter of 1 1/4" (Matching valve with flow-side measuring diaphragm MVT-R d-25 ) |  |  | pc |  | 2.000 |  |  |  |  |  |
| 78 |  | ID04A |  | Ball valve with connectors for draining central heating systems, with a nominal diameter of 1/2"–1" (Caleffi ball valve DN 15 PN16) |  |  | piece |  | 4.000 |  |  |  |  |  |
| 79 |  | ID04A |  | Ball valve with sockets for draining central heating systems, with a nominal diameter of 1/2"–1" (Caleffi DN 20 PN16 ball valve) |  |  | piece |  | 56.000 |  |  |  |  |  |
| 80 |  | ID04A |  | Ball valve with sockets for draining central heating systems, with a nominal diameter of 1/2"–1" (Caleffi DN 25 PN16 ball valve) |  |  | piece |  | 10.000 |  |  |  |  |  |
| 81 |  | ID04B |  | Through-type or check valve with sockets for central heating systems, with a nominal diameter of 1 1/4" – 1 1/2" (Caleffi DN 32 PN16 ball valve) |  |  | piece |  | 7.000 |  |  |  |  |  |
| 82 |  | ID04B |  | Through or shut-off valve with sockets for central heating systems, with a nominal diameter of 1 1/4"–1 1/2" (Caleffi DN 40 PN16 ball valve) |  |  | piece |  | 1.000 |  |  |  |  |  |
| 83 |  | ID04A |  | Ball valve with sockets for central heating systems, with a nominal diameter of 1/2"–1" (3/4" drain valve) |  |  | piece |  | 9.000 |  |  |  |  |  |
| 84 |  | ID03A |  | Three-way plug valve with flanges and compression seal, for central heating systems, with a nominal diameter of 15–20 mm (Ball valve with lever, plug and vent button, 3/4") |  |  | piece |  | 4.000 |  |  |  |  |  |
|  |  |  |  | Total Distribution and Fittings Group Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total Heating system Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Direct costs |  |  | lei |  |  |  |  |  |  |  |
|  |  |  |  | Social fund |  |  | 24.00% |  |  |  |  |  |  |  |
|  |  |  |  | Material transport |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Direct costs |  |  | 100.00 + |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | 100.00 + |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total Plumbing works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total quote: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |