| Major repairs and modernisation of the ‘Mihai Eminescu’ Public Lower Secondary School, cadastral number 2101101.004.01, situated in the town of Cantemir, Cantemir municipality, 10 Trandafirilor Street | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-7

Electrical networks   Interior

Prepared using current prices

| No. No. | Symbol, standards and resource code | Works and expenditure | Unit of measurement | Quantity as per project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction works |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RpCU06A |  | Cutting grooves up to 5 cm deep in 5 x 50 cm brickwork walls2 |  |  | m |  | 2,950.0000 |  |  |  |  |  |
| 2 |  | RpCU07D |  | Filling of grooves in walls up to 50 cm², following installation or reinforcement works |  |  | m |  | 2,950.0000 |  |  |  |  |  |
| 3 |  | RpCU06D |  | Cutting grooves up to 5 cm deep in 5 x 100 cm² brickwork walls |  |  | m |  | 30.0000 |  |  |  |  |  |
| 4 |  | RpCU07E |  | Filling of grooves in walls of 51–100 cm², following installation or reinforcement works |  |  | m |  | 30.0000 |  |  |  |  |  |
| 5 |  | RpCU05A |  | Making penetrations for pipes or tie rods, as part of reinforcement works, in walls or floors made of brickwork up to 15 cm thick |  |  | piece |  | 450.0000 |  |  |  |  |  |
| 6 |  | RpCU07A |  | Filling of holes in walls with plaster mortar, following installation or reinforcement works |  |  | piece |  | 450.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Social security |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | e transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation works |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. TDG-1 Distribution Board |  |  |  |  |  |  |  |  |  |  |
| 7 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Distribution board with lock, surface-mounted, IP-54 ЩМП-5) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 8 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport ( Load switch IP 20, Inom - 400A, 3P QS, 3P ) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 9 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -100A, 3P, QF (3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 10 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -32A, 3P, QF (3P) type C) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 11 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -25A, 3P, QF (3P) type C) |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 12 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -20A, 3P, QF (3P) type C) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 13 |  | 08-03-575- 1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -16A, 3P, QF (3P) type C |  |  | pc |  | 7.0000 |  |  |  |  |  |
| 14 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, QF (1P) type C) |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 15 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, push-button and other, with mounting preparation (Phase control relay IP 20, Inom - 16A, 1P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 16 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 17 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Distribution Board TDG-1 Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Distribution Board TD-A-1 (Air-conditioning) |  |  |  |  |  |  |  |  |  |  |
| 18 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Distribution board with lock, surface-mounted, IP-54 ЩМП-5) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 19 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit-breaker with combined trip mechanism (thermal, electromagnetic), IP 20, Inom -250A, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 20 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -63A, 3P, type "C") |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 21 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 22 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-1 Distribution Board (Air Conditioning) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Distribution Board TD-A-2 (Ventilation) |  |  |  |  |  |  |  |  |  |  |
| 23 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (lockable distribution board, flush-mounted, IP-54 for 24 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 24 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 100A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 25 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 10A, 1P, QF (1P) type C) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 26 |  | 08-03-530-4 |  | General-purpose magnetic starter, separate, wall- or column-mounted, current up to 40 A (Electromagnetic starter KM – 20A (contactor) 230V, Ubobina – 230V, Icontact – 20 A) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 27 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 28 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Distribution Board TD-A-2 (Ventilation) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Distribution Board TD-A-3 (Low-Voltage) |  |  |  |  |  |  |  |  |  |  |
| 29 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (lockable distribution board, flush-mounted, IP-54 for 24 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 30 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 31 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -16A, 1P, type B QF (1P) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 32 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit-breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -10A, 1P, type B QF (1P) type C) |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 33 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 34 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-3 Distribution Board ( Low-Voltage) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Distribution Board TD-A-4 (Ice Melting) |  |  |  |  |  |  |  |  |  |  |
| 35 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (lockable distribution board, surface-mounted, IP-54 for 18 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 36 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 37 |  | 08-03-530-4 |  | Standard-purpose magnetic starter, separate, wall- or column-mounted, current up to 40 A (KM-40A electromagnetic starter (contactor) 400 V, coil – 400 V, contactor – 40 A) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 38 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 10 A (Idif – 30 mA), 2P type C QFD 2P, type C) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 39 |  | 08-01-081-1 |  | Control and signalling device (push-button, control switch, electromagnetic lock, audible alarm, signal lamp), number of connected terminals up to 2 (Start/Stop push-button) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 40 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 41 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-4 Distribution Board (Ice Melting) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. TD-A-5 Distribution Board (Basement Sockets) |  |  |  |  |  |  |  |  |  |  |
| 42 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 48 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 43 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 44 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with IP 20 differential protection, Inom - 16A (Idif - 30 mA), 4P type C QFD 4P, type C) |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 45 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection, IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 46 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 47 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-5 Distribution Board (basement sockets) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.7. TD-A-6 Distribution Board (Basement lighting) |  |  |  |  |  |  |  |  |  |  |
| 48 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (lockable distribution board, flush-mounted, IP-54 for 24 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 49 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 50 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -10A, 1P, type C QF (1P) |  |  | piece |  | 11.0000 |  |  |  |  |  |
| 51 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 52 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-6 Distribution Board (Basement Lighting) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.8. TD-A-7 Distribution Board (Cake Shop) |  |  |  |  |  |  |  |  |  |  |
| 53 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Lockable distribution board, surface-mounted, IP-54 for 36 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 54 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 55 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection, IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 56 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -10A, 1P, type C QF (1P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-7 Distribution Board (Confectionery) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.9. TD-A-8 Distribution Board (Ground floor sockets) |  |  |  |  |  |  |  |  |  |  |
| 57 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 54 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 58 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 59 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with IP 20 differential protection , Inom - 16A (Idif - 30 mA), 4P type C QFD 4P, type C) |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 60 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 17.0000 |  |  |  |  |  |
| 61 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 62 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-8 Distribution Board (Ground floor sockets) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.10. TD-A-9 Distribution Board (Ground floor lighting) |  |  |  |  |  |  |  |  |  |  |
| 63 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 48 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 64 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 65 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, type C QF (1P) |  |  | piece |  | 24.0000 |  |  |  |  |  |
| 66 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 67 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-9 Distribution Board (Ground floor lighting) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.11. TD-A-8 Distribution Board (Upstairs sockets, elevation +3,300) |  |  |  |  |  |  |  |  |  |  |
| 68 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 48 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 69 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 70 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with IP 20 differential , Inom - 16A (Idif - 10 mA), 2P type C QFD 2P type C) |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 71 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 12.0000 |  |  |  |  |  |
| 72 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 73 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-8 Distribution Board (floor sockets, elevation +3,300) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.12. TD-A-11 Distribution Board (Floor lighting, elevation +3,300) |  |  |  |  |  |  |  |  |  |  |
| 74 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 36 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 75 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 76 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport ( Load switch IP 20, Inom - 25A, 1P QS, 1P ) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 77 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, type C QF (1P) |  |  | piece |  | 18.0000 |  |  |  |  |  |
| 78 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 79 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-11 Distribution Board (Floor lighting, elevation +3,300) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.13. TD-A-12 Distribution Board (Upstairs sockets, elevation +6,600) |  |  |  |  |  |  |  |  |  |  |
| 80 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed-mounted, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 48 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 81 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 82 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with IP 20 differential , Inom - 16A (Idif - 10 mA), 2P type C QFD 2P type C) |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 83 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 13.0000 |  |  |  |  |  |
| 84 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 85 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-12 Distribution Board (floor sockets, level +6,600) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.14. TD-A-13 Distribution Board (Floor lighting, elevation +6,600) |  |  |  |  |  |  |  |  |  |  |
| 86 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 36 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 87 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 88 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport ( Load switch IP 20, Inom - 25A, 1P QS, 1P ) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 89 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, type C QF (1P) |  |  | piece |  | 17.0000 |  |  |  |  |  |
| 90 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 91 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-13 Distribution Board (Floor lighting, level +6,600) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.15. Distribution Board TD-A-14 (Computer Room No. 1) |  |  |  |  |  |  |  |  |  |  |
| 92 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Lockable distribution board, surface-mounted, IP-54 for 54 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 93 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 94 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 24.0000 |  |  |  |  |  |
| 95 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 96 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-14 Distribution Board (Computer Room No. 1) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.16. Distribution Board TD-A-15 (Computer Room No. 2) |  |  |  |  |  |  |  |  |  |  |
| 97 |  | 08-03-572-3 |  | Control unit, cabinet-type or distribution point (cabinet), wall-mounted, height and width, mm, up to 600×600 (Lockable distribution board, surface-mounted, IP-54 for 54 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 98 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 99 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Circuit breaker with differential protection , IP 20, Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C) |  |  | piece |  | 24.0000 |  |  |  |  |  |
| 100 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 101 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-15 Distribution Board (Computer Room No. 2) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.17. TD-A-17 Distribution Board ( -evacuation lighting) |  |  |  |  |  |  |  |  |  |  |
| 102 |  | 08-03-572-8 |  | Control unit, cabinet-type or distribution point (cabinet), recessed, height and width, mm, up to 700×850 (Lockable distribution board, flush-mounted, IP-54 for 36 modules) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 103 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Load break switch IP 20, Inom - 63A, 3P QS, 3P) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 104 |  | 08-03-530-4 |  | Standard-purpose magnetic starter, separate, wall- or column-mounted, current up to 40 A (KM-40A electromagnetic starter (contactor) 400 V, coil – 400 V, contactor – 40 A) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 105 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, type C QF (1P) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 106 |  | Supplier price |  | RE SHNI rail |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 107 |  | Supplier price |  | Rail N ШНИ |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-17 Distribution Board ( -evacuation lighting) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.18. Earthing system / Paratrasnet system |  |  |  |  |  |  |  |  |  |  |
| 108 |  | 08-03-572-3 |  | Control unit in cabinet or distribution point (cabinet) design, wall-mounted, height and width, mm, up to 600×600 (Lockable distribution board, surface-mounted, IP-54 ЩМП-1) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 109 |  | 08-02-471-4 |  | Earth electrode, vertical, made of round steel, diameter 20 mm, L=3 m |  |  | 10 pcs |  | 0.3000 |  |  |  |  |  |
| 110 |  | 08-02-472-2 |  | Earth connection conductors: horizontal earth electrode, made of split steel, cross-section 160 mm² ( -galvanised steel strip, 40 × 4 mm) |  |  | 100 m |  | 0.5000 |  |  |  |  |  |
| 111 |  | Market price |  | Down conductor D10 |  |  | m |  | 90.0000 |  |  |  |  |  |
| 112 |  | 08-02-146-2 |  | Cable up to 35 kV, secured with clamps, weight per 1 m up to: 1 kg |  |  | 100 m |  | 0.5000 |  |  |  |  |  |
| 113 |  | Market price |  | Low-flammability copper wire ПВ-1 1x6 mm² |  |  | m |  | 50.0000 |  |  |  |  |  |
| 114 |  | Market price |  | Dispersion resistance test connector – D10 conductor strip |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 115 |  | Market price |  | D10 down conductor fastener for roof installation |  |  | piece |  | 18.0000 |  |  |  |  |  |
| 116 |  | Market price |  | D10 down conductor fixing bracket for wall mounting |  |  | piece |  | 12.0000 |  |  |  |  |  |
| 117 |  | Supplier price |  | Jonostar 45 Level I active lightning arrester |  |  | set |  | 1.0000 |  |  |  |  |  |
| 118 |  | 08-02-408-1 |  | Steel pipe for use in explosive and fire-hazardous areas, on existing structures, secured with clamps, diameter 27 mm |  |  | 100 m |  | 0.0100 |  |  |  |  |  |
| 119 |  | Market price |  | metal pipe for, for the fixing of the D27 lightning conductor |  |  | m |  | 1.0000 |  |  |  |  |  |
| 120 |  | 08-02-408-2 |  | Steel pipe for use in explosive and fire-hazardous areas, on installed structures, secured with clamps, diameter up to 40 mm |  |  | 100 m |  | 0.0200 |  |  |  |  |  |
| 121 |  | Market price |  | metal pipe for, for the fixing of the D32 lightning conductor |  |  | m |  | 2.0000 |  |  |  |  |  |
| 122 |  | Market price |  | Galvanised steel tripod, D40, H=2m |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Earthing system / Lightning protection system Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.19. Cables / Wires |  |  |  |  |  |  |  |  |  |  |
| 123 |  | 08-02-147-10 |  | Cable up to 35 kV on structures and installed cable trays, secured along its entire length, weight per 1 m of cable, up to: 1 kg |  |  | 100 m |  | 1.2000 |  |  |  |  |  |
| 124 |  | Market price |  | -thermal cable 12 W/m, D4.5 RYXON |  |  | m |  | 120.0000 |  |  |  |  |  |
| 125 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and installed boxes, weight per 1 m up to: 1 kg |  |  | 100 m |  | 2.0000 |  |  |  |  |  |
| 126 |  | Market price |  | KВВГнг-FRLS cable, 4x1.5 mm² |  |  | m |  | 200.0000 |  |  |  |  |  |
| 127 |  | 08-02-148-1 |  | Cable up to 35 kV in conduits, blocks and installed boxes, weight per 1 m up to: 1 kg |  |  | 100 m |  | 127.5000 |  |  |  |  |  |
| 128 |  | Market price |  | cable ВВГнг-FRLSLTx 3x1.5 mm² |  |  | m |  | 6,700.0000 |  |  |  |  |  |
| 129 |  | Market price |  | cable ВВГнг-FRLSLTx 3x2.5 mm² |  |  | m |  | 5,500.0000 |  |  |  |  |  |
| 130 |  | Market price |  | cable ВВГнг-FRLSLTx 3x4.0 mm² |  |  | m |  | 150.0000 |  |  |  |  |  |
| 131 |  | Market price |  | BBGng-FRLSLTx cable, 5 × 4.0 mm² |  |  | m |  | 200.0000 |  |  |  |  |  |
| 132 |  | Market price |  | cable ВВГнг-FRLSLTx 5x6.0 mm² |  |  | m |  | 200.0000 |  |  |  |  |  |
| 133 |  | 08-02-148-2 |  | Cable up to 35 kV in conduits, blocks and installed boxes, weight per 1 m up to: 2 kg |  |  | 100 m |  | 4.5000 |  |  |  |  |  |
| 134 |  | Market price |  | cable ВВГнг-FRLSLTx 5x16 mm² |  |  | m |  | 450.0000 |  |  |  |  |  |
| 135 |  | 08-02-148-4 |  | Cable up to 35 kV in conduits, blocks and installed boxes, weight per 1 m up to: 6 kg |  |  | 100 m |  | 0.1500 |  |  |  |  |  |
| 136 |  | Market price |  | cable ВВГнг-FRLSLTx 5x50 mm² |  |  | m |  | 15.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cables / Conductors Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.20. Protective conduits / Trays |  |  |  |  |  |  |  |  |  |  |
| 137 |  | 08-02-409-1 |  | Vinyl-plastic pipe on existing structures, on walls and columns, secured with clamps, diameter up to 25 mm |  |  | 100 m |  | 96.0000 |  |  |  |  |  |
| 138 |  | Market price |  | PVC technical protection conduit, D16 mm |  |  | m |  | 5,000.0000 |  |  |  |  |  |
| 139 |  | Market price |  | PVC technical protective conduit, D20 mm |  |  | m |  | 4,500.0000 |  |  |  |  |  |
| 140 |  | Market price |  | Technical PVC protective conduit, D25 mm |  |  | m |  | 100.0000 |  |  |  |  |  |
| 141 |  | 08-02-409-2 |  | Vinyl-plastic pipe for installation on structures, walls and columns, secured with clamps, diameter up to 50 mm |  |  | 100 m |  | 4.0000 |  |  |  |  |  |
| 142 |  | Market price |  | PVC technical protection conduit, D32 mm |  |  | m |  | 200.0000 |  |  |  |  |  |
| 143 |  | Market price |  | PVC technical protective conduit, D40 mm |  |  | m |  | 200.0000 |  |  |  |  |  |
| 144 |  | 08-02-396-6 |  | Metal duct for walls and ceilings, length 3 m |  |  | 100 m |  | 2.2500 |  |  |  |  |  |
| 145 |  | Market price |  | metal gutter 300x50x3000 |  |  | m |  | 90.0000 |  |  |  |  |  |
| 146 |  | Market price |  | metal gutter 100x50x3000 |  |  | m |  | 45.0000 |  |  |  |  |  |
| 147 |  | Market price |  | metal gutter 50x50x3000 |  |  | m |  | 90.0000 |  |  |  |  |  |
| 148 |  | Market price |  | BML-10 bracket for gutter supports |  |  | piece |  | 300.0000 |  |  |  |  |  |
| 149 |  | Market price |  | M8 26kN wall anchor for C20/25 concrete |  |  | piece |  | 600.0000 |  |  |  |  |  |
| 150 |  | Market price |  | M8 FeZn threaded rod, L=3000mm |  |  | piece |  | 600.0000 |  |  |  |  |  |
| 151 |  | Market price |  | Saiba M8 FeZn |  |  | piece |  | 600.0000 |  |  |  |  |  |
| 152 |  | Market price |  | Grover M8 FeZn |  |  | piece |  | 600.0000 |  |  |  |  |  |
| 153 |  | Market price |  | M8 FeZn nut |  |  | piece |  | 600.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Protective Tubes / Gutters Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.21. Sockets / Switches / Junction boxes / Ceiling lights |  |  |  |  |  |  |  |  |  |  |
| 154 |  | 08-03-591-11 |  | Three-pin socket |  |  | 100 pcs |  | 0.0500 |  |  |  |  |  |
| 155 |  | Supplier price |  | SCAME 513.3257DF fixed l socket, 5-pin, 32A, 230/400V, IP 44 |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 156 |  | 08-03-591-9 |  | Surface-mounted socket, for flush-mounted installation |  |  | 100 pcs |  | 3.6900 |  |  |  |  |  |
| 157 |  | Supplier price |  | -rated socket with earth contact and shutters, flush-mounted, IP 44, Inom - 16A, Unom - 250V |  |  | piece |  | 245.0000 |  |  |  |  |  |
| 158 |  | Supplier price |  | Socket with earth contact, flush-mounted, IP 20, Inom - 16A, Unom - 250V |  |  | piece |  | 124.0000 |  |  |  |  |  |
| 159 |  | 08-03-591-2 |  | Single-pole switch, flush-mounted type, for enclosed installations |  |  | 100 pcs |  | 0.3100 |  |  |  |  |  |
| 160 |  | Market price |  | Single-pole switch for flush-mounted installation, IP 20, Inom – 10A, U – 250V |  |  | piece |  | 31.0000 |  |  |  |  |  |
| 161 |  | 08-03-591-5 |  | Two-pole switch, surface-mounted type, for flush-mounted installation |  |  | 100 pcs |  | 0.1800 |  |  |  |  |  |
| 162 |  | Market price |  | Two-pole switch for flush-mounted installation, IP 20, Inom – 10A, U – 250V |  |  | piece |  | 18.0000 |  |  |  |  |  |
| 163 |  | 08-03-591-5 |  | Three-pole switch, surface-mounted type, for flush-mounted installation |  |  | 100 pcs |  | 0.2000 |  |  |  |  |  |
| 164 |  | Market price |  | Three-pole switch for flush-mounted installation, IP 20, Inom – 10A, U – 250V |  |  | piece |  | 20.0000 |  |  |  |  |  |
| 165 |  | 08-03-591-2 |  | Single-pole switch, flush-mounted type, for enclosed installations |  |  | 100 pcs |  | 0.0400 |  |  |  |  |  |
| 166 |  | Market price |  | Single-pole staircase switch for flush-mounted installation, IP 20, Inom – 10A, U – 250V |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 167 |  | 08-03-592-1 |  | Box for sockets and switches, distribution boxes |  |  | 100 pcs |  | 4.8200 |  |  |  |  |  |
| 168 |  | Supplier price |  | Distribution box (cable distribution box) |  |  | piece |  | 32.0000 |  |  |  |  |  |
| 169 |  | Supplier price |  | Boxes for flush-mounting sockets and switches. |  |  | piece |  | 450.0000 |  |  |  |  |  |
| 170 |  | 08-03-593-10 |  | Wall-mounted lighting indicators |  |  | 100 pcs |  | 0.2100 |  |  |  |  |  |
| 171 |  | Supplier price |  | Battery-powered EXIT sign, 5 W, IP20 |  |  | piece |  | 15.0000 |  |  |  |  |  |
| 172 |  | Supplier price |  | Battery-powered EXIT sign with directional indicator, 5 W, IP20 |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 173 |  | 08-03-594-6 |  | Lighting fixture with fluorescent lamps mounted separately on suspension brackets (bars), number of lamps in the fixture: 1 |  |  | 100 pcs |  | 0.1200 |  |  |  |  |  |
| 174 |  | Supplier price |  | Horoz PROLINE-20 ceiling light, 20 W, IP65 |  |  | piece |  | 12.0000 |  |  |  |  |  |
| 175 |  | 08-03-593-6 |  | Lighting fixture for recessed ceiling or wall lamps with screw mounting, for rooms with normal environmental conditions, single-lamp |  |  | 100 pcs |  | 0.7700 |  |  |  |  |  |
| 176 |  | Supplier price |  | Vargo LED ceiling light, 24 W, IP54 |  |  | piece |  | 77.0000 |  |  |  |  |  |
| 177 |  | 08-03-594-11 |  | Lighting fitting with fluorescent lamps in suspended ceilings, mounted on suspension brackets; number of lamps in the fitting: 4 |  |  | 100 pcs |  | 3.4400 |  |  |  |  |  |
| 178 |  | Supplier price |  | LED ceiling light 595x595, 36 W, 4000 K, IP20 |  |  | piece |  | 344.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Sockets / Switches / Junction Boxes / Ceiling Lights Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e transport of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Budget surplus |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation work Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Equipment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. TDG-1 distribution board |  |  |  |  |  |  |  |  |  |  |
| 179 |  | Market price |  | Lockable distribution board, surface-mounted, IP-54 ЩМП-5 |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 180 |  | Market price |  | Load switch IP 20, Inom - 400A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 181 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 100A, 3P, QF (3P) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 182 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -32A, 3P, QF (3P) type C |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 183 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -25A, 3P, QF (3P) type C |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 184 |  | Market price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -20A, 3P, QF (3P) type C |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 185 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -16A, 3P, QF (3P) type C |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 186 |  | Market price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P type C |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 187 |  | Market price |  | Phase control relay IP 20, Inom - 16A, 1P. |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TDG-1 Distribution Board Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. TD-A-1 Distribution Board (Air Conditioning) |  |  |  |  |  |  |  |  |  |  |
| 188 |  | Market price |  | Lockable distribution board, surface-mounted, IP-54 ЩМП-5 |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 189 |  | Market price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic), IP 20, Inom -250A, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 190 |  | Market price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic), IP 20, Inom -63A, 3P, type "C" |  |  | piece |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-1 Distribution Board (Air Conditioning) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Distribution Board TD-A-2 (Ventilation) |  |  |  |  |  |  |  |  |  |  |
| 191 |  | Market price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 24 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 192 |  | Market price |  | Load switch, IP 20, Inom – 100A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 193 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 10A, 1P, QF (1P) type C |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 194 |  | Supplier price |  | Electromagnetic starter (contactor) 230V, Ubobina – 230V, Icontact – 20 A, KM – 20A |  |  | piece |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-2 Distribution Board (Ventilation) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. TD-A-3 Distribution Board (Low-Voltage Circuits) |  |  |  |  |  |  |  |  |  |  |
| 195 |  | Supplier price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 24 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 196 |  | Market price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 197 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -16A, 1P, QF (1P) type B |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 198 |  | Market price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -10A, 1P, QF (1P) type B |  |  | piece |  | 7.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-3 Distribution Board (Low Current) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.5. TD-A-4 Distribution Board (Ice Melting) |  |  |  |  |  |  |  |  |  |  |
| 199 |  | Market price |  | Distribution board with padlock, surface-mounted, IP-54 for 18 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 200 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 201 |  | Supplier price |  | KM electromagnetic starter – 40A (contactor) 400V, Ubobina – 400V, Icontact – 40 A |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 202 |  | Supplier price |  | Circuit breaker with differential protection , IP 20, Inom - 10A (Idif - 30 mA), 2P type C QFD 2P |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 203 |  | Supplier price |  | Start/Stop button |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-4 Distribution Board (De-icing) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.6. TD-A-5 Distribution Board (Basement Sockets) |  |  |  |  |  |  |  |  |  |  |
| 204 |  | Supplier price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 48 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 205 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 206 |  | Supplier price |  | Automatic circuit breaker with IP 20 differential protection ( ), Inom - 16A (Idif - 30 mA), 4P type C QFD 4P, type C |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 207 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-5 Distribution Board (Basement Sockets) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.7. TD-A-6 Distribution Board (Basement lighting ) |  |  |  |  |  |  |  |  |  |  |
| 208 |  | Market price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 24 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 209 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 210 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 10A, 1P, QF (1P) type C |  |  | piece |  | 11.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-6 Distribution Board (Lighting basement) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.8. Distribution Board TD-A-7 (Café) |  |  |  |  |  |  |  |  |  |  |
| 211 |  | Supplier price |  | Lockable distribution board, surface-mounted, IP-54, for 36 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 212 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 213 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 214 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 10A, 1P, QF (1P) type C |  |  | piece |  | 1.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-7 Distribution Board (Confectionery) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.9. TD-A-8 Distribution Board (Ground floor sockets) |  |  |  |  |  |  |  |  |  |  |
| 215 |  | Supplier price |  | Lockable distribution board, surface-mounted, IP-54, for 54 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 216 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 217 |  | Supplier price |  | Automatic circuit breaker with IP 20 differential protection ( ), Inom - 16A (Idif - 30 mA), 4P type C QFD 4P, type C |  |  | piece |  | 7.0000 |  |  |  |  |  |
| 218 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 17.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-8 Distribution Board (ground floor sockets) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.10. TD-A-9 Distribution Board (Ground floor lighting) |  |  |  |  |  |  |  |  |  |  |
| 219 |  | Supplier price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 48 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 220 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 221 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, QF (1P) type C |  |  | piece |  | 24.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-9 Distribution Board (Ground floor lighting) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.11. TD-A-10 Distribution Board (Upstairs sockets, level +3,300) |  |  |  |  |  |  |  |  |  |  |
| 222 |  | Supplier price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 48 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 223 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 224 |  | Supplier price |  | Circuit breaker with IP 20 differential protection ( ), Inom – 16A (Idif – 10 mA), 2P type C QFD 2P type C |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 225 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 12.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-10 Distribution Board (Upstairs sockets, elevation +3,300) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.12. TD-A-11 Distribution Board (Lighting , floor level +3,300) |  |  |  |  |  |  |  |  |  |  |
| 226 |  | Supplier price |  | Lockable distribution board, built- , IP-54, for 36 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 227 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 228 |  | Supplier price |  | Load break switch IP 20, Inom - 25A, 1P QS, 1P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 229 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, QF (1P) type C |  |  | piece |  | 18.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-11 Distribution Board (Lighting st floor, elevation +3,300) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.13. Distribution Board TD-A-12 (Upstairs sockets, elevation + 6.600) |  |  |  |  |  |  |  |  |  |  |
| 230 |  | Supplier price |  | Lockable distribution board, built-in ‘ ’ mounting, IP-54 rating for 48 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 231 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 232 |  | Supplier price |  | Circuit breaker with IP 20 differential protection ( ), Inom – 16A (Idif – 10 mA), 2P type C QFD 2P type C |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 233 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 13.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-12 Distribution Board (Upstairs sockets, elevation + 6,600) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.14. TD-A-13 Distribution Board (Lighting , floor level + 6,600) |  |  |  |  |  |  |  |  |  |  |
| 234 |  | Supplier price |  | Lockable distribution board, built- , IP-54, for 36 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 235 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 236 |  | Supplier price |  | Load break switch IP 20, Inom - 25A, 1P QS, 1P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 237 |  | Market price |  | Automatic circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom -6A, 1P, QF (1P) type C |  |  | piece |  | 17.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-13 Distribution Board (Lighting st floor, elevation +6,600) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.15. Distribution Board TD-A-14 (Computer Room No. 1) |  |  |  |  |  |  |  |  |  |  |
| 238 |  | Supplier price |  | Lockable distribution board, surface-mounted, IP-54, for 54 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 239 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 240 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 24.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-14 Distribution Board (Computer Room No. 1) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.16. Distribution Board TD-A-15 (Computer Room No. 2) |  |  |  |  |  |  |  |  |  |  |
| 241 |  | Supplier price |  | Lockable distribution board, surface-mounted, IP-54, for 54 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 242 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 243 |  | Supplier price |  | Circuit breaker with IP 20 residual current protection ( ), Inom – 16A (Idif – 30 mA), 2P type C QFD 2P type C |  |  | piece |  | 24.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-15 Distribution Board (Computer Room No. 2) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.17. TD-A-17 Distribution Board (exhaust lighting) |  |  |  |  |  |  |  |  |  |  |
| 244 |  | Supplier price |  | Lockable distribution board, built- , IP-54, for 36 modules |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 245 |  | Supplier price |  | Load switch, IP 20, Inom – 63A, 3P QS, 3P |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 246 |  | Supplier price |  | KM electromagnetic starter – 40A (contactor) 400V, Ubobina – 400V, Icontact – 40 A |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 247 |  | Supplier price |  | Circuit breaker with combined trip mechanism (thermal, electromagnetic) IP 20, Inom - 6A, 1P type C QF 1P |  |  | piece |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total TD-A-17 Distribution Board (Lighting exhaust) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total machinery Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |