| Major repairs and modernisation of the ‘Mihai Eminescu’ Secondary School, a public institution, with cadastral number 2101101.004.01, situated in the town of Cantemir, Cantemir municipality, 10 Trandafirilor Street | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-6

nd fire alarm systems. Block A

| No. No. | Symbol, standards and resource code | Works and expenses | Unit of measurement | Quantity as per project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction- e works |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RCsB21A |  | Mechanical drilling of 5 cm diameter holes in concrete elements up to 20 cm thick |  |  | piece |  | 60.0000 |  |  |  |  |  |
| 2 |  | RpEJ09A |  | Filling of holes in walls with plaster mortar |  |  | piece |  | 60.0000 |  |  |  |  |  |
| 3 |  | RpEJ08C |  | Cutting grooves up to 5 cm deep in stone or reinforced concrete walls, 5x50 cm² |  |  | m |  | 10.0000 |  |  |  |  |  |
| 4 |  | RpEJ09D |  | Filling of grooves in walls up to 50 cmp |  |  | m |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -quote benefit |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation work |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Power supply |  |  |  |  |  |  |  |  |  |  |
| 5 |  | 10-08-001-05 |  | Receiver units: ‘ПС’ device (Back-up power supply for logic device, 4A, 24V (type IFM24160, INIM + PRCAB) ) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 6 |  | 08-01-121-1 |  | Stationary lead-acid battery, type: CB 7.0-12 (sealed battery, 12V, 17 Ah) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 7 |  | 10-08-001-05 |  | Receiver units: ‘PS’ device (addressable module with one input and one supervised output (type EM312SR, INIM)) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 8 |  | 10-08-001-05 |  | Receiver units: "ПС" device (Addressable module with 4 inputs and 4 supervised outputs (type EM344S, INIM)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 9 |  | 10-08-002-02 |  | Automatic "PS" alarms: smoke, photoelectric, radioisotope, light, standard version (addressable smoke detector (type ED100, INIM)) |  |  | piece |  | 90.0000 |  |  |  |  |  |
| 10 |  | 10-08-002-03 |  | Automatic "PS" alarms: thermal, smoke and light, in explosion-proof design (combined smoke and temperature detector, addressable (ED300, INIM type)) |  |  | piece |  | 30.0000 |  |  |  |  |  |
| 11 |  | 10-08-002-06 |  | Alarms: Fitting for alarm installation ( Detector base (type EB0010, INIM) ) |  |  | piece |  | 120.0000 |  |  |  |  |  |
| 12 |  | 10-08-002-06 |  | Alarms: Mounting bracket for alarm installation (Additional box for ceiling-mounted detectors (type EB0030, INIM) ) |  |  | piece |  | 70.0000 |  |  |  |  |  |
| 13 |  | 10-04-066-05 unit |  | Wall-mounted unit: Manual alarm (Addressable manual fire alarm trigger ( , type EC0020, INIM)) |  |  | pc |  | 13.0000 |  |  |  |  |  |
| 14 |  | 10-04-066-05 |  | Wall-mounted unit: Buzzer (Optical and acoustic signalling device, red, addressable, 101 dB, 0.08/20 mA (type ES2020, INIM)) |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 15 |  | 10-04-066-04 |  | Wall-mounted unit: Connection or junction boxes (FBOX100 type box, INIM) |  |  | piece |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Power supply Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Cable |  |  |  |  |  |  |  |  |  |  |
| 16 |  | 08-02-412-2 |  | Laying conductors in installed metal conduits and hoses: the first single-strand or multi-strand conductor in a common braid, with a cross-sectional area of up to 6 mm² |  |  | 100 m |  | 14.0000 |  |  |  |  |  |
| 17 |  | Supplier price |  | Fire alarm cable, with 2 conductors, cross-section S=0.8 mm², compliant with the national standard, with a fire resistance rating of at least 30 minutes, FE180, E30 JE-H(St)H 1x2x0.8 mm |  |  | m |  | 50.0000 |  |  |  |  |  |
| 18 |  | Supplier price |  | Fire alarm cable, with 2 conductors, cross-sectional area S=1.0 mm², complying with the national standard, with a fire resistance rating of at least 30 minutes, FE180, E30 JE-H(St)H 1x2x1.0 mm |  |  | m |  | 50.0000 |  |  |  |  |  |
| 19 |  | Supplier price |  | Fire alarm cable, 2-core, with a cross-sectional area of S = 1.5 mm², complying with the national standard, with a fire resistance rating of at least 30 minutes, FE180, E30 JE-H(St)H 1x2x1.5 mm |  |  | m |  | 1,300.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Cable Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Smoke extraction system (opening of windows and of smoke dampers) |  |  |  |  |  |  |  |  |  |  |
| 20 |  | 10-06-037-05 |  | Cabinets, boxes and junction boxes for pipework installations: Wall-mounted cabinet, dimensions, mm, up to: 640×840 (Controller for opening and closing automation components in the event of a fire, in both automatic and manual modes (type C-SV 13A, PART No. 41918Z, UCS) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 21 |  | 08-03-591-4 |  | Two-pole switch, surface-mounted type, for open-circuit installation |  |  | 100 piece |  | 0.0200 |  |  |  |  |  |
| 22 |  | Supplier price |  | Switch for manual on/off operation (FLUSH MOUNT DOUBLE PUSH BUTTON type, PART No. 41019H, UCS). |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 23 |  | AcB09A |  | Installation of electric valve actuators, for valves with a diameter of 400–600 mm (electric actuation, 24 VDC, 0.8 A, 800 mm, 400 N, Safety Stop, for wall-mounted windows (SUPERMASTER WINDOW ACTUATORS type, UCS) |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 24 |  | 08-02-412-2 |  | Laying conductors in installed metal conduits and hoses: the first single-strand or multi-strand conductor in a common braid, with a cross-sectional area of up to 6 mm² |  |  | 100 m |  | 1.5000 |  |  |  |  |  |
| 25 |  | Supplier price |  | Fire alarm cable, with 2 conductors, cross-section S=0.8 mm², complying with the national standard, with a fire resistance rating of at least 30 minutes, FE180, E30 JE-H(St)H 1x2x1.0 mm |  |  | m |  | 40.0000 |  |  |  |  |  |
| 26 |  | Supplier price |  | Fire alarm cable, with 2 conductors, cross-sectional area S=1.5 mm², complying with the national standard, with a fire resistance rating of at least 30 minutes, FE180, E30 JE-H(St)H 2x2x1.5 mm |  |  | m |  | 100.0000 |  |  |  |  |  |
| 27 |  | Supplier price |  | Cable for connecting equipment in a cabinet, with 3 -core, with a cross-sectional area of S=4.0 mm² |  |  | m |  | 10.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Smoke extraction system (opening of windows and smoke vents) Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Connection of fire alarm zones to the fire station control room |  |  |  |  |  |  |  |  |  |  |
| 28 |  | 10-02-016-01 app |  | Communication devices (dispatch and executive telecommunications switches): Dispatch or executive telecommunications switch with amplifier, capacity 5 lines (wall-mounted intercom in the safety zone, next to the lift for people with disabilities, with the ability to make audio calls (Fanvil i16V + Fanvil EX612 Flush Mount Box type) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 29 |  | 08-02-412-2 |  | Laying conductors in installed metal conduits and hoses: the first single-strand or multi-strand conductor in a common braid, with a cross-sectional area of up to 6 mm² |  |  | 100 m |  | 1.2000 |  |  |  |  |  |
| 30 |  | Supplier price |  | FTP Cat5e-4x2x0.5 cable, in accordance with GOST 31565-2012: -ng(A)-FRHF, with a fire resistance rating of at least 90 minutes, FE180 |  |  | m |  | 120.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Connection of fire alarm zones to the fire station control room Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Components , installation , cable |  |  |  |  |  |  |  |  |  |  |
| 31 |  | 08-02-409-1 |  | Vinyl-plastic conduit on existing structures, mounted on walls and columns, secured with clamps, diameter 25 mm |  |  | 100 m |  | 9.0000 |  |  |  |  |  |
| 32 |  | e price from supplier |  | D20mm PE corrugated pipe |  |  | m |  | 900.0000 |  |  |  |  |  |
| 33 |  | 08-02-390-1 |  | Plastic ducts with a width of up to 40 mm. Specifications with a value of 0 (zero) are to be determined according to the design. |  |  | 100 m |  | 1.4000 |  |  |  |  |  |
| 34 |  | Supplier price |  | PE cable duct 25x16 mm |  |  | m |  | 140.0000 |  |  |  |  |  |
| 35 |  | Supplier price |  | 90° PE corner connector, 25x16 mm, flat |  |  | piece |  | 40.0000 |  |  |  |  |  |
| 36 |  | 08-02-406-1 |  | Metalwork (Металлоизделия) |  |  | t |  | 0.0050 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total cable installation components Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport and of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -quote benefit |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Plant |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Power supply source |  |  |  |  |  |  |  |  |  |  |
| 37 |  | Supplier price |  | 24 V, 4 A backup power supply, for installation of 2 x 12 V/17 Ah batteries (type IFM24160 + PRCAB, INIM) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 38 |  | Supplier price |  | -encapsulated battery, 12 V, 17 Ah |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 39 |  | Supplier price |  | Addressable module with one input and one supervised output (type EM312SR, INIM) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 40 |  | Supplier price |  | Addressable module with 4 inputs and 4 supervised outputs (type EM344S, INIM) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 41 |  | Supplier price |  | Addressable smoke detector (ED100 type, INIM) |  |  | piece |  | 90.0000 |  |  |  |  |  |
| 42 |  | Supplier price |  | Smoke and temperature detector (combined), addressable ( ED300, INIM type) |  |  | pc |  | 30.0000 |  |  |  |  |  |
| 43 |  | Supplier price |  | Detector socket (type EB0010, INIM) |  |  | piece |  | 120.0000 |  |  |  |  |  |
| 44 |  | Supplier price |  | Additional box for installing detectors on the ceiling (type EB0030, INIM) |  |  | piece |  | 70.0000 |  |  |  |  |  |
| 45 |  | Supplier price |  | Manual fire alarm trigger, addressable (type EC0020, INIM) |  |  | piece |  | 13.0000 |  |  |  |  |  |
| 46 |  | Supplier price |  | Visual and audible alarm, red, addressable, 101 dB, 0.08/20 mA (type ES2020, INIM) |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 47 |  | Supplier price |  | Manufacturer’s box for installing addressable modules (type FBOX100, INIM) |  |  | piece |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Power supply Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Smoke extraction system (opening of windows and smoke vents) |  |  |  |  |  |  |  |  |  |  |
| 48 |  | Supplier price |  | Controller for opening and closing automation components in the event of a fire, in both automatic and manual modes, with mains power supply and batteries, with connection management, panel-mounted, U = 24 V, I = 13 A, (type C-SV 13A, PART No. 41918Z, UCS) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 49 |  | Supplier price |  | Electric actuator for opening windows in the evacuation stairwell, the sports hall and the dining hall, 24 VDC, 1.1 A, 500 mm, 300 N, Safety Stop, opens inwards, supplied as a set with fixing components for aluminium frames from the manufacturer (Slimchain L type, 24 V DC, 500 mm, EV1, ID no. 147045 + Slimchain Bracket Set A, art. 147061, GEZE). |  |  | piece |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total smoke extraction system (opening of windows and smoke vents) Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Connection of fire alarm zones to the fire station control room |  |  |  |  |  |  |  |  |  |  |
| 50 |  | Supplier price |  | Wall-mounted intercom in the safety zone, next to the lift for people with disabilities, enabling audio calls to be made via the intercom at the entrance near reception, with a call button; VoIP, SIP endpoint/IP intercom, 1 relay, IP65, IK10, PoE+ (IEEE 802.3at), with manufacturer’s installation box, compatible with IP telephony server (Fanvil i16V + Fanvil EX612 Flush Mount Box) |  |  | piece |  | 2.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Connection of fire alarm zones to the fire station control room Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total machinery Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |