| Major repairs and modernisation of the ‘Mihai Eminescu’ Secondary School, a public institution, with cadastral number 2101101.004.01, situated in the town of Cantemir, Cantemir municipality, 10 Trandafirilor Street | Form No. 7 WinСmeta |
| --- | --- |
| (name of the project) |  |

LOCAL QUOTATION No. 2-1-9

Telephone and signalling systems. Block A

| No. No. | Symbols, standards and resource code | Works and expenditure | Unit of measurement | Quantity as per project data | Estimated value, lei |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measure ———— incl. wages | Total ————— incl. salary |

| 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  |  | 6 |  | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1. Construction- e works |  |  |  |  |  |  |  |  |  |  |
| 1 |  | RCsB21A |  | Mechanical drilling of 5 cm diameter holes in concrete elements up to 20 cm thick |  |  | piece |  | 100.0000 |  |  |  |  |  |
| 2 |  | RpCU07C |  | Filling of voids in floor slabs with cement mortar, following installation of services |  |  | piece |  | 100.0000 |  |  |  |  |  |
| 3 |  | RpEJ08A |  | Cutting grooves up to 5 cm deep in walls of 5x50 cm brickwork |  |  | m |  | 300.0000 |  |  |  |  |  |
| 4 |  | RpEJ09D |  | Filling of grooves in walls up to 50 cm² |  |  | m |  | 300.0000 |  |  |  |  |  |
| 5 |  | RpEG17D |  | Dismantling of electrical panels: metal panel, cabinet, cell, console (19” telecommunications cabinet, 6–12U, mounted on the wall at a height of 1.5–2.2 m) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 6 |  | RpEC16A |  | Removal of a flush-mounted socket with copper or aluminium conductors, up to 3 m room height (RJ45 socket recessed into the wall) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 7 |  | RpED17A Electrical ation |  | Removal of cables for electricity, with a cross-section of 16–35 mmp, laid freely in trenches (Cat. 5e cable with 4x2 copper conductors, shielded, 24 AWG, solid) |  |  | m |  | 400.0000 |  |  |  |  |  |
| 8 |  | RpED17A apl |  | Removal of electrical cables, with a cross-section of 16–35 mmp, laid freely in trenches (4-fibre optical cable, installed in a technical recess) |  |  | m |  | 50.0000 |  |  |  |  |  |
| 9 |  | RpED17A1 |  | Removal of electrical cables with a cross-section of 16–35 mmp, laid loose in trenches. For cables laid in ducts (PE cable duct, 40 × 40 mm) |  |  | m |  | 100.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport ation of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | 2 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on the estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total construction works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2. Installation and assembly work |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.1. Active al equipment |  |  |  |  |  |  |  |  |  |  |
| 10 |  | 11-04-008-01 |  | Detachable and flexible unit (modules, cells, ‘ТЭЗ’), mass, kg, up to: 5 (access switch, 48 RJ45 ports, auto-negotiating 1Gbps + min 2SFR + 10 Gdps) |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 11 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, "ТЭЗ"), weight, kg, up to: 5 (access switch, 24 RJ45 ports, POE+, auto-negotiating 1Gbps + min 2SFR + 10 Gdps) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 12 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, "ТЭЗ"), weight, kg, up to: 5 (access switch, 24 RJ45 ports, POE+, auto-negotiating 1Gbps + min 2SFR + 10 Gdps) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 13 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting bracket (Wi-Fi access point, indoor) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 14 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other items, with mounting points provided (Wi-Fi access point for outdoor use) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 15 |  | 10-02-030-02 |  | ‘ЦБ’ or ‘АТС’ system telephone: wall-mounted |  |  | piece |  | 9.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total active ‘ ’ equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.2. Fixed assets |  |  |  |  |  |  |  |  |  |  |
| 16 |  | 10-06-037-01 |  | Cabinets, boxes and enclosures for pipework installations: Floor-standing cabinet, size, mm, up to: 600×600 (32U floor-standing cabinet, 600×600, 19") |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 17 |  | 10-06-037-01 |  | Cabinets, boxes and junction boxes for conduit installations: Floor-standing cabinet, size, mm, up to: 600×600 (22U floor-standing cabinet, 600×600, 19") |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 18 |  | 10-02-016-06 |  | Communication equipment (dispatch and control room telecommunications switches): Installed separately: converter or power supply unit (UPS, 19", 1U, 1550 VA/1100 W, (Eaton 5P UPS, 5P1550iR type) ) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 19 |  | 10-02-016-06 |  | Telephony equipment (dispatch and executive telecommunications switches): Installed separately: converter or power supply unit (UPS, 19", 1U, 1150 VA/770 W, (Eaton 5P UPS, 5P1150iR type)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 20 |  | 11-04-008-01 |  | Detachable and flexible unit (modules, cells, ‘TEZ’), weight, kg, up to: 5 (PDU 16A, 200–240 V, 50/60 Hz) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 21 |  | 10-04-066-07 apl |  | Wall-mounted equipment: Sockets for computer networks |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 22 |  | Supplier price |  | Electrical socket for mains connection, 220V, 16A, compatible with UPS or PDU |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 23 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, ‘TEZ’), weight, kg, up to: 5 (Cat. 5e patch panel, 24 ports, 1U) |  |  | piece |  | 12.0000 |  |  |  |  |  |
| 24 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, "ТЭЗ"), weight, kg, up to: 5 (cable organiser, 1U) |  |  | piece |  | 25.0000 |  |  |  |  |  |
| 25 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, ‘ТЭЗ’), weight, kg, up to: 5 (shelf for the DT 1 19" telecommunications cabinet, 1U) |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 26 |  | Supplier price |  | Shelf for DT.1 telecommunications cabinet, 19", 1U |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 27 |  | 11-04-008-01 |  | Removable and flexible unit (modules, cells, ‘ТЭЗ’), weight, kg, up to: 5 (ODF Panel, 19", 1U) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 28 |  | 10-01-051-28 |  | Cable and wire termination and connection: “ТСВ” cable, capacity 5×2 when terminating a cable: by soldering |  |  | 10 piece |  | 3.6000 |  |  |  |  |  |
| 29 |  | Supplier price |  | LC duplex adapter |  |  | piece |  | 36.0000 |  |  |  |  |  |
| 30 |  | Supplier price |  | LC/SM duplex fibre optic pigtail |  |  | piece |  | 36.0000 |  |  |  |  |  |
| 31 |  | Supplier price |  | LC/SM Duplex Fibre Patch Cord, 1.0 m |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 32 |  | Supplier price |  | LC/SM duplex fibre patch cord, 3.0 m |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 33 |  | Supplier price |  | Direct Attach Cable 1 m, SFP+ 10G, compatible with access switches |  |  | piece |  | 5.0000 |  |  |  |  |  |
| 34 |  | Supplier price |  | SFP+ 10 Gbps 1310nm 10km transceiver, compatible with access switches and aggregation switches |  |  | piece |  | 8.0000 |  |  |  |  |  |
| 35 |  | Supplier price |  | -cord patch cable, 0.5m, copper, UTP Cat.5e |  |  | piece |  | 288.0000 |  |  |  |  |  |
| 36 |  | Supplier price |  | -cord patch cable, 1.0m, copper, UTP, Cat.5e |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 37 |  | Supplier price |  | -Cord Patch Cable, 3.0m, copper, UTP, Cat.5e |  |  | piece |  | 140.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Passive Equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.3. Cable s and sockets |  |  |  |  |  |  |  |  |  |  |
| 38 |  | 08-02-412-2 |  | Inserting conductors into installed metal conduits and hoses: the first single-core or multi-core conductor in a common braid, with a cross-sectional area of up to 6 mm² |  |  | 100 m |  | 117.8500 |  |  |  |  |  |
| 39 |  | Supplier price |  | 8-fibre optical fibre cable, SM, OS2, single-mode 9/125 ITU-T G.652.D (Legrand LCS OS2 LSZH type) |  |  | m |  | 500.0000 |  |  |  |  |  |
| 40 |  | Supplier price |  | Cat.5e cable with 4x2 copper conductors, shielded, 24 AWG, solid, нг(А)-LSLTx (in accordance with GOST 31565-2012), box: min. 305m |  |  | m |  | 11,285.0000 |  |  |  |  |  |
| 41 |  | 10-01-051-28 |  | Cable and wire termination and connection: “ТСВ” cable, capacity 5×2 when terminating a cable: by soldering |  |  | 10 piece |  | 13.7000 |  |  |  |  |  |
| 42 |  | Supplier price |  | RJ45 Cat.5e module, for installation in RJ45 sockets |  |  | piece |  | 137.0000 |  |  |  |  |  |
| 43 |  | 10-04-066-07 apl |  | Wall-mounted equipment: Sockets for computer networks |  |  | piece |  | 87.0000 |  |  |  |  |  |
| 44 |  | Supplier price |  | Socket for 1 RJ-45 module (wall-mounted, LAN + IP telephony) |  |  | piece |  | 37.0000 |  |  |  |  |  |
| 45 |  | Supplier price |  | Socket for 2 RJ-45 modules (wall-mounted) |  |  | piece |  | 50.0000 |  |  |  |  |  |
| 46 |  | Supplier price |  | box for installation wall sockets, made of PE D80 mm, 70 mm |  |  | piece |  | 87.0000 |  |  |  |  |  |
| 47 |  | 08-03-591-12 apl |  | Blocks with three switches and one flush-mounted socket, for concealed installation (Set for installing 4 RJ45 sockets in the floor (2 RJ45 sockets + 2 sealing plates) |  |  | 100 pieces |  | 0.0800 |  |  |  |  |  |
| 48 |  | 08-03-591-12 apl |  | Blocks with three switches and one flush-mounted socket, for flush-mounted installation (Set for installing 8 RJ45 sockets in the floor) |  |  | 100 pieces |  | 0.0400 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total cables and sockets Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4. Cable installation components |  |  |  |  |  |  |  |  |  |  |
| 49 |  | 08-02-396-6 |  | Metal ducting on walls and ceilings, length 3 m |  |  | 100 m |  | 2.5200 |  |  |  |  |  |
| 50 |  | Market price |  | Unperforated metal guttering 200×50(h)×3000 mm, for installation under the ceiling in a corridor |  |  | m |  | 252.0000 |  |  |  |  |  |
| 51 |  | Market price |  | Cover for metal gutter 200 × 3000 mm |  |  | m |  | 252.0000 |  |  |  |  |  |
| 52 |  | Market price |  | Metal gutter elbow 200×50 mm, 90° horizontal, with cover |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 53 |  | Market price |  | Horizontal T-joint fitting 200×50 |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 54 |  | 08-02-396-6 |  | Metal ducting on walls and ceilings, length 3 m |  |  | 100 m |  | 2.5200 |  |  |  |  |  |
| 55 |  | Market price |  | Non-perforated metal gutter 50×50(h)×3000 mm, for installation beneath the ceiling |  |  | m |  | 252.0000 |  |  |  |  |  |
| 56 |  | Market price |  | Metal gutter cover 50 x 3000 mm |  |  | m |  | 252.0000 |  |  |  |  |  |
| 57 |  | Market price |  | Metal gutter elbow 50×50 mm, 90° horizontal, with cover |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 58 |  | Market price |  | Horizontal T-joint fitting 50×50 |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 59 |  | Supplier price |  | M8 threaded rod, L=1000 mm. |  |  | piece |  | 504.0000 |  |  |  |  |  |
| 60 |  | 08-02-152-7 |  | Metal cable structures: cable tray, mounted on a support, weigh up to: 0.4 kg |  |  | 100 pieces |  | 2.5200 |  |  |  |  |  |
| 61 |  | Supplier price |  | horizontal bracket, L=300 mm. |  |  | piece |  | 252.0000 |  |  |  |  |  |
| 62 |  | Supplier price |  | Connector for joining gutters, 230 mm |  |  | piece |  | 336.0000 |  |  |  |  |  |
| 63 |  | 08-02-409-1 |  | Vinyl-plastic pipe for use in building installations, on walls and columns, secured with clamps, diameter 25 mm |  |  | 100 m |  | 54.0000 |  |  |  |  |  |
| 64 |  | Supplier price |  | Corrugated PE pipe, diameter 20 mm |  |  | m |  | 5,400.0000 |  |  |  |  |  |
| 65 |  | 08-02-390-1 |  | Plastic channels with a width of up to 40 mm. Specifications with a value of 0 (zero) are to be determined according to the design. |  |  | 100 m |  | 3.0000 |  |  |  |  |  |
| 66 |  | Supplier price |  | PE cable duct 25x16 mm |  |  | m |  | 300.0000 |  |  |  |  |  |
| 67 |  | Supplier price |  | Elbow for PE cable duct, 25x16 mm |  |  | piece |  | 150.0000 |  |  |  |  |  |
| 68 |  | 08-01-087-3 |  | Metal structures for machinery |  |  | t |  | 0.0200 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total cable installation items Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.5. Video surveillance |  |  |  |  |  |  |  |  |  |  |
| 69 |  | 10-04-067-22 |  | Colour television equipment: Broadcast television camera (DS-2CD2147G3-LI(S2U)Y camera) |  |  | piece |  | 15.0000 |  |  |  |  |  |
| 70 |  | 10-04-067-22 |  | Colour television equipment: Broadcast television camera (DS-2CD2047G2-L(U) camera) |  |  | piece |  | 9.0000 |  |  |  |  |  |
| 71 |  | 10-04-066-07 apl |  | Wall-mounted equipment: Microphone |  |  | piece |  | 6.0000 |  |  |  |  |  |
| 72 |  | Supplier price |  | 64 GB SD Micro card, Class 10 |  |  | piece |  | 24.0000 |  |  |  |  |  |
| 73 |  | 10-01-051-28 |  | Cable and wire termination and connection: “ТСВ” cable, capacity 5×2 when terminating a cable: by soldering |  |  | 10 pieces |  | 5.0000 |  |  |  |  |  |
| 74 |  | Supplier price |  | RJ45 connector, Cat.5e, for connecting video cameras |  |  | piece |  | 50.0000 |  |  |  |  |  |
| 75 |  | 08-02-412- 2 |  | Inserting conductors into conduits and installed metal hoses: the first single-core or multi-core conductor in a common braid, with a cross-sectional area of up to 6 mm² |  |  | 100 m |  | 18.3000 |  |  |  |  |  |
| 76 |  | Supplier price |  | Cat.5e cable with 4x2 copper conductors, shielded, 24 AWG, solid, нг(А)-LSLTx (in accordance with GOST 31565-2012), box: min. 305m |  |  | m |  | 1,830.0000 |  |  |  |  |  |
| 77 |  | 08-02-409-1 |  | Vinyl-plastic pipe for use in building installations, on walls and columns, secured with clamps, diameter 25 mm |  |  | 100 m |  | 8.0000 |  |  |  |  |  |
| 78 |  | Supplier price |  | Corrugated PE pipe, diameter 20 mm |  |  | m |  | 800.0000 |  |  |  |  |  |
| 79 |  | 08-02-390-1 |  | Plastic channels with a width of up to 40 mm. Specifications with a value of 0 (zero) are to be determined according to the design. |  |  | 100 m |  | 1.0000 |  |  |  |  |  |
| 80 |  | Supplier price |  | PE cable duct 25x16 mm |  |  | m |  | 100.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Video Surveillance Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.6. Call system for staff to assist people with disabilities in the toilet |  |  |  |  |  |  |  |  |  |  |
| 81 |  | 10-02-016-02 app |  | Communication devices (dispatch and executive telecommunications switches): Dispatch or executive telecommunications switch with amplifier and stand, capacity, numbers, up to: 10 (Logic device, in metal panel (type MP-040W1, Hostcall)) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 82 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, push-button and other, with mounting preparation (Call button, IP44 (type GC-0423W1 + MP-010B1, Hostcall) ) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 83 |  | 10-01-039-06 |  | Miscellaneous parts: Relay, switch, button and other, with mounting preparation (Reset button (type GC-0421W1, Hostcall)) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 84 |  | 08-03-593- 10 units |  | Wall-mounted indicator lights (Warning light (type MP-611W1)) |  |  | 100 pieces |  | 0.0300 |  |  |  |  |  |
| 85 |  | 10-02-016-06 |  | Communication devices (dispatcher and executive telecommunications switches): Installed separately: converter or power supply unit (12V/2A power supply) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 86 |  | 08-02-412-1 |  | Feeding conductors into installed metal conduits and hoses: the first single-core or multi-core conductor in a common braid, with a cross-sectional area up to 2.5 mm² |  |  | 100 m |  | 2.4000 |  |  |  |  |  |
| 87 |  | Supplier price |  | 2-conductor cable, with a cross-sectional area of S=0.8 mm², complies with the national standard – GOST 31565-2012, JE-H(St)H-1x2x0.8 |  |  | m |  | 240.0000 |  |  |  |  |  |
| 88 |  | 08-02-409-1 |  | Vinyl-plastic pipe for use in building installations, on walls and columns, secured with clamps, diameter 25 mm |  |  | 100 m |  | 1.8000 |  |  |  |  |  |
| 89 |  | Supplier price |  | D20mm PE corrugated pipe |  |  | m |  | 180.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Toilet assistance call system for people with disabilities Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.7. School bell and timekeeping system. |  |  |  |  |  |  |  |  |  |  |
| 90 |  | 10-08-020-01 |  | Wall-mounted digital electronic clocks |  |  | piece |  | 14.0000 |  |  |  |  |  |
| 91 |  | Supplier price |  | Battery-powered wall clock with a dial diameter of 300 mm |  |  | piece |  | 14.0000 |  |  |  |  |  |
| 92 |  | 08-03-573-4 |  | Wall-mounted control cabinet (console), height, width and depth, mm, up to 600×600×350 (IP65 cabinet, 2×18 modules (36M), with N and PE connection terminals) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 93 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Residual current circuit breaker (RCCB) 2P 40 A 30 mA, type AC (model A9R11240, Schneider Electric (Acti9 iID)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 94 |  | 08-03-575-1 |  | Device or unit dismantled prior to transport (Circuit breaker, 1P, 6 A (type A9F74106, Schneider Electric (Acti9 iC60N) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 95 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (1P busbar, 12-pole, 100 A) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 96 |  | 08-03-575-1 |  | Device or apparatus dismantled prior to transport (Modular installation contactor, 2NO, 24 V AC/DC for doorbell switching (type ESB20-20N-01, ABB)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 97 |  | 10-04-066-05 |  | Wall-mounted unit: Bell (School bell, 230 V AC, IP44, for wall mounting (type 401538 (CB-4), ELMARK) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 98 |  | 08-01-082-1 |  | Assorted terminals: terminal without protective housing (Terminal/PE terminal 2.5 mm² (type UT 2.5-PE-3044076, Phoenix Contact) |  |  | 100 pieces |  | 0.2000 |  |  |  |  |  |
| 99 |  | 08-01-082-1 |  | Assorted terminals: terminal without protective housing (PE terminal 2.5 mm² (type UT 2.5-PE-3044092, Phoenix Contact) |  |  | 100 pieces |  | 0.1000 |  |  |  |  |  |
| 100 |  | 08-02-412-1 |  | Feeding conductors into installed metal conduits and hoses: the first single-core or multi-core conductor in a common braid, with a cross-sectional area up to 2.5 mm² |  |  | 100 m |  | 1.0000 |  |  |  |  |  |
| 101 |  | Supplier price |  | Cable with 3x1.5 solid copper conductors, IEC 61034, IEC 60754-1 |  |  | m |  | 100.0000 |  |  |  |  |  |
| 102 |  | 08-02-409-1 |  | Vinyl-plastic pipe for use in building installations, on walls and columns, secured with clamps, diameter 25 mm |  |  | 100 m |  | 0.4000 |  |  |  |  |  |
| 103 |  | Supplier price |  | D20mm PE corrugated pipe |  |  | m |  | 40.0000 |  |  |  |  |  |
| 104 |  | 08-02-390-1 |  | Plastic ducts with a width of up to 40 mm. The standards with a value of 0 (zero) are determined according to the design. |  |  | 100 m |  | 0.2000 |  |  |  |  |  |
| 105 |  | Supplier price |  | PE cable duct 25x16 mm |  |  | m |  | 20.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total School bell and timekeeping system. Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | e and social insurance |  |  | 24 % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Transport ation of materials |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | ation and storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Overhead costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | e profit on the estimate |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total installation works Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3. Machinery |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.1. Active equipment |  |  |  |  |  |  |  |  |  |  |
| 106 |  | Supplier price |  | Type 1 access switch, with 48 auto-negotiating 1 Gbps RJ-45 ports + min. 2 10 Gbps SFP+ ports; min. 1 GB DDR3; 512 MB NAND flash; min. throughput 130 Mpps; switching capacity 176 Gbps; min. MAC address table size 16k; min. VLAN table size 4k; QoS, SSL, RSTP, BPDU, LLDP, LACP, CLI, ACL, SNMP, DHCP snooping, IPv4 and IPv6 management, Telnet/SSH; 19", 1U; Power consumption: max. 120 W (HPE Networking Instant On Switch 48p Gigabit 2p 10GBT 2p SFP+ 1960 (JL808A)) |  |  | piece |  | 4.0000 |  |  |  |  |  |
| 107 |  | Supplier price |  | Type 2 access switch, with 24 auto-negotiating 1 Gbps RJ-45 ports + min. 2 10 Gbps SFP+ ports; min. 1 GB DDR3; 512 MB NAND flash; Min. throughput 95 Mpps; switching capacity 128 Gbps; MAC address table size min. 16k; VLAN table min. 4K; QoS, SSL, RSTP, BPDU, LLDP, LACP, CLI, ACL, SNMP, DHCP-Snooping, IPv4 and IPv6- -Management, Telnet/SSH; 19", 1U; Power consumption: max. 80 W (HPE Networking Instant On Switch 24p Gigabit 2p 10GBT 2p SFP+ 1960 (JL806A)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 108 |  | Supplier price |  | Type 3 access switch, with 24 RJ-45 POE+ auto-negotiating 1 Gbps ports + min. 2 SFP+ 10 Gbps ports; min. 1 GB DDR3; 512 MB NAND flash; min. throughput 95 Mpps; Switching capacity 128 Gbps; MAC address table size min. 32k; VLAN table min. 4K; QoS, SSL, RSTP, BPDU, LLDP, LACP, CLI, ACL, SNMP, DHCP-Snooping, IPv4 and IPv6 management, Telnet/SSH; PoE budget: 370 W; 19", 1U; Power consumption: max. 480 W (HPE Networking Instant On Switch 24p Gigabit CL4 PoE 2p SFP+ 370W 1960 (JL807A)) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 109 |  | Supplier price |  | Wi-Fi access point; wall and ceiling mounting, indoor use; min. 1 x RJ-45 1/2.5 Gbps PoE (802.3at/af); number of integrated radios – min. 3, 2.4/5.0/6.0 GHz; aggregate wireless speed – min. 10.0 Gbps; channel bandwidth 20/40/80/160/320 MHz; integrated omni-directional antennas 4 dBi for 2.4 GHz, 5.5 dBi for 5 GHz, 5.5 dBi for 6 GHz; Up to 8 SSIDs; 802.11 n/ac/ax/be standards, Wi-Fi 4/5/6/6E/7; features: DL/UL MU-MIMO, OFDMA, BSS colouring, TWT, WPA3; can be managed by network and wireless controllers (such as the Ubiquiti Unifi U7 Pro) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 110 |  | Supplier price |  | Wi-Fi access point; wall and ceiling mounting, outdoor use, dual-band, manageable via UniFi Network Controller; 1 × Gigabit Ethernet RJ-45 port, 802.3af PoE power supply, max. power consumption 9 W; IPX6 outdoor housing, operating temperature -30...+60 °C; supports IEEE 802.11a/b/g/n/ac/ax standards ( -type UniFi U6 Mesh Pro) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 111 |  | Supplier price |  | IP telephone. 2 SIP lines, HD voice, LCD screen (min. 128x48), auto-provisioning via FTP/TFTP/HTTP/HTTPS/DHCP OPT66/SIP PNP/TR-069, web management interface, protocol: SIP 2.0 over UDP/TCP/TLS, RTP/RTCP/SRTP, STUN, LLDP, 802.1x, SNTP; Power Input: PoE 802.3.af (Fanvil X1SP type) |  |  | piece |  | 9.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total active ‘ ’ equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.2. Fixed assets |  |  |  |  |  |  |  |  |  |  |
| 112 |  | Supplier price |  | 32U floor-standing cabinet, 600x600 mm, 19", black, unassembled (set includes legs and castors), with fans, with perforated front and rear doors; with at least 6 blank panels, with tool-less mounting, with 2 vertical cable management systems. |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 113 |  | Supplier price |  | 22U floor-standing cabinet, 600x600 mm, 19", black, unassembled (set includes legs and castors), with fans, with perforated front and rear doors; with at least 6 blank panels, with tool-less mounting, with 2 vertical cable management systems. |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 114 |  | Supplier price |  | UPS with battery units, 19", 1U, 1550 VA/1100 W, max. length/depth 560 mm, output: min. 6 x IEC C13 (10 A) sockets; Single-phase 220V/230V/240V (Eaton 5P UPS type, 5P1550iR) |  |  | piece |  | 2.0000 |  |  |  |  |  |
| 115 |  | Supplier price |  | UPS with battery units, 19", 1U, 1150 VA/770 W, max. length/depth 510 mm, output: min. 6 x IEC C13 (10 A) sockets; Single-phase 220V/230V/240V (Eaton 5P UPS type, 5P1150iR) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 116 |  | Supplier price |  | PDU (Power distribution unit), 1U, 16A, 200–240V, 50/60Hz, Out-min. 6xCEE 7/3, In-min 1xIEC C19/C20 or 1xIEC C13/C14, horizontal mounting for 19" rack, with safety button. |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 117 |  | Supplier price |  | Cat. 5e FTP modular patch panel, fully equipped wit, 24xRJ45, 19", 1U |  |  | piece |  | 12.0000 |  |  |  |  |  |
| 118 |  | Supplier price |  | Perforated Organiser with Cover, 1U 19" (Cable organiser), horizontal |  |  | piece |  | 25.0000 |  |  |  |  |  |
| 119 |  | Supplier price |  | ODF Panel, 19", 1U |  |  | piece |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Passive Equipment Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.3. Cable s and sockets |  |  |  |  |  |  |  |  |  |  |
| 120 |  | Supplier price |  | Kit for installing 4 RJ45 sockets in the floor (2 RJ45 sockets + 2 sealing plates) – Floor-mounted box (base), dimensions 100x122x62(h) mm (Flush-mounting box for Pop-up floor 4 modules, 054001, Legrand), Access panel/cover + Pop-Up mechanism for 4 modules, compatible with box (Pop-Up box to be equipped, 054026, Legrand), RJ45 UTP Cat5e module, Blanking plate – 1 module (077070, Legrand) |  |  | piece |  | 8.0000 |  |  |  |  |  |
| 121 |  | Supplier price |  | Set for installing 8 RJ45 sockets in the floor: Flush-mounting box (base), dimensions 100x255x62(h) mm (Flush-mounting box for Pop-up floor 2x4 modules, 054003, Legrand), Access panel/cover + pop-up mechanism for 8 modules, compatible with the box (Pop-Up box to be equipped, 054013, Legrand), RJ45 UTP Cat5e module, Blanking plate – 1 module (077070, Legrand) |  |  | piece |  | 4.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total cables and sockets Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.4. CCTV |  |  |  |  |  |  |  |  |  |  |
| 122 |  | Supplier price |  | Video camera, min. 2688 x 1520, 1/1.8" CMOS, IR = 30 m, 2.8/4.0 mm, 802.3 af Class 3, max. 9 W, with the option to store data on a video recorder, with micro-SD slot (model DS-2CD2147G3-LI(S2U)Y |  |  | piece |  | 16.0000 |  |  |  |  |  |
| 123 |  | Supplier price |  | Outdoor video camera, min. 2688 x 1520, 1/1.8" CMOS, IR = 40 m, 2.8/4.0/6.0 mm, 802.3 af Class 3, max. 7.5 W, with built-in microphone, with the option to store data on a NAS, with micro-SD slot (model DS-2CD2047G2-L(U)) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 124 |  | Supplier price |  | External microphone for installation in washrooms, POE 2.4 W, 16 kHz, omnidirectional (model: Hikvision DS-UAC-M1P) |  |  | piece |  | 6.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Video Surveillance Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.5. Call system for staff to assist people with disabilities in the toilet |  |  |  |  |  |  |  |  |  |  |
| 125 |  | Supplier price |  | Logic device, in a metal housing (type MP-040W1, Hostcall) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 126 |  | Supplier price |  | Call button, IP44 (type GC-0423W1 + MP-010B1, Hostcall) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 127 |  | Supplier price |  | Reset button (type GC-0421W1, Hostcall) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 128 |  | Supplier price |  | Warning light indicator (type MP-611W1) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 129 |  | Supplier price |  | Power supply, 12V, 2A |  |  | piece |  | 3.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total Toilet assistance call system for people with disabilities Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 3.6. School bell and timekeeping system |  |  |  |  |  |  |  |  |  |  |
| 130 |  | Supplier price |  | IP65 cabinet, 2×18 modules (36M), with N and PE connection terminals (type 13984, Schneider Electric) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 131 |  | Supplier price |  | Residual current circuit breaker (RCCB) 2P 40 A 30 mA, type AC (type A9R11240, Schneider Electric (Acti9 iID)) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 132 |  | Supplier price |  | Circuit breaker, 1P, 6 A (type A9F74106, Schneider Electric (Acti9 iC60N)) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 133 |  | Supplier price |  | 1P busbar, 12-pole, 100 A (type A9XPH112, Schneider Electric) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 134 |  | Supplier price |  | Modular installation contactor, 2NO, 24 V AC/DC for doorbell switching (type ESB20-20N-01, ABB) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 135 |  | Supplier price |  | School siren, 230 V AC, IP44, for wall mounting (type 401538 (CB-4), ELMARK) |  |  | piece |  | 3.0000 |  |  |  |  |  |
| 136 |  | Supplier price |  | Reica DIN NS35/7.5, 2000 mm (type 0801733, Phoenix Contact) |  |  | piece |  | 1.0000 |  |  |  |  |  |
| 137 |  | Supplier price |  | Terminal/Junction block 2.5 mm² (type UT 2.5-3044076, Phoenix Contact) |  |  | piece |  | 20.0000 |  |  |  |  |  |
| 138 |  | Supplier price |  | PE terminal block 2.5 mm² (type UT 2.5-PE-3044092, Phoenix Contact) |  |  | piece |  | 10.0000 |  |  |  |  |  |
| 139 |  | Supplier price |  | Terminal cover (type 3047010, Phoenix Contact) |  |  | piece |  | 6.0000 |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total School bell and timekeeping system Including salary |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Storage costs |  |  | % |  |  |  |  |  |  |  |
|  |  |  |  | Total machinery Including wages |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Total |  |  | Lei |  |  |  |  |  |  |  |
|  |  |  |  | Total estimate: Including wages |  |  |  |  |  |  |  |  |  |  |

| Prepared by |  |
| --- | --- |
| (position, signature, surname, first name) |  |
| Verified |  |
| (position, signature, surname, first name) |  |