RFQ26/03393: Supply of Consumables for the National Air Quality Laboratory in the Republic of Moldova

Annex 2: Technical Responsiveness Table

LOT 1: CONSUMABLES&CHEMICALS

| # | Position | Minimum Technical Requirements | √ Yes | √ No | Please provide the exact technical specifications/ requirements for the proposed goods, including offered Brand and make |
| --- | --- | --- | --- | --- | --- |
| I. Consumables |  |  |  |  |  |
| 1 | Teflon filters | Teflon filters, diameter 47 mm, pore size 3.0 µm, 85% porosity. Suitable for EN14902. 1 set=at least 100 pieces. |  |  |  |
| 2 | Petridishes | Petrislides (non-sterile, transparent, clear polystyrene, 51x76 mm) suitable for use with 47mm filters for ambient air monitoring, preventing damage or contamination during weighing, storage, and transport. Substitude (holder) for 50 petrislides included. 1 set= at least 100 pieces |  |  |  |
| 3 | Tweezers | Filter forceps, blunt end, stainless steel, for weighing filters. 1 set = 3 pcs |  |  |  |
| 4 | Transportation boxes with cool packs for filter transportation | Cooler bag (EPP-termo) including cool packs for filter transportation. Volume min 6 l, max. 8 l. |  |  |  |
| 2. Trace element analysis (heavy metals) PM10 and precipitation |  |  |  |  |  |
| 5 | Test tube holder | A plastic rack for 15 ml centrifuge/test tubes with min. 40 holes and max. 50 holes. |  |  |  |
| 6 | Centrifuge tubes with caps | Centrifuge tubes of 50 ml (PP/HDPE) with screw caps, with graduate, trace metal free, for diluting and storing the sample. 1 set = 25 pcs. |  |  |  |
| 7 | Test tube holder | A plastic rack for 50 ml centrifuge tubes with 15-20 holes. |  |  |  |
| 8 | Plastic tweezers | Plastic white tweezers narrow tipped, lenght >= 14.5 cm, suitable for placing a filter into the digestion vessel. 1 set = 10 pieces. |  |  |  |
| 9 | Volumetric flasks 100 ml | Plastic volumetric flasks 100 ml (PP/PE) with plastic stoppers, for solutions. |  |  |  |
| 10 | Measuring cylinder 1000 ml | Measuring cylinder made of transparent polypopylene, with scale, 1000 ml |  |  |  |
| 11 | Measuring cylinder 2000 ml | Measuring cylinder made of transparent polypopylene, with scale, 2000 ml |  |  |  |
| 12 | Measuring cylinder 100 ml | Measuring cylinder made of transparent polypopylene, with scale, 100 ml |  |  |  |
| 13 | Measuring cylinder 500 ml | Measuring cylinder with scale, made of glass, 500 ml. |  |  |  |
| 14 | Volumetric flask 5 l | Volumetric flask, made of glass, 5 l. |  |  |  |
| 15 | Teflon bottle 250 ml | Wide necked PTFE bottle for storing concentrated nitric acic, 250 ml |  |  |  |
| 16 | Dispenser | Adjustable bottle top dispenser 5 - 25 ml for filter sample extraction. Suitable for acids (HNO3). Precision ≤ ±0,7 ml. |  |  |  |
| 3. PAH (Benzo(a)pyrene measurements in air (PM10) and precipitation) |  |  |  |  |  |
| 17 | GC-column | For Agilent Technologies model 7890A. PAH GC Column. DB-5ms Ultra Inert Columns Part Number:122-5532UI Length: 30 m; Capillary Tubing:Fused Silica Film Thickness: 0.25 µm Format:7 inch Inner Diameter (ID): 0.25 mm Phase: Select PAH Polarity: Low Polarity Temperature Range: -60 °C-325/350 °C |  |  |  |
| 18 | Syringe | Manual syringe for the Agilet GC autosampler, 5 µL, fixed needle with cone tip. 1 set = 5 pieces. Needle Gauge:26, Needle Length: 50 mm |  |  |  |
| 19 | Syringe | Manual syringe, 10 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1490 or analogue Needle Gauge:26, Needle Length: 50 mm |  |  |  |
| 20 | Syringe | Manual syringe, 25 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1497 or analogue Needle Gauge:25, Needle Length: 50 mm |  |  |  |
| 21 | Syringe | Manual syringe, 50 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1503 or analogue Needle Gauge:25, Needle Length: 50 mm |  |  |  |
| 22 | Syringe | Manual syringe, 100 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1510 or analogue Needle Gauge:25, Needle Length: 50 mm |  |  |  |
| 23 | Syringe | Manual syringe, 250 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1517 or analogue Needle Gauge:25, Needle Length: 50 mm |  |  |  |
| 24 | Syringe | Manual syringe, 500 µL, PTFE tipped plunger, fixed needle with bevel tip, Part Number: 5190-1523 or analog Needle Gauge:25, Needle Length:50 mm |  |  |  |
| 25 | Vials | Vial, crimp top, clear, write-on spot, certified, 2 mL. Vial size: 12 x 32 mm (11 mm cap), Part Number: 5183-4492 or analogue, 1 pack= 1000 pieces |  |  |  |
| 26 | Vials | Vial, crimp top, amber, write-on spot, certified, 2 mL. Vial size: 12 x 32 mm (11 mm cap), Part Number: 5181-3376, 1 pack=100 pieces |  |  |  |
| 27 | Capillary bottle | Amber capillary bottle, 1.5 mL, Product Code DE-CE 01, 1 pack = 10 pieces |  |  |  |
| 28 | Caps | Cap, crimp, silver aluminium, certified, PTFE/silicone septa, 11 mm, 1,000 pieces in pack, including a press-fit septum made from red PTFE/white silicone septum bi-layer. Compatibility with Agilent Thermo. Part Number: 5183-4500 |  |  |  |
| 29 | Filament | Filament, high temperature, EI ion source, for cromatograph Agilent Technologies, Part Number:G7005-60061 |  |  |  |
| 30 | Nut for the mass selective detector | Column nut for MS interface, for the mass selective detector Agilent Technologies model 5975C, Part Number:05988-20066 |  |  |  |
| 31 | Ferrules | Capillary, 0.4 mm id, 15% graphite/85% Vespel, 0.1 to 0.25 mm column, 10/pk. For GC oven temperatures up to 350 °C.vPart number 5181-3323 |  |  |  |
| 32 | Ferrules | Ferrule, 0.4 mm id, graphite, 0.05 to 0.25 mm column, 10/pk. For GC oven temperatures up to 450 °C. Part number 500-2114 |  |  |  |
| 33 | GC pre column | GC precolumn, e.g., Agilent FS, lenght 10 m. |  |  |  |
| 34 | GC-column | PAH GC Column for the Agilent GC including a PAH analysis method. Capillary Tubing:Fused Silica Film Thickness: 0.20 µm Format:7 inch Inner Diameter (ID): 0.25 mm Length: 30 m Phase: Select PAH Polarity: Low Polarity Temperature Range: 40 °C-325/350 °C |  |  |  |
| 35 | Sample vials for GC | GC vials, 2 ml, glass and caps PTFE lined stoppers, set=100 |  |  |  |
| 36 | Pasteur pipettes | Pasteur pipettes. Set = 250 |  |  |  |
| 37 | SPE tubes | Solid phase extraction (SPE) tubes for sample cleaning, Florisil 1000 mg, 6 ml, 1 packet = 30 pcs |  |  |  |
| 38 | Syringe Filter | PTFE syringe filter 0.45 µm pore size, 13 mm diameter, housing PP, filter PES, disposable, suitable for GC samples (solvent resistant, female Luer-Lok inlet, male Luer outlet slip. Suitable for the vacuum filtration unit. set = 500 pieces. |  |  |  |
| 39 | Syringe 2.5 ml | Manual glass syringe 2.5 ml, gas tight, PTFE Luer-Lock |  |  |  |
| 40 | Volumetric flasks, 10 ml | Volumetric flask, class A, borosilicate, 10 ml, set=10 |  |  |  |
| 41 | Volumetric flasks, 50 ml | Volumetric glass, class A, borosilicate, 50 ml, set=10 |  |  |  |
| 42 | Volumetric flasks, 100 ml | Volumetric glass, class A, borosilicate, 100 ml, set=10 |  |  |  |
| 43 | Volumetric flasks, 200 ml | Volumetric glass, class A, borosilicate, 200 ml, set=10 |  |  |  |
| 44 | Volumetric flasks, 250 ml | Volumetric glass, class A, borosilicate, 250 ml, set=10 |  |  |  |
| 45 | Beakers, 50 ml | Beaker, glass, 50 ml set=10 |  |  |  |
| 46 | Beakers, 100 ml | Beaker, glass, 100 ml, set=10 |  |  |  |
| 4. Ions in air (PM2,5) |  |  |  |  |  |
| 47 | Syringes 10-13 ml | 10-13 ml disposable bulk, Luer slip syringes, non sterile, two-piece, without needle, PP barrel and PE piston, latex- and silicone oil-free. Set = 100 pcs. |  |  |  |
| 48 | Syringe Filter | PTFE syringe filter 0.45 µm pore size, 13 mm diameter, housing PP, filter PES, disposable, suitable for ion chromatography, no contamination for ions, female Luer-Lok inlet, male Luer outlet slip. Suitable for the vacuum filtration unit. set = 500 pieces. |  |  |  |
| 49 | Dispenser | Adjustable bottle top dispenser 5 - 25 ml for filter sample extraction. Precision ≤ ±0,7 ml. |  |  |  |
| 50 | Bottle for the dispenser | Bottle, 2 liter, plastic (PE/PP) that fits to the dispenser. |  |  |  |
| 51 | Filtration unit | Reusable Filtration unit, polysulfone, 500 ml upper part, 1000 ml bottle (receiver), suitable for 47 mm filters for IC eluent filtration. Two side-arms allow connection to vacuum line. Tubing adapters accept 1/4- to 5/16-inch (6 to 8 mm) I.D. vacuum tubing. |  |  |  |
| 52 | Membrane filter | Filters 0.22 µm GVDF Membrane filter, 47 mm, Millipore. Set= 1000 pcs. |  |  |  |
| 53 | Tweezers | Stainless steel, straight, narrow, flat tipped tweezers (130 ± 10 mm) for packing and unpacking filters (for mass, ions and PAH). 1 set = 10 pieces. |  |  |  |
| 5. Hg deposition |  |  |  |  |  |
| 54 | Bulk collector for Hg | Bulk precipitation collector for EN 15853 funnel (borosilicate or Teflon coated) and sampling bottle (borosilicate/Teflon, 1-2 liter) with heating and cooling down to 5 °C. An overflow bottle (HDPE) included. Diameter of the funnel 100 -300 mm. Power supply: 230 VAC, 50 Hz. max.100 VA. Equipped with 2 extra funnels and 2 extra sampling bottles (borosilicate/Teflon, 1-2 liter) with caps. |  |  |  |
| 6. EMEP precipitation and filters: ions in air (PM10) |  |  |  |  |  |
| 55 | Bulk collector | NILU (Norvegian Institute of Air Research) precipitation collector for monthly heavy metals (Pb, As, Cd, Ni) : bottle 2 - 4 L HDPE or PP, 2 screw caps. Funnel HDPE/PP, UV resistant, easy to clean. Funnel 15-20 cm diameter, 5-10 L volume, 30-50 cm height, Lid, fine inert mesh inside the funnel neck for insects, metal protection frame (bird ring), funnel - bottle adapter. Stand out of stainless steel up to 2 m high. Adjustable collection heights from 1,7-2,6 m above ground. Ground spike. 1 set consist of collector, funnel and stand |  |  |  |
| 56 | Cellulose filters 47 mm | Cellulose filters, 47 mm diameter, contaminant freem suitable for the analysis of anions and cations. 1 set=100 pieces. |  |  |  |
| 57 | Test tubes for PM10 ions | 10-13 ml plastic, test tubes, graduate not needed, suitable for the vacuum filtration unit. 2 test tubes needed for each filter sample (one used when a filter is extracted in ultarasonic bath and the another one is needed for filtration unit). 1 set = 100 pcs. |  |  |  |
| 58 | Test tube caps for PM10 | Caps suitable for above 10-13 ml test tubes. 1 set = 100 pcs. |  |  |  |
| 59 | Syringes 10 ml for PM10 | 10-12 ml disposable bulk, Luer slip syringes, non sterile, two-piece, without needle, PP barrel and PE piston, latex- and silicone oil-free. Set = 100 pcs. |  |  |  |
| 60 | Syringe Filter for PM10 | PTFE syringe filter 0.45 µm pore size, 13 mm diameter, disposable, suitable foi ion chromatography, female Luer-Lok inlet, male Luer outlet slip. 1 set = 500 pieces. |  |  |  |
| 61 | Test tube caps for PM10 | Thin colourless, multi-fit caps for above mentioned test tubes (suitable tubes Ø10-17 mm tubes) suitable for the IC needle. 1 set = 100 pcs. If an instrument autosampler with test tubes is used. |  |  |  |
| 62 | IC column for NH3 | Cations IonPac CS 12A, 4x250 mm |  |  |  |
| 63 | IC pre column for NH3 | Cations IonPac CG12A, RFIC 4x50mm |  |  |  |
| 64 | IC column for SO2,HNO3 | Anion IonPac AS 12A, 4x250 mm |  |  |  |
| 65 | IC precolumn | IonPac AG12A, RFIC 4x50mm |  |  |  |
| 7. General labware |  |  |  |  |  |
| 66 | Adjustable pipette 10-100 µl | Single channel, high precision, manually adjustable pipette for volumes 10-100 µl. Durable for acid use, needs to be self-maintainable. At least 2 year quarantee. |  |  |  |
| 67 | Pipette tips | Pipette tips for 10-100 µl adjustable pipette, set=1000. |  |  |  |
| 68 | Adjustable pipette 100-1000 µl | Adjustable pipette 100-1000 µl |  |  |  |
| 69 | Pipette tips | Pipette tips for 100 -1000 µl adjustable pipette for ions, set=1000. |  |  |  |
| 70 | Pipette tips with filter | Pipette tips with filters for 250 - 1000 µl adjustable pipette for HNO3 and acidic samples, set=1000. |  |  |  |
| 71 | Adjustable pipette 250-1000 µl | Single channel, high precision, manually adjustable pipette for volumes 100-1000 µl. Durable for acid use, needs to be self-maintainable. At least 2 year guarantee. |  |  |  |
| 72 | Pipette tips | Pipette tips for 250 - 1000 µl adjustable pipette for ions, set=1000. |  |  |  |
| 73 | Pipette tips with filter | Pipette tips with filters for 250 - 1000 µl adjustable pipette for HNO3 and acidic samples, set=1000. |  |  |  |
| 74 | Adjustable pipette 0.5-5 ml | Single channel, high precision, manually adjustable pipette for volumes 0.5-5 ml. Durable for acid use, needs to be self-maintainable. At least 2 year guarantee. |  |  |  |
| 75 | Pipette tips | Pipette tips for 0.5-5 ml adjustable pipette for ions, set=1000. |  |  |  |
| 76 | Pipette tips with filter | Pipette tips with filters for 0.5 - 5 ml adjustable pipette for HNO3 and acidic samples, set=1000. |  |  |  |
| 77 | Adjustable pipette 1-10 ml | Single channel, high precision, manually adjustable pipette for volumes 1-10 ml. Durable for acid use, needs to be self-maintainable. At least 2 year quarantee. |  |  |  |
| 78 | Pipette tips with filter | Pipette tips with filters for 1 - 10 ml adjustable pipette for HNO3 and acidic samples, set=1000. |  |  |  |
| 79 | Bottles for precipitations | PP bottles for precipitation 250 ml, 1 set=200 pcs |  |  |  |
| 80 | Bottles, 125 ml | HDPE bottles, wide neck, For cal solutions 125 ml bottles: for IC and ICP-MS. Precipitation samples: ions and HM, set =50 |  |  |  |
| 81 | Bottles, 200-250 ml | HDPE bottles, wide neck, For QC solutions 200-250 ml bottles for IC and ICP-MS, set = 100 |  |  |  |
| 82 | PE-sample bottles, 2 L | Sample bottles for heavy metals and ions (2 l, polyethylene). |  |  |  |
| 83 | PE-sample bottles, 1 L | Sample bottles for heavy metals and ions (1 l, polyethylene). |  |  |  |
| 84 | Tweezers | Stainless steel, straight, narrow, flat tipped tweezers (130 ± 10 mm) for packing and unpacking filters (for mass, ions and PAH). 1 set = 10 pieces. |  |  |  |
| 85 | Tweezers | Stainless steel, straight, narrow, blunt tipped tweezers (160 ± 10 mm) for packing and unpacking filters (for mass, ions and PAH).1 set = 10 pieces. |  |  |  |
| 86 | Wash bottles | Colourless 500 ml wash bottles, non-leaking, joints threaded neck. |  |  |  |
| 87 | Transportation boxes | Small styrox boxes (EPP-termo) (with cool pack) for transporting PAH-filters. 6-8 l. |  |  |  |
| 88 | Transportation boxes | Styrox boxes (EPP-termo) for Hg-sample (HCl included) transpotation. Size: 25-40 l. |  |  |  |
| II. CHEMICALS |  |  |  |  |  |
| 1. Trace element analysis (heavy metals) |  |  |  |  |  |
| 1 | Nitric acid, HNO3, suprapure | Nitric acid HNO3, conc. For ICP applications, purity 67% (ultrapure), assay 67–69%, Cl max. 0.2 ppm; Al max. 1 ppb; Ca max. 1 ppb; Cd max. 0.5 ppb; Cr max. 1 ppb; Cu max. 0.5 ppb; Fe max. 1 ppb; Hg max. 0.1 ppb; K max. 1 ppb; Mg max. 1 ppb; Mn max. 0.1 ppb; Na max. 1 ppb; Ni max. 0.5 ppb; Pb max. 0.1 ppb; Zn max. 0.5 ppb. Supplied in manufacturer-sealed packaging, with a certificate of quality. Packaging volume: 1 piece= 1 L |  |  |  |
| 2 | Nitric acid, HNO3 | Nitric acid, HNO3, 65%, pro analysis, Packaging volume: 2.5 L bottles |  |  |  |
| 3 | Multi Element Standard | Certified ICP multi-element standard solution with at least Pb, Cd, As, Cr, Ni, Cu, Zn, 1000 mg/l, in accordance with ISO 17034 and characterized in accordance with ISO/IEC 17025. traceable to SI unit kg and measured against primary material from an NMI, e.g. NIST. Volume: 100 ml. |  |  |  |
| 4 | Multi Element Standard | Certified ICP multi-element standard solution with at least Pb, Cd, As, Cr, Ni, Cu, Zn, 1000 mg/l, in accordance with ISO 17034 and characterized in accordance with ISO/IEC 17025. traceable to SI unit kg and measured against primary material from an NMI, e.g. NIST. Different producer or different lot than the previous one. Volume: 100 ml |  |  |  |
| 5 | Particulate reference material | Certified reference material NIST 1648a Urban particulate matter which includes heavy metals. Weight = 2 g. |  |  |  |
| 6 | Internal standard | Internal standard (certified reference material) including at least Rh, Sc, Ir 100 ml |  |  |  |
| 2. PAH (Benzo(a)pyrene measurements in air (PM10) and precipitation) |  |  |  |  |  |
| 7 | Reference material for PAH | Certified standard reference material for PAHs, including benzo(a)pyrene. NIST, Standard Reference Material 1649b "Urban dust", or equivalent. |  |  |  |
| 8 | Deuterated internal PAH standard | Deuterated internal PAH standard mix Acenaphthene D10, Chrysene D12, Naphthalene D8, Perylene D12, Phenanthrene D10, 1000-2000 ug/ml in toluene, or equivalent. |  |  |  |
| 9 | PAH standard mixture | PAH EPA method 610 standard mixture: Acenaphthene, Acenaphthylene, Benz[a]anthracene, Benzo(k)fluoranthene, Benzo[a]pyrene, Benzo[b]fluoranthene, Benzo[ghi]perylene, Chrysene, Dibenzo(a,h)anthracene, Fluoranthene, Fluorene, Indeno[1,2,3-cd]pyrene, Naphthalene, Perylene, Phenanthrene, Pyrene 100-2000 ug/ml in methanol/dichloromethane. Or equivalent. Exp. Day more than 2 years. Used for PAH standard solution, (2 standard solutionsin every two months, one standard solution is usable for two months). |  |  |  |
| 10 | Dibenzo[ac]antracene | Deuterated internal PAH standard Dibenzo[a,h]anthracene (D14,98%) 200 µg/mL in toluene-D8. Or equivalent.Exp. Day more than 2 years. 1 piece= 10 ml bottle. |  |  |  |
| 11 | Dichloromethane | Solvent for GC/MS,, purity ≥99.8% (GC), CAS No:75-09-2, 1 piece=bottle 2,5L |  |  |  |
| 12 | Hexane | Solvent for GC/MS, Purity 98 %, CAS No:110-54-3. 1 piece = bottle max 2.5 L. |  |  |  |
| 13 | Methanol | Solvent, HPLC-grade, 1 piece = 2.5 L. |  |  |  |
| 14 | Iso-octane | SupraSolv, 1 piece= bottle max 2.5 L. |  |  |  |
| 15 | Toluene | GC-grade, 1 piece = bottle max 2.5 l. |  |  |  |
| 3. Ions in air (PM2,5 and PM10) |  |  |  |  |  |
| 16 | Nitrate standard solution | Nitrate single-component standard solution (NO3 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 17 | Nitrate standard solution | Nitrate single-component standard solution (NO3 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 18 | Sulfate standard solution | Sulfate single-component standard solution (SO4 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 19 | Sulfate standard solution | Sulfate single-component standard solution (SO4 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 20 | Chloride standard solution | Chloride single-component standard solution (Cl 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 21 | Chloride standard solution | Chloride single-component standard solution (Cl 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 22 | Sodium standard solution | Sodium single-component standard solution (Na 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 23 | Sodium standard solution | Sodium single-component standard solution (Na 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 24 | Potassium standard solution | Potassium single-component standard solution (K 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 25 | Potassium standard solution | Potassium single-component standard solution (K 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 26 | Calcium standard solution | Calsium single-component standard solution (Ca 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 27 | Calcium standard solution | Calsium single-component standard solution (Ca 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Volume not less than 100 ml. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 28 | Magnesium standard solution | Magnesium single-component standard solution (Mg 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 29 | Magnesium standard solution | Magnesium single-component standard solution (Mg 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 30 | Ammonium standard solution | Ammonium single-component standard solution (NH4 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. |  |  |  |
| 31 | Ammonium standard solution | Ammonium single-component standard solution (NH4 1000 mg/L) for IC, traceable through NIST in accordance with ISO/IEC 17025 and ISO 17034. The original quality certificate including expanded uncertainty (k = 2). Factory-sealed packaging. Volume not less than 100 mL. Different manufacturer or at least different lot from the previous solution. |  |  |  |
| 32 | Methanesulfonic acid | 500 ml bottle. Supra pure or eqvivalent. |  |  |  |
| 33 | Sulphuric acid | Sulphuric acid, 1 L bottle, supra pure. |  |  |  |
| 34 | Nitric acid Dipicolonic acid concentrate | HNO3, 34 mM and dipicolonic acid 14 mM in water, 1 L bottle |  |  |  |
| 35 | Sodium Carbonate (Na2CO3) | 0.5 mol/l sodium carbonate, 500 ml |  |  |  |
| 36 | Sodium Bicarbonate (NaHCO3) | 0.5 mol/l sodium bicarbonate, 500 ml |  |  |  |
| 4. Hg analysis precipitation/depositio |  |  |  |  |  |
| 37 | Mercury standard solutions | Hg-standard solution 1000 mg/l in nitric acid (10-12%) treacable to NIST or ISO 17025/ISO 17034. 1 piece = 100 ml |  |  |  |
| 38 | Hydrochloric acid, supra pure | 37%, suprapure (max. 0,000001% Hg), puriss p.a, Reag.ACS, Reag.ISO, Reag.Ph.Eur.Assay min 37%, non-volatile matter max 0.005%, residue on ignition (as sulfates) max 0.0005%, Ammonium max 0.0001%, Cd max 0.000001%, Cr max 0.000002%Pb max 0.000002%, Zn max 0.000005%, Bromide max 0.005%, free chlorine max 0.00005% phosphate max 0.00005%, sulfate max 0.0001%, sulfate max 0.0001% heavy metals (as Pb) max 0.0001% Appearance of the substance complying, APHA max10%, extractable org.substances max 0.0005%, 1 piece=1L |  |  |  |
| 39 | HNO3 p.a. | 65.0%, CAS Number:7697-37-2 1 piece = 1L, pro analysis |  |  |  |
| 40 | Hydrochloric acid, p.a. | 37% , pro analysis, CAS Number:7647-01-0, 1 piece = 1L |  |  |  |
| 41 | Potassium bromate (KBrO3) | CAS Number:7758-01-2 1 piece= 100 g, pro analysis, solid, colorless form, with a density of 3.27 g/cm³ and a molecular weight of 167.01 g/mol. |  |  |  |
| 42 | Potassium bromide (KBr) | CAS Number:7758-02-3 1 piece= 100 g, suprapure Purity grade: ≥99.999%; Bromate(BrO₃): <10 ppm; Chlorides(Cl): <500 ppm; Iodides(I): <200 ppm; Sulfates(SO₄):<10ppm; Nitrogen(N): <2 ppm; Aluminum(Al): <0.02 ppm; Barium(Ba): <0.5 ppm; Calcium(Ca): <0.1 ppm; Cadmium(Cd): <0.005 ppm; Cerium (Ce): <0.005 ppm; Chromium (Cr): <0.005 ppm;Cobalt (Co): <0.005 ppm;Copper (Cu): <0.005 ppm;Iron (Fe): <0.01 ppm;Lanthanum (La): <0.005 ppm;Lead (Pb): <0.005 ppm;Lithium (Li): <0.5 ppm;Magnesium (Mg): <0.05 ppm;Manganese (Mn): <0.005 ppm;Nickel (Ni): <0.005 ppm;Sodium (Na): <5 ppm;Strontium (Sr): <0.1 ppm;Thallium (Tl): <0.01 ppm;Zinc (Zn): <0.01 ppm. |  |  |  |
| 43 | Ascorbic acid | >99%, pro analysis, CAS Number 50-81-7. Piece =250 g |  |  |  |
| 44 | Tin(II)chloride | SnCl2*2H2O, Assay ≥ 99% CAS Number: 10025-69-1 1 piece= 100 g Appearance Colourless crystals Substances insoluble in HCl ≤ 0.05%; Iron (Fe) ≤ 0.002%; Sodium (Na) + Potassium (K) + Calcium (Ca) ≤ 0.02%; Arsenic (As) ≤ 0.00005%; Sulfates (SO4) ≤ 0.0025%; Heavy metals ≤ 0.01% |  |  |  |
| 5. EC/OC analysis |  |  |  |  |  |
| 45 | Sucrose | Sucrose, solid, 100% pro analysis or AnalaR, 500g |  |  |  |
| 6. Cleaning and storing |  |  |  |  |  |
| 48 | Detergent for manual wash | Liquid laboratory manual wash detergent (alkaline, choride and phosphate free) for cleaning organic and inorganic contaminants |  |  |  |
| 47 | Ethanol, 96% | For cleaning purpose |  |  |  |
| 48 | Vaacuum grease | ≥ 50 g |  |  |  |

LOT 2: Gases

| # | Position | Minimum Technical Requirements | √ Yes | √ No | Please provide the exact technical specifications/ requirements for the proposed goods, including offered Brand and make |
| --- | --- | --- | --- | --- | --- |
| 1. Trace element analysis (heavy metals) |  |  |  |  |  |
| 1 | Gas system including and manometer for gas cylinder | 2-stage stainless steel regulator with displays for gas bottle pressure and working pressure 0-15 bar for TGM monitor Compatibility certificate for the regulator and manometer issued by the manufacturer |  |  |  |
| 2 | Argon 5.0 | Purity: Argon 99.999% Unit: Gas bottle size 50 L, or other relevant size 10.7 Nmc/200 bar Conformity and quality certificat(s) for gas issued by the producer Technical certificate for bottles |  |  |  |
| 2. PAH (Benzo(a)pyrene measurements in air (PM10) and precipitation) |  |  |  |  |  |
| 3 | Pressure regulator and manometer for gas cylinder | 2-stage stainless steel regulator with displays for gas bottle pressure and working pressure 0-10 bar Compatibility certificate for the regulator and manometer issued by the manufacturer |  |  |  |
| 4 | Nitrogen 5.0 | For vacuum evaporator Gas Supply nitrogen. Purity 99.998%, Volume of bottle = 50 L. Conformity and quality certificat(s) for gas issued by the producer Technical certificate for bottles |  |  |  |
| 5 | Helium 6.0 | Volume of bottle: 50L Purity: 99.999% Conformity and quality certificat(s) for gas issued by the producer, Technical certificate for bottles |  |  |  |
| 3. EC/OC Analysis |  |  |  |  |  |
| 6 | Pressure regulator and manometer for gas cylinder | 2-stage stainless steel regulator with displays for gas bottle pressure and working pressure 0-5 bar Compatibility certificate for the regulator and manometer |  |  |  |
| 7 | Calibration gas (methane 5% in helium) | Volume of bottle: 10L, Purity: 99.999%, Conformity and quality certificat(s) for gas issued by the producer, Technical certificate for bottles Safety Data Sheet |  |  |  |
| 8 | Oxygen 10% | Volume of bottle: 10L Purity: 99.999% Conformity and quality certificat(s) for gas issued by the producer, Technical certificate for bottles |  |  |  |
| 4. VOC online analyser |  |  |  |  |  |
| 9 | Calibration gas (C2-C10 NMHCs in nitrogen) | Primary reference material, calibration gas (C2-C9 NMHCs in nitrogen), concentrtations of benzene, toluene, xylene 1-5 ppb. bottle=10 L Gases concentration certificat for gas issued by the producer, Technical certificate for bottles Safety Data Sheet |  |  |  |
| 5. General consumables for gases |  |  |  |  |  |
| 10 | 1/8'' stainless steel tubing | 50 meters stainless steel tubing 1/8'' |  |  |  |
| 11 | Ferrules with nuts 1/8'' | Stainless steel ferrules and nuts for 1/8'' gas lines |  |  |  |
| 12 | Seal rings for gas bottles | Teflon seal rings for gas cylinder regulators |  |  |  |
| 13 | Seal rings for gas bottles | Aluminium seal rings for gas cylinder regulators |  |  |  |
| 14 | Bottle holders for 2 gas cylinders | Metal stands for attaching gas cylinders with metal chains |  |  |  |
| 15 | Gas cylinder trolley | Gas cylinder trolly, durable for moving gas bottles. |  |  |  |
| 16 | Cutter for gas lines | Cutter for cutting stainless steel gas tubings |  |  |  |