| Capital repair with reconstruction of roof of the National Centre of Judicial expertise, located at 2 M. Cibotari Str. 2, Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 11.1

Thermo-mechanics

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Mounting works |  |  |
| 1 | IA12B | Wall boiler for the preparation of thermal fluid (hot water 90/70 degrees), with the caloric power of 28 kw, mounted directly on the wall | piece | 2.00 |
| 2 | IA38B | Circulation (recirculation) pump mounted on existing pipe, by flanges, with the diameter of over 2" | piece | 2.00 |
| 3 | IA28A | Expansion tank, mounted on the framework, with the capacity of V=50 l | piece | 1.00 |
| 4 | IA41A | Pressure reducer for central heating installations, with fittings of 50 or 65 mm (hydraulic splitter) | piece | 1.00 |
| 5 | IA23A | Liquid fuel filter | piece | 3.00 |
| 6 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 15mm | piece | 1.00 |
|  |  | Chapter 2. Sanitation works |  |  |
| 7 | ID04B | Passage or retention valve with plugs for central heating installations, with nominal diameter of 40mm (full bore ball valve) | piece | 6.00 |
| 8 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 25 mm (full bore ball valve) | piece | 4.00 |
| 9 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 20 mm (full bore ball valve) | piece | 4.00 |
| 10 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 20 mm (non-return valve for water) | piece | 1.00 |
| 11 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 25 mm (non-return valve for water) | piece | 2.00 |
| 12 | ID04B | Passage or retention valve with plugs for central heating installations, with nominal diameter of 40 mm (non-return valve for water) | piece | 2.00 |
| 13 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 20 mm (safety group of expansion tank) | piece | 1.00 |
| 14 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 25 mm (full bore bleeder ball valve) | piece | 2.00 |
| 15 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 20 mm (stop valve SHTREMAX 3/4", from HERZ) | piece | 3.00 |
| 16 | ID04B | Passage or retention valve with plugs for central heating installations, with nominal diameter of 32 mm (stop valve SHTREMAX 1 1/4", from HERZ) | piece | 1.00 |
| 17 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 20 mm (balance valve SHTREMAX-R, straightway 3/4", from HERZ) | piece | 3.00 |
| 18 | ID04B | Passage or retention valve with plugs for central heating installations, with nominal diameter of 32 mm (balance valve SHTREMAX-R, straightway 1 1/4", from HERZ) | piece | 1.00 |
| 19 | ID04B | Passage or retention valve with plugs for central heating installations, with nominal diameter of 32 mm (anti-vibration inserts of ZKT type from) | piece | 4.00 |
| 20 | ID06A | Air vent valve with mobile key for central heating installations, with nominal diameter of 15 mm (automatic air purge valve) | piece | 4.00 |
| 21 | IC13D | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 45x2.5 mm | m | 8.00 |
| 22 | IC13C | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 38x2.5 mm | m | 10.00 |
| 23 | IC13C | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 32x2.5 mm | m | 36.00 |
| 24 | IE03B | Running pressure tightness test on pipes supplying heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 1 1/4" ... 2" | m | 54.00 |
| 25 | IE04B | Running the expansion-contraction and operation tests on pipes supplying the heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 1 1/4" ... 2" | m | 54.00 |
| 26 | IC46A | Distributor–collector for heating points and stations, mounted on readymade supporter 57x2.5 mm (collectors from electric welding steel pipe on 4 separators L=1200мм) | piece | 1.00 |
| 27 | IC42A | Supports and fixing devices for supporting pipes, boilers, devices and containers, with weight of up to 2 kg / piece | kg | 20.00 |
| 28 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=48 mm and up (heat insulation pipe of Tubolit type, from Armacel TL-48/9-DG (self-adhesive)) | m | 8.00 |
| 29 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=42 mm and up (heat insulation pipe of Tubolit type, from Armacel TL-42/13-DG (self-adhesive)) | m | 10.00 |
| 30 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=28 mm and up (heat insulation pipe of Tubolit type, from Armacel TL-28/9-DG (self-adhesive)) | m | 36.00 |
|  |  | Chapter 3. Equipment |  |  |
| 31 |  | Wall gas boiler “Termona”, 28 kWt, THERM 28 TLX.A | piece | 2.00 |
| 32 |  | Circulating pump WILO-TOP-RL 30/7,5 | piece | 2.00 |
| 33 |  | Expansion close-type tanker V=50 л, Flexcon R 50, from "Flamco" | piece | 1.00 |
| 34 |  | Hydraulic splitter from "MAGRA", WST60-21 | piece | 1.00 |
| 35 |  | Reticular water filter Dn25 mm | piece | 2.00 |
| 36 |  | Reticular water filter Dn50 mm | piece | 1.00 |
| 37 |  | Electromagnetic valve Dn15 mm, WKB from "Danfoss" | piece | 1.00 |
| 38 |  | Coaxial bend Dn60/100 <90' | piece | 1.00 |
| 39 |  | Coaxial bend Dn80/125 <90' | piece | 1.00 |
| 40 |  | Coaxial bend Dn80/125 <45' | piece | 1.00 |
| 41 |  | Flange adapter from Dn60/100 to Dn80/125 with change of places and condensate discharge | piece | 1.00 |
| 42 |  | Extension tube Dn60/100 I=0.5 m | piece | 1.00 |
| 43 |  | Extension tube Dn60/100 I=1.0 m | piece | 1.00 |
| 44 |  | Extension tube Dn80/125 I=0.5 m | piece | 1.00 |
| 45 |  | Extension tube Dn80/125 I=1.0 m | piece | 2.00 |