| Natural gas supply to the National Centre of Judicial Expertise, located at 2 M.Cebotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 3.2

Gas supply. External piping.

(name works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Mounting of networks |  |  |
| 1 | AcE51A | Connecting to existing steal pipeline (with nozzle), with nozzle diameter of 57 mm | unit | 1.00 |
| 2 | AcE51A | Connecting to existing steal pipeline (with nozzle), with nozzle diameter of 32 mm | unit | 3.00 |
| 3 | TfA01A1 | Steel pipe, mounted in the channel, 1 m deep, or above ground, 3 m high, including the cold pressure test, the leak test and the complex test with flowing fluid, with the diameter of 32x3.2 mm | m | 55.00 |
| 4 | TfA01A1 | Steel pipe, mounted in the channel, 1 m deep, or above ground, 3 m high, including the cold pressure test, the leak test and the complex test with flowing fluid, with the diameter of 20x2.8 mm | m | 48.00 |
| 5 | TfA01A1 | Steel pipe, mounted in the channel, 1 m deep, or above ground, 3 m high, including the cold pressure test, the leak test and the complex test with flowing fluid, with the diameter of 15x2.8 mm | m | 2.00 |
| 6 | GC03A | Endurance and regime test, run with air, with compressor, to check the tightness of joints and fittings, on steel pipes with Dn of 63 mm | km | 0.105 |
| 7 | IA52A | Metal box for the protection of the gas meter | piece | 1.00 |
| 8 | GB01A | Tap with flanges Pn 10-16, with Dn of 32 mm (collared ball valve GSHК-32) | piece | 2.00 |
| 9 | GB01A | Tap with flanges Pn 10-16, with Dn of 15 mm (collared ball valve GSHК-15) | piece | 1.00 |
| 10 | GB05A | Steel flanges for Pn 10,16, 25, 40, 64, mounted by electric welding to steal pipes, with the Dn of 32 mm | piece | 1.00 |
| 11 | GD05A | Welding curve, mounted to pipes, with the Dn of 32x20 mm (reduction) | piece | 1.00 |
| 12 | GD05A | Welding curve, mounted to pipes, with the Dn of 20x15 mm (reduction) | piece | 1.00 |
| 13 | GD05A | Welding curve, mounted to pipes, with the Dn of 32 mm (bend) | piece | 4.00 |
| 14 | IC42A | Supports and fixing devices to support the pipes, boilers, devices and containers, with the weight of up to 2 kg / piece | kg | 0.60 |
| 15 | IzJ07A | Applying primer of red lead in one layer to pipes and devices | m2 | 10.56 |
| 16 | IzJ09B | Dying sheet-metal cover on pipes and oil paint devices in two layers, including priming | m2 | 10.56 |
|  |  | Chapter 2. Mounting works |  |  |
| 17 | IA43B | Volumetric gas meter, with Dutch connectors, mounted on existing meter plate, the connectors having the diameter of 1 1/4" | piece | 1.00 |
|  |  | Chapter 3. Equipment |  |  |
| 18 |  | Gas Meter G6-Т | piece | 1.00 |