| Capital repair with reconstruction of roof of the National Centre of Judicial expertise, located at 2 M. Cibotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 4.1

Dismounting and recovery of layer.

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Covering asphalt sidewalk |  |  |
| 1 | DC04B | Cutting by diamond disc machine the contraction and expansion joints in worn concrete on roads | m | 100.00 |
| 2 | Dl109 | Mechanized uncapping of asphaltic concrete bedding | m3 | 1.20 |
| 3 | Dl118 | Mechanized uncapping of crushed stone bedding | m3 | 5.00 |
| 4 | DA06A1 | Layer of natural rolled aggregates having the function of filtering, insulating, ventilating, antifreeze and anti-capillary endurance, with manual ballast layering | m3 | 5.00 |
| 5 | DB16B | Asphaltic concrete bedding with small aggregate, hot execution, 3.0 cm thick, manual layering | m2 | 50.00 |
|  |  | Chapter 2. Sidewalk covering |  |  |
| 6 | DG03A | Dismounting sidewalks of concrete dales or basalt, mounted on concrete | m2 | 8.10 |
| 7 | DA06A2 | Layer of natural rolled aggregates having the function of filtering, insulating, ventilating, antifreeze and anti-capillary endurance, with manual layering, with sand | m3 | 0.81 |
| 8 | DE17A | Pavements made of sidewalk concrete plates of precast concrete 4 cm thick, layered on layer of dry blend of cement and sand, in 1:3 ratio, jointed with dry blend of cement and sand, layer 5 cm thick | m2 | 8.10 |
|  |  | Chapter 3. Asphalt layer |  |  |
| 9 | DC04B | Cutting by diamond disc machine the contraction and expansion joints in worn concrete on roads | m | 21.20 |
| 10 | Dl118 | Mechanized uncapping of crushed stone bedding | m3 | 1.70 |
| 11 | Dl109 | Mechanized uncapping of asphaltic concrete bedding | m3 | 0.34 |
| 12 | DA06A2 | Layer of natural rolled aggregates having the function of filtering, insulating, ventilating, antifreeze and anti-capillary endurance, with manual layering, with sand | m3 | 0.85 |
| 13 | DA06A1 | Layer of natural rolled aggregates having the function of filtering, insulating, ventilating, antifreeze and anti-capillary endurance, with manual ballast layering | m3 | 1.70 |
| 14 | DB16D | Asphaltic concrete bedding with small aggregate, hot execution, 4.0 cm thick, manual layering | m2 | 8.50 |
|  |  | Chapter 4. Rubble |  |  |
| 15 | TRI1AA04C3 | Loading materials of group A – light and small, by throwing – from ramp or site – into car of category 3 | t | 12.50 |
| 16 | TsI50C5 | Transporting ground by dumper of 5 t at a distance of 25 km | t | 12.50 |