| Capital repair with reconstruction of roof of the National Centre of Judicial expertise, located at 2 M. Cibotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 4.2

Exterior sewerage networks

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| 1 | TsC03B1 | Mechanic excavation of 0.40-0.70 cubic meters, with combustion-engine and hydraulic control, in ground with natural humidity, unloading into ground storage 2nd category | 100 m3 | 1.27 |
| 2 | TsA20B | Manual ground digging, in embankments, in dugs either excavated or scraped out, for completing the excavation at the embankment profile, in middle ground | m3 | 3.90 |
| 3 | TsD02A1 | Spreading of ground derived from ground of 1st or 2nd categories, executed by bulldozer on tractor on tracks of 65-80 CP, in layers 15-20 cm thick | 100 m3 | 0.79 |
| 4 | TsD05A | Compacting by mechanic beater of 150-200 kg of fillings in successive layers 20-30 cm thick, excluding watering of each layer in part, with fillings executed of non-cohesive ground | 100 m3 | 0.79 |
| 5 | TsD01B | Shovel spreading of loose ground, in uniform layers, 10-30 cm thick, by throwing up to 3 m from heaps, including lump crushing, middle ground | m3 | 19.70 |
| 6 | TsD04B | Compacting by hand beater of fillings made in horizontal excavations or inclined at ј, including watering of each ground layer in part, 10 cm thick of cohesive ground | m3 | 19.70 |
| 7 | AcF03A | Fillings in trenches for water supply or sewerage pipes, as substrate, protection layer, insulation layer or filtering layer for drainage tubes, made with sand | m3 | 24.60 |
| 8 | AcE13A | Execution of manholes of reinforced precast concrete for sewerage, circular (ring-shaped) with the diameter of 1.0 m, in ground without underground water | m3 | 3.26 |
| 9 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTSD-10 | piece | 5.00 |
| 10 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTS-10-9 | piece | 3.00 |
| 11 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTS-10-6 | piece | 6.00 |
| 12 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTS-10-2 | piece | 2.00 |
| 13 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTS-10-1 | piece | 3.00 |
| 14 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project KTSO-1 | piece | 9.00 |
| 15 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project skylight L | piece | 3.00 |
| 16 | AcE13A1 | Elements of precast reinforced concrete of manholes, circular (ring-shaped), with the diameter of 1.0 m, for sewerage, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to project skylight T | piece | 2.00 |
| 17 |  | Ladder with steel stringers | kg | 61.90 |
| 18 | CL57A | Mounting and fixing pieces encompassed in monolithic reinforced concrete: weight under 4 kg | kg | 126.88 |
| 19 | IzD05A | Manual priming with a layer of paint of red lead on technological equipment | t | 0.19 |
| 20 | IzD04A | Hand painting of metal items and constructions using oil paint in two layers, executed from profiles, thickness between 8 mm and 12 mm, inclusively | t | 0.19 |
| 21 | TsC54B | Crushed stone foundation layer | m3 | 0.57 |
| 22 | CA02C | Regular concrete poured into equalizers, slopes, screeds at heights up to 35 m inclusively, prepared by the concrete plant according to art. CA01 or commodity concrete, poured by classical means B12.5 | m3 | 0.57 |
| 23 | AcA07B | Mounting in the ground, outside the buildings, PVC pipes of types 4(G) or 3(M), with the diameter of 160 mm SN4 | m | 53.00 |
| 24 | AcA07B | Mounting in the ground, outside the buildings, PVC pipes of types 4(G) or 3(M), with the diameter of 110 mm SN4 | m | 4.50 |
| 25 |  | Warning strip | m | 61.50 |
| 26 | GD54E | Joining the pipe by electrofusion welding to polyethylene fitting (plugs, T-joint, bend) the pipes having the diameter of 160х160 mm (PVC T-joint of type SN4 SDR41) | piece | 2.00 |
| 27 | GD54E | Joining the pipe by electrofusion welding to polyethylene fitting (plugs, T-joint, bend) the pipes having the diameter of 160 mm (offset of PVC of type SN4 SDR41) | piece | 2.00 |
| 28 | RCsU05B | Execution of penetration by pipes or tension rods, at enforcements, in regular concrete walls up to 15 cm thick | piece | 1.00 |
| 29 | RCsU07C | Caulking holes in floors, using cement mortar, after the installations | piece | 1.00 |
| 30 | SA34I | Manufacturing, mounting and cementing the protection pipe at passage of pipes through walls, having the diameter of 250x9.1 mm (PN6 L=0.2m) | piece | 6.00 |
| 31 | SA34I | Manufacturing, mounting and cementing the protection pipe at passage of pipes through walls, having the diameter 225x8.2 mm (PN6 L=0.2m) | piece | 2.00 |