| Capital repair with reconstruction of roof of the National Centre of Judicial expertise, located at 2 M. Cibotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 4.3

Exterior water supply networks

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Mounting of networks |  |  |
| 1 | TsC03B1 | Mechanic excavation of 0.40-0.70 cubic meters, with combustion-engine and hydraulic control, in ground with natural humidity, unloading into ground storage 2nd category | 100 m3 | 0.52 |
| 2 | TsA20B | Manual ground digging, in embankments, in dugs either excavated or scraped out, for completing the excavation at the embankment profile, in middle ground | m3 | 1.60 |
| 3 | TsD02A1 | Spreading of ground derived from ground of 1st or 2nd categories, executed by bulldozer on tractor on tracks of 65-80 CP, in layers 15-20 cm thick | 100 m3 | 0.33 |
| 4 | TsD05A | Compacting by mechanic beater of 150-200 kg of fillings in successive layers 20-30 cm thick, excluding watering of each layer in part, with fillings executed of non-cohesive ground | 100 m3 | 0.33 |
| 5 | TsD01B | Shovel spreading of loose ground, in uniform layers, 10-30 cm thick, by throwing up to 3 m from heaps, including lump crushing, middle ground | m3 | 8.20 |
| 6 | TsD04B | Compacting by hand beater of fillings made in horizontal excavations or inclined at ј, including watering of each ground layer in part, 10 cm thick of cohesive ground | m3 | 8.20 |
| 7 | AcF03A | Fillings in trenches for water supply or sewerage pipes, as substrate, protection layer, insulation layer or filtering layer for drainage tubes, made with sand | m3 | 7.80 |
| 8 | AcE11A | Execution of sewer manholes of reinforced precast concrete for water supply circular (ring-shaped) with the diameter of 1.5 m, in ground without underground water | m3 | 1.44 |
| 9 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project KTSD-15 | piece | 1.00 |
| 10 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project KTS-15-9 | piece | 1.00 |
| 11 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project KTS-15-9а | piece | 1.00 |
| 12 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project KTSP2-15-1 | piece | 1.00 |
| 13 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project KTSО-1 | piece | 4.00 |
| 14 | AcE11A1 | Elements of precast concrete of sewer manholes, circular (ring-shaped) with the diameter of 1.5 m, for water supply, in ground without underground water. Note: the resource with the norm 0.00 (zero) is taken according to the project skylight "T" | piece | 1.00 |
| 15 |  | Ladder with steel stringers C-2 | kg | 17.08 |
| 16 | CL57A | Mounting and fixing pieces encompassed in monolithic reinforced concrete: weight under 4 kg 4 kg | kg | 20.80 |
| 17 | IzD05A | Manual priming with a layer of paint of red lead on technological equipment | t | 0.04 |
| 18 | IzD04A | Hand painting of metal items and constructions using oil paint in two layers, executed from profiles, thickness between 8 mm and 12 mm, inclusively | t | 0.04 |
| 19 | AcA52A | Polyethylene pipe for water supply pipes, mounted in the trench, with the diameter of 32 mm. Note: the type of polyethylene pipe and warning strip will be included according to project (PE100 SDR17 PN10) | m | 23.70 |
| 20 | AcF11C | Washing of PVC, cast iron, asbestos-cement etc., pipes, 20-75 mm, of drinking water after mounting and joining, before reception | m | 23.70 |
| 21 | AcF12A | Running pressure test of polyethylene pipes mounted in trenches for water supply and sewerage pipes, with the diameter up to 100 mm | m | 23.70 |
| 22 | AcA24B | Joining cast iron joining pieces or fittings to plugs, in pressure pipes, having the diameter of 250x100 mm (cast iron flange T-joint with fire stand) | piece | 1.00 |
| 23 | AcB01A | Mounting fittings of manual or mechanic action (cock with plug) in water supply or sewerage pipes, with the diameter of 25 mm (cast iron flange valve PN16) | piece | 1.00 |
| 24 | AcA31B | Mounting flanges or steel joining pieces at pipe ends with the diameter of 250 mm by electric welding | piece | 2.00 |
| 25 | AcA26A | Joining pieces to flanges, including blind flanges and fittings, having the diameter of 25 mm (loose steel flange) | piece | 1.00 |
| 26 | AcA53A | Mounting fittings by electrofusion. Joining the pipe to the fitting of polyethylene (flange sleeve) by electrofusion welding, the pipe having the diameter of 32 mm. Note: the type of polyethylene fitting (flange sleeve) will be included according to the project. | piece | 1.00 |
| 27 | GD08C | Domed cover of welded steel sheet-metal, in pipes, with the Dn 110 mm | piece | 1.00 |
| 28 | AcA25A | Mounting by electric welding of steel joints, in position, with the diameter of 25 mm (steel pipe nipple L=0.1) | piece | 1.00 |
| 29 | SA34E | Manufacturing, mounting and cementing protection pipes at passage of pipes through walls, having the diameter of 63x3.8 mm (PN6, L=0.1) | piece | 1.00 |
| 30 | CA03F | Regular concrete, poured by classical means, into foundations, caps, support walls, walls under zero share, prepared by concrete plant or commodity concrete according to art.CA01, pouring using classical means, ordinary concrete of class.... B7.5 | m3 | 0.16 |
| 31 | DF18B | Planting poles for road signs of reinforced concrete, industrially produced (informative sign of fire hydrant with fluorescent cover) | piece | 1.00 |
| 32 | AcE51F | Connection to existing pipe of steel tubes (with a nozzle), nozzle diameter 250 mm | piece | 1.00 |
| 33 | AcA10A | Mounting pressure, high density, polyethylene pipes in the ground, designed for water supply, welded head to head, according to rule I-6-PE, with the diameter 63x3.8 mm PE100 SDR25.6 PN6 | m | 16.70 |
| 34 | AcF13A | Embedding heads with bitumen and bituminous lumps on protection tubes with the diameters of 63 mm | piece | 1.00 |
| 35 | AcA53A | Mounting fittings by electrofusion. Joining the pipe to the fitting of polyethylene by electrofusion welding, the pipe having the diameter of 32 mm. Note: the type of polyethylene fitting (pipe separator) will be included according to the project. | piece | 5.00 |
|  |  | Chapter 2. Mounting works |  |  |
| 36 | AcB06B | Mounting above-ground fire hydrant on existing position (fire hydrant) | piece | 1.00 |
|  |  | Chapter 3. Equipment |  |  |
| 37 |  | Fire hydrant Н=1500, Dn125 mm | set | 1.00 |