| Capital repair with reconstruction of roof of the National Centre of Judicial Expertise, located at 2 M. Cibotari Str., Chisinau | Form Nr.1 |
| --- | --- |
| (name of objective) |  |

Bill of Quantities № 6.1

Land planning

(name of work)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Pavements |  |  |
| 1 | DI109 | Mechanized stripping of asphaltic concrete cover | m3 | 23.04 |
| 2 | TsC35C1 | Excavator with front loader, at loading distances into vehicle with frontal loader on tracks of 0.5-0.99 m3, hard and very hard rocks, up to 25 kg at distance < 10 m | 100 m3 | 0.24 |
| 3 | TsI50A10 | Transportation of ground by a dumper of 5 t at a distance of 10 km away | t | 48.00 |
| 4 | TsE06B | Preparation of ground platform for laying an insulating layer or a repair layer of sand or ballast, by manual leveling and compacting by self-propelled static compression roller, of 10-12 t, in cohesive land | 100m2 | 2.48 |
| 5 | DA12B | Foundation or re-profiling layer of crushed stone M400, for roads, GOST8267-93, fraction 5-40 mm, with mechanical bedding, executed with wedging and without mudding | m3 | 34.72 |
| 6 | DE18B | Pavements made of sidewalk plates of vibro-pressed precast concrete, with water repellence, red, plate sizes LxBxH: 200x100x60 mm, laid on a layer of dry blend of cement and sand, in a 1:6 ratio, jointed with a dry blend of cement and sand, layer 10 cm thick | m2 | 248.00 |
| 7 | DE15A | Excavation in road platform for executing the covering fitting box | m3 | 1.05 |
| 8 | DE10C | Precast concrete edgings, for sidewalks of 30x15 cm, on concrete foundation C12/15 35x20(h) cm | m | 19.00 |
|  |  | Chapter 2. Green areas |  |  |
| 9 | TsC02C1 | Mechanical excavation by wheel excavator of 0.21-0.39 mc, with hydraulic command in ground with natural humidity, unloading ground by car category I. | 100 m3 | 0.08 |
| 10 | TsI50A10 | Transportation of ground by dumper of 5 t at a distance of 10 km away | t | 13.20 |
| 11 | TsH05C | Uniform bedding of green cover, on horizontal ground or with slope of 20%, by preserving the structure, in layers of 20 cm thick | m2 | 40.00 |
| 12 | TsH09A | Sowing the lawn on horizontal areas or slopes under 30% | 100m2 | 0.40 |
| 13 | TsH12B | Watering the areas with hose from tanker | 100m2 | 0.40 |