| Capital repair with reconstruction of roof of the National Centre of Judicial Expertise, located at 2 M. Cibotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 7.4

Sewerage

(name of work)

| № crt. | Symbol of rules and resource code | Name of works | Unit of measurement | Volume |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Sanitation works |  |  |
| 1 | SC07A | Toilet pan, fully equipped, of semi porcelain, sanitary porcelain etc. including for the disabled, set on the floor, with water tank mounted at height or semi-height, with an inside syphon of S type (toilet seat for disabled of 3rd type 3; 650х500; high bowl with horizontal outflow) | piece | 1.00 |
| 2 |  | Handrail for the disabled | piece | 1.00 |
| 3 | SC04A | Washbasin of semi-porcelain, sanitary porcelain etc. including for the disabled, with drain pipe of plastic material, mounted on consoles fixed on brick or autoclaved aerated concrete (AAC) walls (ceramic washbasin for the disabled 650х590 with plastic bottle syphon) | piece | 1.00 |
| 4 | SD04A | Mixer tap with standing rocking arm for lavatory or washer, regardless of the shut-off mode, including for disabled, with the diameter of 1/2" (central tabletop cubital mixer) | piece | 1.00 |
| 5 | SC04A | Washbasin of semi-porcelain, sanitary porcelain etc. including for the disabled, with drain pipe of plastic material, mounted on consoles fixed on brick or autoclaved aerated concrete (AAC) walls (ceramic semicircular washbasin for the disabled 570х480 with plastic syphon) | piece | 6.00 |
| 6 | SD04A | Mixer tap with standing rocking arm for lavatory or washer, regardless of the shut-off mode, including for disabled, with the diameter of 1/2" (central tabletop mixer) | piece | 6.00 |
| 7 | SD04A | Mixer tap with standing rocking arm for lavatory or washer, regardless of the shut-off mode, including for disabled, with the diameter of 1/2" (central tabletop mixer (for technological equipment)) | piece | 3.00 |
| 8 | SB28A | Polypropylene floor syphon, with exit diameter of 50 mm. (plastic bottle syphon (for technological equipment)) | piece | 4.00 |
| 9 | AcA25A | Mounting steel joining pieces by electric welding, in position, with the diameter of 100x50 mm (steel funnel) | piece | 4.00 |
| 10 | SD17A | Inside hydrant, for buildings, with the diameter of 2", mounted in niches (with frame) (internal watering tap Du15 mm) | piece | 1.00 |
| 11 | SB28B | Polypropylene floor syphon, with exit diameter of 100 mm. (plastic ladder with vertical exit) | piece | 2.00 |
| 12 | SD12A | Safety valves with lever and counter-weight, for connection with threaded sleeves, with the diameter of 15 mm (automatic air vent) | piece | 2.00 |
| 13 | SC06A | Dropper washer (one compartment) for pans, of enameled cast iron, enameled sheet-metal, stainless steel etc., with drain pipe of plastic material, mounted on consoles fixed on walls of brick framework (enamel steel sink 500х500 with plastic bottle syphon) | piece | 1.00 |
| 14 | SD04A | Mixer tap with standing rocking arm for lavatory or washer, regardless of the shut-off mode, including for disabled, with the diameter of 1/2" (central tabletop mixer (central tabletop mixer for sink) | piece | 1.00 |
| 15 | SD19G | Retention valves with threaded sleeves, with diameter of 100 mm (sewerage air valve) | piece | 1.00 |
| 16 | SB05E | Non-plasticized PVC pipe of light type (U) for sewerage, joined by gumming, mounted visibly or buried under floor, with the diameter of 110 mm (sewerage PVC SN4) | m | 33.00 |
| 17 | SB08E | Sewerage plastic pipe, joined with rubber gasket, mounted visibly or buried under floor, with the diameter of 110 mm (polypropylene) | m | 35.00 |
| 18 | SB08C | Sewerage plastic pipe, joined with rubber gasket, mounted visibly or buried under floor, with the diameter of 50 mm (polypropylene) | m | 17.00 |
| 19 | SB05A | Non-plasticized PVC pipe of light type (U) for sewerage, joined by gumming, mounted visibly or buried under floor, with the diameter of 32 mm (PVC overhead water supply PN6) | m | 6.00 |
| 20 | SF04A | Running tightness and operation test for the sewerage installation made of cast iron tubes, for drainage, non-plasticized PVC pipe, of light type or of plastic material, ductile cast iron, with the diameter of up to 100 mm inclusively | 10 m | 91.00 |
| 21 | SB08E | Sewerage plastic pipe, joined with rubber gasket, mounted visibly or buried under floor, with the diameter of 110 mm (polypropylene fittings 40%) | m | 14.00 |
| 22 | SB08C | Sewerage plastic pipe, joined with rubber gasket, mounted visibly or buried under floor, with the diameter of 50 mm (polypropylene fittings 40%) | m | 7.00 |
| 23 | SA40A | Installing anti-fire sleeves by fixing to floor top with dowels (anti-fire sleeve) | piece | 3.00 |
| 24 | SA34I | Manufacturing, mounting and cementing protection pipe at passage through wall, with the diameter of 219х5.0 mm (L=1.5 m) | piece | 1.00 |
| 25 | AcA10C | Mounting pressure, high density, polyethylene pipes in the ground, designed for water supply, welded head to head, according to rule I-6-PE, with the diameter 225х8.2 mm (case of polyethylene pipes PE100 PN6) | m | 5.00 |
| 26 | AcF13C | Embedding heads with bitumen and bituminous lumps on protection tubes with the diameters of 225 mm | piece | 2.00 |
| 27 | SA37I | Bracelet for fixing water and gas supply pipes, of steel or PVC mounted by embedding, pipes having the diameter of 110 mm (polypropylene) | piece | 23.00 |
| 28 | SA37F | Bracelet for fixing water and gas supply pipes, of steel or PVC mounted by embedding, pipes having the diameter of 50 mm (polypropylene) | piece | 12.00 |
| 29 | TsA02E | Manual ground digging in limited space, under or over 1.00 m wide, executed without supports, with vertical slope, on foundations, channels, undergrounds, water drainage channels, twinning steps, in cohesive or very cohesive ground, depth < 1.5 m medium ground | m3 | 2.40 |
| 30 | TsD01B | Spreading loose ground by shovel, in uniform layers, by throwing up to 3 m away from heaps, including breaking lumps, medium ground | m3 | 2.40 |
| 31 | TsD04A | Compacting by hand-beater of filings made in horizontal or ј inclined diggings, including by watering separately each ground layer, 10 cm thick, non-cohesive ground | m3 | 2.40 |
|  |  | Chapter 2. Mounting works |  |  |
| 32 | SE52C | Manual keyboard pump, mounted on wall, with the diameter of 1 1/2" - 2" (pump from Salolift D-2 P-0.28 kWt) | piece | 1.00 |
|  |  | Chapter 3. Equipment |  |  |
| 33 |  | Pump from Salolift D-2 P-0.28 kWt, "Grundfos" | piece | 1.00 |