| Capital repair with reconstruction of roof of the National Centre of Judicial Expertise, located at 2 M. Cibotari Str., Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 7.5

Hot aqueduct T3.

(name of works)

| № crt. | Symbol of rules and resource code | Name of works | Unit of measurement | Amount | Price per unit of measurement, lei (including salary) | Total, lei (col.5 x col.6) |
| --- | --- | --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Sanitation works |  |  |  |  |
| 1 | SA17B | Tube of plastic material joint by polifusion welding in distribution pipes, in residential and social cultural buildings, with the diameter of 32x4.4 mm (polypropylene PN16) | m | 1.00 |  |  |
| 2 | SA17A | Tube of plastic material joint by polifusion welding in distribution pipes, in residential and social cultural buildings, with the diameter of 20x2.8 mm (polypropylene PN16) | m | 20.00 |  |  |
| 3 | SF01C | Running pressure tightness test of warm or cold water installation made of PVC pipe of hard type of from plastic material, with the diameter of 16-110 mm | m | 21.00 |  |  |
| 4 | SF05C | Washing of cold or warm water installation, made of pipes of plastic material, with the diameter of 20-75 mm | m | 21.00 |  |  |
| 5 | SD19A | Retention valves with threaded sleeves, with diameter of 15 mm (collared brass ball valve) | piece | 6.00 |  |  |
| 6 | RpIF09A | Insulating pipes with special insulation sleeves, dressed on the pipes, with diameter and thickness of D=32 mm (hose PEP) | m | 1.00 |  |  |
| 7 | RpIF09A | Insulating pipes with special insulation sleeves, dressed on the pipes, with diameter and thickness of D=20 mm (hose PEP) | m | 17.00 |  |  |
| 8 | SA37B | Bracelet for fixing water and gas supply pipes, of steel or PVC mounted by embedding, pipes having the diameter of 20 mm (polypropylene) | piece | 12.00 |  |  |
|  |  | Chapter 2. Mounting works |  |  |  |  |
| 9 | SD19C | Retention valves with threaded sleeves, with diameter of 40 mm | piece | 1.00 |  |  |
|  |  | Chapter 3. Equipment |  |  |  |  |
| 10 |  | Valve with rubber wedge "Tecofi" with motor controller "Bernard" V=0.03kWt, Dn40 mm | piece | 1.00 |  |  |
| 11 |  | Instantaneous tap water heater; V=3.0-4,0 l/min; N=3,0 kWt; W=220V | piece | 8.00 |  |  |