| Capital repair with reconstruction of roof of the National Centre of Judicial Expertise, located at 2 M. Cibotari Str. 2, Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 8.1

Heating

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| 1 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 22 500x500, Vogel&Noot) | piece | 6.00 |
| 2 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 22 500x600, Vogel&Noot) | piece | 11.00 |
| 3 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 22 500x700, Vogel&Noot) | piece | 6.00 |
| 4 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 22 500x800, Vogel&Noot) | piece | 11.00 |
| 5 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type22 500x900, Vogel&Noot) | piece | 4.00 |
| 6 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 22 500x1000, Vogel&Noot) | piece | 5.00 |
| 7 | IB06B | Steel radiators, single block, with length of 1001 – 1500 mm (type 22 500x1200, Vogel&Noot) | piece | 2.00 |
| 8 | IB06B | Steel radiators, single block, with length of 1001 – 1500 mm (type 22 500x1400, Vogel&Noot) | piece | 2.00 |
| 9 | IB06A | Steel radiators, single block, length up to 1000 mm inclusive (type 11 500x500, Vogel&Noot) | piece | 1.00 |
| 10 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 1/2" (angular radiator stop valve HERZ-RL-5) | piece | 48.00 |
| 11 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 1/2" (thermostatic angular valve HERZ-TS-90) | piece | 48.00 |
| 12 | ID06A | Air vent valves with mobile key for central heating installations, with nominal diameter of 1/4" (thermostatic head HERZ-"MINI" с.9000) | piece | 48.00 |
| 13 | IC40A | Bracelet for fixing steel pipes for central heating or gas installations, mounted through dowels of up to 16 mm inclusively, on PVC dowels o brick wall (connector for plastic pipes) | piece | 96.00 |
| 14 | IC35B | Reinforced polyethylene pipe of high density or of reinforced or non-reinforced polypropylene, mounted, at joints of heating bodies or devices, in central heating installations, having external diameter of 16х2.0 mm (metalloplastics PE-Xc/AI/-Xc HERZ) | m | 170.00 |
| 15 | IC35B | Reinforced polyethylene pipe of high density or of reinforced or non-reinforced polypropylene, mounted, at joints of heating bodies or devices, in central heating installations, having external diameter of 20x2.0 mm (metalloplastics PE-Xc/AI/-Xc HERZ) | m | 270.00 |
| 16 | IC35C | Reinforced polyethylene pipe of high density or of reinforced or non-reinforced polypropylene, mounted, at joints of heating bodies or devices, in central heating installations, having external diameter of 26x3.0 mm (metalloplastics PE-Xc/AI/-Xc HERZ) | m | 300.00 |
| 17 | IE03A | Running pressure tightness test on pipes supplying heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 3/8" ... 1" | m | 740.00 |
| 18 | IE04A | Running the expansion-contraction and operation tests on pipes supplying the heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 3/8" ... 1" | m | 740.00 |
| 19 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=16x10 mm (tubular insulation 10 mm thick) | m | 170.00 |
| 20 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=20x10 mm (tubular insulation 10 mm thick) | m | 270.00 |
| 21 | RpIF09B | Insulating pipes with special insulation sleeves, dressed on pipes, with diameter and thickness of D=26x10 mm (tubular insulation 10 mm thick) | m | 300.00 |
| 22 | IC38H | Fitting piece of polypropylene, with 3 joints, joined by multiple spot fusion with the reinforced polypropylene pipe, having the exterior diameter of 26x16x26 mm (T-joint under pressure) | piece | 38.00 |
| 23 | IC38H | Fitting piece of polypropylene, with 3 joints, joined by multiple spot fusion with the reinforced polypropylene pipe, having the exterior diameter of 26x16x20 mm (T-joint under pressure) | piece | 16.00 |
| 24 | IC38G | Fitting piece of polypropylene, with 3 joints, joined by multiple spot fusion with the reinforced polypropylene pipe, having the exterior diameter of 20х16х20 mm, inclusively (T-joint under pressure) | piece | 24.00 |
| 25 | IC38G | Fitting piece of polypropylene, with 3 joints, joined by multiple spot fusion with the reinforced polypropylene pipe, having the exterior diameter of 20х16х16 mm, inclusively (T-joint under pressure) | piece | 20.00 |
| 26 | IC38B | Fitting piece of polypropylene, with 2 joints, joined by multiple spot fusion with the reinforced polypropylene pipe, having the exterior diameter of 26 mm (offtake <90') | piece | 26.00 |
| 27 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 3/4" (balance valve SHTREMAX-R, straightway, HERZ company) | piece | 3.00 |
| 28 | ID04A | Passage or retention valve with plugs for central heating installations, with nominal diameter of 3/4" (stop valve SHTREMAX, HERZ company) | piece | 3.00 |
| 29 | IC13C | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 38x2.5 mm | m | 40.00 |
| 30 | IC13C | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 32x2.2 mm | m | 20.00 |
| 31 | IE03B | Running pressure tightness test on pipes supplying heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 1 1/4" ... 2" | m | 60.00 |
| 32 | IE04B | Running the expansion-contraction and operation tests on pipes supplying the heating devices (warm air heaters, thermal convectors, plinth convectors etc.) with the diameter of 1 1/4" ... 2" | m | 60.00 |
| 33 | IzJ09B | Dying sheet-metal cover on pipes and devices with oil paint in two layers, including priming | m2 | 8.00 |