| Capital repair with reconstruction of roof of the National Centre of Judicial Expertise, located at 2 M. Cibotari Str. 2, Chisinau | Form No.1 WinCmeta2000 |
| --- | --- |
| (name of object) |  |

Bill of Quantities № 8.2

Ventilation

(name of works)

| № crt. | Symbol of rules, resource code | Name of works | Measurement unit | Amount |
| --- | --- | --- | --- | --- |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
|  |  | Chapter 1. Sanitation works |  |  |
| 1 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (protective grating SG100) | piece | 2.00 |
| 2 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (blinds VK 10) | piece | 2.00 |
| 3 |  | Quickly connectable straps FK 100 | piece | 6.00 |
| 4 |  | Quickly connectable straps FK 125 | piece | 10.00 |
| 5 |  | Quickly connectable straps FK 160 | piece | 2.00 |
| 6 |  | Quickly connectable straps FK 250 | piece | 2.00 |
| 7 |  | Flexible connecting inserts ISE 225 | piece | 1.00 |
| 8 |  | Flexible connecting inserts USE 225 | piece | 1.00 |
| 9 |  | Vibration insulating support SD-PRF 160 | piece | 4.00 |
| 10 |  | Protective case VP 160 | piece | 1.00 |
| 11 |  | Flexible connecting inserts ASS-P 160 | piece | 2.00 |
| 12 | VB01A | Control valve, butterfly, mounted on circular channels of CFC-I and CFC-II types, with perimeter of 800-1600 mm (automatic air valve VKS-P 160) | piece | 1.00 |
| 13 | VB01A | Control valve, butterfly, mounted on circular channels of CFC-I and CFC-II types, with perimeter of 800-1600 mm (shutdown valve 300х300 from Belimo) | piece | 1.00 |
| 14 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (air intake DT OZR 300x300) | piece | 1.00 |
| 15 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (ventilation frame RAG 150х100) | piece | 6.00 |
| 16 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (ventilation frame RAG 150х150) | piece | 2.00 |
| 17 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (ventilation frame RAG 200х150) | piece | 9.00 |
| 18 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (ventilation frame RAG 250х150) | piece | 2.00 |
| 19 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (ventilation frame RAG 500х300) | piece | 1.00 |
| 20 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (plastic frame Р 100х100) | piece | 2.00 |
| 21 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (plastic frame Р 150х100) | piece | 1.00 |
| 22 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (plastic frame Р 150х150) | piece | 2.00 |
| 23 | VB15B | Frame with simultaneously adjustable blinds, readymade, with perimeter of 800 - 2500 mm, mounted on the channel (plastic frame Р 200х200) | piece | 2.00 |
| 24 | VB27A | Protective cap for circular channels with the perimeter of 230 – 700 mm (umbrella ЗК 00.000) | piece | 1.00 |
| 25 | CL20A | Ventilation frames, readymade, of black plate, with manually adjustable blinds, dyed and mounted in the brickwork (door grill МВМ475х80) | piece | 6.00 |
| 26 |  | Window ventilator PО 400 (air supplying device) | piece | 34.00 |
| 27 | VA02A | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of rectangular section of 250–700 mm (rectangular ducts 0.5mm) | m2 | 8.50 |
| 28 | VA02E | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of circular section of 250–700 mm (round ducts 0.5mm) | m2 | 50.27 |
| 29 | VA02F | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of circular section of 700 - 1600 mm (round ducts 0.5mm) | m2 | 7.85 |
| 30 | VA02B | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of rectangular section of 700–1600 mm (rectangular ducts 0.7mm) | m2 | 37.30 |
| 31 | VA02C | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of rectangular section of 1600–2500 mm (rectangular ducts 0.7mm) | m2 | 10.20 |
| 32 | VA02E | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of rectangular section of 250–700 mm (round ducts 0.7mm) | m2 | 21.07 |
| 33 | VA02F | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of circular section of 700-1600 mm (round ducts 0.7mm) | m2 | 7.07 |
| 34 | IzH05B | Insulating pipes with matrasses of glass wool, mineral wool of type I or type P sewed on front, on zinc wire braid, manufactured on the site, 40 mm thick, on pipes with circumference of over 35 cm over thermal insulation (plates of mineral wool of synthetic binder of 125 type) (insulation of ducts 30 mm thick) | m2 | 38.60 |
| 35 | IzH05B | Insulating pipes with matrasses of glass wool, mineral wool of type I or type P sewed on front, on zinc wire braid, manufactured on the site, 40 mm thick, on pipes with circumference of over 35 cm over thermal insulation (plates of mineral wool of synthetic binder of 125 type) (insulation of ducts 100 mm thick) | m2 | 11.00 |
| 36 | VA11D | Special piece (reduction), of black plate of 0.5 - 1.5 mm thick, with perimeter of rectangular section of 1600–2500 mm (stainless steel passage, 0.1 mm thick, 600х250-1400х300) | m2 | 2.50 |
| 37 | VA02F | Manufacturing and mounting straight ventilation channels, of zinc or aluminum plates of 0.3–2 mm thick, with perimeter of circular section of 700-1600 mm (round ducts 1.0mm) | m2 | 44.24 |
| 38 | VB27A | Protective cap for circular channels with the perimeter of 230 – 700 mm (umbrella №1 thickness 0.1 GOST 19903-74) | piece | 1.00 |
| 39 | VC34A | Manufacturing and mounting of support and anchorage device, for devices, channels, special pieces etc. of section steel, with weight per piece of up to 5 kg (steel for duct mounting) | kg | 30.00 |
| 40 | IC13D | Black steel pipe welded longitudinally for installations with screw-thread and valve screwed to device joints, in central heating installations, pipe having the diameter of 45x2.5 mm (Freon-duct) | m | 30.00 |
| 41 | IC35B | Reinforced polyethylene pipe of high density or of reinforced or non-reinforced polypropylene, mounted, at joints of heating bodies or devices, in central heating installations, having external diameter of 20.0 mm (condensate drain pipe ПВХ 20 mm) | m | 120.00 |
| 42 | IC35C | Reinforced polyethylene pipe of high density or of reinforced or non-reinforced polypropylene, mounted, at joints of heating bodies or devices, in central heating installations, having external diameter of 25.0 mm (PVC condensate drain pipe 20 mm) | m | 10.00 |
|  |  | Chapter 2. Mounting works |  |  |
| 43 |  |
| 44 | VC37A | Mounting of household air-conditioners (split system), engine power up to 4.5 kw, from the stairs (wall air-conditioner 1.27 кWt, Kentatsu, exterior set–KSRMA21HZAN1, interior set–KSGMA21HZAN1) | piece | 22.00 |
| 45 | VC37D | Mounting of household air-conditioners (split system), engine power up to 8.5 kw, from the stairs (duct air-conditioner 5.35 кWt, Kentatsu, exterior set -KSUT140HFAN3, interior set -KSKR140HFAN3) | piece | 1.00 |
| 46 | VC37A | Mounting of household air-conditioners (split system), engine power up to 4.5 kw, from the stairs (wall air-conditioner 1.71 кWt, Kentatsu, exterior set -KSRMA35HZAN1, interior set -KSGMA35HZAN1) | piece | 2.00 |
| 47 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (duct ventilator К 125 ЕС SILEO) | piece | 5.00 |
| 48 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (duct ventilator К 100 ЕС SILEO) | piece | 2.00 |
| 49 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (duct ventilator К 250 ЕС SILEO) | piece | 1.00 |
| 50 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 (duct ventilator DKEX 225-4) | piece | 1.00 |
| 51 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (centrifuge ventilator PRF 160D2 IE2) | piece | 1.00 |
| 52 | VC05B | Mounting of axial ventilators, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (duct ventilator К 160 ЕС SILEO) | piece | 1.00 |
| 53 | VC05B | Mounting of axial ventilator, with clover sheet pallets, in axial anti-explosive construction, low pressure, both engaged directly with coupling, with debit of 6.000-20.000 mc/h, electric engine of 0.77 – 5.5 kw (duct ventilator КV 100 ЕС SILEO) | piece | 2.00 |
| 54 | VB18A | Noise suppression device, rectangular with perimeter 1000 - 2000 mm (sound attenuator LDС 125-600) | piece | 3.00 |
| 55 | VB18A | Noise suppression device, rectangular with perimeter 1000 - 2000 mm (sound attenuator LDС 160-600) | piece | 1.00 |
|  |  | Chapter 3. Equipment | piece |  |
| 56 |  | Wall conditioner 1.27 кWt, Kentatsu, exterior set -KSRMA21HZAN1, interior set -KSGMA21HZAN1 | piece | 22.00 |
| 57 |  | Duct conditioner 5.35 кWt, Kentatsu, exterior set -KSUT140HFAN3, interior set -KSKR140HFAN3 | piece | 1.00 |
| 58 |  | Wall conditioner 1.71 кWt, Kentatsu, exterior set -KSRMA35HZAN1, interior set -KSGMA35HZAN1 | piece | 2.00 |
| 59 |  | Duct ventilator К 125 ЕС SILEO | piece | 5.00 |
| 60 |  | Duct ventilator К 100 ЕС SILEO | piece | 2.00 |
| 61 |  | Duct ventilator К 250 ЕС SILEO | piece | 1.00 |
| 62 |  | Duct ventilator DKEX 225-4 | piece | 1.00 |
| 63 |  | Centrifuge ventilator PRF 160D2 IE2 | piece | 1.00 |
| 64 |  | Duct ventilator К 160 ЕС SILEO | piece | 1.00 |
| 65 |  | Duct ventilator КV 100 M SILEO | piece | 2.00 |
| 66 |  | Sound attenuator LDС 125-600 | piece | 3.00 |
| 67 |  | Sound attenuator LDС 160-600 | piece | 1.00 |