Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ Construction works – Park development /

Bid value: USD

Date: 27.04.2020

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Demolishing works |  |  |  |  |  |
| 1 | DG01B | Displacing the paving or the foundation of raw stone, cobbles or chipping of raw stone or cobbles laid on concrete | m2 | 480,00 |  |  |  |
| 2 | DG04B | Displacing the edging of stone or concrete of any size, laid on concrete | m | 1 283,00 |  |  |  |
| 3 | RpCB18G | Demolishing the old concrete with mechanic means, reinforced concrete | m3 | 128,50 |  |  |  |
| 4 | RpCS20G | Dismantling the metal, simple, wrought or ornamental fencing (metal fence) | kg | 8 850,00 |  |  |  |
| 5 | RpCP45B | Dismantling (stalls, meta dome, metal box., ....) | kg | 750,00 |  |  |  |
| 6 | DI109 | Mechanized scrapping of asphalt concrete covering | m3 | 433,00 |  |  |  |
| 7 | TsC03G1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, and unloading in motor-cars (foundation under asphalt) | 100 m3 | 16,48 |  |  |  |
| 8 | TrI1AA02C2 | Loading materials from group A – heavy, in lumps, by throwing – from the ramp or field, in trucks category 2 | t | 1 326,00 |  |  |  |
| 9 | TsI50A10 | Transportation of the ground with a 5t dumper at a distance of 10 km | t | 1 326,00 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 2. Paving |  |  |  |  |  |
|  |  | Chapter 2.1. Stage I |  |  |  |  |  |
| 10 | TsE04A | Levelling the natural land field and the groundwork platforms with bulldozer on wheeled tractor 65-80 hp, by cutting the bumps and pushing the dug soil in the holes, land cat. I and II | 100m2 | 55,25 |  |  |  |
| 11 | DA06B2 | Layer of cylindered natural aggregates, having the function of filtering, insulation, ventilation, anti-freezing and anti-capillary resistance, with mechanic coverage, with sand (50 mm) | m3 | 276,27 |  |  |  |
| 12 | DA12B | Foundation or re-profiling layer of crushed stone, for roads, M-400, with mechanic coverage, executed with breading without renewal - (150 mm) | m3 | 828,80 |  |  |  |
| 13 | DE10A | Pre-manufactured concrete borders, for pavements, pressed through vibration, 100x30x15 cm, on concrete foundation 30x15 cm | m | 70,00 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 2.2. Stage II |  |  |  |  |  |
|  |  | Chapter 2.2.1. PV-1 |  |  |  |  |  |
| 14 | DA06B2 | Layer of cylindered natural aggregates, having the function of filtering, insulation, ventilation, anti-freezing and anti-capillary resistance, with mechanic coverage, with sand (50 mm) | m3 | 13,45 |  |  |  |
| 15 | DA12B | Foundation or re-profiling layer of crushed stone, for roads, M-400, with mechanic coverage, executed with breading without renewal - (150 mm) | m3 | 40,34 |  |  |  |
| 16 | CA02C | Simple concrete poured in equalization, slabs at the height of 35m inclusively, prepared with the concrete plant according to art. CA01 or concrete-commodity, poured with classical means (100 mm) | m3 | 26,89 |  |  |  |
| 17 | CG29A | Wear layer of washed concrete | m2 | 268,90 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 2.2.2. PV- 2, 6 |  |  |  |  |  |
| 18 | DE18B | Pavement made of precast concrete paving slabs laid on a layer of dry cement and sand mixture in the proportion 1: 6, embroidered with dry mixture of cement and sand, 10 cm thick layer (vibro-pressed concrete paving – gray breaks 20*10*4) | m2 | 5 516,25 |  |  |  |
| 19 | DE11A | Small edging, precast from concrete with section 10x15 cm, for framing green spaces, sidewalks, alleys, etc., placed on a concrete foundation – B 7,5, (grey edging B5 100*20*8) | m | 2 346,00 |  |  |  |
| 20 | DE11A | Small edging, precast from concrete with section 10x15 cm, for framing green spaces, sidewalks, alleys, etc., placed on a concrete foundation (concrete B7,5 - 88,92 m3) - (vibro-pressed grey edging B5 100*25*8) | m | 2 650,00 |  |  |  |
| 21 | DE18B | Pavement made of precast concrete paving slabs laid on a layer of dray cement and sand mixture, in the proportion 1:6, embroidered with dry mixture of cement and sand, 10 cm thick layer (Tactile model paving for blind persons 30*30*6 cm of yellow color) | m2 | 425.00 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 2.2.3. PV-3 |  |  |  |  |  |
| 22 | DE18B | Pavement made of precast concrete paving slabs laid on a layer of dry cement and sand mixture in the proportion 1: 6, embroidered with dry mixture of cement and sand, 10 cm thick layer (vibro-pressed concrete paving – black breaks 20*10*4) | m2 | 2 266,20 |  |  |  |
|  |  | Chapter 2.2.4. PV-4 |  |  |  |  |  |
| 23 | DE18B | Pavement made of precast concrete paving slabs laid on a layer of dry cement and sand mixture in the proportion 1: 6, embroidered with dry mixture of cement and sand, 10 cm thick layer (vibro-pressed concrete paving – grey breaks 20*10*6) | m2 | 1 473,20 |  |  |  |
|  |  | Chapter 2.2.5. PV-5 |  |  |  |  |  |
| 24 | DE18B | Pavement made of precast concrete paving slabs laid on a layer of dry cement and sand mixture in the proportion 1: 6, embroidered with dry mixture of cement and sand, 10 cm thick layer (vibro-pressed concrete paving – black breaks 20*10*6) | m2 | 719,80 |  |  |  |
|  |  | Total Chapter 2 |  |  |  |  |  |
|  |  | Chapter 3. Road marking |  |  |  |  |  |
| 25 | 47-02-040-3 | Manual digging of holes, size 0.6x0.6 in light soil | 100 pieces | 0,23 |  |  |  |
| 26 | DF23A | Planting pillars for signs, made of galvanized metal | piece | 23,00 |  |  |  |
| 27 | DF19A | Assembling the road signs made of steel or aluminum plates on ready-planted pillars | piece | 23,00 |  |  |  |
|  |  | Total Chapter 3 |  |  |  |  |  |
|  |  | Chapter 4. Greening |  |  |  |  |  |
| 28 | DE11A | Small edging, precast from concrete with section 10x15 cm, for framing green spaces, sidewalks, alleys, etc., placed on a concrete foundation (Decorative vibro-pressed colored edging 47*25*7 model Iacobas, for flowerbeds) | m | 580,00 |  |  |  |
| 28.1 | Market price | Decorative gravel 8 – 35 mm, (white, red, green, purple, M-600-700) | kg | 3800.00 |  |  |  |
| 29 | TsH05C | Uniform laying of vegetal soil on horizontal fields or fields with a slope of 20%, preserving the structure, in layers of 20 cm thickness | m2 | 3 884,00 |  |  |  |
| 30 | Market price | Vegetal soil | m3 | 776,80 |  |  |  |
| 31 | TsH09A | Sowing lawns on horizontal surfaces or in slopes under 30% | 100m2 | 388,40 |  |  |  |
| 31.1 | 47-02-040-10 | Manual digging of wholes, 1.0x0.60 (m) in medium category of ground | 100 pieces | 1,55 |  |  |  |
| 32 | TsH26E | Planting the hedge, Buxus in one row ( h=0.4-0.6 m) | m | 800,00 |  |  |  |
| 33 | TsH30D | Planting perennial plants with ballots in the field | 100 pieces | 4,00 |  |  |  |
| 34 | TsH39B | Ornamental rockery made of isolated pieces with medium size of 50 cm | piece | 100,00 |  |  |  |
| 35 | TsH110A | Planting trees in ready-dug holes: deciduous trees (Picea pungens Glauca) (h=1.1-1.5 m) | piece | 20,00 |  |  |  |
| 36 | TsH110A | Planting trees in ready-dug holes: deciduous trees (Pyramidalis Compacta) (h=1.5-1.8 m) | piece | 30,00 |  |  |  |
| 37 | TsH110B | Planting trees in ready-dug holes: resinous trees (Butela Pendula) (h=2.8-3.0 m) | piece | 15,00 |  |  |  |
| 38 | TsH110B | Planting trees in ready-dug holes: resinous trees (Weeping Willow) (h=1.8-2.0 m) | piece | 10,00 |  |  |  |
| 39 | TsH110B | Planting trees in ready-dug holes: resinous trees (Turtuosa Nana) (h=2.2-2.5 m) | piece | 20,00 |  |  |  |
| 40 | TsH110B | Planting trees in ready-dug holes: resinous trees (Sharp-leaved maple "Columnare") (h=2.8-3.0 m) | piece | 40,00 |  |  |  |
| 41 | TsH110B | Planting trees in ready-dug holes: resinous trees (Sacura) (h=2.2-2.5 m) | piece | 20,00 |  |  |  |
| 42 | TsH137A | Planting perennial flowers with ground ballots according to the scheme 0.25 x 0.25 m. in soil of category I-a | 100 pieces | 100,00 |  |  |  |
| 43 | TsH24B | Planting of roses and thorny shrubs, without ballots | piece | 500,00 |  |  |  |
|  |  | Total Chapter 4 |  |  |  |  |  |
|  |  | Total works |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Transportation |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Overhead costs |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Estimate benefit |  |  |  | % |  |
|  |  | Total |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ Outdoor water supply networks /

Bid value: USD

Date: 27.04.20

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Value of equipment |  |  |  |  |  |
| 1 | Market price | Ultrasound water-meter d15 mm | piece | 1,0000 |  |  |  |
| 2 | Market price | Mobile trolley with wheels and hose L=20m | piece | 2,0000 |  |  |  |
|  |  | Chapter 2. Assembling works |  |  |  |  |  |
| 3 | AcB04D | Installing the water meter on the embedded concession tap, D 25 mm | piece | 1,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3. Construction works |  |  |  |  |  |
|  |  | Chapter 3.1. Earthworks |  |  |  |  |  |
| 4 | TsC03B1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in the storage of ground cat. II | 100 m3 | 10,6800 |  |  |  |
| 5 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 33,0000 |  |  |  |
| 6 | AcF03A | Fillings in the trenches of the pipes for water supply or sewerage, as substrate, protection layer, insulating layer or filtering layer for the drainage tubes, made with sand | m3 | 91,5000 |  |  |  |
| 7 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor cars of ground cat. II | 100 m3 | 1,0200 |  |  |  |
| 8 | TsI50A5 | Transportation of the ground with the 5t dumper at a distance of 5 km | t | 173,4000 |  |  |  |
| 9 | TsC51B | Unloading soil in storage, ground category II | 100 m3 | 1,0200 |  |  |  |
| 10 | TsD01B | Spreading with the shovel of light soil in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from middle ground | m3 | 269,0000 |  |  |  |
| 11 | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of soil separately, with the thickness of 10 cm of cohesive ground | m3 | 269,0000 |  |  |  |
| 12 | TsD02A1 | Spreading the loose land coming from the fields of category I and II, executed with caterpillar tractor-based bulldozer 65-80 HP, in layers with thickness of 15-20 cm | 100 m3 | 7,3000 |  |  |  |
| 13 | TsD05B | Compacting with the mechanical knocker of 150-200 kg filling in the successive layers of 20-30 cm thickness, excluding the watering of every layer separately, the earth fillings being executed from cohesive ground | 100 m3 | 7,3000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3.2. Accessory pipes |  |  |  |  |  |
| 14 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Bend t FI dn. 25 x 3/4" / 10 A compression | piece | 46,0000 |  |  |  |
| 15 | SD17A | Garden hydrant installed in the ground with diameter 1" (Watering installation hydrant (cap 3/4) Gardena | piece | 46,0000 |  |  |  |
| 16 | AcA52A | Water supply polyethylene pipe mounted in ditch, with diameter of 20, 25, 32, 40, 50, 63 mm. PE80 PN10 d25 mm | m | 1 120,0000 |  |  |  |
| 17 | AcA52A | Water supply polyethylene pipe mounted in ditch, with diameter of 20, 25, 32, 40, 50, 63 mm. PE80 PN10 d32 mm | m | 710,0000 |  |  |  |
| 18 | Market price | Warning tape | m | 1 830,0000 |  |  |  |
| 19 | AcF11C | Washing the PVC, cast iron, asbestos -cement, polyethylene, etc. pipes of 20-75 mm, for drinking water after installation and coupling, before reception | m | 1 830,0000 |  |  |  |
| 20 | AcF12A | The pressure-test for the polyethylene pipes mounted in channels for the water and sewerage supply, with diameter up to 100 mm | m | 1 830,0000 |  |  |  |
| 21 | AcA52A | Water supply polyethylene pipe mounted in ditch, with diameter of 20, 25, 32, 40, 50, 63 mm. PE80 PN10 d25 mm (to install the garden hydrant, L=0.8m-46 piece) | m | 36,8000 |  |  |  |
| 22 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction sleeve PE D25 mm | piece | 46,0000 |  |  |  |
| 23 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction bend PE D25 mm | piece | 46,0000 |  |  |  |
| 24 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction T-bend PE D25 mm | piece | 18,0000 |  |  |  |
| 25 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction T-bend PE D32 mm | piece | 3,0000 |  |  |  |
| 26 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduced reduction sleeve PE D32/25 mm | piece | 4,0000 |  |  |  |
| 27 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction T-bend PE D32/25/32 mm | piece | 27,0000 |  |  |  |
| 28 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction inlet PE D25/32 mm | piece | 2,0000 |  |  |  |
| 29 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Hydrant connection cap PE d25 mm | piece | 2,0000 |  |  |  |
| 30 | IC44A | Manufacturing, mounting, and cementing the protection tube when the pipes go through the walls, the pipe having the diameter 50 mm | piece | 4,0000 |  |  |  |
| 31 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets) on the water supply or sewerage pipes, Ball valve with lever d 25 mm | piece | 4,0000 |  |  |  |
| 32 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets) on the water supply or sewerage pipes, Return valve with bow d 25 mm | piece | 1,0000 |  |  |  |
| 33 | SE56A | Filter for drinking water, with threaded sleeves to be installed on the pipe, with the diameter 25 mm | piece | 1,0000 |  |  |  |
| 34 | AcE51E | Connecting to the existing tube of steel pipes (with stud) with the diameter of the stud of 200 mm | unit | 1,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3.3. Circular manholes |  |  |  |  |  |
| 35 | AcE11B | Making the manholes out of pre-manufactured reinforced concrete elements, for water supply, circular (ring-type) with diameter 1,5 m, in the field with underground water | m3 | 4,0700 |  |  |  |
| 36 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦД-15 (0,38 m3) | piece | 3,0000 |  |  |  |
| 37 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦ-15-6 | piece | 4,0000 |  |  |  |
| 38 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦ-15-9 | piece | 2,0000 |  |  |  |
| 39 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦП1-15-1(0,27 m3) | piece | 3,0000 |  |  |  |
| 40 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦО-1(0,02 m3) | piece | 8,0000 |  |  |  |
| 41 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦ-7-3 (0,05 m3) | piece | 2,0000 |  |  |  |
| 42 | AcE07B | Installing the cast iron or concrete-cast iron caps without support piece, for the manholes to visit the water supply and sewerage installations, non-roadway type II A and B Ductile cast iron cap c-250 | piece | 3,0000 |  |  |  |
| 43 | Market price | Consumption of metals for reinforcing the built manholes | kg | 114,0600 |  |  |  |
| 44 | Market price | Ladder | kg | 58,3000 |  |  |  |
| 45 | IzD10A | Anticorrosive painting with the manual brush of the metallic garments and constructions with one layer of anti-corrosive primer based on lead minium and two layers of chlorinated rubber enamel, of the metallic garments and constructions, executed on profiles with thicknesses between 8 mm and 12 mm inclusively | t | 0,1720 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Transportation |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Overhead costs |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Estimate benefit |  |  |  | % |  |
|  |  | Total |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ Outdoor water supply networks for the fountain /

Bid value: USD

Date: 27.04,2020

| №. | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter1. Value of equipment i |  |  |  |  |  |
| 1 | Market price | Ultrasound water meter d15 mm | piece | 1,0000 |  |  |  |
|  |  | Total Value of the equipment Including wage |  |  |  |  |  |
|  |  | Chapter 2. Assembling works |  |  |  |  |  |
| 2 | AcB04D | Installing the water meter on the embedded concession tap, D 25 mm | piece | 1,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |  |
|  |  | Total Capitol 2 |  |  |  |  |  |
|  |  | Chapter 3. Construction works |  |  |  |  |  |
|  |  | Chapter 3.1. Earthworks |  |  |  |  |  |
| 3 | TsC03B1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in the storage of ground cat. II | 100 m3 | 1,5200 |  |  |  |
| 4 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 1,6800 |  |  |  |
| 5 | AcF03A | Fillings in the trenches of the pipes for water supply or sewerage, as substrate, protection layer, insulating layer or filtering layer for the drainage tubes, made with sand | m3 | 6,0000 |  |  |  |
| 6 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor cars of ground cat. II | 100 m3 | 0,0800 |  |  |  |
| 7 | TsI50A5 | Transportation of the ground with the 5t dumper at a distance of 5 km | t | 13,6000 |  |  |  |
| 8 | TsC51B | Unloading soil in storage, ground category II | 100 m3 | 0,0800 |  |  |  |
| 9 | TsD01B | Spreading with the shovel of light soil in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from middle ground | m3 | 10,0000 |  |  |  |
| 10 | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of soil separately, with the thickness of 10 cm of cohesive ground | m3 | 10,0000 |  |  |  |
| 11 | TsD02A1 | Spreading the loose land coming from the fields of category I and II, executed with caterpillar tractor-based bulldozer 65-80 HP, in layers with thickness of 15-20 cm | 100 m3 | 1,4200 |  |  |  |
| 12 | TsD05B | Compacting with the mechanical knocker of 150-200 kg filling in the successive layers of 20-30 cm thickness, excluding the watering of every layer separately, the earth fillings being executed from cohesive ground | 100 m3 | 1,4200 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3.2. Accessory pipes |  |  |  |  |  |
| 13 | AcA52A | Water supply polyethylene pipe mounted in ditch, with diameter of 20, 25, 32, 40, 50, 63 mm. PE80 PN10 d32 mm | m | 50,0000 |  |  |  |
| 14 | AcA52A | Water supply polyethylene pipe mounted in ditch, with diameter of 20, 25, 32, 40, 50, 63 mm. PE80 PN10 d25 mm | m | 11,0000 |  |  |  |
| 15 | Market price | Warning tape | m | 61,0000 |  |  |  |
| 16 | AcF11C | Washing the PVC, cast iron, asbestos -cement, polyethylene, etc. pipes of 20-75 mm, for drinking water after installation and coupling, before reception | m | 61,0000 |  |  |  |
| 17 | AcF12A | The pressure-test for the polyethylene pipes mounted in channels for the water and sewerage supply, with diameter up to 100 mm | m | 61,0000 |  |  |  |
| 18 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction T-bend PE D32 mm | piece | 1,0000 |  |  |  |
| 19 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets) on the water supply or sewerage pipes, Ball valve with lever d 25 mm | piece | 2,0000 |  |  |  |
| 20 | IC30N | Malleable cast iron fittings, having 3 screws, installed through screwing on the steel pipe, having diameter 1" Brass T-bend Fi D25 mm | piece | 1,0000 |  |  |  |
| 21 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduced reduction sleeve PE D32/25 mm | piece | 1,0000 |  |  |  |
| 22 | IC44A | Manufacturing, mounting, and cementing the protection tube when the pipes go through the walls, the pipe having the diameter 50 mm | piece | 8,0000 |  |  |  |
| 23 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Connecting piece Fi 25/15 | piece | 2,0000 |  |  |  |
| 24 | IC30L | Malleable cast iron fittings, having 3 screws, installed through screwing on the steel pipe, having diameter 1/2" Brass T-bend Fi D15 mm | piece | 2,0000 |  |  |  |
| 25 | SD07A | The passing tap with valve and coupling, with or without discharge, for the steel pipe, with the diameter of 3/8" - 1/2" Ball valve with level d 15 mm | piece | 4,0000 |  |  |  |
| 26 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction joint PE D20/15 mm | piece | 4,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3.3. Water-metering node |  |  |  |  |  |
| 27 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Filter D 15 | piece | 1,0000 |  |  |  |
| 28 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Submersible shutter D 15 | piece | 1,0000 |  |  |  |
| 29 | SD01A | Service tap, simple or double with adapter, regardless of the closing modality, having the diameter 3/8" - 1/2" Faucet D15 FI-FE | buc | 2,0000 |  |  |  |
| 30 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Reduction joint FI 25x15 | piece | 2,0000 |  |  |  |
| 31 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Nipple FE D15 | piece | 2,0000 |  |  |  |
| 32 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Steel threaded pipe FE D15 | piece | 2,0000 |  |  |  |
| 33 | AcA54A | Mechanic installation of fittings (screwing). Mechanic combination of the pipe and polyethylene fitting (sleeve, T-bend, bend), the pipes having the diameter of 20-40 mm. Sleeve D15 | piece | 2,0000 |  |  |  |
| 34 | AcE13A | Executing the manholes from the reinforced concrete pre-manufactured elements, for sewerage, circular (ring-type) with diameter of 1,0 m, in the field without underground water | m3 | 0,7500 |  |  |  |
| 35 | AcE13A1 | Precast plates for manholes КЦД-10 | piece | 1,0000 |  |  |  |
| 36 | AcE13A1 | Precast ring for manholes КЦ-10-6 | piece | 1,0000 |  |  |  |
| 37 | AcE13A1 | Precast ring for manholes КЦ-10-9 | piece | 1,0000 |  |  |  |
| 38 | AcE13A1 | Precast plates for manholes КЦП1-10-1 | piece | 1,0000 |  |  |  |
| 39 | AcE13A1 | Bearing ring КЦО-1 | piece | 1,0000 |  |  |  |
| 40 | AcE07B | Installing the cast iron or concrete-cast iron caps without support piece, for the manholes to visit the water supply and sewerage installations, non-roadway type II A and B Ductile cast iron cap c-250 | piece | 1,0000 |  |  |  |
| 41 | Market price | Consumption of metals for reinforcing the built manholes | kg | 38,0200 |  |  |  |
| 42 | Market price | Ladder | kg | 12,9000 |  |  |  |
| 43 | IzD10A | Anticorrosive painting with the manual brush of the metallic garments and constructions with one layer of anti-corrosive primer based on lead minium and two layers of chlorinated rubber enamel, of the metallic garments and constructions, executed on profiles with thicknesses between 8 mm and 12 mm inclusively | t | 0,0510 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Chapter 3.4. Circular manholes |  |  |  |  |  |
| 44 | AcE10B | Making the manholes out of pre-manufactured reinforced concrete elements, for water supply, circular (ring-type) with diameter 1.0 m, in the field with underground water | m3 | 3,7200 |  |  |  |
| 45 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦД-10 (0.18 m3) | piece | 5,0000 |  |  |  |
| 46 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦ-10-6 | piece | 8,0000 |  |  |  |
| 47 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦ-10-9 | piece | 2,0000 |  |  |  |
| 48 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦП1-10-1 | piece | 5,0000 |  |  |  |
| 49 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦО-1(0.02 m3) | piece | 5,0000 |  |  |  |
| 50 | AcE10A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for water supply, in the field without underground water. КЦ-7-3 (0.05 m3) | piece | 5,0000 |  |  |  |
| 51 | AcE07B | Installing the cast iron or concrete-cast iron caps without support piece, for the manholes to visit the water supply and sewerage installations, non-roadway type II A and B Ductile cast iron cap c-250 | piece | 5,0000 |  |  |  |
| 52 | Market price | Consumption of metals for reinforcing the built manholes | kg | 105,6000 |  |  |  |
| 53 | IzD10A | Anticorrosive painting with the manual brush of the metallic garments and constructions with one layer of anti-corrosive primer based on lead minium and two layers of chlorinated rubber enamel, of the metallic garments and constructions, executed on profiles with thicknesses between 8 mm and 12 mm inclusively | t | 0,1060 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Transportation |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Overhead costs |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Estimate benefit |  |  |  | % |  |
|  |  | Total |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ Outdoor sewerage networks – fountain /

Bid value: USD

Date: 27.04,2020

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Construction works |  |  |  |  |  |
|  |  | Chapter 1.1. Earthworks |  |  |  |  |  |
| 1 | TsC03B1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in the storage of ground cat. II | 100 m3 | 1,0000 |  |  |  |
| 2 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 3,7200 |  |  |  |
| 3 | AcF03A | Fillings in the trenches of the pipes for water supply or sewerage, as substrate, protection layer, insulating layer or filtering layer for the drainage tubes, made with sand | m3 | 19,0000 |  |  |  |
| 4 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor cars of ground cat. II | 100 m3 | 0,4000 |  |  |  |
| 5 | TsI50A5 | Transportation of the ground with the 5t dumper at a distance of 5 km | t | 68,0000 |  |  |  |
| 6 | TsC51B | Unloading soil in storage, ground category II | 100 m3 | 0,4000 |  |  |  |
| 7 | TsD01B | Spreading with the shovel of light soil in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from middle ground | m3 | 27,0000 |  |  |  |
| 8 | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of soil separately, with the thickness of 10 cm of cohesive ground | m3 | 27,0000 |  |  |  |
| 9 | TsD02A1 | Spreading the loose land coming from the fields of category I and II, executed with caterpillar tractor-based bulldozer 65-80 HP, in layers with thickness of 15-20 cm | 100 m3 | 0,7700 |  |  |  |
| 10 | TsD05B | Compacting with the mechanical knocker of 150-200 kg filling in the successive layers of 20-30 cm thickness, excluding the watering of every layer separately, the earth fillings being executed from cohesive ground | 100 m3 | 0,7700 |  |  |  |
|  |  | Chapter 1.2. Accessory pipes |  |  |  |  |  |
| 11 | AcA08A | Assembling in the ground, outside the building, the PVC pipes of 9m, sealed with rubber fittings, with the diameter 125-160 mm PVC pipe SN4 D 160 mm | m | 81,5000 |  |  |  |
| 12 | SB08C | Plastic sewer pipe for sewerage, combined with rubber case, surface-mounted or buried under the floor, having a diameter of 50 mm PVC pipe SN4 D 50 mm | m | 10,0000 |  |  |  |
| 13 | SB24E | Paving siphon for water collection, with frost-resistant and bower-resistant hydraulic lock, with diameter 100 mm | piece | 2,0000 |  |  |  |
| 14 | CA03C3 | Concrete poured in foundations, basements, support walls, under zero share walls, manufactured manually and poured with classical means, reinforced concrete class C 10/8 (Bc 10/B 150) | m3 | 0,1300 |  |  |  |
| 15 | SB09E | The connecting piece from plastic for sewerage, combined with rubber case, with diameter 110 mm Fillet SN4 110/50 | piece | 2,0000 |  |  |  |
| 16 | SB09C | The connecting piece from plastic for sewerage, combined with rubber case, with diameter 50 mm Cot 45* SN4 D50 mm | piece | 4,0000 |  |  |  |
|  |  | Chapter 1.3. Circular manholes |  |  |  |  |  |
| 17 | AcE13B | Making the manholes out of pre-manufactured reinforced concrete elements, for sewerage supply, circular (ring-type) with diameter 1,0 m, in the field with underground water | m3 | 1,7800 |  |  |  |
| 18 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦД-10 (0,18 m3) | piece | 3,0000 |  |  |  |
| 19 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦ-10-6 | piece | 2,0000 |  |  |  |
| 20 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦ-10-9 | piece | 1,0000 |  |  |  |
| 21 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦП1-10-1 | piece | 3,0000 |  |  |  |
| 22 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦО-1(0,02 m3) | piece | 7,0000 |  |  |  |
| 23 | AcE13A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.0 m, for sewerage supply, in the field without underground water. КЦ-7-3 (0,05 m3) | piece | 3,0000 |  |  |  |
| 24 | AcE07B | Installing the cast iron or concrete-cast iron caps without support piece, for the manholes to visit the water supply and sewerage installations, non-roadway type II A and B Ductile cast iron cap c-250 | piece | 3,0000 |  |  |  |
| 25 | Market price | Consumption of metals for reinforcing the built manholes | kg | 80,8800 |  |  |  |
| 26 | Market price | Ladder | kg | 48,6000 |  |  |  |
| 27 | IzD10A | Anticorrosive painting with the manual brush of the metallic garments and constructions with one layer of anti-corrosive primer based on lead minium and two layers of chlorinated rubber enamel, of the metallic garments and constructions, executed on profiles with thicknesses between 8 mm and 12 mm inclusively | t | 0,1300 |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Transportation |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Overhead costs |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Estimate benefit |  |  |  | % |  |
|  |  | Total |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ Electrical lighting – fountain /

Bid value: USD

Date: 27.04,2020

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Construction works |  |  |  |  |
| 1 | TsA16D1 | Manual digging of soil, in limited spaces, in tranches up to 4 m depth, for electrical high-voltage cables, in grounds with natural humidity without supporters width < 1 m, depth < 1,5 m , very hard grounds | m3 | 100,0000 |  |  |
| 2 | TsD18B | Compacted filling in ditches, for embedded high-voltage cables, made with soil from middle ground | m3 | 100,0000 |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Transportation |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Chapter 2. Assembling works |  |  |  |  |
| 3 | 08-03-572-3 | Command switchboard of closet-type or as distribution point type (case), mounted on the wall, with specific height and width, mm, up 600х600 BZUM-TF-02-100-17 | piece | 1,0000 |  |  |
| 4 | 08-03-521-1 | Switcher with lever on the plate with central or lateral hand lever, or bar handling, mounted on metallic support, mono-pole, ВР32 | piece | 1,0000 |  |  |
| 5 | 08-03-526-1 | Mono-, bi-, three-pole automate, mounted on wall or column constructions, power up to 100 A | piece | 2,0000 |  |  |
| 6 | 08-03-600-2 | Meters mounted on prepared support, with three phases | piece | 1,0000 |  |  |
| 7 | 08-02-142-1 | Making the bed for one single cable in the ditch | 100 m | 2,3400 |  |  |
| 8 | 08-02-148-1 | Cable up to 35 kV in laid pipe, blocks and boxes, weight 1 m up to: 1 kg | 100 m | 0,1000 | 8 | 08-02-148-1 |
| 9 | Market price | Cable АВБбЩВ-4*10 mm2 | m | 10,0000 | 9 |  |
| 10 | Market price | Cable АВБбЩВ-5*16 mm2 | m | 210,0000 | 10 |  |
| 11 | 08-03-591-8 | Plug socket of unburied type, in open installation | 100 pieces | 0,0800 | 11 | 08-03-591-8 |
| 12 | Market price | Exterior plug socket with earth contact in open installation IP68 16A (Resistant to weather conditions) | piece | 8,0000 |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Transportation |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Chapter 3. Value of equipment |  |  |  |  |
| 13 | Market price | Case BZUM-TF-02-100-17 | piece | 1,0000 |  |  |
| 14 | Market price | Switcher BP-32-31-3P-63 | piece | 1,0000 |  |  |
| 15 | Market price | Automaton BA-47-29-3P 63A C | piece | 1,0000 |  |  |
| 16 | Market price | Automaton диф АД 30 мА 16А | piece | 1,0000 |  |  |
| 17 | Market price | Electricity meter 380B 5-30A | piece | 1,0000 |  |  |
|  |  | Total |  | USD |  |  |
|  |  | Total Value equipment Including wage |  | USD |  |  |
|  |  | Total estimate: Including wage |  | USD |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ electrical lighting /

Bid value: USD

Date: 27.04,2020

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Construction works |  |  |  |  |
| 1 | TsA16D1 | Manual digging of soil, in limited spaces, in tranches up to 4 m depth, for electrical high-voltage cables, in grounds with natural humidity without supporters width < 1 m, depth < 1,5 m , very hard grounds | m3 | 267,3000 |  |  |
| 2 | TsD18B | Compacted filling in ditches, for embedded high-voltage cables, made with soil from middle ground | m3 | 267,3000 |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Transportation |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Chapter 2. Assembling works |  |  |  |  |
| 3 | 08-03-572-3 | Command switchboard of closet-type or as distribution point type (case), mounted on the wall, ЩО70-1-01 | piece | 1,0000 |  |  |
| 4 | 08-03-572-3 | Command switchboard of closet-type or as distribution point type (case), mounted on the wall, with specific height and width, mm, up to 600х600 BZUM-TF-01-63-09 | piece | 1,0000 |  |  |
| 5 | 08-03-521-1 | Switcher with lever on the plate with central or lateral hand lever, or bar handling, mounted on metallic support, mono-pole, ВР32 | piece | 1,0000 |  |  |
| 6 | 08-03-526-1 | Mono-, bi-, three-pole automate, mounted on wall or column constructions, power up to 100 A | piece | 1,0000 |  |  |
| 7 | 08-03-600-2 | Meters mounted on prepared support, with three phases | piece | 1,0000 |  |  |
| 8 | 08-03-572-3 | Command switchboard of closet-type or as distribution point type (case), mounted on the wall, with specific height and width, mm, up to 600х600 (ЯУО9601-3574-54У3) | piece | 1,0000 |  |  |
| 9 | 08-03-575-1 | Device or appliance dismantled before transportation | piece | 3,0000 |  |  |
| 10 | 08-02-142-1 | Making the bed for one single cable in the ditch | 100 m | 12,5800 |  |  |
| 11 | 08-02-141-1 | Cable up to 35 kV in ditches, executed without covering, weight 1 m, up to: 1 kg | 100 m | 12,5800 |  |  |
| 12 | 08-02-148-1 | Cable up to 35 kV in laid tubes, blocks, and boxes, weight 1 m up to: 1 kg | 100 m | 0,1200 |  |  |
| 13 | Market price | Cable АВБбЩВ-5*25 mm2 | m | 1 309,0000 |  |  |
| 14 | Market price | Cable КВБбЩВ-5*25 mm2 | m | 70,0000 |  |  |
| 15 | 08-02-165-1 | Terminal collar from synthetic resins (epoxy) for cable with 3 conductors, voltage up to 1 kV, section of one conductor, up to: 35 mm2 | piece | 2,0000 |  |  |
| 16 | 08-02-370-2 | Board with up to three groups, installed in the basement corbel ЩМ59 | piece | 41,0000 |  |  |
| 17 | 08-02-146-2 | Cable up to 35 kV, fixing with applied clips, weight 1 m up to: 1 kg | 100 m | 2,0000 |  |  |
| 18 |  | Cable VVGng-LS-3*2.5 mm2 | m | 200,0000 |  |  |
| 19 | 08-02-471-4 | Vertical grounding, from round steel, diameter 16 mm | 10 pieces | 0,8000 |  |  |
| 20 | 08-02-472-9 | Grounding conductor, open, on construction supports, from round steel, diameter 12 mm | 100 m | 0,0300 |  |  |
| 21 | 08-02-472-7 | Grounding conductor, open, on construction supports, from steel strips, section 160 mm2 | 100 m | 0,0600 |  |  |
| 22 | 08-02-472-7 | Grounding conductor, open, on construction supports, from steel strips, section 200 mm2 | 100 m | 0,0300 |  |  |
| 23 | 08-02-407-5 | Steel pipe on installed on construction, on walls, fixing with clamps, diameter up to 100 mm | 100 m | 0,0200 |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Transportation |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Chapter 3. Value of equipment |  |  |  |  |
| 24 | Market price | Switchboard ЩО70-1-01 | piece | 1,0000 |  |  |
| 25 | Market price | Case BZUM-TF-01-63-09 | piece | 1,0000 |  |  |
| 26 | Market price | Switcher BP-32-31-3P-63 | piece | 1,0000 |  |  |
| 27 | Market price | Automaton BA-47-29-3P 20A C | piece | 1,0000 |  |  |
| 28 | Market price | Electricity meter 380B 5-30A | piece | 1,0000 |  |  |
| 29 | Market price | Case ЯУО9601-3574-54У3 | piece | 1,0000 |  |  |
|  |  | Total |  | USD |  |  |
|  |  | Total Value of equipment Including wage |  | USD |  |  |
|  |  | Total estimate: Including wage |  | USD |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.

Development of the central park ''Grigore Vieru''

mun. Cahul, 31 August str. I. Voda cel Cumplit str.

Bill of work quantities

/ equipment /

Bid value: USD

Date: 27.04,2020

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Chapter 1. Construction works |  |  |  |  |
|  |  | Chapter 1.1. Repairing bench type 1 |  |  |  |  |
| 1 | RpCR29A | Removing the oil-based paint, with remover substance | m2 | 0,560 |  |  |
| 2 | RpCR22A | Painting with alkyd enamel the metal joinery, from inside and outside, in existing constructions, in one layer of metal joinery (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 0,560 |  |  |
| 3 | TsH90A | Repairing the benches of different forms (repairing the benches of type 1 –existing legs of cast iron, and the seat of new wood (oak, acacia) | m3 | 0,160 |  |  |
| 4 | RpCR13B | Painting with alkyd enamel the wooden joinery, from inside and outside, in existing constructions, in two layers (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 12,600 |  |  |
|  |  | Chapter 1.2. Repairing bench type 2 |  |  |  |  |
| 5 | RpCR29A | Removing the oil-based paint, with remover substance | m2 | 1,040 |  |  |
| 6 | RpCR22A | Painting with alkyd enamel the metal joinery, from inside and outside, in existing constructions, in one layer of metal joinery (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 1,040 |  |  |
| 7 | TsH90A | Repairing the benches of different forms (repairing the benches of type 1 –existing legs of cast iron, and the seat of new treated wood (oak, acacia) | m3 | 0,276 |  |  |
| 8 | RpCR13B | Painting with alkyd enamel the wooden joinery, from inside and outside, in existing constructions, in two layers (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 22,100 |  |  |
|  |  | Chapter 1.3 Repairing bench type 3 |  |  |  |  |
| 9 | RpCR29A | Removing the oil-based paint, with remover substance | m2 | 7,200 |  |  |
| 10 | RpCR22A | Painting with alkyd enamel the metal joinery, from inside and outside, in existing constructions, in one layer of metal joinery (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 7,200 |  |  |
| 11 | TsH90A | Repairing the benches of different forms (repairing the benches of type 1 –existing legs of cast iron, and the seat of new treated wood (oak, acacia) | m3 | 2,200 |  |  |
| 12 | RpCR13B | Painting with alkyd enamel the wooden joinery, from inside and outside, in existing constructions, in two layers (sustainable painting resistant to ultraviolet radiations and weather conditions) | m2 | 477,000 |  |  |
|  |  | Chapter 1.4. Diverse |  |  |  |  |
| 13 | TsH91A | Dismantling the wooden benches with cast-iron legs | piece | 91,000 |  |  |
| 14 | TsH91B | Dismantling the waste bins | piece | 40,000 |  |  |
|  |  | Chapter 1.5. Installing new and old benches |  |  |  |  |
| 15 | 47-02-040-3 | Manual digging of holes, size 0.6x0.6 on light soil | 100 pieces | 5,640 |  |  |
| 16 | TsC54C | Foundation layer of gravel | m3 | 2,256 |  |  |
| 17 | TsH91A | Installing benches on 2 legs (without bench price) | piece | 141,000 |  |  |
| 18 | CL18A | Diverse metallic confections from rolled profiles, plate, checker plate, steel, concrete, pipes for supporting or covering, totally or partially embedded in concrete | kg | 112,800 |  |  |
|  |  | Chapter 1.6. Installing waste bins |  |  |  |  |
| 19 | 47-02-040-3 | Manual digging of holes, size 0.6x0.6 on light soil | 100 pieces | 0,720 |  |  |
| 20 | TsC54C | Foundation layer of gravel | m3 | 0,288 |  |  |
| 21 | CL18A | Diverse metallic confections from rolled profiles, plate, checker plate, steel, concrete, pipes for supporting or covering, totally or partially embedded in concrete | kg | 14,400 |  |  |
| 22 | TsH91B | Installing the waste bins (without the price of the waste bin) | piece | 72,000 |  |  |
|  |  | Chapter 1.7. Installing lighting pillars |  |  |  |  |
| 23 | 47-02-040-3 | Manual digging of holes, size 0.6x0.6 on light soil | 100 pieces | 0,410 |  |  |
| 24 | TsC54C | Foundation layer of gravel | m3 | 0,328 |  |  |
| 25 | CL18A | Diverse metallic confections from rolled profiles, plate, checker plate, steel, concrete, pipes for supporting or covering, totally or partially embedded in concrete | kg | 73,600 |  |  |
| 26 | CA03B3 | Concrete poured in foundations, basements, support walls, under zero share walls, manufactured manually and poured with classical means, simple concrete class C 10/8 (Bc 10/B 150) | m3 | 6,200 |  |  |
| 27 | 33-04-003-1 | Installing the lighting pillars (without the price of the pillar) | piece | 41,000 |  |  |
|  |  | Chapter 1.8. Installing the drinking fountain including the constructions for the cesspool |  |  |  |  |
|  |  | Chapter 1.8.1. Installing the drinking fountain |  |  |  |  |
| 28 | 47-02-040-3 | Manual digging of holes, size 0.6x0.6 on light soil | 100 pieces | 0,020 |  |  |
| 29 | TsH91B | Installing the drinking fountain (without the price of the drinking fountain) | piece | 2,000 |  |  |
|  |  | Chapter 1.8.2. Cesspool - 2 pieces |  |  |  |  |
| 30 | TsC03B1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in the storage of ground cat. II | 100 m3 | 2,210 |  |  |
| 31 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 6,400 |  |  |
| 32 | AcF03A | Fillings in the trenches of the pipes for water supply or sewerage, as substrate, protection layer, insulating layer or filtering layer for the drainage tubes, made with sand | m3 | 32,000 |  |  |
| 33 | TsC03F1 | Mechanic digging with excavator of 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds with natural humidity, and unloading in motor cars of ground cat. II | 100 m3 | 0,400 |  |  |
| 34 | TsI50A5 | Transportation of the ground with the 5t dumper at a distance of 5 km | t | 68,000 |  |  |
| 35 | TsC51B | Unloading soil in storage, ground category II | 100 m3 | 0,400 |  |  |
| 36 | TsD01B | Spreading with the shovel of light soil in uniform layers, 10-30 cm thick, with a throw of up to 3 m of piles , including smashing of earth bolls from middle ground | m3 | 56,400 |  |  |
| 37 | TsD04B | Compacting with manual knocker of the embankments in horizontal of inclined digs to 1/4, including watering every layer of soil separately, with the thickness of 10 cm of cohesive ground | m3 | 56,400 |  |  |
| 38 | TsD02A1 | Spreading the loose land coming from the fields of category I and II, executed with caterpillar tractor-based bulldozer 65-80 HP, in layers with thickness of 15-20 cm | 100 m3 | 1,710 |  |  |
| 39 | TsD05B | Compacting with the mechanical knocker of 150-200 kg filling in the successive layers of 20-30 cm thickness, excluding the watering of every layer separately, the earth fillings being executed from cohesive ground | 100 m3 | 1,710 |  |  |
| 40 | AcA08A | Assembling in the ground, outside the building, the PVC pipes of 9m, sealed with rubber fittings, with the diameter 125-160 mm PVC SN4 D 110 mm | m | 150,000 |  |  |
| 41 | AcE11B | Making the manholes out of pre-manufactured reinforced concrete elements, for water supply, circular (ring-type) with diameter 1,5 m, in the field with underground water | m3 | 2,900 |  |  |
| 42 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦД-15 (0,38 m3) | piece | 2,000 |  |  |
| 43 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦ-15-9 | piece | 4,000 |  |  |
| 44 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦП1-15-1(0,27 m3) | piece | 2,000 |  |  |
| 44 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦП1-15-1(0,27 m3) | piece | 2,000 |  |  |
| 45 | AcE11A1 | Reinforced concrete pre-manufactured elements of the circular (ring-type) manholes with diameter of 1.5 m, for water supply, in the field without underground water. КЦО-1(0,02 m3) | piece | 2,000 |  |  |
| 46 | AcE07B | Installing the cast iron or concrete-cast iron caps without support piece, for the manholes to visit the water supply and sewerage installations, non-roadway type II A and B Ductile cast iron cap c-250 | piece | 2,000 |  |  |
| 47 | Market price | Consumption of metals for reinforcing the built manholes | kg | 76,040 |  |  |
| 48 | Market price | Ladder | kg | 38,870 |  |  |
| 49 | IzD10A | Anticorrosive painting with the manual brush of the metallic garments and constructions with one layer of anti-corrosive primer based on lead minium and two layers of chlorinated rubber enamel, of the metallic garments and constructions, executed on profiles with thicknesses between 8 mm and 12 mm inclusively | t | 0,115 |  |  |
|  |  | Total | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Transportation |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Overhead costs |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Estimate benefit |  |  |  |  |
|  |  | Total |  |  |  |  |
|  |  | Chapter 2. Equipment |  |  |  |  |
| 50 | Market price | Bench type 4 similar to model (Park bench 700-FR-urban furniture) (see attached photo-sketch) | piece | 61,000 |  |  |
| 51 | Market price | Decorative lighting pillar for the park, made of cast iron similar to model R1-C2, weight: 132.00kg (see attached photo-sketch) | piece | 31,000 |  |  |
| 52 | Market price | Ornamental public lighting pillars similar to model B1-1B: B1-1b weight: 102.0kg (see attached photo-sketch) | piece | 10,000 |  |  |
| 53 | Market price | LED bulbs 18-25Wt | piece | 72,000 |  |  |
| 54 | Market price | Waste bin with tipping basket Artmet no. 2300kg (see attached photo-sketch) | piece | 72,000 |  |  |
| 55 | Market price | Arch-type pergola for roses with 2 flower pots, vidaXL, wooden, 180 x40 x 205 cm 00kg (see attached photo-sketch) | piece | 10,000 |  |  |
| 56 | Market price | Cast iron drinking fountain FORBAN 1A00kg (see attached photo-sketch) | piece | 2,000 |  |  |
| 57 | Market price | Public WC (double mobile boxes) ecologic, (for 2 persons), similar to model from photo-sketch attached; each section: 1100x1100x2350h(mm); Metalic frame, sandwich pannels, mineral wool (MW), 50mm, anticorozive wrinkled metal, door pannel with a ventilation glill, pandlock handle; floor –aluminium non-slip boards, δ=1,5mm; WC-equiped with technical water sistem, washbasin, puclic toilet bowl, (without chair), acumulation tank and sewerage; | piece | 4,000 |  |  |
|  |  | Total |  | USD |  |  |
|  |  | Total Value of equipment Including wage |  | USD |  |  |
|  |  | Total estimate: Including wage |  | USD |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.