Building a dry fountain in “Gr. Vieru” Park

in Cahul municipality

Bill of Quantities

/ Reinforced concrete constructions /

In current prices                                                                                            Valued of the bid: USD

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate value, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurements ———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1. Earthworks |  |  |  |  |  |
| 1 | TsC58B | Mechanic excavation of soil in the storage with the “dragline” of “reversed bin” excavator with the volume of the bin of 0.15 m3: ground category 2 | 100 m3 | 1,1072 |  |  |  |
| 2 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 15,4400 |  |  |  |
| 3 | TsC59A | Mechanic excavation of soil with and excavator with the volume of the bin of 0.15 m3 and unloading in the dumper: ground category 1 (collecting and loading the ground) | 100 m3 | 0,5207 |  |  |  |
| 4 | TsI50A10 | Transportation of the ground with a dumper of 5 t at a distance up to 10 km | t | 68,5100 |  |  |  |
| 4.1 | TsC58B | Mechanic excavation of soil in the storage with the “dragline” of “reversed bin” excavator with the volume of the bin of 0.15 m3: ground category 2 (clay) | 100 m3 | 0,4507 |  |  |  |
| 5 | TsI50C10 | Transportation of the ground with a dumper of 5 t at a distance of 30 km (clay) | t | 58.6400 |  |  |  |
| 6 | TsD08A1 | Mechanical compacting of fillings with self-propelled static tire-based compactor of 10.1-16 t, in successive layers of 15-25 cm thickness after compacting, excluding the watering of every layer separately, the earth fillings being executed from non-cohesive ground (clay) | 100 m3 | 0,4507 |  |  |  |
| 7 | TsD02A1 | Spreading the loose land coming from grounds of category I and II, executed with caterpillar tractor-based bulldozer 65-80 HP, in layers with thickness of 15-20 cm | 100 m3 | 0,5200 |  |  |  |
| 8 | TsD05B | Compacting with the mechanical knocker of 150-200 kg filling in the successive layers of 20-30 cm thickness, excluding the watering of every layer separately, the earth filling being executed from cohesive ground | 100 m3 | 0,5200 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total Earthworks Including wage |  |  |  |  |  |
|  |  | 2. Construction of the technical room and fountain – foundation, walls, flooring |  |  |  |  |  |
| 9 | TsC54B | Foundation layer of crushed stone, fraction (5-10)mm | m3 | 4,0000 |  |  |  |
| 10 | CB03J applied | Shuttering of plywood panels of 15 mm for pouring the concrete (plywood n=1.01 m2) | m2 | 119,0000 |  |  |  |
| 11 | CA03I1 | Concrete poured in foundation, basement, support walls and walls under zero level, prepared with the concrete plant and pump-based pouring on the vertical up to 10 m inclusively, and on the horizontal – on lengths over 15 m, reinforced concrete class C 7.5 | m3 | 5,0000 |  |  |  |
| 12 | CC01A | Concrete-steel reinforcement shaped in OB 37 construction shops, with bars up to 8 mm diameter inclusively | kg | 1 316,3000 |  |  |  |
| 13 | CC01B | Concrete-steel reinforcement shaped in OB 37 construction shops, with bars over 8 mm diameter inclusively | kg | 184,3000 |  |  |  |
| 14 | CL17B | Diverse metallic confections (Pi1) | kg | 11,0000 |  |  |  |
| 15 | CB11A | Supporters with extended inventory props used for installation of the prefabricated plates, of the floor plates, when casting the slabs which are partially or totally monolith with beams or monolith beams with prefabricated slabs of type PE 3100 R | piece | 25,0000 |  |  |  |
| 16 | CA03I1 | Concrete poured in foundation, basement, support walls and walls under zero level, prepared with the concrete plant and pump-based pouring on the vertical up to 10 m inclusively and on the horizontal for lengths over 15 m, reinforced concrete cl. C25 W8 F100 adding waterproof additive Penetron Admix | m3 | 20,8000 |  |  |  |
| 17 | RpCM05A adjusted | Smoothing concrete surfaces | m2 | 32,0000 |  |  |  |
| 18 | IzF04C | Waterproof layer made in hot conditions for terraces, roofs, or foundations and slabs, in grounds without groundwater, including molding and valleys from the current waterproofing protection on horizontal or inclined surfaces up to 40% flat or curved, with covered bitumen cloth or with bitumen fiberglass fabric, glued on the entire surface with bitumen putty | m2 | 40,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total Construction of the technical room and fountain – foundation, walls, flooring Including wage |  |  |  |  |  |
|  |  | 3. Works for making the trenches |  |  |  |  |  |
| 19 | TsC03B31 | Mechanic digging with excavator 0.40-0.70 m3, with internal combustion engine and hydraulic command, in grounds heavily saturated with water, with excavator moving on decking, and unloading in the storage of ground cat. I with water management conditions | 100 m3 | 0,2000 |  |  |  |
| 20 | TsA20B | Manual digging of soil, in breakers, with canal embankment dug with the excavator or scraper for completing the cutting slopes, in middle ground | m3 | 2,1400 |  |  |  |
| 21 | TsI50A10 | Transportation of the ground with a dumper of 5 t at a distance up to 10 km | t | 32,2000 |  |  |  |
| 22 | TsC54A | Foundation layer of sand | m3 | 10,0000 |  |  |  |
| 23 | RCsB21C k=2 | Mechanical drilling of holes with diameter of 10cm, in concrete elements, with thickness of 30 cm | piece | 30,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total Works for making the trenches Including wage |  |  |  |  |  |
|  |  | 4. Waterproofing works |  |  |  |  |  |
| 24 | CN53A applied | Coating the internal surfaces, walls/flooring of type "Primer" G n=0,2 kg/m2 | m2 | 32,0000 |  |  |  |
| 25 | RpCE03A applied | Waterproofing the vertical surfaces or those with a slope over 20 degrees, flat or curved, in concrete roofs or subsoils (attics, sheds, lunettes, dead walls, etc.), made in hot conditions with cement mortar with two components with rubber plastering in 2 layers (cement-based adhesive with two components of type Elastocem A+B) | m2 | 32,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total Waterproofing works Including wage |  |  |  |  |  |
|  |  | 5. Tiles’ installation works |  |  |  |  |  |
| 26 | CF17C | Miscellaneous - fleece layer of fiberglass applied to the surface of pre-manufactured elements from autoclaved aerated concrete, bonded with glue, including the primer layer | m2 | 25,0000 |  |  |  |
| 27 | CI21B | Wall cladding with structured ceramics-granite plates: size 600 x 600 x 20 mm, type R11, Norr | m2 | 25,0000 |  |  |  |
| 28 | market price | Support for tiles, of type SE9, h-2-5-345 mm | piece | 264,0000 |  |  |  |
| 28.1 | market price | Support element, closing the T profile edge, stainless steel, size 30x50mm, (20m) | kg | 67,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Total Tiles’ installation works Including wage |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Social and health insurance fund |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Transportation |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Overhead costs |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Estimate benefit |  |  |  | % |  |
|  |  | Total estimate: without VAT Including wage |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

Building a dry fountain in “Gr. Vieru” Park

in Cahul municipality

Bill of Quantities

/ Electricity works. Technical-sanitary works /

In current prices                                                                                             Bid value: USD

| № | Symbol of the norm and resources code | Works and expenses | U.M. | Quantity according to design data | Estimate vale, USD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per unit of measurement ———— wage | Total ———— wage |

| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | - Electrical works 1.1. Installation works |  |  |  |  |  |
| 1 | 10-06-037-07 | Cases, boxes and doses for pipe-through installation: wall-based case, size, mm, up to: 800х1800 | piece | 2,0000 |  |  |  |
| 2 | 08-03-594-10 | Installing the projector for the fountain LED RGB DMX control | 100 pieces | 0,1800 |  |  |  |
| 3 | Market price | Electrical connection box 400x400x155 mm | piece | 18,0000 |  |  |  |
| 4 | RpEQ04A | Installing the box with clamps | piece | 18,0000 |  |  |  |
| 5 | 08-02-401-1 | Cable fixed with applied staples, strips with installation of branch cases, with 2-4 wires, wire section up to 16 mm2 | 100 m | 10,6000 |  |  |  |
| 6 | Market price | Underwater cable 4x1,5 | m | 500,0000 |  |  |  |
| 7 | Market price | Cable PVS 3x1,5 | m | 100,0000 |  |  |  |
| 8 | Market price | Cable UTP 6x0,5 | m | 100,0000 |  |  |  |
| 9 | Market price | Underwater cable 2x2,5 | m | 180,0000 |  |  |  |
| 10 | Market price | Cable PVS 4x2,5 | m | 80,0000 |  |  |  |
| 11 | Market price | Cable FTP 6x0,5 | m | 60,0000 |  |  |  |
| 12 | Market price | Cable VVG 5x6 | m | 40,0000 |  |  |  |
| 13 | 08-02-390-1 | Plastic ducts | 100 m | 0,3000 |  |  |  |
| 14 | 08-03-591-10 | Single socket with grounding | 100 pieces | 0,0200 |  |  |  |
| 15 |  | Single socket with grounding | piece | 2,0000 |  |  |  |
| 16 | 08-03-591-10 | Double socket with grounding | 100 pieces | 0,0200 |  |  |  |
| 17 |  | Double socket with grounding | piece | 2,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation of materials | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Estimate benefit | % |  |  |  |  |
|  |  | Total Installation works Including wage |  |  |  |  |  |
|  |  | 1.2. Equipment |  |  |  |  |  |
| 18 | Market price | Pump control board (with frequency variation) and lighting | piece | 1,0000 |  |  |  |
| 19 | Market price | Water level control board | piece | 1,0000 |  |  |  |
| 20 | Market price | Audio pre-booster | piece | 1,0000 |  |  |  |
| 21 | Market price | Audio booster Revamp 4240t | piece | 1,0000 |  |  |  |
| 22 | Market price | Rack case | piece | 1,0000 |  |  |  |
| 23 | Market price | Outdoor audio speakers Apart COLW81 | piece | 8,0000 |  |  |  |
| 24 | Market price | Projector for the fountain LED RGB DMX control | piece | 18,0000 |  |  |  |
| 25 | Market price | Pillar for the speaker, of modern type, galvanized metal, sustainable, sect. 80x80mm, δ=2mm, H=4,5m, anchoring the galvanized details on the concrete foundation, brand B12.5 | piece | 8,0000 |  |  |  |
| 26 | Market price | Fountain nozzle | piece | 18,0000 |  |  |  |
| 27 | Market price | Grate for fountain | piece | 18,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Procurement-storage costs | % |  |  |  |  |
|  |  | Total Equipment Including wage |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
| 27.1 | Market price | Individual testing and launching the system into operation | set | 1 |  |  |  |
|  |  | Total Electrical works Including wage |  |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation of materials | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Estimate benefit | % |  |  |  |  |
|  |  | 2. Technical-sanitary works |  |  |  |  |  |
|  |  | 2.1. Construction works |  |  |  |  |  |
| 28 | AcD18A | Installing the PVC pipe o type M-3, for drainage, combined through gluing, outside the buildings, buried at a depth up to 2m, having the diameter of 63 mm - rigid | m | 220,0000 |  |  |  |
| 29 | AcD18A | Installing the PVC pipe o type M-3, for drainage, combined through gluing, outside the buildings, buried at a depth up to 2m, having the diameter of 63 mm – semi-rigid | m | 700,0000 |  |  |  |
| 30 | AcD18A | Installing the PVC pipe o type M-3, for drainage, combined through gluing, outside the buildings, buried at a depth up to 2m, having the diameter of 50 mm - rigid | m | 60,0000 |  |  |  |
| 31 | AcD18A | Installing the PVC pipe o type M-3, for drainage, combined through gluing, outside the buildings, buried at a depth up to 2m, having the diameter of 50 mm – semi-rigid | m | 200,0000 |  |  |  |
| 32 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets), on water supply and sewerage pipes, - PVC valve d 50 | piece | 4,0000 |  |  |  |
| 33 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets), on water supply and sewerage pipes, - PVC valve d 63 | piece | 10,0000 |  |  |  |
| 34 | AcE08B | Installing the lids for technical rooms HAGODECK BVA 88-80x80 (92x92) | piece | 1,0000 |  |  |  |
| 35 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets), on water supply and sewerage pipes, - PVC Valve d 32 | piece | 1,0000 |  |  |  |
| 36 | SD07E | Passing tap with valve and plug, with or without discharge, for the steel pipe, with diameter 1 1/2" | piece | 1,0000 |  |  |  |
| 37 | SA37F | Bracelet for fixing the pipes for water and gas supply, from steel or PVC, flush mounted through ducts having the diameter of 2" | piece | 35,0000 |  |  |  |
| 38 | SA37G | Bracelet for fixing the pipes for water and gas supply, from steel or PVC, flush mounted through ducts having the diameter of 2 1/2" | piece | 53,0000 |  |  |  |
| 39 | market price | Plastic clamp 4,8x300 | piece | 200,0000 |  |  |  |
| 40 | market price | Support stand for electrical pump (polypropylene) | piece | 6,0000 |  |  |  |
| 41 | market price | Spacing dowel 16x100 | piece | 130,0000 |  |  |  |
| 42 | market price | Flexible tube d32 | m | 10,0000 |  |  |  |
| 43 | market price | Taping pin d8x1000 | m | 12,0000 |  |  |  |
| 44 | market price | Taping pin d10x1000 | m | 10,0000 |  |  |  |
| 45 | market price | Screw bolt 6x60 | piece | 135,0000 |  |  |  |
| 46 | market price | Stainless clamp with screw d52 | piece | 80,0000 |  |  |  |
| 47 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets) on the water supply or sewerage pipes, - not-return valve d 50 | piece | 1,0000 |  |  |  |
| 48 | AcB01A | Assembling the fitting with manual or mechanic triggering (valves, taps, faucets) on the water supply or sewerage pipes, - PVC Valve d 63 1R (CH) | piece | 5,0000 |  |  |  |
| 49 | market price | Taping pin d8x1000 stainless | m | 8,0000 |  |  |  |
| 50 | market price | Taping pin d10x1000 stainless | m | 5,0000 |  |  |  |
| 51 | market price | Screw bolt 6x35 stainless | piece | 150,0000 |  |  |  |
| 52 | AcA52A | Polyethylene pipe for water supply, mounted in ditch, with diameter 20, 25, 32, 40, 50, 63 mm. (Pipe PE100, PN10 SDR 17 D=40 mm) | m | 12,0000 |  |  |  |
| 53 | AcA54B | Assembling the fittings mechanically, screwing up. Mechanic combination between the pipe and the fitting (valves, T-bends, joints) from polyethylene, the pipes having the diameter of 50-110 mm. (Reduction joint D 50x1" PT) | piece | 20,0000 |  |  |  |
| 54 | AcA54B | Assembling the fittings mechanically, screwing up. Mechanic combination between the pipe and the fitting (valves, T-bends, joints) from polyethylene, the pipes having the diameter of 50-110 mm. (T-bend 40x90 PE) | piece | 2,0000 |  |  |  |
| 55 | AcA54B | Assembling the fittings mechanically, screwing up. Mechanic combination between the pipe and the fitting (valves, T-bends, joints) from polyethylene, the pipes having the diameter of 50-110 mm. (Passage joint 50x32 PE) | piece | 2,0000 |  |  |  |
| 56 | AcA08A | Assembling in the ground, outside the building, the PVC pipes of 9m, sealed with rubber fittings, with the diameter of 125-160 mm (PVC pipe with sleeve joint and saddle SN 4, SDR41, D=110 mm) | m | 6,0000 |  |  |  |
| 57 | AcA54B | Assembling the fittings mechanically, screwing up. Mechanic combination between the pipe and the fitting (valves, T-bends, joints) from polyethylene, the pipes having the diameter of 50-110 mm. (Inspection lid D100 mm PE) | piece | 5,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation of materials | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Estimate benefit | % |  |  |  |  |
|  |  | Total Construction works Including wage |  |  |  |  |  |
|  |  | 2.2. Installation works |  |  |  |  |  |
| 58 | AcC07A | Installing on the existing position the centrifuge pumps, with horizontal axis, with aspiration diameter up to 80 mm (material in the equipment) | piece | 5,0000 |  |  |  |
| 59 | IA23A applied | Filtering system | piece | 1,0000 |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Social and health insurance fund | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Transportation of materials | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Overhead costs | % |  |  |  |  |
|  |  | Total |  |  |  |  |  |
|  |  | Estimate benefit | % |  |  |  |  |
|  |  | Total Installation works Including wage |  |  |  |  |  |
|  |  | 2.3. Equipment |  |  |  |  |  |
| 60 | market price | Pump ND2-28T 2HP 400V | piece | 5,0000 |  |  |  |
| 61 | market price | Pump-based filtering installation LX 25 m3/h | piece | 1,0000 |  |  |  |
| 62 | market price | Quarts sand for filter | kg | 225,0000 |  |  |  |
| 63 | market price | Dosimeter for disinfecting substances | piece | 1,0000 |  |  |  |
| 64 | market price | Air dryer FDC 32 | piece | 2,0000 |  |  |  |
| 65 | market price | Pump of type Karcher SP7 | piece | 2,0000 |  |  |  |
| 66 | market price | Ladder 4,68 m | piece | 1,0000 |  |  |  |
| 67 | market price | Adhesive for pipes, of type PVC U cement | kg | 3,0000 |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Total |  | USD |  |  |  |
|  |  | Procurement-storage costs |  |  |  | % |  |
|  |  | Total Equipment Including wage |  |  |  | USD |  |
| 67.1 | market price | Individual testing and launching the system into operation | set | 1 |  |  |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Total Technical-sanitary works Including wage |  |  |  | % |  |
|  |  | Total |  |  |  | USD |  |
|  |  | Total estimate: without VAT Including wage |  | USD |  |  |  |

Note: The bidder’s costs will include all the expenditures, including: materials and/or equipment, employees, social payments, indirect payments

Bidder

(position, signature, name, surname)

S.P.